3GPP TSG-RAN WG4 Meeting NR #2	            R4-1706525

Qingdao, China, 27-29 May 2017

Agenda item:	3.2.5

Source: 	Intel Corporation

Title: 	UE ACS for Sub-6 GHz

Document for:	Discussion

# Introduction

UE ACS for sub-6 GHz was discussed in the last RAN4 meetings [1-6]. A WF on sub-6 GHz ACLR and ACS was agreed [7]. This document provides our views on the remaining aspects regarding sub-6 GHz UE ACS.

# Discussion on sub-6 GHz ACS

The WF agreement regarding sub-6 GHz UE ACS is captured below [7].

- ACS = 33dB for wanted channel bandwidth up to 20MHz.

  - ACS for other NR wanted channel bandwidth are FFS.

  - The adjacent channel bandwidth the same as the wanted signal.

  - ACS for different adjacent channel bandwidth are FFS.

  - ACS may be updated as SNR for Refsens is currently TBD.

- Case 1: Refsens+14dB for wanted signal and Refsens+45.5 dB for interferer.

- Case 2: -56.5dBm for wanted signal and -25dBm for interferer.

  - Case 1 and 2 is based on IM=2.5dB and SNR=-1dB.

The following table captures the current agreement and our proposals for bandwidths wider than 20 MHz. Those highlighted in yellow are new or different from the existing LTE tables in TS36.101 [8].

Table 1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz |
| ACS | dB | 33.0 | 33.0 | 33.0 | 33.0 | 33.0 | 33.0 |

The agreement is that the adjacent channel bandwidth is the same as the wanted signal. The following two tables give the test parameters for Case 1 and Case 2, which are based on the assumptions of IM=2.5dB and SNR=-1dB. Those highlighted in yellow are new or different from the existing LTE tables in TS36.101 [8].

Table 2: Test parameters for adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |
| PInterferer | dBm | REFSENS +45.5dB | REFSENS +45.5dB | REFSENS +45.5dB | REFSENS +45.5dB | REFSENS +45.5dB | REFSENS +45.5dB |
| BWInterferer | MHz | 5 | 10 | 15 | 20 | 50 | 100 |
| FInterferer (offset) For SCS of 15kHz | MHz | 5+0.0025 / -5-0.0025 | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 | 50+0.0025 / -50-0.0025 | 100+0.0125 / -100-0.0125 |
| FInterferer (offset) For SCS of 30kHz | MHz | 5+0.025 / -5-0.025 | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 | 50+0.025 / -50-0.025 | 100+0.005 / -100-0.005 |
| FInterferer (offset) For SCS of 60kHz | MHz | 5+0.010 / -5-0.010 | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 | 50+0.010 / -50-0.010 | 100+0.050 / -100-0.050 |

Table 3: Test parameters for adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz | 50 Hz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -56.5 | -56.5 | -56.5 |  | -56.5 | -56.5 | -56.5 |
| PInterferer | dBm | -25 |  |  |  |  |  |  |
| BWInterferer | MHz | 5 | 10 | 15 | 20 |  | 50 | 100 |
| FInterferer (offset) For SCS of 15kHz | MHz | 5+0.0025 / -5-0.0025 | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 | 20+0.0025 / -20-0.0025 |  | 50+0.0025 / -50-0.0025 | 100+0.0125 / -100-0.0125 |
| FInterferer (offset) For SCS of 30kHz | MHz | 5+0.025 / -5-0.025 | 10+0.005 / -10-0.005 | 15+0.015 / -15-0.015 | 20+0.025 / -20-0.025 |  | 50+0.025 / -50-0.025 | 100+0.005 / -100-0.005 |
| FInterferer (offset) For SCS of 60kHz | MHz | 5+0.010 / -5-0.010 | 10+0.050 / -10-0.050 | 15+0.030 / -15-0.030 | 20+0.010 / -20-0.010 |  | 50+0.010 / -50-0.010 | 100+0.050 / -100-0.050 |

Proposal:  RAN4 should agree on the values in the above tables, which are highlighted in yellow. ACS may be updated as SNR for REFSENS is currently TBD.

# Conclusion

Proposal:  RAN4 should agree on the values in the above tables, which are highlighted in yellow. ACS may be updated as SNR for REFSENS is currently TBD.

# References

- R4-1703174: "UE sub-6 GHz ACS considerations and proposals", Intel Corporation

- R4-1703560, “Conducted UE ACS below for frequency range 1”, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1705743, “Discussion on co-existence study for NR uplink”, NTT DOCOMO, INC

- R4-1705671, “Sub-6 ACS”, Qualcomm Incorporated

- R4-1705657, “UE sub-6 GHz ACS considerations and proposals”, Intel Corporation

- R4-1704929, “Co-existence performance for NR_NR co-existence on sub-6GHz”, CATT

- R4-1706316, WF on sub-6GHz ACLR and ACS”, Nokia, Alcatel-Lucent Shanghai Bell

- 3GPP TS 36.101 V14.2.1 (2017-01)