3GPP TSG-RAN WG4 Meeting NR #2	            R4-1706526

Qingdao, China, 27-29 May 2017

Agenda item:	3.2.5

Source: 	Intel Corporation

Title: 	UE ACS for mmWave

Document for:	Discussion

# Introduction

UE ACS for mmWave was discussed in the last RAN4 meeting [1]. A WF on UE ACLR and ACS in mmWave was agreed [2]. This document provides our views on the remaining aspects regarding mmWave UE ACS.

# Discussion on mmWave UE ACS

The WF agreement regarding mmWave UE ACS is captured below [2].

- UE ACS

  - 23dBc for 30GHz frequency range

  - 22dBc for 45GHz frequency range

- For mmW specifications the bandwidths of wanted and unwanted signals are assumed to be the same

The following table captures the current agreement and our proposals for different bandwidths.

Table 1: Adjacent channel selectivity

|  |  |  | Channel bandwidth |  |  |
| --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | Frequency range | 50 MHz | 100 MHz | 200 MHz |
| ACS | dB | 30GHz | 23.0 | 23.0 | 23.0 |
| ACS | dB | 45GHz | 22.0 | 22.0 | 22.0 |

The agreement is that the adjacent channel bandwidth is the same as the wanted signal. The following two tables give the test parameters for Case 1 and Case 2. For Case 1, the desire signal power is kept the same as LTE, which is REFSENS+14dB, and the interference power is adjusted according to the ACS level of 23dB and 22dB, assuming IM of 2.5dB and SNR of -1dB. For Case 2, the interference power is kept the same, which is [-25] dBm, and the desire signal power is adjusted according to the ACS level of 22dB, assuming IM of 2.5dB and SNR of -1dB. It is noted that if the assumptions of IM, SNR and maximum input power are changed later on, the values in these two table will be adjusted accordingly.

Table 2: Test parameters for adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz |  | 200 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| PInterferer For ACS of 23dB | dBm | REFSENS + 35.5dB |  | REFSENS + 35.5dB |  | REFSENS + 35.5dB |
| PInterferer For ACS of 22dB | dBm | REFSENS + 34.5dB |  | REFSENS + 34.5dB |  | REFSENS + 34.5dB |
| BWInterferer | MHz | 50 |  | 100 |  | 200 |
| FInterferer (offset) For SCS of 60kHz | MHz | 50+0.010 / -50-0.010 |  | 100+0.050 / -100-0.050 |  | 200+0.010 / -200-0.010 |
| FInterferer (offset) For SCS of 120kHz | MHz | 50+0.100 / -50-0.100 |  | 100+0.020 / -100-0.020 |  | 200+0.100 / -200-0.100 |

Table 3: Test parameters for adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |
| --- | --- | --- | --- | --- |
|  |  | 50 Hz | 100 MHz | 200 MHz |
| Power in Transmission Bandwidth Configuration, for ACS of 23dB | dBm | [-46.5] | [-46.5] | [-46.5] |
| Power in Transmission Bandwidth Configuration, for ACS of 22dB | dBm | [-45.5] | [-45.5] | [-45.5] |
| PInterferer | dBm | [-25] |  |  |
| BWInterferer | MHz | 50 | 100 | 200 |
| FInterferer (offset) For SCS of 60kHz | MHz | 50+0.010 / -50-0.010 | 100+0.050 / -100-0.050 | 200+0.010 / -200-0.010 |
| FInterferer (offset) For SCS of 120kHz | MHz | 50+0.100 / -50-0.100 | 100+0.020 / -100-0.020 | 200+0.100 / -200-0.100 |

Proposal:  RAN4 should agree on the values in the above tables.

# Conclusion

Proposal:  RAN4 should agree on the values in the above tables.

# References

- R4-1705743, “Discussion on co-existence study for NR uplink”, NTT DOCOMO, INC

- R4-1706063, “WF on mmW ACLR and ACS”, Qualcomm Incorporated, Huawei