3GPP TSG-RAN WG4 NR AH Meeting#2	R4-1706570

Qingdao, China, 27 - 29 June, 2017

Agenda Item:	3.4.4.3

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Proposal on mmWave NR BS Receiver Requirements

Document for:	Approval

# 1.	Introduction

mmWave NR BS receiver OTA requirements were further discussed in RAN4#83, and a way forward was agreed [1].

Proposals for the receiver dynamic range, ACS and in-band blocking requirements are provided in [2-4], respectively. This contribution provides discussion on the necessities and proposals to specify the other mmWave NR BS receiver OTA requirements in the RAN4 specifications per the agreed way forward.

# 2.	Discussion

The necessities and proposals to specify the mmWave NR BS receiver OTA requirements other than the receiver dynamic range, ACS and in-band blocking requirements are provided below.

2.1	Reference sensitivity level

This is a necessary requirement to measure the receiver ability to receive a wanted signal with low signal and noise levels, mainly to verify the BS noise figure. The SNR for the BS reference sensitivity can be obtained at 95% relative throughput from link level simulations of the reference measurement channel following the physical layer designed in RAN1.

2.2	In-channel selectivity

This is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density, mainly to verify the BS RF impairments of RX image leakage, EVM, IMD3 and LO phase noise, as well as the BS baseband filter implementation. Considering mixed numerology is supported in NR UL, this is likely a necessary requirement for NR BS. The wanted and interfering signal power levels can be obtained from the BS reference sensitivity, the allowed sensitivity degradation (3dB for E-UTRA BS) and the required selectivity (25dB for E-UTRA BS), following the physical layer (higher order modulation etc.) designed in RAN1.

2.3	Narrow-band blocking

This is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent narrow-band channel signal at a given frequency offset from the center frequency of the assigned channel, mainly to verify the BS RF impairments of filter selectivity and LO phase noise. The interfering signal for E-UTRA BS narrow-band blocking requirement is single resource block transmission from an adjacent E-UTRA system. For operating band without adjacent narrow-band systems in the same geographical area (as is likely in mmWave), this requirement is not necessary.

2.4	Out-of-band blocking

This is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those ”close-in” to the wanted channel, mainly to verify the BS RF filter implementation. The interfering signal for E-UTRA BS out-of-band blocking requirement is CW signal. This is a necessary requirement. The wanted signal power level can be obtained from the BS reference sensitivity and the allowed sensitivity degradation (6dB for E-UTRA BS), while the interfering signal power level (-15dBm for E-UTRA BS) can be decided per the expected signal power from the coexisting systems (in term of OTA signal level), together with the expected isolation between the interferer and the BS.

On the other hand, testing of this requirement has been a time-consuming exercise as the measurements are required for a very wide frequency range (1 to 12750 MHz for E-UTRA). To reduce the amount of time used for unnecessary measurement, using a larger measurement step size than current 1MHz step for E-UTRA can be considered on frequencies that are “further away” from the wanted channel (e.g. 10MHz step on frequencies further than two times BS maximum radio bandwidth from the wanted channel).

2.5	Co-location with other base stations

This requirement aims to protect the receiver when other BS operating in a different frequency band are co-located with a BS, mainly to verify the BS RF filter implementation. For BS co-located with other BS, this can be specified as an optional requirement. The wanted signal power level can be obtained from the BS reference sensitivity and the allowed sensitivity degradation (6dB for E-UTRA BS), while the interfering signal power level (+16dBm for E-UTRA Wide Area BS) can be decided per the expected signal power from the co-located BS (in term of OTA signal level), together with the expected isolation between two BS.

2.6	Receiver spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the BS receiver antenna connector. This is mainly a measure of the receiver ability to suppress generation or amplification of emissions that appear at the BS receiver antenna connector. This is a necessary requirement to protect other systems in the same geographical area. The transmitter spurious emission limits can be applied also for receiver.

2.7	Receiver intermodulation

This is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal, mainly to verify the BS RF impairment of IMD3. The interfering signals for E-UTRA BS intermodulation performance requirement are CW and 5MHz E-UTRA signal. This is a necessary requirement. The wanted signal power level can be obtained from the BS reference sensitivity and the allowed sensitivity degradation (6dB for E-UTRA Wide Area BS), while the interfering signal power level (-52dBm for E-UTRA Wide Area BS) can be decided per the expected signal power from the adjacent systems (in term of OTA signal level), together with the expected isolation between the interferer and the BS. The offset between the interfering signal centre frequency and the nominal band edge of the wanted carrier can be specified based on the worst-case scenario, i.e. the intermodulation products fall (almost) on the edge resource blocks of an operating channel bandwidth.

2.8	Receiver narrowband intermodulation

This is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of two narrowband interfering signals which have a specific frequency relationship to the wanted signal, mainly to verify the BS RF impairment of IMD3. The interfering signals for E-UTRA BS intermodulation performance requirement are CW and single resource block transmission from an adjacent E-UTRA system. For operating band without adjacent narrow-band systems in the same geographical area (as is likely in mmWave), this requirement is not necessary.

# 3.	Conclusion and proposals

This contribution has provided discussion on the necessities and proposals to specify the other mmWave NR BS receiver OTA requirements in the RAN4 specifications.

# References

[1]	3GPP TR 38.803 v14.0.0, “Study on New Radio Access Technology; RF and co-existence aspects”.

[2]	R4-1705430, “Proposal on mmWave NR BS Receiver Dynamic Range”, Nokia, Alcatel-Lucent Shanghai Bell.

[3]	R4-1706565, “Proposals on mmWave NR BS ACLR and ACS”, Nokia, Alcatel-Lucent Shanghai Bell.

[4]	R4-1706568, “Proposal on mmWave NR BS Receiver In-band Blocking”, Nokia, Alcatel-Lucent Shanghai Bell.