3GPP TSG RAN WG4 Meeting NR#2	R4-1706618
27 - 29 June 2017, Qingdao, China

Source:		Fraunhofer-HHI, Germany

Title:			Measurement Control Channel and Associated Message Space

Agenda Item:		3.6.1

Document for:	Discussion

# Introduction

For frequencies above 6 GHz, the baseline for New Radio (NR) user equipment (UE) radio frequency (RF) testing requirements is over-the-air (OTA) [1]. Radio and antenna performance measurements are therefore inseparable since, in many cases, the antenna ports of individual antenna elements are either inaccessible for measurement or a measurement of their isolated RF performance provides marginal value. In contrast, the characterization of the joint performance of multiple antenna elements and their associated transceiver chains is of much more interest. This is especially true for systems comprised of a large number of antenna elements such as those that form the basis of beam-forming and massive MIMO equipment. OTA test and measurement (T&M) thus becomes an important aspect for the performance and compliance evaluation of such wireless devices, especially those used for 4G, 5G and other current and future wireless communication systems. This is true for user equipment [1] and base station (BS) equipment [2] alike, both of which are considered in this document.

In order to perform tests and measurements OTA, it is necessary for the device under test (DUT), the measurement equipment, the measurement system and the measurement environment to communicate with each other and to perform reproducible measurement steps and procedures. Ideally, this should be done in a standardized way and one that is vendor independent.

For such procedures, the following should be considered as prerequisites:

a measurement control channel (MCC) necessary to command the measurement process; and

an associated message space (AMS) designed such that current and future T&M procedures can be realized in an appropriate and efficient manner.

This document presents the rationale for an MCC and an AMS and proposes options for their realization.

# Rationale

Current discussions, such as those reflected in 3GPP TR37.842 [2], propose OTA test methods for Active Antenna Systems (AAS). However, to the best of the authors’ knowledge, an OTA interface between a measurement system and a device under test (such as a BS or a UE) does not exist at present. For example, TR37.842 [2] assumes that BS parameters are set manually and while TS36.141 [3] discloses details of current BS conformance testing, it does not define a communication protocol for T&M purposes per se. Therefore, and by way of an example, in order to provide the means to control a BS or a measurement system externally (or OTA), a standardized communication protocol for T&M purposes is required.

Current T&M procedures mainly follow two options, as illustrated in Figures 1 and 2.

Figure 1: Extension of 3GPP TR 37.842 [2], in which the DUT is controlled from measurement equipment via an OTA interface.

Figure 2: Extension of 3GPP TR 37.976 [4] in which the DUT is controlled from measurement equipment via a wireless extension of the OTA interface

Fig. 1 shows an extension of 3GPP TR 37.842 [2] in which a DUT is controlled from measurement equipment via an OTA interface and Fig. 2 shows a schematic diagram of an extension of 3GPP TR 37.976 [4] in which the DUT is controlled from measurement equipment via a wireless extension of the OTA interface of Fig. 1.

Using the available communication between the measurement equipment and the DUT, several parameters and predefined messages can be exchanged over the existing interfaces. This may involve a list of different key performance indicators (KPls), such as: mode of test (test cases); transmit (Tx) power; waveform; channel allocation; data pattern; and reference signal. There is thus a need to enhance tests, test methods and test results for wireless communications.

Furthermore, in order to control a base station (BS) from some form of user equipment (UE) emulator, further extensions of TR37.976 would be needed to define a new control channel, for example such as the measurement control channel (MCC) option, described herein.

# Solution options

The particular solution option defines a generalization and an extension of existing interfaces in the context of TR37.842 and TR37.976. Herein the solution option proposal adds further capabilities for the control of OTA T&M and in-field optimization (IFO) of wireless devices that use multiple or massive numbers of antennas. In addition to the current parameters of the existing interfaces, such as mode of test (test cases), Tx power and waveform, a flexible message space is required to support both current and future T&M procedures. Further extensions of TR37.976 are therefore needed to define a new control interface that will support the control of a BS from a UE emulator as well as the control of a UE from a BS emulator.

The proposed solution consists of two main components:

a measurement control channel (MCC); and

an associated message space (AMS).

## Measurement Control Channel (MCC)

The MCC defines the logical channel between the DUT and the measurement system(s) and can be realized in various ways including the following:

RAN4 standardized, proprietary or via the air interface channel (for example NR air interface (AI)); and

auxiliary channel (non-integrated) outside the radio access network (RAN) under test (for example, WiFi).

The T&M system collects pre-configured KPIs and stores them for further benchmarks and/or system performance evaluations.

The MCC design has the following principles:

Separated from the primary AI and the RAN under test.

For further optimization of the MCC, data can be compressed and stored on the DUT and transported to the measurement system at an earlier or later stage (for example pre-configuration for production purposes).

The MCC may use any available communication connection between the measurement system and the DUT.

Security levels provided by the MCC should be considered.

Multi-RAT interworking mechanisms such as those defined in 3GPP-non-3GPP interworking should be supported.

## Associated message space (AMS)

It might be necessary to define an Associated Message Space (AMS) that extends the current definition of messages in TS37.320 [5] (Radio Measurement collection for minimization of drive test (MDT), also known as “MDT mode”).

The following list provides examples of the purpose for which additional AMS messages might require definition: triggering; synchronizing; configuring; requesting; reporting; and coordination of measurements and measurement equipment.

An extension of messages in the AMS might include the following: beam forming index; reference signals; signal strengths; waveforms; timestamps; and locations.

Additional consideration of the extended AMS should be given for interfacing requirements with several control interfaces at the DUT involved in the T&M procedures. This includes for example interfacing with the internal interfaces of the UEs for example under the framework of ETSI RRS [6][7][8].

# Conclusion

This document identifies the need for RAN4 to consider the definition of an extended MCC and AMS and proposes that this item is the subject of further study.

# References

- 3GPP Technical Report TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”

- 3GPP Technical Report TR 37.842, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”

- 3GPP Technical Specification TS 36.141, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”

- 3GPP Technical Report TR 37.976, “Measurement of radiated performance for Multiple Input Multiple Output (MIMO) and multi-antenna reception for High Speed Packet Access (HSPA) and LTE terminals”

- 3GPP Technical Specification TS 37.320, “Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2”

- DIRECTIVE 2014/53/EU OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 16 April 2014 on the harmonisation of the laws of the Member States relating to the making available on the market of radio equipment and repealing Directive 1999/5/EC

- ETSI EN 303 095: Reconfigurable Radio Systems (RRS); Radio Reconfiguration Related Architecture for Mobile Devices, 2015

- ETSI TR 102 967: Reconfigurable Radio Systems (RRS); Use Cases for dynamic equipment reconfiguration, ETSI, 2015