3GPP TSG-RAN WG4 Meeting NR#2                                                                  R4-1706706

Qingdao, China, 27 - 29 May, 2017

Title:                      Discussion on SUL and UL sharing band definition

Source: 	CMCC

Agenda Item:	3.1.1

Document for:	Discussion and decision

1    Introduction

A few way forwards on SUL and uplink sharing were agreed in RAN1#89 meeting [1][2][3]. A lision was also sent to RAN2 and RAN4 to consider the following RAN1 agreements:

Specify mechanisms for supporting supplementary Uplink frequency - Note: SUL herein refers to the case when there is only UL resource for a carrier from NR perspective - Use SUL as complimentary access link (including from random access point of view) to NR TDD and to NR FDD, where the UE may select PRACH resources either in the NR TDD/FDD uplink frequency or the SUL frequency. - Note: The SUL frequency can be a frequency shared with LTE UL (at least for the case when NR spectrum is below 6 Ghz). - Minimize impact to NR physical layer design to enable this co-existence - Note: whether or not UE has to support simultaneous transmission on uplink frequencies is a separate discussion

This contribution discuss how to specify the band defintion for SUL and UL sharing in RAN4.

2    Discussion

Accoring to current E-UTRA specification, there are two potential ways to define the band for SUL and UL sharing.

Alternative 1: define the SUL and uplink sharing bands in the general NR operating bands table.

Table 2-1: NR operating bands

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 2 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| 3 | 880 MHz | – | 915 MHz |  | N/A |  | XDD |
|  | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

Alternative 2: define the SUL bands in the general NR operating bands table and define SUL and uplink sharing bands in a separte table.

Table 2-2: NR operating bands

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 2 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| 3 | 880 MHz | – | 915 MHz |  | N/A |  | XDD |
| 4 | 1710 MHz | – | 1785 MHz |  | N/A |  | XDD |

Table 2-3: SUL and uplink sharing operating bands

| NR SUL and uplink sharing Band | NR Band (Table x) |
| --- | --- |
| SUL_2-3 | 2, 3 |
| SUL_2-4 | 2, 4 |

We will further analysis pros and cons of the two band definition methods from the following aspects.

## 2.1 Sytem information broadcasting

E-UTRA indicates its operating band(s) via the IE freqBandIndicator and multiBandInfoList defined in SIB1, SIB2, SIB3 and SIB5. The operating bands for carrier aggregation and dual connectivity are not broadcasted.

Alternative 1

- NR can indicate its operating band(s) including SUL and uplink sharing via the similar approach as E-UTRA. e.g. NR send the IE multiBandInfolist that contains the NR operating band (NR band 1, 2) together with the SUL/uplink sharing bands (NR band 3).

Alternative 2

- NR can only indicate its normal operating band(s) (NR band 1, 2) in the IE multiBandInfolist. The bands for SUL and uplink sharing can’t be broadcasted in SIB.

## 2.2 Procedures in RRC_IDLE state

In RRC_IDLE state, the main procedure related to this issue is random access. In E-UTRA, UE initial the random access procedure on fixed uplink carrier (TDD: same as downlink carrier, FDD: downlink carrier + duplex gap). The main purpose of SUL and uplink sharing is to enlarge the uplink coverage especially for the higher frequency band. UE can initial the random access procedure in either the SUL carrier or the normal uplink carrier following BS’s indication.

Alternative 1

- BS can directly indicate the frequency range for intial random access or indicate the criterion of uplink resource selection for initial random access. UE can initial the random access procedure either in SUL carrier (lower frequency range) or in the normal uplink carrier (higher frequency range) following BS’s indication.

Alternative 2

- BS will only broadcast its normal operating bands (NR band 1,2). Therefore, UE can only initial the random access in the normal uplink carrier (higher frequency range). In some area, UE may not successfully send the random access due to link budget limition of higher frequency range.

Observation 1: alternative 1 is better than alternative 2 in RRC_IDLE state procedures, e.g. random access.

## 2.3 Procedures in RRC_CONNECTED state

In RRC_CONNECTED state, there are several procedures reltated to this issue, e.g. UE capability report, scheduling and handover.

(1) UE capability report

E-UTRAN initiates the procedure to a UE in RRC_CONNECTED when it needs (additional) UE radio access capability information. UE shall submit the UECapabilityInformation message which contains UE capability of support operating bands of single band, carrier aggregation and dual connectivity to E-UTRAN once UE receives the UECapabilityEnquiry. Futhermore, the capabilities of support of uplink 64QAM, MIMO layers and power class related to operating bands are also send via the same IE.

For NR, the operating bands and reltaed capabilities (uplink 64QAM, MIMO layers, power class, etc.) may be transferred in the following ways:

Alternative 1

- since SUL and normal band share the same band number, we can use E-UTRA UE capabilities reporting aproach for single band as a baseline.

- bandParametersSUL-r15: conveys SUL and uplink sharing frequency range related UE capabilities

- bandParametersUL-r15: conveys normal uplink frequency range related UE capabilities

BandParameters-r15 ::= SEQUENCE {

bandNR-r15						FreqBandIndicator-r15,

bandParametersSUL-r15			BandParametersSUL-r15					OPTIONAL,

bandParametersUL-r15			BandParametersUL-r15					OPTIONAL,

bandParametersDL-r15			BandParametersDL-r15					OPTIONAL,

}

BandParametersSUL-r15 ::= SEQUENCE {

ul-64QAM-r15				ENUMERATED {supported}		OPTIONAL

ue-PowerClass-r15			ENUMERATED {class1, class2, class4}		OPTIONAL

MIMO-CapabilityUL-r15 ::= 	ENUMERATED {twoLayers, fourLayers}

}

BandParametersUL-r15 ::= SEQUENCE {

ul-64QAM-r15				ENUMERATED {supported}		OPTIONAL

ue-PowerClass-r15			ENUMERATED {class1, class2, class4}		OPTIONAL

MIMO-CapabilityUL-r15 ::= 	ENUMERATED {twoLayers, fourLayers}

}

BandParametersDL-r15 ::= SEQUENCE {

dl-256QAM-r15				ENUMERATED {supported}		OPTIONAL

MIMO-CapabilityDL-r15 ::= 	ENUMERATED {twoLayers, fourLayers}

}

- Usually, the normal uplink and downlink frequency range has its own NR band number. For example, in table 2-1, band 3 is a band that contain a SUL frequency range of 880-915MHz and a normal uplink and downlink frequency range of 3300-3800MHz. In addition, 3300-3800MHz is also a NR band (band 2). In that case, for uplink sharing band, UE can report the capabilities for normal freuqency range and SUL frequency range seperately. As the following exmaple, for normal freuqnecy range, both downlink and uplink capabilities are reported; for SUL freuqnecy range, only uplink capabilities are reported.

- bandParametersUL-r15: only conveys SUL frequency range related UE capabilities of band 3

- bandParametersDL-r15: N/A of band 3

- bandParametersUL-r15: only conveys normal uplink frequency range related UE capabilities of band 2

- bandParametersDL-r15: only conveys normal downlink frequency range related UE capabilities of band 2

- BS can get the full knowleges of UE capabilities of band 3 (SUL frequency range and normal frequency range) when it receives related UE capabillities of band 2 and band 3

BandParameters-r15 ::= SEQUENCE {

bandNR-r15						FreqBandIndicator-r15,

bandParametersUL-r15			BandParametersUL-r15					OPTIONAL,

bandParametersDL-r15			BandParametersDL-r15					OPTIONAL,

}

BandParametersUL-r15 ::= SEQUENCE {

ul-64QAM-r15				ENUMERATED {supported}		OPTIONAL

ue-PowerClass-r15			ENUMERATED {class1, class2, class4}		OPTIONAL

MIMO-CapabilityUL-r15 ::= 	ENUMERATED {twoLayers, fourLayers}

}

BandParametersDL-r15 ::= SEQUENCE {

dl-256QAM-r15				ENUMERATED {supported}		OPTIONAL

MIMO-CapabilityDL-r15 ::= 	ENUMERATED {twoLayers, fourLayers}

}

Alternative 2

- since SUL has its own band number, we can use similar aproach as carrier aggregation and dual connectivity. UE can indicate the support of uplink sharing in bandCombinationParameters. Then the UE capabilities for SUL band and normal band can be reported seperately.

  - SUL band number and related UE capabilities

  - normal band number and related UE capabilities

BandCombinationParameters-r15 ::=	SEQUENCE {

sul-Support-r15				ENUMERATED {supported}					OPTIONAL,

bandParameterList-r15		SEQUENCE (SIZE (1..maxSimultaneousBands-r15)) OF BandParameters-r15,

...

}

BandParameters-r15 ::= SEQUENCE {

bandNR-r15					FreqBandIndicator,

bandParametersUL-r15		BandParametersUL-r15					OPTIONAL,

bandParametersDL-r15		BandParametersDL-r15					OPTIONAL

}

BandParametersUL-r15 ::= SEQUENCE {

ul-64QAM-r15				ENUMERATED {supported}		OPTIONAL

ue-PowerClass-r15			ENUMERATED {class1, class2, class4}		OPTIONAL

MIMO-CapabilityUL-r15 ::= 	ENUMERATED {twoLayers, fourLayers}

}

BandParametersDL-r15 ::= SEQUENCE {

dl-256QAM-r15				ENUMERATED {supported}		OPTIONAL

MIMO-CapabilityDL-r15 ::= 	ENUMERATED {twoLayers, fourLayers}

}

(2) Scheduling

In uplink sharing scenario, a NR BS can schedule uplink resource either on a SUL carrier or a normal uplink carrier via dedicate scheduling signalling. The procedure is considered to be the same for both alternative 1 and 2.

(3) Handover

During NR handover procedure, if target cell is deployed with two different uplink frequency ranges in uplink sharing scenario, the source cell may send the indication of desired uplink carrier information (SUL or normal uplink carrier) of the target cell to UE for uplink transmission. The procedure is considered to be the same for both alternative 1 and 2.

Observation 2: alternative 1 and 2 have similar performance in RRC_CONNECTED state procedures, e.g. UE capability report, uplink scheduling and handover.

## 2.4 Others

RAN4 is considered to define the maximum number of operating band for NR. It’s proposed to also consider the need of NR band number for SUL and uplink sharing.

Alternative 1 directly defines the SUL and uplink sharing combinations into the NR operating band table (seen in table 2-4). Alternative 2 needs to define all the SUL bands into the NR operating band table (seen in table 2-5) and then capture the SUL related band combinations into another table (seen in table 2-6).

Alternative 1

Table 2-4: NR operating bands

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 2 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 3 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| 4 | 880 MHz | – | 915 MHz |  | N/A |  | XDD |
|  | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |
| 5 | 1710 MHz | – | 1785 MHz |  | N/A |  | XDD |
|  | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz |  |

Alternative 2

Table 2-5: NR operating bands

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 2 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 3 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| 4 | 880 MHz | – | 915 MHz |  | N/A |  | XDD |
| 5 | 1710 MHz | – | 1785 MHz |  | N/A |  | XDD |

Table 2-6: SUL and uplink sharing operating bands

| NR SUL and uplink sharing Band | NR Band (Table x) |
| --- | --- |
| SUL_3-4 | 3, 4 |
| SUL_3-5 | 3, 5 |

If the number of SUL and uplink sharing band combination is small, alternative 1 can has the similar overhead of NR operating band number as alternative 2. According to current agreement, only 4 SUL and uplink sharing band combinations were agreed. Alternative 1 can has similar overhead of NR operating band number as alternative 2. However, no matter which alternative is applied,  we need to reserve more spaces for SUL bands or uplink sharing bands for future use when we define the maximum NR operating band number.

Observation 3: since only 4 SUL and uplink sharing band combinations were agreed, alternative 1 could have similar overhead of NR operating band number as alternative 2. More spaces are proposed to be reserved for SUL and uplink sharing for future use.

3   Conclusions

According to above discussions, we propose to consider the following observations.

Observation 1: alternative 1 is better than alternative 2 in RRC_IDLE state procedures, e.g. random access.

Observation 2: alternative 1 and 2 have similar performance in RRC_CONNECTED state procedures, e.g. UE capability report, uplink scheduling and handover.

Observation 3: since only 4 SUL and uplink sharing band combinations were agreed, alternative 1 could have similar overhead of NR operating band number as alternative 2. More spaces are proposed to be reserved for SUL and uplink sharing for future use.

Based on the above observations, we propose to adopt alternative 1 to define SUL and uplink sharing band to enlarge uplink coverage and achieve good performance.

4   References

R1-1709732, Way forward on the support of Supplementary Uplink in NR, CMCC

R1-1709825, WF on LTE-NR Coexistence with UL sharing, Huawei, HiSilicon, AsusTek, CATR, MediaTek, vivo, Spreadtrum, OPPO, Intel, III, Ericsson, Orange

R1-1709586 , WF on UL Sharing for NSA NR UE, Apple Inc, Huawei, HiSilicon, Deusche Telecom, Samsung, Orange, OPPO, vivo, INL

R1-1709843, LS on the support of supplementary uplink in NR, CMCC