3GPP TSG-RAN WG4 Meeting NR #2                                                                 R4-1706756

# Qingdao, China, 27 - 29 June, 2017

Title:                        Measurement coordination between LTE and NR

Source: 	CMCC

Agenda Item:	3.5.8

Document for:	Discussion

1    Introduction

Last meeting, RAN2 discussed the measurement configuration coordination between MN and SN. A liaison was sent to RAN4 to ask which UE capabilities related are required for coordination that is required across LTE and NR. The LS is provided as follows [1].

For LTE-NR Dual Connectivity covering all options (i.e. Option 3/4/7 series, now also known as Multi-RAT Dual Connectivity, MR-DC), RAN2 has been investigating how measurements are configured for MCG and SCG. In terms of UE measurement capabilities, RAN2 understands that at least, the total number of measured carriers needs to be coordinated between the Master Node (MN) and the Secondary Node (SN) so that it does not go beyond the UE capability. Apart from this, it is not clear if there are any other UE capabilities related to measurements for which coordination is required across LTE and NR. Since the measurement capability falls into the realm of RAN4, RAN4 feedback is deemed as necessary, in particular to the following questions: When a UE is configured with MR-DC: Q1: Will RAN4 specify UE requirements on; - the total number of measurable objects across LTE and NR? - the total number of configurable measurement events across LTE and NR? Q2: if the answer to Q1-a) is Yes, and if both the MN and SN separately configure a measurement object on the same carrier frequency (e.g. the MN eNB RRC configures an inter-RAT NR measurement on a given carrier and the SN gNB RRC configures an intra-RAT NR measurements on the same - serving or non-serving - carrier frequency), should it be counted as 1 or 2 measured objects? Q3: Would the answer to Q2 be dependent on differences in configuration of the measurement object? Q4: If MN and SN are to separately configure a measurement object on the same carrier frequency as in Q2, which parameters need to be configured with the same value (i.e., would need to be coordinated between the MN and SN) and which can be allowed to differ, in order to regard the two measurement object configurations from both MN and SN as one measurement object? - For example, the parameters included in E-UTRA measurement object are listed in Annex. - Any other parameters to be specified for NR, if any. Q5: In addition to Q1, will RAN4 specify additional UE requirements for which the UE requirement across inter-RATs is not the union of the one for each RAT (like the number of measurable carriers)?

In this contribution, we discuss the measurement related information required for coordination between MN and SN for multi-RAT DC.

2    Discussion

Q1:		Will RAN4 specify UE requirements on;

- the total number of measurable objects across LTE and NR?

- the total number of configurable measurement events across LTE and NR?

According to RAN2 agreements, a measurement object is corresponding to one carrier frequency in NR. The total number of measurable objects mentioned in Q1 is the maximum allowed layers for UE monitoring specified in RAN4. For NR, RAN4 will specify number of layers for UE monitoring in each RAT, as well as the total number of carrier frequency layers for UE monitoring comprising of both NR and LTE.

In LTE, RAN4 specifies the UE capabilities for support of event triggering and reporting criteria, including intra-RAT and inter-RAT measurement. Such UE capabilities is to set some limits on the number of different event, periodic, logged measurement and no reporting criteria the UE may be requested to track in parallel. As long as the measurement configuration does not exceed the limits, UE shall meet the measurement performance requirements defined in TS36.133. The similar UE capabilities will also be considered in NR.

Answer to Q1: Yes

Q2:	if the answer to Q1-a) is Yes, and if both the MN and SN separately configure a measurement object on the same carrier frequency (e.g. the MN eNB RRC configures an inter-RAT NR measurement on a given carrier and the SN gNB RRC configures an intra-RAT NR measurements on the same - serving or non-serving - carrier frequency), should it be counted as 1 or 2 measured objects?

According to RAN2 current agreements, SN does not provide the NR measurement results to the MN. In order to support SN change controlled by MN, MN needs to configure inter-RAT NR measurements on the PSCell serving or non-serving carrier frequencies. That may lead to the situation that MN and SN separately configure a measurement object on the same carrier frequency. So it seems that such situation is inevitable.

If duplicated measurement objects on the same carrier frequency are configured, UE should be able to correlate the measurement configuration from MN and SN, then measure such carrier frequency once but send measurement report to MN and SN separately. Hence, from UE perspective, it should be counted as 1 measurement object if both MN and SN separately configure a measurement object on the same carrier frequency.

Answer to Q2: It should be counted as 1 measured object from UE perspective.

Q3:	Would the answer to Q2 be dependent on differences in configuration of the measurement object?

As we discussed in Q2, for measurement objects on the same carrier frequency, UE should only measure once. If the different parameters are configured in each measurement object, e.g. allowed measurement bandwidth, blacklisted cells and whitelisted cells, etc, it is unpredictable for the UE behaviour on how to do measurements based on different parameters.

For the measurement objects on the same carrier frequency, MN and SN should take the responsible for coordinating the configurations in the measurement objects. So UE could count the duplicated measured carrier frequency as one measurement object.

Answer to Q3: If different parameters are configured in measurement objects on the same carrier frequency, UE measurement behaviour is unpredictable. MN and SN should take responsible for measurement configuration coordination in order to prevent the unpredictable UE measurement behaviour.

Q4:	If MN and SN are to separately configure a measurement object on the same carrier frequency as in Q2, which parameters need to be configured with the same value (i.e., would need to be coordinated between the MN and SN) and which can be allowed to differ, in order to regard the two measurement object configurations from both MN and SN as one measurement object?

- For example, the parameters included in E-UTRA measurement object are listed in Annex.

- Any other parameters to be specified for NR, if any.

In order to regard the two measurement objects configurations from both MN and SN as one measurement object, some parameters in measurement object need to be consistent. Hence, coordination is needed between MN and SN. Firstly, the configured measurement carrier frequencies need to be coordinated in order to find whether duplicated measured frequencies exist or not. If duplicated measured frequencies are found, then MN and SN can further coordinate the detailed parameters in measurement objects on the same carrier frequency.

Observation 1: Measured carrier frequencies need to be coordinated between MN and SN in order to find whether duplicated measured frequencies exist or not.

In RAN2 LS, a list of parameters included in measurement object configuration is provided.  In Table 1, we provide our views on whether same values are required for each parameter.

Table 1: Parameters included in measurement object for E-UTRA

| Parameters | Descriptions | Whether same value is needed |
| --- | --- | --- |
| E-UTRA carrier frequency | Identifies E UTRA carrier frequency for which this configuration is valid. | Same |
| Allowed measurement bandwidth | indicate the maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 | Same |
| Presence of antenna port 1 | indicate whether all the neighbouring cells use Antenna Port 1. | Same |
| Neighbour cell configuration | provide the information related to MBSFN and TDD UL/DL configuration of neighbour cells. | Same |
| Offset to the carrier frequency | Offset value applicable to the carrier frequency. | Same |
| Cell list | List of cells to add/ modify in the cell list. | Same |
| Black list | List of cells to add/ modify in the black list of cells. | Same |
| Cell for which to report CGI | Target cell to report CGI | Could be different. |
| Measurement cycle of SCell | Measurement cycle for deactivated SCell(s) | Applied for carrier aggregation. It is not needed for inter-RAT measurement. |
| Wideband RSRQ measurements | RSRQ measurement bandwidth wider than 6 PRBs | Same |
| Cells to apply alternative TTT | Target cells to apply different TTT | Could be different. |
| T312 | Short RLM timer | Used for RLM. It is not needed for inter-RAT measurement. |
| Reduced measurement performance | Indicate to measure with reduced performance | Used for IncMon. If IncMon is supported in NSA NR, it should be the same. |
| Measurement DS configuration | Discovery Signal configuration for measurements | Used for small cell. Not sure whether it is needed for NR. |
| White cells | Target cell to report measurements | Same |
| RMTC configuration | RSSI measurement timing configuration | Used for LAA. Not sure whether it is needed in NR |

According to Table 1, most parameters used for UE performing measurements should be the same. In addition, NR may define some new measurement parameters, such as beam related parameters. RAN2 is discussing some threshold and/or beam number parameters used for deriving cell quality, which may also need to be considered when the final agreements are made.

Observation 2: For measurements objects on the same carrier frequency configured by both MN and SN, parameters related to UE performing measurement should be the consistent.

3  Conclusions

In this contribution, we discuss the measurement related information required for coordination between MN and SN for multi-RAT DC. The reply answers to RAN2 LS are provided as follows:

Q1:		Will RAN4 specify UE requirements on;

- the total number of measurable objects across LTE and NR?

- the total number of configurable measurement events across LTE and NR?

[RAN4 answer]: Yes

Q2:	if the answer to Q1-a) is Yes, and if both the MN and SN separately configure a measurement object on the same carrier frequency (e.g. the MN eNB RRC configures an inter-RAT NR measurement on a given carrier and the SN gNB RRC configures an intra-RAT NR measurements on the same - serving or non-serving - carrier frequency), should it be counted as 1 or 2 measured objects?

[RAN4 answer]: It should be counted as 1 measured objects from UE perspective.

Q3:	Would the answer to Q2 be dependent on differences in configuration of the measurement object?

[RAN4 answer]: If different parameters are configured in measurement objects on the same carrier frequency, UE measurement behaviour is unpredictable. MN and SN should take responsible for measurement configuration coordination in order to prevent the unpredictable UE measurement behaviour.

Q4:	If MN and SN are to separately configure a measurement object on the same carrier frequency as in Q2, which parameters need to be configured with the same value (i.e., would need to be coordinated between the MN and SN) and which can be allowed to differ, in order to regard the two measurement object configurations from both MN and SN as one measurement object?

- For example, the parameters included in E-UTRA measurement object are listed in Annex.

- Any other parameters to be specified for NR, if any.

[RAN4 answer]: Regarding the parameters in measurement object, firstly, measured carrier frequencies need to be coordinated in order to find whether duplicated measured frequencies exist or not. Then, parameters related to UE performing measurement should be the consistent. The detailed information is provided in the following table.

RAN4 didn’t agree on any new NR specific parameters so far. It can be further discussed after the agreement.

| Parameters | Descriptions | Whether same value is needed |
| --- | --- | --- |
| E-UTRA carrier frequency | Identifies E UTRA carrier frequency for which this configuration is valid. | Same |
| Allowed measurement bandwidth | indicate the maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 | Same |
| Presence of antenna port 1 | indicate whether all the neighbouring cells use Antenna Port 1. | Same |
| Neighbour cell configuration | provide the information related to MBSFN and TDD UL/DL configuration of neighbour cells. | Same |
| Offset to the carrier frequency | Offset value applicable to the carrier frequency. | Same |
| Cell list | List of cells to add/ modify in the cell list. | Same |
| Black list | List of cells to add/ modify in the black list of cells. | Same |
| Cell for which to report CGI | Target cell to report CGI | Could be different. |
| Measurement cycle of SCell | Measurement cycle for deactivated SCell(s) | Used for carrier aggregation. It is not needed for inter-RAT measurement. |
| Wideband RSRQ measurements | RSRQ measurement bandwidth wider than 6 PRBs | Same |
| Cells to apply alternative TTT | Target cells to apply different TTT | Could be different. |
| T312 | Short RLM timer | Used for RLM. It is not needed for inter-RAT measurement. |
| Reduced measurement performance | Indicate to measure with reduced performance | Used for IncMon. If IncMon is supported in NSA NR, it should be the same. |
| Measurement DS configuration | Discovery Signal configuration for measurements | Used for small cell. Not sure whether it is needed for NR. |
| White cells | Target cell to report measurements | Same |
| RMTC configuration | RSSI measurement timing configuration | Used for LAA. Not sure whether it is needed in NR |

Q5:	In addition to Q1, will RAN4 specify additional UE requirements for which the UE requirement across inter-RATs is not the union of the one for each RAT (like the number of measurable carriers)?

[RAN4 answer]: So far, there are no additional UE requirements across inter-RATs for RRM.

4   References

R2-1706140, LS on UE measurement capabilities across LTE and NR, RAN2.