3GPP TSG-RAN WG4 NR AH#3	R4-1709670

# Nagoya, Japan, 18 - 21 Sep, 2017

Source: 	ZTE

Title:	Correction TP of Conducted Tx spurious requirements for NR BS, Range 1

Agenda Item:	3.4.1.1 (BS RF: Unwanted emission requirements, Sub 6GHz)

Document for:	Endorsement

Introduction

TP of conducted Tx spurious emission for NR BS was technically endorsed in the RAN4#84 meeting, however some concerns are raised for measurement bandwidth 10KHz from 9KHz to 150KHz which was approved in the NR AH#1 meeting. As no technical reasons are indentified for that modification, therefore we propose to follow the recommendation from ITU-R SM.329. General Tx spur requirements are only considered here, i.e. no additional, co-location nor regional requirements are considered in this contribution.

Conclusion

It is proposed that the attached text proposal is included in TR38.xxx.

References

- R4-1709137,TP to draft general TR 38.xxx: Conducted Tx spurious requirements for NR BS, Huawei, ZTE, RAN4#84

- ------------------------------ Next modified section ------------------------------

6.6.4.1	Spurious emissions for FR1

NR BS Tx spurious emission conducted requirement for FR1 should comply with the following:

- For conducted requirements (at antenna connector/ at the transceiver array boundary) use same limits as 37.104/105 i.e. 9kHz lower limit, 5th harmonic upper limit

- The corresponding reference bandwidths for the specified spurious emission domain level measurement are:

  - 1khz between 9khz and 150khz

  - 10khz between 150khz and 30MHz

  - 100 kHz between 30 MHz and 1 GHz

  - 1 MHz for above 1 GHz

Furthermore, For NR BS in the Range1, the same spurious limit as for E-UTRA will be reused.

Based on the above agreements, the following is concluded:

The transmitter spurious emission limits for conducted requirement apply from 9 kHz to 12.75 GHz, excluding the frequency range from ΔfUEM below the lowest frequency of each supported downlink operating band, up to ΔfUEM above the highest frequency of each supported downlink operating band, where the ΔfUEM is defined in sub-clause 6.6.4.

For NR BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, this exclusion applies for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the single-band requirements apply and the multi-band exclusions and provisions are not applicable.

The requirements shall apply whatever the type of transmitter considered (single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification. Unless otherwise stated, all requirements are measured as mean power (RMS).

Category A or Category B limits are defined for the conducted Tx spurious emissions requirements for NR BS in Range 1, as outlined in Tables 6.6.5-1 and 6.6.5-2, respectively.

Table 6.6.5-1: Conducted Tx spurious emission limits for Range 1 NR BS, Category A

| Spurious frequency range | Maximum level | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150kHz | -13 dBm | 1kHz | Note 1 |
| 150kHz – 30 MHz |  | 10 kHz | Note 1 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz - 12.75 GHz |  | 1 MHz | Note 1, Note 2 |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: Applies only for operating bands for which the 5th harmonic of the upper frequency edge is reaching beyond 12.75 GHz. |  |  |  |

Table 6.6.5-2: Conducted Tx spurious emission limits for Range 1 NR BS, Category B

| Spurious frequency range | Maximum level | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150kHz | -36 dBm | 1kHz | Note 1 |
| 150kHz – 30 MHz |  | 10 kHz | Note 1 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz - 12.75 GHz | -30 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [x], s2.5 table 1. NOTE 3: Applies only for operating bands for which the 5th harmonic of the upper frequency edge is reaching beyond 12.75 GHz. |  |  |  |

- ------------------------------ End of modified sections ------------------------------