3GPP TSG-RAN WG4 Meeting #NR-AH3	R4-1709685

Nagoya, Japan, 18 - 21 Sep, 2017

Agenda item:	3.3.4.4.2

Source:	Dish Network

Title:	Considerations on Sub-6GHz ACS and blocking requirements

Document for:	Approval

# 1	Introduction

We provide our views on Sub-6GHz ACS and blocking in this contribution.

# 2	Discussion

The WF for Sub-6GHz ACS was agreed in RAN4#NR-AH2 [1] is as follows.

- ACS = 33dB for wanted channel bandwidth up to 20MHz.

  - ACS for other NR wanted channel bandwidth are FFS.

  - The adjacent channel bandwidth the same as the wanted signal.

  - ACS for different adjacent channel bandwidth are FFS.

  - ACS may be updated as SNR for Refsens is currently TBD.

- Case 1: Refsens+14dB for wanted signal and Refsens+45.5 dB for interferer.

- Case 2: -56.5dBm for wanted signal and -25dBm for interferer.

  - Case 1 and 2 is based on IM=2.5dB and SNR=-1dB.

The WF was generic, i.e. it is valid for all NR bands including LTE re-farming bands. After some afterthoughts, we believe that the LTE-style ACS definition where the blocker BW is limited to 5MHz would be better for LTE re-farming bands.

NR deployments at LTE bands won’t impact operator’s spectrum holdings. Hence there will be 5, 10, 15, and 20MHz blockers next to wanted signal. We believe that it would be most straightforward to apply LTE ACS for NR bands i.e. assuming 5MHz aggressor BW. It would for instance reduce testing when the ACS/blocking for the same band were not necessarily tested both for LTE and NR. Another point is that in many cases, even the ACS aggressor would not fit within the LTE re-farming band.

In LTE, the power difference between wanted signal and ACS aggressor is calculated according to formula (1) below:

Power difference = ACS – IM - SNR				(1)

The same formula should be applied for LTE re-farming bands.

For bandwidths above 20MHz, scaling in ACS is used.

Proposal1: ACS for LTE re-farming bands is defined according to LTE. NR IM and SNR are accounted in aggressor power according to formula above. For bandwidths above 20MHz, scaling in ACS is used.

For new frequency ranges defined for NR the situation is somewhat simpler. In general, the operator specific spectrum allocations for each band tend to be larger than those for LTE re-farming bands. Thus the agreed WF [1] still fits well for new NR bands.

In previous meeting, there was quite a lot of discussion on whether the ACS blocker BW should be limited to max 20MHz or if it should follow the principle of [1] even for wider than 20MHz CC’s. We don’t have a strong opinion on that. Either way is ok as long as the respective requirement is not overly relaxed.

Proposal2: ACS for new NR bands shall follow WF [1]. The ACS blocker BW and respective ACS requirements for CC’s >20MHz are FFS.

The principles applied in ACS definition pretty much define IBB and some of OOB as well. It is natural to assume that ACS, IBB1, and IBB2 aggressor BW’s are the same. Then OOB begins where IBB region ends. Accounting proposal 1 above, IBB1 and IBB2 are always 5MHz for LTE re-farming bands which is the same as ACS BW.

Proposal3: For both LTE re-farming and new NR bands, IBB1 and IBB2 aggressor BW’s are same as respective ACS aggressor BW.

IBB1, IBB2, and OOB aggressor and wanted signal levels and OOB frequency ranges for NR re-farming bands could be copied from LTE despite ample margins on most bands. For wider than 20MHz wanted signals, scaling is applied to calculate wanted signal power level.

Proposal4: IBB1, IBB2, and OOB aggressor and wanted signal levels and OOB frequency ranges for NR re-farming bands are copied from LTE. For CC’s >20MHz, scaling is applied to calculate wanted signal power level.

Reference [2] is pretty well aligned with our proposals 1-4. For ACS, we believe the same IM and SNR as in LTE are assumed in [2]. According to WF and our proposal1, the power difference between the wanted signal and aggressor would change if different IM and/or SNR are chosen for NR. For instance, should the IM be 2dB and SNR be -3dB for NR, then the power difference between the wanted signal and the aggressor for 33dB ACS would be 34dB.

The remaining open issues for new NR bands are IBB and OBB. To start off with likely the easiest one, we suggest IBB power levels could be the same as in LTE. With the principle of scaling the aggressor BW according to wanted signal, the wanted signal power level for all BW’s can be the same as for 5MHz CC in LTE (REFSENS+6)

Proposal5: IBB1 and IBB2 power levels are same as in LTE for new NR bands. The wanted signal power is REFSENS+6 for all BW’s when the aggressor BW scaling is used.

OOB range 3 blocker level has been extensive discussed in LTE a couple of years ago. Back then it was agreed to relax the aggressor power level from -15dBm to -20dBm for OOB3 interferers 2800<F<4200 MHz for 3.5GHz bands and later on for OOB3 interferers >4400MHz for B46. It was then confirmed by UE vendors that UE can meet OOB3 without help from RF filter. Thus, the OOB3 aggressor power should be not smaller than -20dBm. There should be no reason to relax OOB1 or OOB2 aggressor power levels either. The wanted signal power level for CC BW’s up to 20MHz can be copied from LTE and scaling can be applies for wider CC’s.

Proposal6: OOB1, OOB2, and OOB3 aggressor power levels for new NR bands are -44dBm, -30dBm, and -15dBm. OOB3 is relaxed to -20dBm in vicinity of bands >3GHz like in LTE.  The wanted signal power level for CC BW’s up to 20MHz is copied from LTE and scaling is applied for CC’s >20MHz.

An important aspect that needs considerations is how to define OOB region for new NR bands when the total bandwidth of ACS, IBB1, and IBB2 is basically 3x time wanted signal BW. The topics to consider are whether the OOB region offset from the applicable CC edge is fixed or whether it varies according to CC BW. If it varies according to CC BW, will all three OOB ranges vary or will for instance only range 1 vary? Our suggestion is future proof i.e. it allows to add wider CC BW’s later without revising the requirements. Hence, we suggest the OOB region offset varies according to CC BW. OOB1 is the first 45MHz starting from IBB2 edge, OOB2 is next 25MHz from OOB1 edge, and finally OOB3 starts from OOB2 other edge. Figure 1 illustrates the proposal.

Figure 1 Out-of-band blocking illustration

Proposal7: For new NR bands, OOB range begins where IBB2 range ends. OOB1 range is 45MHz wide, OOB2 range is 25MHz wide, and OOB3 begins where OOB2 ends.

# 3	Conclusions

Sub-6GHz ACS and blocking for LTE re-farming and new NR bands was considered with the following proposals.

Proposal1: ACS for LTE re-farming bands is defined according to LTE. NR IM and SNR are accounted in aggressor power according to formula above. For bandwidths above 20MHz, scaling in ACS is used.

Proposal2: ACS for new NR bands shall follow WF [1]. The ACS blocker BW and respective ACS requirements for CC’s >20MHz are FFS.

Proposal3: For both LTE re-farming and new NR bands, IBB1 and IBB2 aggressor BW’s are same as respective ACS aggressor BW.

Proposal4: IBB1, IBB2, and OOB aggressor and wanted signal levels and OOB frequency ranges for NR re-farming bands are copied from LTE. For CC’s >20MHz, scaling is applied to calculate wanted signal power level.

Proposal5: IBB1 and IBB2 power levels are same as in LTE for new NR bands. The wanted signal power is REFSENS+6 for all BW’s when the aggressor BW scaling is used.

Proposal6: OOB1, OOB2, and OOB3 aggressor power levels for new NR bands are -44dBm, -30dBm, and -15dBm. OOB3 is relaxed to -20dBm in vicinity of bands >3GHz like in LTE.  The wanted signal power level for CC BW’s up to 20MHz is copied from LTE and scaling is applied for CC’s >20MHz.

Proposal7: For new NR bands, OOB range begins where IBB2 range ends. OOB1 range is 45MHz wide, OOB2 range is 25MHz wide, and OOB3 begins where OOB2 ends.

# 4	References

[1] R4-1709149, “WF on sub-6GHz ACLR and ACS”, Nokia, Alcatel-Lucent Shanghai-Bell, RAN4#83, Hangzhou, China

[2] R-1708448, “Sub-6 ACS & IBB Re-farmed LTE bands2, Qualcomm Inc, RAN4#84, Berlin, Germany