3GPP TSG-RAN WG4 Meeting NR ad-hoc #3                                                   R4-1709719

# Nagoya, Japan, 18 - 21 Sep, 2017

Source:	KT

Title:	Channel BW for 28GHz

Agenda item:	3.2.1.3

Document for:	Approval

1.  Introduction

In RAN4 #84 meeting, there was agreement regarding BS channel BW. However it seems that we need further clarification regarding UE mandatory single component channel BW.

In this paper, we propose BW set that UE needs to support as with single component carrier as mandatory and several combinations that UE and BS need to support from release 15.

2. Mandatory channel BW

In RAN4 #84 meeting, following CH BW set was confirmed as single component carrier that BS needs to support in the range of 28GHz.

- 50MHz, 100MHz, 200MHz, 400MHz (only for 120KHz SCS)

KT understands that above four channel BWs also mandatory channel BWs that UE will support as single component carrier form Rel 15.

In addition, KT wants to have clear view regarding CA which UE and BS will support from Rel 15 in 28GHz range. If we consider current 5G spectrum that might be available in day 1 deployment in Korea, KT proposes below.

- UE needs to support 50MHz, 100MHz, 200MHz, 400MHz (only for 120KHz SCS) as single component carrier from Rel 15

- BS and UE will supports up to 800MHz with 2DL contiguous carrier aggregation manner from Rel 15. Detailed BW combination set will be clarified in following RAN4 meeting.

- For NSA operation, below combos need to be supported from Rel 15.

|  | LTE | NR (28GHz) |
| --- | --- | --- |
| Spectrum | Proposed LTE band in NR WID | 50MHz, 100MHz, 200MHz, 400MHz (only for 120KHz SCS) |
|  |  | Up to 800MHz with 2DL contiguous carrier aggregation |

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"