3GPP TSG RAN WG4 NR#3		           			             		R4-1709788
18 - 21 September 2017, Nagoya, Japan

Source:		Rohde & Schwarz

Title:	TP on DUT Positioning Guidelines for NR

Agenda Item:		3.6.2

Document for:	Approval

# Introduction

This contribution is a text proposal to TP38.810 [1] to define the DUT positioning guidelines for NR. A respective discussion paper was presented in [2].

# References

- R4-1704666, “TR38.810 v0.0.2,” Intel Corporation, CATR, 3GPP RAN4 #83, May 2017

- R4- 1709774, DUT Positioning Guidelines for NR, Rohde & Schwarz, 3GPP TSG RAN WG4 NR#3, Nagoya, Japan, September 2017

# Text proposal for TR 38.810

<<unchanged text omitted>>

- Annex C: UE coordinate system

# C.1	Reference coordinate system

This annex defines the measurement coordinate system for the NR UE.  The reference coordinate system, reused from the LTE MIMO OTA definition in [5], is provided in Figure C.1-1 below.

Figure C.1-1: Reference coordinate system

The following aspects are necessary:

-	A basic understanding of the top and bottom of the device is needed in order to define unambiguous DUT positioning requirements for the test

-	An understanding of the origin of the test system (i.e. the direction in which the x-axis points inside the test chamber) is needed in order to define unambiguous DUT orientation, DUT beam, signal, interference, and measurement angles

# C.2	Test conditions and angle definitions

Table C.2-1 below provides the test conditions and angle definitions.

Table C.2-1: Test conditions and angle definitions

| Test condition | DUT orientation | Link angle | Measurement angle | Diagram |
| --- | --- | --- | --- | --- |
| Free space | Ψ=0; Θ=0; Φ=0 | θLink; φLink with polarization reference PolLink = θ or ; φ | ΘMeas; φMeas with polarization reference PolMeas = θ or ; φ |  |
| NOTE 1: A polarization reference, as defined in relation to the reference coordinate system in E.1, is maintained for each signal angle, link or interferer angle, and measurement angle |  |  |  |  |

For each UE requirement and test case, each of the parameters in Table C.2-1 are defined as single values or ranges of values, such that DUT positioning, DUT beam direction, and angles of the signal, link/interferer, and measurement are specified.

# C.3	DUT Positioning Guidelines

The centre of the reference coordinate system shall be aligned with the geometric centre of the DUT in order to minimize the offset between antenna arrays integrated at any position of the UE and the centre of the quiet zone.

<<unchanged text omitted>>