3GPP TS 38.141-1 V0.0.1 (2017-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group RAN;

NR;

Base Station (BS) conformance testing

Part 1: Conducted conformance testing

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Radio, NR

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	General conducted test conditions and declarations	7

4.1	Measurement uncertainties and test requirements	7

4.2	Conducted requirement reference points	7

4.3	Base station classes	7

4.4	Regional requirements	7

4.5	BS configurations	7

4.6	Manufacturer’s declarations of regional and optional requirements	7

4.7	Applicability of requirements	7

4.8	Test configurations	7

4.9	 RF channels and test models	7

4.10	Format and interpretation of tests	7

4.11	Requirements for contiguous and non-contiguous spectrum	7

4.12	Requirements for BS capable of multi-band operation	8

5	Operating bands and channel arrangement	8

6	Conducted transmitter characteristics	8

6.1	General	8

6.2	Base station output power	8

6.3	Output power dynamics	8

6.4	Transmit ON/OFF power	8

6.5	Transmitted signal quality	8

6.6	Unwanted emissions	8

6.6.1	General	8

6.6.2	Occupied bandwidth	8

6.6.3	Adjacent Channel Leakage Power Ratio (ACLR)	8

6.6.4	Operating band unwanted emissions	9

6.6.5	Transmitter spurious emissions	9

6.7	Transmitter intermodulation	9

7	Conducted receiver characteristics	10

7.1	General	10

7.2	Reference sensitivity level	10

7.3	Dynamic range	10

7.4	In-band selectivity and blocking	10

7.5	Out-of-band blocking	10

7.6	Receiver spurious emissions	10

7.7	Receiver intermodulation	10

7.8	In-channel selectivity	10

8	Conducted performance requirements	11

Annex A (normative): Characteristics of interfering signals	12

Annex B (normative): Environmental requirements for the BS equipment	12

Annex C (informative): Test tolerances and derivation of test requirements	12

C.1	Measurement of transmitter	12

C.2	Measurement of receiver	12

Annex D (informative): Measurement system set-up	12

Annex E (informative): Change history	13

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

The present document specifies the Radio Frequency (RF) test methods and conformance requirements for NR Base Station (BS). These have been derived from, and are consistent with the NR BS specification defined in TS 38.104 [2]. The technical specification TS 38.141 is in 2 parts, part TS 37.141-1 (the present document) covers conducted requirements and part TS 37.145-2 covers radiated requirements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.104: "NR Base Station (BS) radio transmission and reception"

[3]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

1-C type NR BS: non-AAS BS in single RAT NR operation in Frequency Range 1, conforming to the conducted requirements set defined at the antenna connectors

1-H type NR BS: hybrid AAS BS in single RAT NR operation in Frequency Range 1, conforming to the hybrid requirements set defined at TAB connectors and RIB interface

1-O type NR BS: OTA AAS BS in single RAT NR operation in Frequency Range 1, conforming to the OTA requirements set defined at RIB interface

hybrid AAS BS:	AAS BS which has both a conducted RF interface and a radiated RF interface in the far field and conforms to a hybrid requirements set

hybrid requirements set: complete set of requirements applied to a hybrid AAS BS with both conducted and radiated  requirements

non-AAS BS: BS conforming to one of the specifications in 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4], 3GPP TS 37.104 [5], or to the 1-C core requirements set in TS 38.104 [x]

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements

OTA requirements set:	 complete set of OTA requirements applied to an OTA AAS BS

single RAT NR operation: operation of AAS BS declared to be single RAT NR in the operating band

TAB connector: transceiver array boundary connector

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AA	Antenna Array

DUT	Device Under Test

RDN	Radio Distribution Network

RIB	Radiated Interface Boundary

TAB	Transceiver Array Boundary

TT	Test Tolerance

# 4	General conducted test conditions and declarations

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in TS 38.141-1 (the present document), i.e. to all conducted tests defined for NR BS type 1-C and NR BS type 1-H.

The minimum requirements are given in TS 38.104 [2]. Test Tolerances (TT) for the conducted test requirements explicitly stated in the present document are given in annex C.

Test Tolerances are individually calculated for each test. Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

### 4.1.2	Acceptable uncertainty of Test System

The maximum acceptable uncertainty of the Test System is specified below for each conducted test defined explicitly in the present specification, where appropriate.

When a requirement is applied per antenna connector (TAB connector) then the conducted test uncertainty is applied to the measured value. When a requirement is applied for a group of antenna connectors (TAB connectors) then the test uncertainty is applied to sum of the measured power on each antenna connector (TAB connector) in the group.

The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the DUT to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95% is the measurement uncertainty tolerance interval for a specific measurement that contains 95% of the performance of a population of test equipment.

For RF tests, it should be noted that the uncertainties in sub-clause 4.1.2 apply to the Test System operating into a nominal 50 ohm load and do not include system effects due to mismatch between the DUT and the Test System.

#### 4.1.2.1	Measurement of transmitter

# Table 4.1.2-1: Maximum Test System uncertainty for conducted transmitter tests

| Sub-clause | Maximum Test System uncertainty | Derivation of Test System uncertainty |
| --- | --- | --- |

#### 4.1.2.2	Measurement of receiver

# Table 4.1.2-2: Maximum Test System uncertainty for conducted receiver tests

| Sub-clause | Maximum Test System uncertainty | Derivation of Test System uncertainty |
| --- | --- | --- |

### 4.1.3	Interpretation of measurement results

The measurement results returned by the Test System are compared - without any modification - against the test requirements as defined by the Shared Risk principle, which is defined in ITU-R M.1545 [3].

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each conducted measurement, equal to or lower than the appropriate figure in sub-clause 4.1.2 of this specification.

If the Test System for a conducted test is known to have a measurement uncertainty greater than that specified in sub-clause 4.1.2, it is still permitted to use this apparatus provided that an adjustment is made as follows:

Any additional uncertainty in the Test System over and above that specified in sub-clause 4.1.2 shall be used to tighten the conducted test requirement, making the test harder to pass. For some tests e.g. receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with sub-clause 4.1.2 does not increase the chance of passing a DUT where that device would otherwise have failed the test if a Test System compliant with sub-clause 4.1.2 had been used.

## 4.2	Conducted requirement reference points

For 1-C type NR BS the conducted requirements apply at the BS antenna connector (port A) for a single transmitter or receiver with a full complement of transceivers for the configuration in normal operating conditions. If any external apparatus such as an amplifier, a filter or the combination of such devices is used, requirements apply at the far end antenna connector (port B).

BS

cabinet

Port A

Port B

External

device

e.g.

TX filter

(if any

)

External

PA

(if any)

Towards

antenna connector



Figure 4.2-1: Transmitter interface for 1-C type NR BS

BS

cabinet

Port A

Port B

External

device

e.g.

RX filter

(if any

)

External

LNA

(if any)

From

antenna connector



Figure 4.2-2: Receiver interface for 1-C type NR BS

For 1-H type NR BS the conducted requirements apply at the conducted reference point called Transmitter Array Boundary (TAB), while the OTA requirements apply at the radiated reference point called radiated interface boundary (RIB).

This specification details the test requirements of the conducted requirements only and hence only requires the conducted reference points.

Figure 4.2-3: General architecture of 1-H type NR BS

Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna.

The transceiver unit array is part of the composite transceiver functionality generating modulated transmit signal structures and performing receiver combining and demodulation.

The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to receive/send parallel independent modulated symbol streams.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network that distributes the RF power between the transceiver array boundary and the antenna array, in an implementation specific way.

How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement sub-clause.

## 4.3	Base station classes for NR BS

Conducted test requirements for NR BS apply to Wide Area Base Stations, Medium Range Base Stations and Local Area Base Stations unless otherwise stated. NR BS classes for BS with antenna connectors (for non-AAS BS) or for BS with TAB connectors (for AAS BS) are defined as indicated below:

-	Wide Area BS is characterised by requirements derived from Macro Cell scenarios. For Wide Area BS class the minimum coupling loss between any antenna connector (TAB connector) and the UE is 70 dB.

-	Medium Range BS is characterised by requirements derived from Micro Cell scenarios. For Medium Range BS class the minimum coupling loss between any antenna connector (TAB connector) and the UE is 53 dB.

-	Local Area BS is characterised by requirements derived from Pico Cell scenarios. For Local Area BS class the minimum coupling loss between any antenna connector (TAB connector) and the UE is 45 dB.

## 4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or set by local and regional regulation as mandatory requirements. It is normally not stated in the 3GPP specifications under what exact circumstances that the requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.4-1: List of regional requirements

| Sub-clause | Requirement | Comments |
| --- | --- | --- |
| TBD |  |  |

## 4.5	NR BS configurations

## 4.6	Manufacturer’s declarations of regional and optional requirements

## 4.7	Applicability of requirements

## 4.8	Test configurations

Editor’s note: to capture multi-carrier/CA operation, contiguous/non-contiguous operation

## 4.9		RF channels and test models

Editor’s note: to capture multi-carrier/CA operation, contiguous/non-contiguous operation

### 4.9.1	RF channels

Detailed structure of the subclause is TBD.

### 4.9.2	Test models

Detailed structure of the subclause is TBD.

## 4.10	Format and interpretation of tests

## 4.11	Requirements for contiguous and non-contiguous spectrum

Editor’s note: whether this subclause is needed will depend on the decision for TS 38.104

## 4.12	Requirements for BS capable of multi-band operation

Editor’s note: whether this subclause is needed will depend on the decision for TS 38.104

# 5	Operating bands and channel arrangement

Detailed structure of the subclause is TBD.

# 6	Conducted transmitter characteristics

This subclause describes any general aspects of conducted transmitter characteristics and relations between requirements.

## 6.1	General

Detailed structure of the subclause is TBD.

## 6.2	Base station output power

Detailed structure of the subclause is TBD.

## 6.3	Output power dynamics

Detailed structure of the subclause is TBD.

## 6.4	Transmit ON/OFF power

Detailed structure of the subclause is TBD.

## 6.5	Transmitted signal quality

Detailed structure of the subclause is TBD.

## 6.6	Unwanted emissions

### 6.6.1	General

This subclause describes relations between unwanted emissions requirements.

### 6.6.2	Occupied bandwidth

Detailed structure of the subclause is TBD.

### 6.6.3	Adjacent Channel Leakage Power Ratio (ACLR)

Detailed structure of the subclause is TBD.

### 6.6.4	Operating band unwanted emissions

Detailed structure of the subclause is TBD.

### 6.6.5	Transmitter spurious emissions

Detailed structure of the subclause is TBD.

## 6.7	Transmitter intermodulation

Detailed structure of the subclause is TBD.

# 7	Conducted receiver characteristics

## 7.1	General

This subclause describes any general aspects of conducted receiver characteristics and relations between requirements.

## 7.2	Reference sensitivity level

Detailed structure of the subclause is TBD.

## 7.3	Dynamic range

Detailed structure of the subclause is TBD.

## 7.4	In-band selectivity and blocking

Detailed structure of the subclause is TBD.

## 7.5	Out-of-band blocking

Detailed structure of the subclause is TBD.

## 7.6	Receiver spurious emissions

Detailed structure of the subclause is TBD.

## 7.7	Receiver intermodulation

Detailed structure of the subclause is TBD.

## 7.8	In-channel selectivity

Detailed structure of the subclause is TBD.

# 8	Conducted performance requirements

Detailed structure of the subclause is TBD.

Annex A (normative):
Characteristics of interfering signals

Annex B (normative):
Environmental requirements for the BS equipment

Annex C (informative):
Test tolerances and derivation of test requirements

The test requirements explicitly defined in this specification have been calculated by relaxing the minimum requirements of the core specification TS 38.104 [2] using the test tolerances (TT) defined here. When the TT value is zero, the test requirement will be the same as the minimum requirement. When the TT value is non-zero, the test requirements will differ from the minimum requirements, and the formula used for this relaxation is given in the following tables.

The TT values are derived from Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the TT values may sometimes be set to zero.

The TT values should not be modified for any reason e.g. to take account of commonly known Test System errors (such as mismatch, cable loss, etc.).

Note that a formula for applying TT values is provided for all conducted tests, even those with a test tolerance of zero. This is necessary in the case where the conducted Test System uncertainty is greater than that allowed in sub-clause 4.1.2. In this event, the excess error shall be subtracted from the defined TT value in order to generate the correct tightened test requirements as defined in this annex.

# C.1	Measurement of transmitter

Detailed structure of the subclause is TBD.

# C.2	Measurement of receiver

Detailed structure of the subclause is TBD.

Annex D (informative):
Measurement system set-up

Annex E (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-09 | RP-77 | R4-170xxxx | - | - | - | TS skeleton | 0.0.1 |