3GPP TSG-RAN WG4 Meeting NR#3	R4-1709820

Nagoya, Japan, 18th – 21st September 2017

Agenda item:	3.4.4.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On blocking requirements for below 6 GHz NR bands

Document for:	Approval

# 1	Introduction

During the last 3GPP RAN4 #84 meeting, references [1][2] discussed and proposed to extend the frequency offset for out-of-band blocking requirements for NR operating bands that are equal to or wider than 100 MHz. However, no consensus was reached.

In this document, we provide further justifications for our proposed frequency offset for out of band blocking.

# 2         Discussion

For the NR BS Tx side, it was agreed in [3] that the frequency offset (ΔfUEM ) for spurious emissions is extended from 10 MHz to 40 MHz for NR bands ≥ 100 MHz as shown in Table 1.

Table 1: ΔfUEM of spurious emissions

|  | Before extension (ΔfUEM ) | After extension (ΔfUEM ) |
| --- | --- | --- |
| NR bands ≥ 100 MHz | 10 MHz | 40 MHz |

For the NR BS Rx side, reference [1] proposes to extend the frequency offset (ΔfOOB) for out-of-band blocking from 20 MHz to 80 MHz as shown in Table 2.

Table 2: ΔfOOB of out-of-band blocking

|  | Before extension (ΔfOOB ) | After extension (ΔfOOB ) |
| --- | --- | --- |
| NR bands ≥ 100 MHz | 20 MHz | 80 MHz |

The rationale behind such an extension is to maintain the same ratio ∆fOOB∆fUEM=2, and the same increase factor i.e.,

ΔfUEM (after extension) = 4 × ΔfUEM (before extension) and

ΔfOOB (after extension) = 4 × ΔfOOB (before extension).

Further, filter design is simplified and is not constrained by limited space when there is a potentially large number of receivers.

# 3	Conclusions

In this document, we have provided further technical justifications for the following proposals:

Proposal 1: ΔfOOB  = 80 MHz for NR bands with bandwidth equal to or wider than 100 MHz as a baseline for the boundary between in-band and out-of-band blocking requirement. That is,

1 MHz to (FUL_low - ΔfOOB) MHz and (FUL_high + ΔfOOB) MHz to 12750 MHz

(FUL_low - ΔfOOB)       to     (FUL_high + ΔfOOB)  	MHz

Proposal 2: the blocking requirement for such NR bands is the same as E-UTRA.

# References

- R4-1708492, On blocking requirements for below 6 GHz NR bands, Nokia, Nokia Shanghai Bell

- R4-1707967, Discussion on out of band blocking requirement for NR BS, ZTE

- R4-1706843, On NR BS boundary between UEM and spurious emission below 6 GHz, Nokia, Alcatel-Lucent Shanghai Bell