3GPP TSG-RAN WG4 (Radio) Meeting NR#3  	R4-1709842

Nagoya, Japan, 18-21September, 2017

Agenda item:	3.2.4

Source: 	Ericsson

Title: 		Simulation assumptions of Co-existence study for 55dBm CPE in mmWave spectrum

Document for:	Approval

# Introduction

In RAN4#84, a WF has been agreed on conducting simulations related to 55dBm CPE in mmWave scenario.

The contribution lists down the simulation assumptions of coexistence study for 55dBm CPE in mmWave spectrum. We propose to agree on these simulation assumptions so that companies can provide simulation results in the coming meeting which can be compared with each other.

# Network layout model

## Urban macro

### Single operator layout

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 300m | Note 1 |
| BS antenna height |  | 25 m |  |
| CPE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor CPE ratio | 20% | Note 1 |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.900 |
|  | CPE antenna height | Same as 3D-UMa in TR 36.873 | Elevation height is 4.5m |
| CPE distribution (horizontal) |  | Uniform |  |
| Minimum BS - CPE distance (2D) |  | [35 m] | [Note: Preferably much higher than 35m, e.g. 100m] |
| Channel model |  | UMa | Specified in TR38.900 |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Note: If we find any issue, then we can revisit parameters. Other cases are not precluded in addition to Case 1 and 2. |  |  |  |

### Multi operators layout

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) |  |
|  | coordinated operation (100% Grid Shift) |  |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |
| Non-coordinated operation |  |

## Dense urban

## There are a number of issues related to dense urban micro deployment whern 55dBm CPE is also used:

- There will be many more micro BSs within CPE coverage areas

- 3m minimum distance between CPE and micro BS is unrealistic case

- CPE has higher max power compared to BS in case of micro BS deployment scenatio.

Thus, we propose the following:

Proposal: Do not consider dense urban micro deployment for 55dBm CPE evaluations.

# Propagation model

## Path loss

The pathloss models are summarized in Table 7.4.1-1 and the distance definitions are indicated in Figure 7.4.1-1 and Figure 7.4.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 7.4.1-1.

| Figure7.4.1-1: Definition of d2D and d3D for outdoor CPEs | Figure 7.4.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor CPEs. Note that d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2 |
| --- | --- |

Observation:

Can we reuse UE-BS propagation model for BS-CPE since CPE height is 4.5m (instead of 1.5m of normal UE)?

Table 7.4.1-1: Pathloss models

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default valCPEs |
| --- | --- | --- | --- |
| UMa LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre freqCPEncy in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable freqCPEncy range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified formatand and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre freqCPEncy in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center freqCPEncy normalized by 1GHz, all distance related valCPEs are normalized by 1m, unless it is stated otherwise. |  |  |  |

## LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 7.4.2-1.

Table 7.4.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMi – Street canyon | Outdoor users: |
| UMa | Outdoor users: where and |
| Indoor – Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

## O-to-I penetration loss

The pathloss incorporating O-to-I building penetration loss is modelled as in the following:

PL = PLb + PLtw + PLin + N(0, σP2)

where PLb is the basic outdoor path loss given in Section 7.4.1. PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP  is the standard deviation for the penetration loss.

PLtw is characterized as:

is an additional loss is added to the external wall loss to account for non-perpendicular incidence;

, is the penetration loss of material i, example valCPEs of which can be found in Table 7.4.3-1.

pi is proportion of i-th materials, where ; and

N is the number of materials.

Table 7.4.3-1. Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2⋅f |
| IRR glass | LIRRglass=23+0.3⋅f |
| Concrete | Lconcrete=5+4⋅f |
| Wood | Lwood=4.85+0.12⋅f |
| Note: f is in GHz |  |

Table 7.4.3-2 gives PLtw, PLin and σP  for two O-to-I penetration loss models. The O-to-I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 7.4.3-2 O-to-I penetration loss model

|  | Path loss through external wall: PLtw [dB] | Indoor loss: PLin [dB] | Standard deviation: σP [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 | 0.5d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for RMa, UMa and UMi-Street Canyon. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models, and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world.

The pathloss incorporating O-to-I car penetration loss is modelled as in the following:

PL = PLb + N(μ, σP2)

where PLb is the basic outdoor path loss given in Section 7.4.1. μ = 9, and σP = 5. Optionally, for metallized car windows, μ = 20 can be used. The O-to-I car penetration loss models are applicable for at least 0.6-60 GHz.

# Transmission power control model

For uplink, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

- CL-xile = 106+ 10*log10(200/X)

  - X: UL transmission BW

- Gamma = 1

Note: we need discuss KPI based on simulation results.

# Received signal power model

The following model is applied.

RX_PWR = TX_PWR – pathloss + G_TX + G_RX

where:

RX_PWR is the received signal power

TX_PWR is the transmitted signal power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

# Other simulation parameters

| Parameters | Urban macro |
| --- | --- |
| Channel bandwidth | 200MHz |
| Scheduled channel bandwidth per CPE (DL) | 200MHz |
| Scheduled channel bandwidth per CPE (UL) | 200MHz |
| The number of active CPE (DL) | Same as the number of BS beam |
| The number of active CPE (UL) | Same as the number of BS beam |
| Traffic model | Full buffer |
| DL power control | NO |
| UL power control | YES |
| BS max TX power in dBm | 43dBm |
| CPE max TX power in dBm | 35dBm |
| CPE min TX power in dBm | -40dBm |
| BS Noise figure in dB | 10 |
| CPE Noise figure in dB | 10 |
| Handover margin Is it needed since CPEs are fixed? | 3dB |

# Simulation description

Adopt following simulation steps. If companies find issues in the following simulation steps, we can revisit in RAN4#80bis. Note: detailed simulation description is captured in Section 5.1.5 TR25.942.

- Aggressor and victim network are generated.

- CPE associations: CPE are associated to base station based on coupling loss.

  - Associations are made assuming a single element at both CPE and BS.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-CPE. This done for both victim and aggressor networks.

- SINR Throughput are measured in the victim systems without considering ACI, i.e. ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

- SINR and throughput are computed considering ACI: ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based degradation cause by ACI: LossACI=1-ThputACIThputSINGLE.

# Evaluation metric

Assume scaled Shannon's formula specified in TR36.942 with following parameters.

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMAX, dB | 30 | 22 | Based on 256QAM 0.93(UL) & 64QAM 0.93 (UL) |

# ACIR range to be evaluated

ACIR range: 5 ~ 45dB, with 5dB as step. Note that: we do not have simulation per 1dB step.

# Antenna configuration

Assumptions captured in page 8 - 11 of [R4-168794] are assumed as BS antenna configurations. For Uma base station, we keep this same antenna model.

For CPE, the current UE antenna model is not valid, since CPE is deployed much higher than normal UE height. One Option is to use micro BS antenna model for CPE,

Proposal: Use micro BS antenna assumptions for 55dBm CPE.

Assumptions captured in [R4-167123] are assumed as CPE antenna configurations without an additional 3dB gain. Note: R4-167750 provides further explanation.

# Co-existence scenarios

Evaluate following scenarios for 55dBm CPE:

| No. | Aggressor | Victim | Simulation freqCPEncy | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Urban macro |