3GPP TSG-RAN WG4 Meeting NR#3	R4-1709844

Nagoya, Japan, 18th-21st September 2017

Source: 	Ericsson

Title:  	Proposals on absolute level ACLR for mm-wave NR base station

Agenda Item:	3.4.3.3.1

Document for:	Approval

# Introduction

The discussion on mm-wave frequency band ACLR has been ongoing for some time in RAN4, and there is consensus on relative ACLR level. RAN4 has discussed the necessity for absolute levels for ACLR requirements in mmWave spectrum. In our previous contribution[1], we elaborated on the mm-wave transmitter chain and describe in detail the contributions to transmitter noise floor and considerations needed before the absolute ACLR level is settled.

In this contribution, we discuss further considerations on the importance of introducing absolute limits for mmWave NR BS ACLR levels and proposed a way forward on this.

# Acievable levels of TX leakage in mmWave

We have provided detailed analysis on achievable levels in terms of TX leakage for mmWave BS. Consider the following simple example [1]: Assume an 7 ENOB DAC, which translates to roughly 44dB of of maximum SNR due to quantization noise using the full Nyquist bandwidth. With an RF signal bandwidth BWRF=400MHz (both for desired and adjacent channel) and a sampling rate of fs=2GHz  the oversampling rate (fs/BWRF) becomes 5 times (~7dB) and, thus, the maximum SNR improves to 51dB. To accommodate an OFDM signal, a 13dB of backoff is applied yielding a quantization noise level within one channel bandwidth of -38dBc (disregarding the sinc roll-off due to zero-order hold of the DAC). In practice, further backoff may be required to account for a predistorted signal in case DPD is used. Now, quantization noise is commonly only one major source of noise and is commonly designed to be lower or comparable to other noise sources. Here, consider a total noise budget that is 7dB above the quantization noise to accommodate other noise source and the noise level in an adjacent channel becomes -31dBc. Due to TX dynamic range we may operate at lower output power levels than maximum output power Pmax but the noise floor is unaffected. For example, at 6dB below Pmax the noise in the adjacent channel becomes -25dBc, and thus will comparable to relative ACLR requirements. In terms of absolute power levels, with Pout=Pmax=12dBm and 512 transceivers (27dB), the noise floor becomes 12dBm+27dB-31dBc-10log10(400MHz/1MHz)=-18dBm/MHz. Note that additional contributions could occur.

# Revisiting NR coexistence studies

For Rel-14 NR SI phase, extensive studies have been done on the coexistence of mmWave NR systems. The table below shows the interpolated ACIR value for DL.

Table 5.5-7: Interpolated ACIR values for DL to meet the 5% throughput loss criteria at 30GHz

| Scenario |  | Indoor |  | Urban macro, ISD=200m |  | Urban macro, ISD=300m |  | Dense urban |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF [dB] |  | 9 | 11 | 9 | 11 | 9 | 11 | 9 | 11 |
| Nokia, ALU | Average | 15.73 | 15.73 | 11.85 | 11.81 | 14.29 | 14.11 | 5.00 | 5.00 |
|  | 5%-tile | 14.88 | 14.87 | 19.90 | 19.95 | 23.02 | 19.59 | 7.37 | 8.52 |
| Ericsson * | Average | 15.63 |  | 14.53 |  |  |  | 12.23 |  |
|  | 5%-tile | 17.32 |  | 18.72 |  |  |  | 14.63 |  |
| NEC | Average | 15.19 | 16.10 | 12.47 | 12.47 | 12.62 | 12.55 | 9.66 | 9.24 |
|  | 5%-tile | 13.79 | 16.10 | 12.32 | 11.41 | 8.83 | 7.62 | 10.68 | 7.56 |
| China Telecom | Average |  |  | 14.11 | 14.06 | 16.57 | 16.53 |  |  |
|  | 5%-tile |  |  | 20.80 | 20.04 | 26.47 | 26.13 |  |  |
| Huawei | Average | 15.72 | 15.72 | 13.57 | 13.53 | 15.95 | 15.83 | 7.10 | 6.83 |
|  | 5%-tile | 14.47 | 14.47 | 23.38 | 22.22 | 25.52 | 26.34 | 11.92 | 12.73 |
| ZTE | Average | 16.21 | 16.23 | 9.19 | 9.43 |  |  | 5.05 | 5.49 |
|  | 5%-tile | 14.75 | 15.92 | 24.79 | 27.89 |  |  | 18.81 | 17.17 |
| Samsung | Average | 15.68 | 15.68 | 8.89 | 8.87 | 9.39 | 9.37 | 5.00 | 5.00 |
|  | 5%-tile | 14.62 | 14.62 | 13.62 | 13.02 | 9.14 | 8.35 | 8.93 | 7.51 |
| CATT | Average | 15.45 | 15.45 | 15.13 | 15.12 |  |  | 10.10 | 9.91 |
|  | 5%-tile | 13.14 | 13.14 | 20.58 | 23.85 |  |  | 14.14 | 13.51 |
| Qualcomm | Average | 16.17 | 16.18 | 12.32 | 12.30 | 12.47 | 12.43 | 6.12 | 5.80 |
|  | 5%-tile | 13.47 | 13.47 | 13.49 | 13.16 | 9.55 | 9.14 | 10.33 | 8.96 |
| Intel | Average | 15.64 | 15.90 | 8.16 | 8.35 | 10.70 | 9.71 | 5.00 | 5.00 |
|  | 5%-tile | 15.78 | 14.55 | 16.48 | 17.99 | 19.08 | 19.17 | 6.47 | 5.00 |
| LG | Average | 19.59 | 19.59 | 8.91 | 9.42 | 14.49 | 16.32 | 7.83 | 10.46 |
|  | 5%-tile | 15.51 | 15.51 | 23.43 | 22.90 | 23.36 | 23.05 | 15.46 | 14.60 |
| * NF=10 is assumed |  |  |  |  |  |  |  |  |  |

Table 5.5-10: UE ACS

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| UE ACS value [dB] | 22.5 | 21.5 | 20.5 |

Table 5.5-11: BS ACLR

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| BS ACLR value [dB] | 27.5 | 25.5 | 23.5 |

Looking at the ACIR results above and along with thr decided ACLR values as in Table above, according to investigations by majority of the companies, there is 8~10dB margin in ACLR for 30GHz case. The margins are large for low power cases, i.e. dense urban and indoor cells. Similar margins can also be seen for other frequency cases. Thus, this can be ensured that, the absolute limit to BS ACLR will not cause degradation in coexistence performance for NR, when low power BS is used.

Observation: Introducing absolute limit for BS ACLR will not cause degradation to NR coexistence performance.

# Proposal

Based on the above discussions, we propose the following:

Proposal: Absolute ACLR limits for mmWave NR BS shall be equal to in-band spurious emission limit.

The above proposal is inline with previous proposal from other companies, such as [3].

# References

R4-1708123, On absolute level ACLR for mm-wave bands, Ericsson

3GPP TR 38.803 V2.0.0 (2017-03), Study on New Radio Access Technology; RF and co-existence aspects (Release 14)

R4-1707961, Discussion on ACLR and ACS requirement for mmWave NR BS, ZTE Corporation