3GPP TSG-WG RAN4 Meeting #NR3	R4-1709949

# Nagoya, Japan, Date 18th  – 21st Sept, 2017

Source: 	Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc.

Title: 					TP to TR General aspects for UE RF for NR: NR range 1 IQ Image and Carrier leakage

Agenda Item: 			3.3.3.7.2

Document for:	Approval

# 1	Introduction

NR Range 1 IQ-image and carrier leakage requirement has been discussed in RAN4 during the MPR simulation assumtions topic. Given the fact that these requirements are rather old and chipsets have evolved in recent years it was proposed to use LTE 256-QAM assumptions as a baseline [1]. This was seen however as a too stringent minimum requirement. To accommodate the concearns of chipset vendors new propssal was made [2] with IQ-Image = 30 dBc and carrier leakage = 28 dBc. Finally it was agreed to use 28 dBc requirement for NR range 1 IQ Image and Carrier leakage for MPR studies [3]. In this contribution we propose to adopt this agreement also for actual requirement and to be specified in TS 38.101.

# 2	Discussion

In current LTE specification carrier leakage is specified twice, firstly in clause 6.5.2.2 Carrier leakage and then again in clause 6.5.2.3	In-band emissions. Perhaps it is sufficient that in NR specification it is only specified once as a part in in-band emission requirement.

As mentioned in introduction we propose to specify IQ Image and Carrier leakage for NR Range 1 as follows

Table 1: NR range 1 IQ Image and Carrier leakage requirement

| IQ Image | dB | -28 | Image frequencies when carrier center frequency < 6 GHz and Output power > 10 dBm |
| --- | --- | --- | --- |
|  |  | -25 | Image frequencies when carrier center frequency < 6 GHz and Output power ≤ 10 dBm |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier center frequency < 6 GHz |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |
|  |  | -10 | -40 dBm Output power < -30 dBm |

# 3	Conclusion

In this contribution we have made proposal for NR Range 1 IQ Image and Carrier leakage requirement and have associated TP.

# 4	References

[1] R4-1706691, Simulation and measurement results for NR Sub-6 GHz CP-OFDM and DFT-s-OFDM waveforms, Nokia, Alcatel-Lucent Shanghai Bell, Skyworks Solutions, Inc., RAN4#NR AH2

[2] R4-1707598, S6 Contiguous MPR and UE assumptions, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., RAN4#84

[3] R4-1708912, NR MPR simulation assumptions for sub-6 GHz operation, Nokia, RAN4#84

****************************** TP to TR ***********************************************

## 5.4	Transmit signal quality

Detailed structure of the subclause is TBD.

### 5.4.1	IQ-Image and Carrier leakage

IQ Image and Carrier leakage for NR Range 1 are specified as follows

Table 5.4.1-1: NR range 1 IQ Image and Carrier leakage requirement

| IQ Image | dB | -28 | Image frequencies when carrier center frequency < 6 GHz and Output power > 10 dBm |
| --- | --- | --- | --- |
|  |  | -25 | Image frequencies when carrier center frequency < 6 GHz and Output power ≤ 10 dBm |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier center frequency < 6 GHz |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |
|  |  | -10 | -40 dBm Output power < -30 dBm |
| Note: Positions of carrier leakage will be captured later. |  |  |  |

****************************** End of TP to TR ***********************************************