3GPP TSG RAN WG5 Meeting #61	 R5-134643

# San Francisco, 11th –15th   Nov 2013

| CR-Form-v11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 36.523-1 |  | CR |  | CRNum | rev |  | - |  |  | Current version: |  |  |  |  |  |  | 11.4.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  | UICC apps |  |  | ME |  |  |  |  | Radio Access Network |  |  |  |  | Core Network |  |  |  |
| Title: |  | Addition of test case “Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MT call / SRVCC HO failure” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  | CATR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  | R5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  | TEI10_Test |  |  |  |  |  |  |  |  |  |  | Date: |  | 2013-10-31 |  |  |  |  |  |
| Category: |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  | Rel- |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |  |  |  |  |  |

| Reason for change: | The test case cover “Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MT call / SRVCC HO failure” need to be added. |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | Added test case cover “Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MT call / SRVCC HO failure” |  |  |  |
| Consequences if not approved: | The test case cover “Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MT call / SRVCC HO failure” will be missed. |  |  |  |
| Clauses affected: | 13.4.3.22 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | X | Other core specifications | TS/TR ... CR ... |
| affected: | X |  | Test specifications | 36.523-2 |
| (show related CRs) |  | X | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |

<Start of modified section>

13.4.3.22	Inter-system mobility / E-UTRA voice to GERAN CS voice / aSRVCC / MT call / SRVCC HO failure

13.4.3.22.1	Test Purpose (TP)

(1)

with { UE is in E-UTRA RRC_CONNECTED state, an IMS MT speech call is in alerting phase and UE receives a MobilityFromEUTRACommand message }

ensure that {
  when { UE detects radio link failure }

then { UE transmits SIP UPDATE message after RRC connection re-establishment procedure }

}

13.4.3.22.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.4.3.3, 5.4.3.5, 5.3.7.2, 5.3.7.3, 5.3.7.4, 5.3.7.5 and TS 24.237, clause 12.2.4.2.

[TS 36.331, clause 5.4.3.3]

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to handover:

2>	if the targetRAT-Type is set to utra or geran:

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

[TS 36.331, clause 5.4.3.5]

The UE shall:

1>	if T304 expires (mobility from E-UTRA failure); or

1>	if the UE does not succeed in establishing the connection to the target radio access technology; or

1>	if the UE is unable to comply with (part of) the configuration included in the MobilityFromEUTRACommand message; or

1>	if there is a protocol error in the inter RAT information included in the MobilityFromEUTRACommand message, causing the UE to fail the procedure according to the specifications applicable for the target RAT:

2>	stop T304, if running;

2>	if the cs-FallbackIndicator in the MobilityFromEUTRACommand message was set to TRUE:

3>	 indicate to upper layers that the CS Fallback procedure has failed;

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, mac-MainConfig and sps-Config;

2>	initiate the connection re-establishment procedure as specified in 5.3.7;

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure when AS security has been activated. The UE initiates the procedure when one of the following conditions is met:

...

1>	upon mobility from E-UTRA failure, in accordance with 5.4.3.5; or

...

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	start timer T311;

1> suspend all RBs except SRB0;

1>	reset MAC;

1>	release the SCell(s), if configured, in accordance with 5.3.10.3a;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	release reportProximityConfig and clear any associated proximity status reporting timer;

1>	release measSubframePatternPCell, if configured;

...

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

[TS 36.331, clause 5.3.7.3]

Upon selecting a suitable E-UTRA cell, the UE shall:

1>	stop timer T311;

1>	start timer T301;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	initiate transmission of the RRCConnectionReestablishmentRequest message in accordance with 5.3.7.4;

[TS 36.331, clause 5.3.7.4]

If the procedure was initiated due to radio link failure or handover failure, the UE shall:

1>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

The UE shall set the contents of RRCConnectionReestablishmentRequest message as follows:

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source PCell (handover and mobility from E-UTRA failure) or used in the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source PCell (handover and mobility from E-UTRA failure) or of the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source PCell (handover and mobility from E-UTRA failure) or of the PCell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

...

2>	else if the re-establishment procedure was initiated due to handover failure as specified in 5.3.5.6 (intra-LTE handover failure) or 5.4.3.5 (inter-RAT mobility from EUTRA failure):

3>	set the reestablishmentCause to the value handoverFailure;

The UE shall submit the RRCConnectionReestablishmentRequest message to lower layers for transmission.

[TS 36.331, clause 5.3.7.5]

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1>	re-establish PDCP for SRB1;

1>	re-establish RLC for SRB1;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	resume SRB1;

NOTE:	E-UTRAN should not transmit any message on SRB1 prior to receiving the RRCConnectionReestablishmentComplete message.

1>	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionReestablishment message, as specified in TS 33.401 [32];

1>	store the nextHopChainingCount value;

1>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

...

1>	configure lower layers to activate integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

...

1>	configure lower layers to apply ciphering using the previously configured algorithm, the KRRCenc key and the KUPenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	set the content of RRCConnectionReestablishmentComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include the rlf-InfoAvailable;

...

1>	perform the measurement related actions as specified in 5.5.6.1;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission, upon which the procedure ends;

[TS 24.237, clause 12.2.4.2]

If the SC UE is engaged in a session in early dialog state and:

...

-	does not successfully retune to the 3GPP UTRAN or 3GPP GERAN after it receives the handover command from the eNodeB (as described in 3GPP TS 36.331 [62]) or from the NodeB (as described in 3GPP TS 25.331 [61]);

then if the SC UE the SC UE shall send a SIP UPDATE request containing:

1)	an SDP offer, including the media characteristics as used in the existing dialog; and

2)	a Reason header field containing protocol "SIP" and reason parameter "cause" with value "487" as specified in IETF RFC 3326 [57], and with reason-text set to either "handover cancelled" or "failure to transition to CS domain";

by following the rules of 3GPP TS 24.229 [2] in each transferred session.

13.4.3.22.3	Test description

13.4.3.22.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 24.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

13.4.3.22.3.2	Test procedure sequence

Table 13.4.3.22.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 13.4.3.22.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RSEPRE | dBm/15kHz | -65 | - | The power level values are such that entering conditions for event B2 are not satisfied. |
|  | RSSI | dBm | - | -85 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | RSSI | dBm | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | Only Cell 1 is available. |
|  | RSSI | dBm | - | "Off" |  |

Table 13.4.3.22.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures GERAN Cell 24 to reference configuration according 36.508 Table 4.8.4-1, condition GERAN Speech. | - | - | - | - |
| 2-23 | Steps 1 to 22 of the generic test procedure for IMS MT speech call (TS 36.508 4.5A.7.3-1). | - | - | - | - |
| 24 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter-RAT measurement and reporting for event B2. | <-- | RRCConnectionReconfiguration | - | - |
| 25 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 26 | The SS changes the power level for Cell 1 and Cell 24 according to the row "T1" in Table 13.4.3.22.3.2-1 | - | - | - | - |
| 27 | The UE transmits a MeasurementReport message on Cell 1 to report event B2 for Cell 24. | --> | MeasurementReport | - | - |
| 28 | The SS changes the power level for Cell 1 and Cell 24 according to the row "T2" in Table 13.4.3.22.3.2-1. | - | - | - | - |
| 29 | The SS transmits a UECapabilityEnquiry message on Cell 1 to request UE radio access capability information for E-UTRA and GERAN. | <-- | UECapabilityEnquiry | - | - |
| 30 | The UE transmit a UECapabilityInformation message on Cell 1. NOTE: The start-CS values received, should be used to configure ciphering on Cell 24. | --> | UECapabilityInformation | - | - |
| 31 | The SS transmits a MobilityFromEUTRACommand message on Cell 1. | <-- | MobilityFromEUTRACommand | - | - |
| 32 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 1. | --> | RRCConnectionReestablishmentRequest | - | - |
| 33 | The SS transmits an RRCConnectionReestablishment message on Cell 1. | <-- | RRCConnectionReestablishment | - | - |
| 34 | The UE transmits an RRCConnectionReestablishmentComplete message on Cell 1. | --> | RRCConnectionReestablishmentComplete | - | - |
| 35 | The SS transmits an RRCConnectionReconfiguration message on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 36 the steps specified in Table 13.4.3.22.3.2-3 should take place. | - | - | - | - |
| 36 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 37-38 | Steps 12-13 expected sequence defined in annex C.21 of TS 34.229-1 [35]. | - | - | - | - |

Table 13.4.3.22.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit SIP UPDATE request on Cell 1? NOTE: Step 1 defined in annex C.28 of TS 34.229-1 [35] is performed. | - | - | 1 | P |
| 2 | Step 2 expected sequence defined in annex C.28 of TS 34.229-1 [35]. | - | - | - | - |

13.4.3.22.3.3	Specific message contents

Table 13.4.3.22.3.3-1: ATTACH REQUEST (preamble)

| Derivation path: 36.508 Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MS network capability |  | SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported |  |  |
| Mobile station classmark 2 |  | Any allowed value |  |  |
| Supported Codecs |  | Any allowed value |  |  |
| Old GUTI type |  | Any allowed value |  |  |

Table 13.4.3.22.3.3-2: RRCConnectionReconfiguration (step 24, Table 13.4.3.22.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 13.4.3.22.3.3-3: MeasConfig (Table 13.4.3.22.3.3-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 with condition GERAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measurementConfiguration ::= SEQUENCE { |  |  |  |
| measObjectToAddModifyList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f11 |  |  |
| measObject[1] | MeasObjectGERAN-GENERIC(f11) |  |  |
| measObjectId[2] | IdMeasObject-f1 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| reportConfigToAddModifyList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfigInterRAT-B2-GERAN |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-GERAN (-90, -100) |  |  |
| } |  |  |  |
| measIdToAddModifyList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f11 |  |  |
| reportConfigId[1] | IdReportConfigInterRAT-B2-GERAN |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.22.3.3-4: MeasObjectGERAN-f11 (Table 13.4.3.22.3.3-3)

| Derivation Path: 36.508clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectGERAN-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| carrierFreqs SEQUENCE { |  |  |  |
| startingARFCN | Downlink GERAN ARFCN of Freq |  |  |
| bandIndicator | Set according to the band used for cell24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| explicitListOfARFCNs | Set the corresponding ARFCN of cell24 |  |  |
| } |  |  |  |
| } |  |  |  |
| offsetFreq | 0 (dB 0) |  |  |
| ncc-Permitted | ‘01000000'B | NCC=1 permitted |  |
| cellForWhichToReportCGI | Not present |  |  |
| } |  |  |  |

Table 13.4.3.22.3.3-5: MeasurementReport (step 27, Table 13.4.3.22.3.2-2)

| Derivation Path: 36.508, table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListGERAN SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 24 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rssi | The value of rssi is present but contents not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.22.3.3-6: UECapabilityEnquiry (step 29, Table 13.4.3.22.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 2 entries |  |  |
| RAT-Type[1] | eutra |  |  |
| RAT-Type[2] | Geran |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.22.3.3-7: MobilityFromEUTRACommand (step 31, Table 13.4.3.22.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| cs-FallbackIndicator | True |  |  |
| purpose CHOICE { |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | Geran |  |  |
| targetRAT-MessageContainer | HANDOVER COMMAND(GERAN RR message) |  |  |
| nas-SecurityParamFromEUTRA | The 4 least significant bits of the NAS downlink COUNT value |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.22.3.3-8: HANDOVER COMMAND (Table 13.4.3.22.3.3-7)

| Derivation Path: 51.010, Table 40.2.4.33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Cell Description |  |  |  |
| Network Colour Code | 1 |  |  |
| Base Station Colour Code | 5 |  |  |
| BCCH Carrier Number | The BCCH Carrier ARFCN as per table in clause 40.1.1 of 51.010-1. |  |  |
| Description of the First Channel, after time |  |  |  |
| Channel Description |  |  |  |
| Channel Type and TDMA offset | TCH/F + ACCH’s |  |  |
| Timeslot Number | Chosen arbitrarily, but not Zero. |  |  |
| Training Sequence Code | Same as the BCCH |  |  |
| Hopping channel | Single RF channel |  |  |
| ARFCN | The first ARFCN in the cell allocation as per table in clause 40.2.1.1.1 of 51.010-1 |  |  |
| Cipher Mode Setting | 1001xxxy | See TS 44.018 §9.1.15.10 xxx - px_GSM_CipherAlg y - px_GSM_CipheringOnOff |  |

Table 13.4.3.22.3.3-9: RRCConnectionReestablishmentRequest (step 32, Table 13.4.3.22.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.22.3.3-10: RRCConnectionReestablishmentComplete (step 34, Table 13.4.3.22.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.22.3.3-11: RRCConnectionReconfiguration (step 35, Table 13.4.3.22.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>