3GPP TSG-RAN WG5 Meeting #65	R5-145216

# Venice, Italy, 10th – 14th November 2014

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | CRNum | rev | - | Current version: | 11.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UTRA CSG test case 12.3.1.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, TF 160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  |  | Date: |  | 2014-10-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | At step 11, the UE initiates a PS detach. This can trigger the UE to perform a GPRS detach. Secondly, procedure to trigger the attach request sent by UE is missing at step 16 Also, at Step 16 the ATTACH REQ is expected from the UE. The ATTACH REQ expects the mobile ID to be IMSI, which should actually be P-TMSI-1. Similarly, UE should have valid TMSI if it has remained IMSI-attached, hence TMSI status = “ no valid TMSI available ’’ is removed from the Test prose |  |  |  |  |  |  |  |  |
| Summary of change: |  | “Normal detach, GPRS Detach” added at step 12. Step 15a is included for UE to initiate an attach. In Step 16, changed the expected Mobile Identity to P-TMSI-1 and TMSI status = “ no valid TMSI available ’’ is removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |

#### 12.3.1.10	UE initiated detach/abnormal case/ Not authorized for this CSG

12.3.1.10.1	Definition

12.3.1.10.2	Conformance requirement

The following abnormal cases can be identified:

a)	CSG ID of the CSG cell is not in the Allowed CSG list of the MS which sends the detach request

If the MS initiates a detach procedure in a CSG cell the CSG ID of which is not in the MS's Allowed CSG list and the detach procedure is not due to "switch off", the network shall initiate the detach procedure. The network shall send a DETACH REQUEST message including the GMM cause value #25 "not authorized for this CSG", to indicate to the MS to remove the CSG ID of the cell, where the MS has sent the DETACH REQUEST message, from the allowed CSG list.

Reference

3GPP TS 24.008 clause 4.7.4.1.5

12.3.1.10.3	Test purpose

To test the behaviour of the MS if the MS initiates a detach procedure in a CSG cell not due to "switch off", and the network send a DETACH REQUEST message including the GMM cause value #25 "not authorized for this CSG".

12.3.1.10.4	Method of test

Initial condition

System Simulator:

Two cells cell A with MCC1/MNC1/LAC1/RAC1 (RAI-1), cell B in MCC1/MNC1/LAC1/RAC1 (RAI-1)/CSG1.

Both cells are operating in network operation mode I.

Both Cell A and Cell B belong to PLMN 1.

The cell B shall also include:

-"CSG Indicator" in its Master Information Block

-"CSG Identity" and "CSG PSC Split Information" in its System Information Block type 3 with the following contents:

Start PSC: 144

Number of PSCs: alltheRest

On the SS side, CSG1 is not in the Allowed CSG List which is stored in the CSG subscription data.

User Equipment:

The UE has a valid IMSI.

The UE’s Allowed CSG List contains CSG1

The UE is equipped with a USIM containing default values except for those listed below.

| USIM field | PLMN | CSG ID |
| --- | --- | --- |
| EFACSGL | PLMN 1 | CSG1 |

Related ICS/IXIT statements

Support of PS service 	Yes/No
UE operation mode A 	Yes/No
Switch off/on button 	Yes/No
Automatic PS attach procedure at switch on or power on 	Yes/No
User requested combined PS and non-PS detached without powering off 	Yes/No

Test procedure

The UE performs a combined PS attach procedure (for PS and non-PS services) on Cell A.

The UE performs cell reselection to Cell B and then initiates a PS detach without power off. The SS does not answer the detach procedure, but initiates a detach procedure (cause “not authorized for this CSG”). The UE shall continue with the network initiated detach procedure.

The UE shall not camp again on Cell B whose CSG ID is not in its Allowed CSG list and it camps on a suitable Cell A. The UE performs the combined PS attach procedure on Cell A.

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
|  |  |  |  | The following messages are sent and shall be received on cell A. |
| 1 | SS |  |  | Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Non-suitable cell". (See note) |
| 2 | UE |  |  | The UE is set in UE operation mode A (see ICS). The UE is powered up or switched on and initiates an attach (see ICS). |
| 3 | -> |  | ATTACH REQUEST | Attach type = 'Combined GPRS/IMSI attach' Mobile identity = IMSI TMSI status = no valid TMSI available |
| 4 | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 5 | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 6 | SS |  |  | The SS starts integrity protection. |
| 7 | <- |  | ATTACH ACCEPT | Attach result = 'Combined GPRS/IMSI attached' Allocated P-TMSI = P-TMSI-1 P-TMSI Signature = P-TMSI-1 signature MS identity = TMSI-1 Routing area identity = RAI-1 |
| 8 | -> |  | ATTACH COMPLETE |  |
|  |  |  |  | The following messages are sent and shall be received on cell B. |
| 9 | SS |  |  | Set the cell type of cell B to the "Serving cell". Set the cell type of cell A to the "Suitable neighbour cell". The SS configures power level of each Cell as follows. Cell B > Cell A |
| 10 | UE |  |  | UE reselects to Cell B |
| 11 | UE |  |  | The UE initiates a PS detach (without power off) by MMI or AT command. |
| 12 | -> |  | DETACH REQUEST | Detach type = 'normal detach, combined GPRS / IMSI detach' or ‘normal detach, GPRS detach’ |
| 12a | SS |  |  | SS starts integrity protection |
| 13 | <- |  | DETACH REQUEST | GMM cause = 'not authorized for this CSG' |
| 14 | -> |  | DETACH ACCEPT | The UE answers the network initiated detach. |
| 15 | SS |  |  | The SS initiates the RRC connection release. |
|  |  |  |  | The following message are sent and shall be received on cell A |
| 15a | UE |  |  | The UE Initiates an attach by MMI or AT command |
| 16 | -> |  | ATTACH REQUEST | Attach type = 'Combined GPRS/IMSI attach' Mobile identity = P-TMSI-1 |
| 17 | <- |  | AUTHENTICATION AND CIPHERING REQUEST |  |
| 18 | -> |  | AUTHENTICATION AND CIPHERING RESPONSE |  |
| 19 | SS |  |  | The SS starts integrity protection. |
| 20 | <- |  | ATTACH ACCEPT | Attach result = 'Combined GPRS/IMSI attached' Allocated P-TMSI = P-TMSI-1 P-TMSI Signature = P-TMSI-1 signature MS identity = TMSI-1 Routing area identity = RAI-1 |
| 21 | -> |  | ATTACH COMPLETE |  |
| 22 | SS |  |  | The SS initiates the RRC Connection release |
| 23 | UE |  |  | The UE is Switched OFF or Power is removed |
| 24 | SS |  |  | SS checks that the IE "Establishment cause" in any received RRC CONNECTION REQUEST message is set to "Detach" (message not sent if power is removed). |
| 25 | -> |  | DETACH REQUEST | Message not sent if power is removed. Detach type = 'power switched off, GPRS detach' |
| 26 | SS |  |  | The SS releases the RRC connection. If no RRC CONNECTION RELEASE COMPLETE message have been received within 1 second then the SS shall consider the UE as switched off. |
| 27 | SS |  |  | Set the cell type of cell B to the "Serving cell". Set the cell type of cell A to the "Non-Suitable neighbour cell". |
| 28 | UE |  |  | The UE is powered up or switched ON. |
| 29 |  |  |  | SS Checks that the UE does not initiate a RRC CONNECTION REQ on Cell B |
| NOTE: The definitions for "Suitable neighbour cell", "Non-suitable cell" and "Serving cell" are specified in TS 34.108 clause 6.1 in the relevant to the RAT of the cells "Reference Radio Conditions" subclauses. |  |  |  |  |

Specific message contents

None.

12.3.1.10.5	Test requirements

At step 1-12, UE Allowed CSG List shall contain only CSG 1.

At step 13, when the UE receives DETACH REQUEST, UE shall:

-	delete the CSG1 from the Allowed CSG List.

At step 14, the UE shall continue the network initiated detach procedure.

At step 16, when the UE camps on a suitable cell in the same PLMN, UE shall:

-	perform the Combined PS attach procedure.

At step 29, when the UE is switched ON, it should not send a RRC CONNECTION REQ on Cell B.