3GPP TSG-RAN WG5 Meeting #69	R5-155480

# Anaheim, CA, USA, 16th – 20th November 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0132 | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LBS-Perf: Corrections to A-BDS navigation files |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_BDS-UEConTest |  |  |  |  |  |  | Date: |  | 2015-11-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 1. The URA in the BDS Rinex files “Perf GNSS Navigation Model subtest9 01/02.rnx.txt” is in meter, while given as an index in .csv-files “Perf GNSS Navigation Model subtest9 01/02.csv”. 2. AODC and AODE have been defined, but not yet included in the files “Perf GNSS Navigation Model subtest9 01/02.csv”. 3. The IOD Value in the files“Perf GNSS Navigation Model subtest9 01/02.csv” is wrong and not conformant with its definition in the latest TS 36.355 (v12.4.0). 4. The Leap Second value in the BDS Rinex Files is wrong (“2” instead of “1”). This erroneous value may lead to an inconsistency between the Leap Second of BDS and the one of the other GNSS standard. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. URAI in the files “Perf GNSS Navigation Model subtest9 01/02.csv” changed to meters to be consistent with the corresponding Rinex files. 2. Added AODC and AODE to the files “Perf GNSS Navigation Model subtest9 01/02.csv”. 3. The IOD Value in the files “Perf GNSS Navigation Model subtest9 01/02.csv” corrected according latest TS 36.355. 4. Leap Second value in BDS Rinex Files “Perf GNSS Navigation Model subtest9 01/02.rnx.txt” corrected from “2” to “1”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and missing parameters will be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Following files within the .zip-File GNSS_Data_Perf_V11.zip - Perf GNSS Navigation Model subtest9 01.csv. - Perf GNSS Navigation Model subtest9 02.csv. - Perf GNSS Navigation Model subtest9 01.rnx.txt - Perf GNSS Navigation Model subtest9 02.rnx.txt |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Note to editor: - Please add the attached files to the zip file GNSS_Data_Perf_V11.zip (replacing the exisitng files with the same name). - Then rename it to GNSS_Data_Perf_V12.zip. - Note that other CRs might also make changes to this zip file, so care must be taken when creating the new zip file!! |  |  |  |  |  |  |  |  |