3GPP TSG-RAN5 Meeting #75	R5-172455

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4027 | rev | - | Current version: | 14.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NB-IoT test case 22.2.6. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-UEConTest |  |  |  |  |  |  | Date: |  | 2017-04-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UE may choose not to perform inter-frequency measurements according to TS 36.304 subclause 5.2.4.2a if the serving cell fulfils Srxlev > SnonIntraSearchP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the SnonIntraSearchP of test case 22.2.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be correctly implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

22.2.6	NB-IoT / Cell reselection using cell status and cell reservations / Access control class 0 to 9

22.2.6.1	Test Purpose (TP)

(1)

with { UE camped normally in state RRC_IDLE and UE fitted with a USIM with access class 0..9}

ensure that {

when { a higher ranked cell is found with cell status "barred" }

then { UE does not attempt to reselect to the higher ranked cell }

}

(2)

with { UE camped normally in state E-UTRA RRC_IDLE and UE fitted with a USIM with access class 0..9}

ensure that {

when { a higher ranked cell is found "reserved" for Operator use }

then { UE does not attempt to reselect to the higher ranked cell }

}

22.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clauses 5.2.4.4 and 5.3.1.

[TS 36.304, clause 5.2.4.4]

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in subclause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in subclause 5.2.4.5, the UE shall check if the access is restricted according to the rules in subclause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in subclause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

[TS 36.304, clause 5.3.1]

Cell status and cell reservations are indicated in the SystemInformationBlockType1 message (or SystemInformationBlockType1-NB message) [3] by means of two fields:

-	cellBarred (IE type: "barred" or "not barred") 
In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN.

When cell status is indicated as "not barred" and "not reserved" for operator use,

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN,

-	….

-	UEs assigned to an Access Class in the range of 0 to 9, 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN.

….

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-If the cell is a CSG cell:

…

-else

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-NB) message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in field cellAccessRelatedInfo in SystemInformationBlockType1 (or SystemInformationBlockType1-NB) message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell;

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

22.2.6.3	Test description

22.2.6.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency cells as specified in TS 36.508 clause 8.1.4.1.2 are configured broadcasting default NAS parameters as indicated in TS 36.508 Table 8.1.4.2-2, except that TAC values use the codes in TS 36.508 Table 8.1.4.2-6.

-	SIB 1-NB of Ncell 3 and Ncell 6 indicate cellBarred-r13 = barred.

- Each cell has only a single PLMN identity.

- All cells are high quality.

- The cell power levels are configured as shown in Table 22.2.6.3.1-1.

- System information combination 3 as defined in TS 36.508 [18] clause 8.1.4.3.1 is used in NB-IoT cells.

Table 22.2.6.3.1-1: Cell power configuration

| Parameter | Unit | Ncell 1 | Ncell 3 | Ncell 6 | Remarks |
| --- | --- | --- | --- | --- | --- |
| NRS-EPRE | dBm/15kHz | -97 | -82 | -67 | S>0 for all cells |
| Note 1: The default values (including “not present”) for all other parameters influencing cell reselection are suitable for this test. The values are defined in TS 36.508 clauses 8.1.4.3.2 and 8.1.4.3.3 |  |  |  |  |  |

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those shown in Table 22.2.6.3.1-2.

Table 22.2.6.3.1–2: USIM Configuration

| USIM field | Value |
| --- | --- |
| EFACC | Type “A” as defined in TS 34.108 clause 8.3.2.15 |

Preamble:

-	The UE is in state Registered, Idle Mode (State 3-NB) on Ncell 1 according to [18].

22.2.6.3.2	Test procedure sequence

Table 22.2.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts SIB1-NB of Ncell 3 to indicate cellBarred-r13 = notBarred | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on NB-IoT Ncell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 3 | The SS notifies the UE of change of System Information. | <-- | Paging-NB | - | - |
| 4 | SS adjusts SIB1-NB of Ncell 3 to indicate cellBarred-r13 = barred. (Ncell 3 and Ncell 6 are now both barred). | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on NB-IoT Ncell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |
| 6 | SS adjusts SIB1-NB of both Ncell 3 and Ncell 6: Ncell 3 indicates cellBarred-r13 = notBarred; Ncell 6 indicates cellBarred-r13 = notBarred and cellReservedForOperatorUse-r13 = reserved. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on NB-IoT Ncell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | - |
| 8 | The SS notifies the UE of change of System Information. | <-- | Paging-NB | - | - |
| 9 | SS adjusts SIB1-NB of Ncell 3 to indicate cellReservedForOperatorUse-r13 = reserved. | - | - | - | - |
| 10 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.5 indicate that the UE is camped on NB-IoT Ncell 1? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 2 | - |

22.2.6.3.3	Specific message contents

Table 22.2.6.3.3-0: Conditions for specific message contents
in Tables 22.2.6.3.3-1, 22.2.6.3.3-2, 22.2.6.3.3-4, 22.2.6.3.3-5 and 22.2.6.3.3-7

| Condition | Explanation |
| --- | --- |
| Ncell 3 | This condition applies to system information transmitted on Ncell 3. |
| Ncell 6 | This condition applies to system information transmitted on Ncell 6. |

Table 22.2.6.3.3-1: SystemInformationBlockType1-NB for Ncell 3 and Ncell 6 (pre-test conditions, Table 22.2.6.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| PLMN-IdentityList-NB-r13 ::= |  |  |  |
| PLMN-IdentityInfo-NB-r13 ::= SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse-r13 | notReserved |  | Ncell 3 Ncell 6 |
| } |  |  |  |
| cellBarred-r13 | barred |  | Ncell 3 Ncell 6 |
| } |  |  |  |
| } |  |  |  |

Table 22.2.6.3.3-1A: SystemInformationBlockType3-NB for Ncell 1 , Ncell 3 and Ncell 6 (pre-test conditions, Table 22.2.6.3.2-1)

| Derivation path: 36.508 table 8.1.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo-r13 SEQUENCE { |  |  |  |
| s-NonIntraSearch-r13 | 31 (62 dB) |  | Ncell 1 Ncell 3 Ncell 6 |
| } |  |  |  |
| } |  |  |  |

Table 22.2.6.3.3-2: SystemInformationBlockType1-NB for Ncell 3 and Ncell 6 (step 1, Table 22.2.6.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| PLMN-IdentityList-NB-r13 ::= |  |  |  |
| PLMN-IdentityInfo-NB-r13 ::= SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse-r13 | notReserved |  | Ncell 3 Ncell 6 |
| } |  |  |  |
| cellBarred-r13 | notBarred |  | Ncell 3 |
|  | Barred |  | Ncell 6 |
| } |  |  |  |
| } |  |  |  |

Table 22.2.6.3.3-3: Paging-NB for Ncell 3 (step 3, Table 22.2.6.3.2-1)

| Derivation Path: 36.508 clause 8.1.6.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging-NB ::= SEQUENCE { |  |  |  |
| systemInfoModification-r13 | True |  | Ncell 3 |
| } |  |  |  |

Table 22.2.6.3.3-4: SystemInformationBlockType1-NB for Ncell 3 (step 4, Table 22.2.6.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| PLMN-IdentityList-NB-r13 ::= |  |  |  |
| PLMN-IdentityInfo-NB-r13 ::= SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse-r13 | notReserved |  | Ncell 3 Ncell 6 |
| } |  |  |  |
| cellBarred-r13 | Barred |  | Ncell 3 Ncell 6 |
| } |  |  |  |
| } |  |  |  |

Table 22.2.6.3.3-5: SystemInformationBlockType1-NB for Ncell 3 and Ncell 6 (step 6, Table 22.2.6.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| PLMN-IdentityList-NB-r13 ::= |  |  |  |
| PLMN-IdentityInfo-NB-r13 ::= SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | notReserved |  | Ncell 3 |
|  | reserved |  | Ncell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Ncell 3 Ncell 6 |
| } |  |  |  |
| } |  |  |  |

Table 22.2.6.3.3-6: Paging-NB for Ncell 3 (step 8, Table 22.2.6.3.2-1)

| Derivation Path: 36.508 clause 8.1.6.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging-NB ::= SEQUENCE { |  |  |  |
| systemInfoModification-r13 | True |  | Ncell 3 |
| } |  |  |  |

Table 22.2.6.3.3-7: SystemInformationBlockType1-NB for Ncell 3 and Ncell 6 (step 9, Table 22.2.6.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| PLMN-IdentityList-NB-r13 ::= |  |  |  |
| PLMN-IdentityInfo-NB-r13 ::= SEQUENCE { | 1 entry |  |  |
| cellReservedForOperatorUse | reserved |  | Ncell 3 Ncell 6 |
| } |  |  |  |
| cellBarred | notBarred |  | Ncell 3 Ncell 6 |
| } |  |  |  |
| } |  |  |  |

PAG