3GPP TSG-RAN5 Meeting #75	R5-172540

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 0946 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update initial UE state of state 5A for DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SC_enh_dualC-UEConTest |  |  |  |  |  | Date: |  |  | 2017-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Acoording to cl. 6.4.1.2B.3 the procedure of DC Split Dedicated Bearer Establishment (to state 5B), there are two dedicated EPS bearers been established during this procedure. However, there only one default EPS bearer and one dedicated EPS bearer context(s) active as a Split DRB on the PCell and the PSCell with UL transmission of PDCP SDUs on PSCell show in Table 6.4.1.1-1 for state 5B, one dedicated EPS bearer context(s) active on the PCell is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the initial UE state of state 5A for DC by adding one more dedicated EPS bearer context(s) active on the PCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The state after procedure of DC Split Dedicated Bearer Establishment (to state 5B) will be conflict with the state shows in Table 6.4.1.1-1, and will be confused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

6.4.1	Initial UE states and setup procedures

6.4.1.1	Initial UE states and setup procedures

Figure 6.4.1.1-1: Initial UE states

In order that the UE can set up a call or session in E-UTRAN, there are a number of setup procedures to be undertaken in a hierarchical sequence to move between known states. The sequences are shown in figure 6.4.1.1-1 and the status of the relevant protocols in the UE in the different states are given in table 6.4.1.1-1. State 3 is defined in clause 4.5.1.

Table 6.4.1.1-1: Protocol state for each initial UE state

|  |  | RRC | ECM | EMM | ESM |
| --- | --- | --- | --- | --- | --- |
| State 5 | Dedicated RB established | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active N dedicated EPS bearer context(s) active |
| State 5A | DC MCG/SCG Dedicated RB established | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active on the PCell 1 dedicated EPS bearer context(s) active on the PCell 1 dedicated EPS bearer context(s) active on the PSCell |
| State 5B | DC Split Dedicated RB established | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active on the PCell 1 dedicated EPS bearer context(s) active on the PCell 1 dedicated EPS bearer context(s) active as a Split DRB on the PCell and the PSCell with UL transmission of PDCP SDUs on PSCell |
| State 6 | Loopback Activated | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active N dedicated EPS bearer context(s) active |
| State 6A | DC MCG/SCG DRB Loopback Activated | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active 1 dedicated EPS bearer context(s) active on the PCell 1 dedicated EPS bearer context(s) active on the PSCell |
| State 6B | DC Split DRB Loopback Activated | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active 1 dedicated EPS bearer context(s) active as a Split DRB on the PCell and the PSCell with UL transmission of PDCP SDUs on PSCell |