3GPP TSG-RAN5 Meeting #78	R5-180720

Athens, Greece, 26th Feb – 2nd Mar 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4425 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of eMBMS SC Rel-11 test cases 17.4.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 07-02-2018 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the Noted Discussion paper R5-177063: TTCN Handling for Service Activation using AT commands for eMBMS test cases, proposal 2 was accepted in principle, for TTCN changes to happen prose may need to be updated. Current CR propose those required changes in the Prose for handling AT command inTTCN for test case execution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MBMS service activation has been removed from the test case pre-amble UE initial condition and added in the test steps on the test case main behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A compliant UE will unfairly fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.4.2, 17.4.3, 17.4.5, 17.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |

### <Start of changed section>

### 17.4.2	Cell reselection to inter- frequency cell to start MBMS service reception

17.4.2.1	Test Purpose (TP)

(1)

with { UE in E-UTRAN RRC IDLE state on a cell broadcasting SIB15 and interested to receive a MBMS service}

ensure that {

when { SIB15 indicates that the MBMS service is available on a frequency of an inter-frequency neighbour cell }

then { UE performs cell reselection to the inter-frequency neighbour cell even if the serving cell is better and starts MBMS reception }

}

17.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1 and TS 36.331, clause 5.2.2.4. Unless otherwise stated these are Rel-11 requirements.

[TS 36.304 clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCConnectionReject unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. If the UE is capable of MBMS Service Continuity and receiving or interested to receive an MBMS service and can only receive this MBMS service while camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBMS session [2] as long as the reselected cell is broadcasting SIB13 and as long as:

-	SIB15 of the serving cell indicates for that frequency one or more MBMS SAIs included in the MBMS User Service Description (USD) [22] of this service; or

-	SIB15 is not broadcast in the serving cell and that frequency is included in the USD of this service.

If the UE is not capable of MBMS Service Continuity but has knowledge on which frequency an MBMS service of interest is provided, it may consider that frequency to be the highest priority during the MBMS session [2] as long as the reselected cell is broadcasting SIB13.

NOTE:	The UE considers that the MBMS session is ongoing using the session start and end times as provided by upper layers in the USD i.e. the UE does not verify if the session is indicated on MCCH.

…

[TS 36.331 clause 5.2.2.4]

The UE shall:

…

1>	if the UE is interested to receive MBMS services:

2>	if schedulingInfoList indicates that SystemInformationBlockType13 is present and the UE does not have stored a valid version of this system information block:

3>	acquire SystemInformationBlockType13;

2>	if the UE is capable of MBMS Service Continuity:

3>	if schedulingInfoList indicates that SystemInformationBlockType15 is present and the UE does not have stored a valid version of this system information block:

4>	acquire SystemInformationBlockType15;

…

17.4.2.3	Test description

17.4.2.3.1	Pre-test conditions

System Simulator:

-	2 E-UTRA cells with the same PLMN. Cell 1 and Cell 23 are inter-frequency cells. Cell 1 is "Serving cell" and Cell 23 is "Non-suitable cell" as defined in TS 36.508 Table 6.2.2.1-1.

- MBSFNAreaConfiguration message as defined in TS 36.508 [18] Table 4.6.1-4A is transmitted on Cell 23.

-	System information combination 3 as defined in TS 36.508[18] clause 4.4.3.1 is used in Cell 1.

-	System information combination 16 as defined in TS 36.508[18] clause 4.4.3.1 is used in Cell 23.

UE:

-	The UE is configured to receive MBMS services.

Preamble:

-	UE is in Registered, Idle mode, Test Mode Activated (State 2A) according to [18] in Cell 1(serving cell) with the UE TEST LOOP MODE C.

17.4.2.3.2	Test procedure sequence

Table 17.4.2.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble.

Table 17.4.2.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 23 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are assigned to satisfy R Cell 23 < RCell 1. |

Table 17.4.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 0A | The SS transmits a Paging message including a systemInfoModification for Cell1 and Cell 23. | <-- | Paging | - | - |  |
| 0B | From the beginning of the next modification period the SS starts broadcast of SystemInformationBlockType15 according to system information combination 18 on Cell 1 and according to system information combination 20 on Cell 23 including mbms-SAI-IntraFreq-r11 list indicating MBMS SAI=1. Note: System information combination 18 and 20 is defined in TS 36.508[18] clause 4.4.3.1. | <-- | SystemInformationBlockType15 | - | - |  |
| 0C | Wait 13 s for UE to receive modified system information (Note). | - | - | - | - |  |
| 0D | The UE is made interested in receiving a MBMS service with MBMS Service ID=0 associated with the MBMS SAI (1) broadcasted in SIB15 mbms-SAI-InterFreq list on Cell 1 and Cell 23. | - | - | - | - |  |
| 0E | The UE is made aware that the MBMS service is active. | - | - | - | - |  |
| - | EXCEPTION: the behaviour in table 17.4.2.3.2-3 runs in parallel with steps 1 to 3 below. | - | - | - | - |  |
| 1 | The UE executes the generic test procedure described in TS 36.508 subclause 6.4.2.7 and UE should camp on E-UTRA Cell 23. NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | - | - |  |
| 2 | Void | - | - | - | - |  |
| 3 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSFNAreaConfiguration message | - | - | - | - |  |
| - | EXCEPTION: In parallel to the events described in step 4,The steps described in Table 17.4.2.3.2-3 may takeplace, depending on the UE implementation. | - | - | - | - |
| 4 | The generic procedures described in TS 36.508 subclause 4.5.3A.3 and 4.5.4.3 are performed on Cell 23 activating UE test loop Mode C. | - | - | - | - |  |
| 4A | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |  |
| - | Exception; Step 5 is repeated 5 times | - | - | - | - |  |
| 5 | The SS transmits 2 MBMS Packets on the MTCH in the next MCH Scheduling Period, with MCH Scheduling Information MAC Control Element with LCID=’00001’, Stop MTCH= ‘00000000001’ in the first MAC PDU of the MCH Scheduling Period. | <-- | MBMS Packets | - | - |  |
| 6 | Void | - | - | - | - |  |
| - | EXCEPTION: In parallel to the events described in steps 6A, 7 and 8, the steps described in Table 17.4.2.3.2-3 may takeplace, depending on the UE implementation. | - | - | - | - |
| 6A | Generic test procedure Generic Radio Bearer Establishment as described in TS 36.508 subclause 4.5.3 is executed. | - | - | - | - |  |
| 7 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |  |
| 8 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |  |
| 9 | Check: Is the number of reported MBMS Packets received on the MTCH in step 8 greater than zero? (Note: This verifies that UE has selected Cell 23 providing the MBMS service and starts MBMS reception) | - | - | 1 | P |  |
| Note: Minimum delay 12,8 sec. = 2.5 * BCCH modification period (512 rf) to ensure UE detected SIB update. |  |  |  |  |  |  |

Table 17.4.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE transmits a MBMSInterestIndication message. | --> | MBMSInterestIndication | - | - |

17.4.2.3.3	Specific message contents

Table 17.4.2.3.3-1: SystemInformationBlockType2 for Cell 23 (preamble and all steps, Table 17.4.2.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-1, condition MBMS.

Table 17.4.2.3.3-2: SystemInformationBlockType15 for Cell 1 (Step 0B and later steps preamble and all steps, Table 17.4.2.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-14, condition MBMS_interFreq. |  |  |  |
| --- | --- | --- | --- |
| SystemInformationBlockType15 ::= SEQUENCE { |  |  |  |
| mbms-SAI-IntraFreq-r11 SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | Not present |  |  |
| mbms-SAI-InterFreqList-r11[1] SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  | 1 entry |  |
| dl-CarrierFreq-r11 | Downlink EARFCN for Cell 23, see table 6.3.1.2-1. |  |  |
| mbms-SAI-List-r11[1] SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | 1 | 1 entry INTEGER (0..65535) |  |

Table 17.4.2.3.3-3: SystemInformationBlockType15 for Cell 23 (Step 0B and later steps preamble and all steps, Table 17.4.2.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-14, condition MBMS_intraFreq.

Table 17.4.2.3.3-4: ACTIVATE TEST MODE (preamble)

Derivation Path: 36.508, Table 4.7A-1, condition UE TEST LOOP MODE C.

Table 17.4.2.3.3-5: CLOSE UE TEST LOOP (step 4, Table 17.4.2.3.2-2)

Derivation Path: 36.508, Table 4.7A-3, condition UE TEST LOOP MODE C

### 17.4.2a	Cell reselection to inter- band cell to start MBMS service reception

17.4.2a.1	Test Purpose (TP)

(1)

with { UE in E-UTRAN RRC IDLE state on a cell broadcasting SIB15 and interested to receive a MBMS service}

ensure that {

when { SIB15 indicates that the MBMS service is available on a frequency of a neighbour cell on a different frequency band }

then { UE performs cell reselection to the neighbour cell on the different frequency band even if the serving cell is better and starts MBMS reception }

}

17.4.2a.2	Conformance requirements

Same as test case 17.4.2.

17.4.2a.3	Test description

17.4.2a.3.1	Pre-test conditions

Same as test case 17.4.2 with the following differences:

-	Cells configuration: Cell 10 replaces Cell 23 with TA# set to TAI-2.

NOTE: 	TA# of Cell 10 shall be different from Cell 1 (TAI-1) to trigger TAU procedure in step 1 in Table 17.4.2.3.2-2.

17.4.2a.3.2	Test procedure sequence

Same as test case 17.4.2 with the following differences:

-	Cells configuration: Cell 10 replaces Cell 23

17.4.2a.3.3	Specific message contents

Same as test case 17.4.2 with the following differences:

-	Cells configuration: Cell 10 replaces Cell 23

### 17.4.3	Handover to inter-frequency cell to start MBMS service reception

17.4.3.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_Connected state AND on a cell broadcasting SIB15 and interested to receive a MBMS service }

ensure that {

when { SIB15 indicates that the MBMS service is available on a frequency of an inter-frequency neighbour cell }

then { UE transmits a MBMSInterestIndication message indicating interest in MBMS reception on the frequency }

}

(2)

with { UE in E-UTRA RRC_Connected state AND having transmitted a MBMSInterestIndication message indicating interest in MBMS reception on a frequency of an inter-frequency neighbour cell }

ensure that {

when { 1s after the UE has transmitted the MBMSInterestIndication message the UE receives RRCConnectionReconfiguration message including a mobilityControlInfo indicating a the E-UTRA frequency of the inter-frequency neighbour cell }

then { UE performs inter-frequency handover and starts MBMS reception }

}

17.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.3.5.4, 5.8.5.2, 5.8.5.3, 5.8.5.4. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331 clause 5.3.5.4]

- submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure:

….

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	if the UE has transmitted a MBMSInterestIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	the procedure ends;

[TS 36.331 clause 5.8.5.2]

An MBMS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment, upon entering or leaving the service area, upon session start or stop, upon change of interest, upon change of priority between MBMS reception and unicast reception or upon change to a PCell broadcasting SystemInformationBlockType15.

Upon initiating the procedure, the UE shall:

1>	if SystemInformationBlockType15 is broadcast by the PCell:

2>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

2>	if the UE did not transmit an MBMSInterestIndication message since last entering RRC_CONNECTED state; or

2>	if since the last time the UE transmitted an MBMSInterestIndication message, the UE connected to a PCell not broadcasting SystemInformationBlockType15:

3> if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty:

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	else:

3>	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, has changed since the last transmission of the MBMSInterestIndication message; or

3>	if the prioritisation of reception of all indicated MBMS frequencies compared to reception of any of the established unicast bearers has changed since the last transmission of the MBMSInterestIndication message:

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

NOTE:	The UE may send an MBMSInterestIndication even when it is able to receive the MBMS services it is interested in i.e. to avoid that the network allocates a configuration inhibiting MBMS reception.

[TS 36.331 clause 5.8.5.3]

The UE shall:

1>	consider a frequency to be part of the MBMS frequencies of interest if the following conditions are met:

2>	at least one MBMS session the UE is receiving or interested to receive via an MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see 3GPP TS 36.300 [9] or 3GPP TS 26.346 [57].

2>	for at least one of these MBMS sessions SystemInformationBlockType15 acquired from the PCell includes for the concerned frequency one or more MBMS SAIs as indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBMS frequencies of interest even though E-UTRAN may (temporarily) not employ an MRB for the concerned session. I.e. the UE does not verify if the session is indicated on MCCH.

2>	the UE is capable of simultaneously receiving the set of MBMS frequencies of interest, regardless of whether a serving cell is configured on each of these frequencies or not; and

2>	the supportedBandCombination the UE included in UE-EUTRA-Capability contains at least one band combination including the set of MBMS frequencies of interest;

NOTE 3:	Indicating a frequency implies that the UE supports SystemInformationBlockType13 acquisition for the concerned frequency i.e. the indication should be independent of whether a serving cell is configured on that frequency.

NOTE 4:	When evaluating which frequencies it can receive simultaneously, the UE does not take into account the serving frequencies that are currently configured i.e. it only considers MBMS frequencies it is interested to receive.

NOTE 5:	The set of MBMS frequencies of interest includes at most one frequency for a given physical frequency. The UE only considers a physical frequency to be part of the MBMS frequencies of interest if it supports at least one of the bands indicated for this physical frequency in SystemInformationBlockType1 (for serving frequency) or SystemInformationBlockType15 (for neighbouring frequencies). In this case, E-UTRAN may assume the UE supports MBMS reception on any of the bands supported by the UE (i.e. according to supportedBandCombination).

[TS 36.331 clause 5.8.5.4]

The UE shall set the contents of the MBMSInterestIndication message as follows:

1>	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty:

2>	include mbms-FreqList and set it to include the MBMS frequencies of interest, using the EARFCN corresponding with freqBandIndicator included in SystemInformationBlockType1 (for serving frequency), if applicable, and the EARFCN(s) as included in SystemInformationBlockType15 (for neighbouring frequencies);

NOTE 1:	The EARFCN included in mbms-FreqList is merely used to indicate a physical frequency the UE is interested to receive i.e. the UE may not support the band corresponding to the included EARFCN (but it does support at least one of the bands indicated in system information for the concerned physical frequency).

2>	include mbms-Priority if the UE prioritises reception of all indicated MBMS frequencies above reception of any of the unicast bearers;

NOTE 2:	If the UE prioritises MBMS reception and unicast data cannot be supported because of congestion on the MBMS carrier(s), E-UTRAN may initiate release of unicast bearers. It is up to E-UTRAN implementation whether all bearers or only GBR bearers are released. E-UTRAN does not initiate re-establishment of the released unicast bearers upon alleviation of the congestion.

The UE shall submit the MBMSInterestIndication message to lower layers for transmission.

17.4.3.3	Test description

17.4.3.3.1	Pre-test conditions

System Simulator:

-	2 E-UTRA cells with the same PLMN belonging to same MBSFN area. Cell 1 and Cell 3 are inter-frequency cells. Cell 1 is "Serving cell" and Cell 3 is "Suitable cell" as defined in TS 36.508 Table 6.2.2.1-1.

- MBSFNAreaConfiguration message as defined in TS 36.508 [18] Table 4.6.1-4A is transmitted on Cell 1 and Cell 3.

-	System information combination 16 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA Cell 1 and Cell 3.

UE:

-	The UE is configured to receive MBMS services.

Preamble:

-	UE is in state Loopback Activated (State 4) with UE TEST LOOP MODE C on Cell 1 according to [18].

17.4.3.3.2	Test procedure sequence

Table 17.4.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T0", and "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 17.4.3.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy exit condition for event A3 (M3 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 3 (M3) satisfy entry condition for event A3 (M3 > M1). |

Table 17.4.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS transmits a Paging message including a systemInfoModification for Cell1 and Cell 3. | <-- | Paging | - | - |
| 1 | From the beginning of the next modification period the SS starts broadcast of SystemInformationBlockType15 according to System information combination 20 as defined in TS 36.508[18] clause 4.4.3.1 on Cell 1 and Cell 3. SystemInformationBlockType15 on Cell 1 is including mbms-SAI-InterFreq list for the frequency of Cell 3 indicating MBMS SAI=1. SystemInformationBlockType15 on Cell 3 is including mbms-SAI-IntraFreq list indicating MBMS SAI=1. | <-- | SystemInformationBlockType15 | - | - |
| 1A | Wait 13 s for UE to receive modified system information (Note). | - | - | - | - |
| 1B | The UE is made interested in receiving a MBMS service with MBMS Service ID=0 associated with the MBMS SAI (1) broadcasted in SIB15 mbms-SAI-InterFreq list on Cell 1 and Cell 3. | - | - | - | - |
| 1C | The UE is made aware that the MBMS service is active. | - | - | - | - |
| 2 | Check: Does the UE transmit MBMSInterestIndication message. | --> | MBMSInterestIndication | 1 | P |
| 2A | The SS waits for 1s. |  |  |  |  |
| 3 | The SS transmits an RRCConnectionReconfiguration message to setup inter frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the setup of inter frequency measurement. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 1 and Cell 3 level according to the row "T1" in table 17.4.3.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message to report event A3 on Cell 1 with the measured RSRP, RSRQ value for Cell 3. | --> | MeasurementReport |  |  |
| 7 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter-frequency handover to Cell 3. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 3? | --> | RRCConnectionReconfigurationComplete | - | - |
| - | Exception; Step 9 is repeated 5 times | - | - | - | - |
| 9 | The SS transmits 2 MBMS Packets on the MTCH in the next MCH Scheduling Period, with MCH Scheduling Information MAC Control Element with LCID=’00001’, Stop MTCH= ‘00000000001’ in the first MAC PDU of the MCH Scheduling Period. | <-- | MBMS Packets | - | - |
| 10 | Void | - | - | - | - |
| 11 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 12 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 13 | Check: Is the number of reported MBMS Packets received on the MTCH in step12 greater than zero? | - | - | 2 | P |
| Note: Minimum delay 12,8 sec. = 2.5 * BCCH modification period (512 rf) to ensure UE detected SIB update. |  |  |  |  |  |

17.4.3.3.3	Specific message contents

Table 17.4.3.3.3-0: Conditions for specific message contents
in Tables 17.4.3.3.3-1B, 17.4.3.3.3-6 and 17.4.3.3.3-9

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 17.4.3.3.3-1: SystemInformationBlockType2 for Cells 1 and 3 (Preamble and all steps, Table 17.4.3.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-1, condition MBMS.

Table 17.4.3.3.3-1A: SystemInformationBlockType3 for Cells 1 and 3 (Preamble and all steps, Table 17.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| neighCellConfig | '10'B | The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell on this frequency, if configured, and of that in the PCell otherwise |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.3.3.3-1B: SystemInformationBlockType5 for Cell 1 and Cell 3 (Preamble and all steps, Table 17.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |  |
| neighCellConfig[n] | '10'B | The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell on this frequency, if configured, and of that in the PCell otherwise |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 17.4.3.3.3-2: SystemInformationBlockType15 for Cell 1 (step 1 and all subsequent steps, Table 17.4.3.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-14, condition MBMS_interFreq. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType15 ::= SEQUENCE { |  |  |  |  |
| mbms-SAI-IntraFreq-r11[1] SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | 2 | 1 entry INTEGER (0..65535) |  |  |
| mbms-SAI-InterFreqList-r11 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry | n denotes the index of the entry |  |  |
| dl-CarrierFreq-r11 | Downlink EARFCN of Cell 3, see table 6.3.1.2-1. |  |  |  |
| } |  |  |  |  |
| mbms-SAI-List-r11[1] SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | 1 | INTEGER (0..65535) |  |  |
| } |  |  |  |  |

Table 17.4.3.3.3-3: SystemInformationBlockType15 for Cell 3 (step 1 and all subsequent steps, Table 17.4.3.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-14, condition MBMS_interFreq. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType15 ::= SEQUENCE { |  |  |  |  |
| mbms-SAI-IntraFreq-r11[1] SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | 1 | 1 entry INTEGER (0..65535) |  |  |
| mbms-SAI-InterFreqList-r11 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry | n denotes the index of the entry |  |  |
| dl-CarrierFreq-r11 | Downlink EARFCN of Cell 1, see table 6.3.1.2-1. |  |  |  |
| } |  |  |  |  |
| mbms-SAI-List-r11[1] SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF { INTEGER (0..65535) } | 2 | INTEGER (0..65535) |  |  |
| } |  |  |  |  |

Table 17.4.3.3.3-4: MBMSInterestIndication (step 2, Table 17.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-4C |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| mbms-FreqList-r11[1] SEQUENCE (SIZE (1..maxFreqMBMS-r11)) OF { INTEGER (0..maxEARFCN2) } |  | Same EARFCN as used for Cell 3 | INTEGER (0..maxEARFCN2) |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 17.4.3.3.3-5: RRCConnectionReconfiguration (step 3, Table 17.4.3.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 17.4.3.3.3-6: MeasConfig (Table 17.4.3.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) | Cell 1 |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) | Cell 3 |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.3.3.3-6A: MeasObjectEUTRA-GENERIC(f1/f2) (Table 17.4.3.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| neighbourCellConfig | '10'B | The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell on this frequency, if configured, and of that in the PCell otherwise |  |
| } |  |  |  |

Table 17.4.3.3.3-7: MeasurementReport (step 6, Table 17.4.3.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.3.3.3-8: RRCConnectionReconfiguration (step 7, Table 17.4.3.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 17.4.3.3.3-9: MobilityControlInfo (Table 17.4.3.3.3-6)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 3 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 3. |  |  |
| } |  |  |  |
| } |  |  |  |

### 17.4.3a	Handover to inter-band cell to start MBMS service reception

17.4.3a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_Connected state AND on a cell broadcasting SIB15 and interested to receive a MBMS service }

ensure that {

when { SIB15 indicates that the MBMS service is available on a frequency of a neighbour cell on a different frequency band }

then { UE transmits a MBMSInterestIndication message indicating interest in MBMS reception on the frequency }

}

(2)

with { UE in E-UTRA RRC_Connected state AND having transmitted a MBMSInterestIndication message indicating interest in MBMS reception on a frequency of a neighbour cell on a different frequency band }

ensure that {

when { UE receives RRCConnectionReconfiguration message including a mobilityControlInfo indicating a the E-UTRA frequency of the inter-frequency neighbour cell }

then { UE performs inter-band handover and starts MBMS reception }

}

17.4.3a.2	Conformance requirements

Same as test case 17.4.3.

17.4.3a.3	Test description

17.4.3a.3.1	Pre-test conditions

Same as test case 17.4.3 with the following differences:

-	Cells configuration: Cell 10 replaces Cell 3

17.4.3a.3.2	Test procedure sequence

Same as test case 17.4.3 with the following differences:

-	Cells configuration: Cell 10 replaces Cell 3

17.4.3a.3.3	Specific message contents

Same as test case 17.4.3 with the following differences:

-	Cells configuration: Cell 10 replaces Cell 3

### 17.4.4	Handover to intra-frequency cell to continue MBMS service reception

17.4.4.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_Connected state AND is receiving MBMS service }

ensure that {

when { UE receives RRCConnectionReconfiguration message including a mobilityControlInfo for intra frequency neighbour cell providing MBMS service }

then { UE performs intra frequency handover and continues to receive MBMS service }

}

17.4.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.306 clause 4.3.5.2. Unless otherwise stated these are Rel-11 requirements.

[TS 36.306 clause 4.3.5.2]

This field defines the carrier aggregation, MIMO and MBMS reception capabilities supported by the UE for configurations with inter-band, intra-band non-contiguous, intra-band contiguous carrier aggregation and without carrier aggregation. For each band in a band combination the UE provides the supported CA bandwidth classes and the corresponding MIMO capabilities for downlink. The UE also has to provide the supported uplink CA bandwidth class and the corresponding MIMO capability for at least one band in the band combination. A MIMO capability applies to all carriers of a bandwidth class of a band in a band combination.

In all non-CA band combinations the UE shall indicate a bandwidth class supporting the maximum channel bandwidth defined for the band.

In all non-CA band combinations the UE shall indicate at least the number of layers for spatial multiplexing according to the UE’s Rel-8/9 category (Cat. 1-5). If the UE provides a Rel-10 category (Cat. 6-8) it shall indicate at least the number of layers according to that category for at least one band combination. In all other band combinations a UE indicating a category between 2 and 8 shall indicate support for at least 2 layers for downlink spatial multiplexing for all bands. The indicated number of layers for spatial multiplexing may exceed the number of layers required according to the category indicated by the UE. The carrier aggregation and MIMO capabilities indicated for at least one band combination shall meet the processing requirements defined by the physical layer parameter values in the UE category (i.e., maximum number of DL-SCH/UL-SCH transport block bits received/transmitted within a TTI, maximum number of bits of a DL-SCH/UL-SCH transport block received/transmitted within a TTI, and total number of soft channel bits for downlink).

The UE supporting MBMS procedures shall support MBMS reception on any serving cell and on any cell that may be additionally configured as serving cell according to this field.

17.4.4.3	Test description

17.4.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and  Cell 2 are on the same E-UTRA frequency and belongs to same MBSFN area.

- MBSFNAreaConfiguration message as defined in TS 36.508 [18] Table 4.6.1-4A is transmitted on Cell 1 and Cell 2.

-	System information combination 15 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA Cell 1 and Cell 2.

UE:

-	The UE is configured to receive MBMS services.

Preamble:

-	UE is in state Loopback Activated (State 4) with UE TEST LOOP MODE C on Cell 1 according to [18].

-	The UE is made interested in receiving a MBMS service with MBMS Service ID=0 associated with the MBMS SAI (1) broadcasted in SIB15 mbms-SAI-IntraFreq list on Cell 1 and Cell 2.

-	The UE is made aware that the MBMS service is active.

17.4.4.3.2	Test procedure sequence

Table 17.4.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T0", and "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 17.4.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |

Table 17.4.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a systemInfoModification for Cell1 and Cell 2. | <-- | Paging | - | - |
| 1A | From the beginning of the next modification period the SS starts broadcast of SystemInformationBlockType15 according to System information combination 19 as defined in TS 36.508[18] clause 4.4.3.1 on Cell 1 and Cell 2.. | <-- | SystemInformationBlockType15 | - | - |
| 1B | The UE transmits MBMSInterestIndication message. | --> | MBMSInterestIndication | - | - |
| - | Exception; Step 2 is repeated 5 times | - | - | - | - |
| 2 | The SS transmits 2 MBMS Packets on the MTCH in the next MCH Scheduling Period, with MCH Scheduling Information MAC Control Element with LCID=’00001’, Stop MTCH= ‘00000000001’ in the first MAC PDU of the MCH Scheduling Period. | <-- | MBMS Packets | - | - |
| 3 | Void | - | - | - | - |
| 4 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 5 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 6 | Check: Is the number of reported MBMS Packets received on the MTCH in step5 greater than zero? (NOTE 1) | - | - | 1 | P |
| 7 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the setup of intra frequency measurement. | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | The SS changes Cell 1 and Cell 2 level according to the row "T1" in table 17.4.3.3.2-1. | - | - | - | - |
| 10 | The UE transmits a MeasurementReport message to report event A3 on Cell 1 with the measured RSRP, RSRQ value for Cell 2. | --> | MeasurementReport |  |  |
| 11 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra-frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2? | --> | RRCConnectionReconfigurationComplete | - | - |
| - | Exception; Step 13 is repeated 5 times | - | - | - | - |
| 13 | The SS transmits 2 MBMS Packets on the MTCH in the next MCH Scheduling Period, with MCH Scheduling Information MAC Control Element with LCID=’00001’, Stop MTCH= ‘00000000001’ in the first MAC PDU of the MCH Scheduling Period. | <-- | MBMS Packets | - | - |
| 14 | Void | - | - | - | - |
| 15 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 16 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 17 | Check: Is the number of reported MBMS Packets received on the MTCH in step16 greater than the number of MBMS Packets reported in step5? (NOTE 2) | - | - | 1 | P |
| NOTE 1: This verifies that UE is receiving active MBMS reception on Cell 1 before Intra-frequency handover to Cell 2. NOTE 2: This verifies that UE has performed intra-frequency handover to Cell 2 providing the MBMS service and continue MBMS reception. |  |  |  |  |  |

17.4.4.3.3	Specific message contents

Table 17.4.4.3.3-1: SystemInformationBlockType2 for Cells 1 and 2 (Preamble and all steps, Table 17.4.4.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-1, condition MBMS.

Table 17.4.4.3.3-1A: SystemInformationBlockType3 for Cells 1 and 2 (Preamble and all steps)

| Derivation Path: 36.508, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| neighCellConfig | '10'B | The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell on this frequency, if configured, and of that in the PCell otherwise |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.4.3.3-2: SystemInformationBlockType15 for Cells 1 and 2 (step 1A and all subsequent steps, Table 17.4.4.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-14, condition MBMS_intraFreq.

Table 17.4.4.3.3-3: RRCConnectionReconfiguration (step 7, Table 17.4.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 17.4.4.3.3-4: MeasConfig (Table 17.4.4.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  | Band > 64 |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 17.4.4.3.3-4A: MeasObjectEUTRA-GENERIC(f1) (Table 17.4.4.3.3-4)

| Derivation Path: 36.508, Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| neighbourCellConfig | '10'B | The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell on this frequency, if configured, and of that in the PCell otherwise |  |
| } |  |  |  |

Table 17.4.4.3.3-5: MeasurementReport (step 10, Table 17.4.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.4.3.3-6: RRCConnectionReconfiguration (step 12, Table 17.4.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 17.4.4.3.3-7: MobilityControlInfo (Table 17.4.4.3.3-6)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

### 17.4.5	Conditional retransmission of MBMS Interest Indication after handover

17.4.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_Connected state AND SystemInformationBlockType15 have been acquired by the UE on the Pcell AND the UE has transmitted a MBMSInterestIndication message on the Pcell }

ensure that {

when { UE receives a RRCConnectionReconfiguration message including mobilityControlInfo less than 1 second after the last transmission of an MBMSInterestIndication message AND UE has completed the intra frequency handover procedure }

then { UE should re-transmit a MBMSInterestIndication message }

}

17.4.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clause 5.8.5.3, 5.3.5.4, and 5.8.5.4. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331 clause 5.8.5.3]

The UE shall:

1>	consider a frequency to be part of the MBMS frequencies of interest if the following conditions are met:

2>	at least one MBMS session the UE is receiving or interested to receive via an MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see 3GPP TS 36.300 [9] or 3GPP TS 26.346 [57].

2>	for at least one of these MBMS sessions SystemInformationBlockType15 acquired from the PCell includes for the concerned frequency one or more MBMS SAIs as indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBMS frequencies of interest even though E-UTRAN may (temporarily) not employ an MRB for the concerned session. I.e. the UE does not verify if the session is indicated on MCCH.

2>	the UE is capable of simultaneously receiving the set of MBMS frequencies of interest, regardless of whether a serving cell is configured on each of these frequencies or not; and

2>	the supportedBandCombination the UE included in UE-EUTRA-Capability contains at least one band combination including the set of MBMS frequencies of interest;

NOTE 3:	Indicating a frequency implies that the UE supports SystemInformationBlockType13 acquisition for the concerned frequency i.e. the indication should be independent of whether a serving cell is configured on that frequency.

NOTE 4:	When evaluating which frequencies it can receive simultaneously, the UE does not take into account the serving frequencies that are currently configured i.e. it only considers MBMS frequencies it is interested to receive.

[TS 36.331 clause 5.3.5.4]

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

….

1>	if MAC successfully completes the random access procedure:

….

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	if the UE has transmitted a MBMSInterestIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

[TS 36.331 clause 5.8.5.4]

The UE shall set the contents of the MBMSInterestIndication message as follows:

1>	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty:

2>	include mbms-FreqList and set it to include the MBMS frequencies of interest;

2>	include mbms-Priority if the UE prioritises reception of all indicated MBMS frequencies above reception of any of the unicast bearers;

NOTE:	If the UE prioritises MBMS reception and unicast data cannot be supported because of congestion on the MBMS carrier(s), E-UTRAN may initiate release of unicast bearers. It is up to E-UTRAN implementation whether all bearers or only GBR bearers are released. E-UTRAN does not initiate re-establishment of the released unicast bearers upon alleviation of the congestion.

The UE shall submit the MBMSInterestIndication message to lower layers for transmission.

17.4.5.3	Test description

17.4.5.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 are intra-frequency cells and belongs to same MBSFN area.

- MBSFNAreaConfiguration message as defined in TS 36.508 [18] Table 4.6.1-4A is transmitted on Cell 1 and Cell 2.

-	System information combination 19 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA Cell 1 and Cell 2.

UE:

- 	The UE is configured to receive MBMS services.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

17.4.5.3.2	Test procedure sequence

Table 17.4.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T0", and "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 17.4.5.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |

Table 17.4.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSFNAreaConfiguration message. | - | - | - | - |
| 2 | The generic procedures described in TS 36.508 sub clause 4.5.3.3 are performed on Cell 1.. | - | - | - | - |
| 3 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the setup of intra frequency measurement. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 1 and Cell 2 level according to the row "T1" in table 17.4.5.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message to report event A3 on Cell 1 with the measured RSRP, RSRQ value for Cell 2. | --> | MeasurementReport |  |  |
| 7 | The UE is made interested in receiving MBMS service with MBMS Service ID=0 associated with the MBMS SAI (1) broadcasted in SIB15 mbms-SAI-IntraFreq list on Cell 1 and Cell 2 | - | - | - | - |
| 7A | The UE is made aware that the MBMS service is active. | - | - | - | - |
| 8 | Check: Does the UE transmit MBMSInterestIndication message? | --> | MBMSInterestIndication | 1 | P |
| 9 -10 | Void | - | - |  |  |
| - | EXCEPTION: the behaviour in table 17.4.5.3.2-3 runs in parallel with step 11 below. | - | - | - | - |
| 11 | Check: Does the UE transmit MBMSInterestIndication message? | --> | MBMSInterestIndication | 1 | P |
| NOTE 1: To secure that UE receives the RRCConnectionReconfiguration message less than 1 second after the transmission of an MBMSInterestIndication message in step 8. |  |  |  |  |  |

Table 17.4.5.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionReconfiguration message message on Cell 1 to order the UE to perform intra-frequency handover to Cell 2 less than [600ms] after reception of the MBMSInterestIndication message in step 8 of Table 17.4.5.3.2-2 | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |

17.4.5.3.3	Specific message contents

Table 17.4.5.3.3-1: SystemInformationBlockType2 for Cells 1 and 2 (Preamble and all steps, Table 17.4.5.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-1, condition MBMS.

Table 17.4.5.3.3-1A: SystemInformationBlockType3 for Cells 1 and 2 (Preamble and all steps, Table 17.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| neighCellConfig | '10'B | The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell on this frequency, if configured, and of that in the PCell otherwise |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.5.3.3-2: SystemInformationBlockType15 for Cells 1 and 2 (Preamble and all steps, Table 17.4.5.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-14, condition MBMS_intraFreq.

Table 17.4.5.3.3-3: RRCConnectionReconfiguration (step 2, Table 17.4.5.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 17.4.5.3.3-4: MeasConfig (Table 17.4.5.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 17.4.5.3.3-4A: MeasObjectEUTRA-GENERIC(f1) (Table 17.4.5.3.3-4)

| Derivation Path: 36.508, Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| neighbourCellConfig | '10'B | The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell on this frequency, if configured, and of that in the PCell otherwise |  |
| } |  |  |  |

Table 17.4.5.3.3-5: MeasurementReport (step 6, Table 17.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.5.3.3-6: MBMSInterestIndication (step 8 & 11, Table 17.4.5.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-4AC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| interestIndication-r11 OF SEQUENCE { |  |  |  |
| mbms-FreqList-r11[n] SEQUENCE (SIZE (1..maxFreqMBMS-r11)) OF { INTEGER (0..maxEARFCN2) } | Same EARFCN as used for Cell 2 |  |  |
| mbms-Priority-r11 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.5.3.3-7: RRCConnectionReconfiguration (step 1, Table 17.4.5.3.2-3)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 17.4.5.3.3-8: MobilityControlInfo (Table 17.4.5.3.3-7)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

### 17.4.6	MBMS Interest Indication retransmission after returning from cell not broadcasting SIB15

17.4.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_Connected state AND is receiving MBMS service and having transmitted a MBMSInterestIndication message }

ensure that {

when { UE performs handover to a Pcell not broadcasting SystemInformationBlockType15 followed by a handover to a Pcell broadcasting SystemInformationBlockType15 }

then { UE transmits a MBMSInterestIndication message }

}

17.4.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.8.5.2, 5.8.5.3 and 5.8.5.4. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331 clause 5.8.5.3]

The UE shall:

1>	consider a frequency to be part of the MBMS frequencies of interest if the following conditions are met:

2>	at least one MBMS session the UE is receiving or interested to receive via an MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see 3GPP TS 36.300 [9] or 3GPP TS 26.346 [57].

2>	for at least one of these MBMS sessions SystemInformationBlockType15 acquired from the PCell includes for the concerned frequency one or more MBMS SAIs as indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBMS frequencies of interest even though E-UTRAN may (temporarily) not employ an MRB for the concerned session. I.e. the UE does not verify if the session is indicated on MCCH.

2>	the UE is capable of simultaneously receiving the set of MBMS frequencies of interest, regardless of whether a serving cell is configured on each of these frequencies or not; and

2>	the supportedBandCombination the UE included in UE-EUTRA-Capability contains at least one band combination including the set of MBMS frequencies of interest;

NOTE 3:	Indicating a frequency implies that the UE supports SystemInformationBlockType13 acquisition for the concerned frequency i.e. the indication should be independent of whether a serving cell is configured on that frequency.

NOTE 4:	When evaluating which frequencies it can receive simultaneously, the UE does not take into account the serving frequencies that are currently configured i.e. it only considers MBMS frequencies it is interested to receive.

[TS 36.331 clause 5.8.5.4]

The UE shall set the contents of the MBMSInterestIndication message as follows:

1>	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty:

2>	include mbms-FreqList and set it to include the MBMS frequencies of interest;

2>	include mbms-Priority if the UE prioritises reception of all indicated MBMS frequencies above reception of any of the unicast bearers;

NOTE:	If the UE prioritises MBMS reception and unicast data cannot be supported because of congestion on the MBMS carrier(s), E-UTRAN may initiate release of unicast bearers. It is up to E-UTRAN implementation whether all bearers or only GBR bearers are released. E-UTRAN does not initiate re-establishment of the released unicast bearers upon alleviation of the congestion.

The UE shall submit the MBMSInterestIndication message to lower layers for transmission.

17.4.6.3	Test description

17.4.6.3.1	Pre-test conditions

System Simulator:

-	2 E-UTRA cells with the same PLMN, Cell 1 and Cell 2 are intra-frequency cells. Cell 1 is a MBMS cell and Cell 2 is a non-MBMS cell.

- MBSFNAreaConfiguration message as defined in TS 36.508 [18] Table 4.6.1-4A is transmitted on Cell 1.

-	System information combination 15 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA Cell 1.

-	System information combination 1 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA Cell 2.

UE:

-	The UE is configured to receive MBMS services.

Preamble:

-	UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

-	The UE is made interested in receiving a MBMS service with MBMS Service ID=0 associated with the MBMS SAI (1) broadcasted in SIB15 mbms-SAI-IntraFreq list on Cell 1.

-	The UE is made aware that the MBMS service is active.

17.4.6.3.2	Test procedure sequence

Table 17.4.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T0","T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 17.4.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M1 > M2). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M1 > M2). |

Table 17.4.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS transmits a Paging message including a systemInfoModification for Cell 1. | <-- | Paging | - | - |
| 1 | From the beginning of the next modification period the SS starts broadcast of SystemInformationBlockType15 according to System information combination 19 as defined in TS 36.508[18] clause 4.4.3.1 on Cell 1 including mbms-SAI-IntraFreq-r11 list indicating MBMS SAI=1. | <-- | SystemInformationBlockType15 | - | - |
| 2 | Check: Does the UE transmit MBMSInterestIndication message. | --> | MBMSInterestIndication | 1 | P |
| 3 | The SS transmits an RRCConnectionReconfiguration message to setup intra- frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 4 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the setup of intra-frequency measurement. | --> | RRCConnectionReconfigurationComplete | - | - |
| 5 | The SS changes Cell 1 and Cell 2 level according to the row "T1" in table 17.4.6.3.2-1. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message to report event A3 on Cell 1 with the measured RSRP, RSRQ value for Cell 2. | --> | MeasurementReport |  |  |
| 7 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra-frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2 | --> | RRCConnectionReconfigurationComplete | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to setup intra frequency measurement on Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 2 to confirm the setup of intra frequency measurement. | --> | RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS changes Cell 1 and Cell 2 levels according to the row "T2" in table 17.4.6.3.2-1. | - | - | - | - |
| 12 | The UE transmits a MeasurementReport message to report event A3 on Cell 2 with the measured RSRP, RSRQ value for Cell 1. | --> | MeasurementReport | - | - |
| 13 | Void | - | - |  |  |
| 14 | Void | - | - |  |  |
| - | EXCEPTION: the behaviour in table 17.4.6.3.2-3 runs in parallel with step 15 below. | - | - | - | - |
| 15 | Check: Does the UE transmit MBMSInterestIndication message. | --> | MBMSInterestIndication | 2 | P |

Table 17.4.6.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionReconfiguration message on Cell 2 to order the UE to perform intra-frequency handover to Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |

17.4.6.3.3	Specific message contents

Table 17.4.6.3.3-1: SystemInformationBlockType2 for Cells 1 (Preamble and all steps, Table 17.4.6.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-1, condition MBMS.

Table 17.4.6.3.3-1a: SystemInformationBlockType15 for Cell 1 (Step 1 and all the subsequent steps, Table 17.4.6.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-14, condition MBMS_intraFreq.

Table 17.4.6.3.3-2: RRCConnectionReconfiguration (step 3 and 9, Table 17.4.6.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 17.4.6.3.3-3: MeasConfig (step 3 and 9, Table 17.4.4.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 17.4.6.3.3-4: MeasurementReport (step 6, Table 17.4.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.6.3.3-5: MeasurementReport (step 12, Table 17.4.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 1 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.6.3.3-6: MBMSInterestIndication (step 2 and 15, Table 17.4.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-4AC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| interestIndication-r11 OF SEQUENCE { |  |  |  |
| mbms-FreqList-r11[1] SEQUENCE (SIZE (1..maxFreqMBMS-r11)) OF { INTEGER (0..maxEARFCN2) } | EARFCN of Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 17.4.7	MBMS Interest Indication after Radio Link Failure

17.4.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED }

ensure that {

when { the UE detects a radio link failure less than 1 second after the last transmission of an MBMSInterestIndication message }

then { the UE transmits a MBMSInterestIndication message }

}

17.4.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.3.7.5, 5.8.5.3 and 5.8.5.4. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.7.5]

1>	if SystemInformationBlockType15 is broadcast by the PCell:

2>	if the UE has transmitted an MBMSInterestIndication message during the last 1 second preceding detection of radio link failure:

3>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

3>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

3>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

[TS 36.331, clause 5.8.5.3]

The UE shall:

1>	consider a frequency to be part of the MBMS frequencies of interest if the following conditions are met:

2>	at least one MBMS session the UE is receiving or interested to receive via an MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see 3GPP TS 36.300 [9] or 3GPP TS 26.346 [57].

2>	for at least one of these MBMS sessions SystemInformationBlockType15 acquired from the PCell includes for the concerned frequency one or more MBMS SAIs as indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBMS frequencies of interest even though E-UTRAN may (temporarily) not employ an MRB for the concerned session. I.e. the UE does not verify if the session is indicated on MCCH.

2>	the UE is capable of simultaneously receiving the set of MBMS frequencies of interest, regardless of whether a serving cell is configured on each of these frequencies or not; and

2>	the supportedBandCombination the UE included in UE-EUTRA-Capability contains at least one band combination including the set of MBMS frequencies of interest;

NOTE 3:	Indicating a frequency implies that the UE supports SystemInformationBlockType13 acquisition for the concerned frequency i.e. the indication should be independent of whether a serving cell is configured on that frequency.

NOTE 4:	When evaluating which frequencies it can receive simultaneously, the UE does not take into account the serving frequencies that are currently configured i.e. it only considers MBMS frequencies it is interested to receive.

NOTE 5:	Within this section, the term frequency does not indicate a physical frequency but covers the associated band(s), noting that additional bands may be indicated in SystemInformationBlockType1 (serving frequency) or SystemInformationBlockType15 (neighbouring frequencies).

[TS 36.331, clause 5.8.5.4]

The UE shall set the contents of the MBMSInterestIndication message as follows:

1>	if the set of MBMS frequencies of interest, determined in accordance with 5.8.5.3, is not empty:

2>	include mbms-FreqList and set it to include the MBMS frequencies of interest, using the EARFCN corresponding with freqBandIndicator included in SystemInformationBlockType1, if applicable, and the EARFCN(s) as included in SystemInformationBlockType15;

NOTE 1:	The mbms-FreqList merely indicates the physical frequencies the UE is interested to receive and does not imply the UE supports the associated band.

2>	include mbms-Priority if the UE prioritises reception of all indicated MBMS frequencies above reception of any of the unicast bearers;

NOTE 2:	If the UE prioritises MBMS reception and unicast data cannot be supported because of congestion on the MBMS carrier(s), E-UTRAN may initiate release of unicast bearers. It is up to E-UTRAN implementation whether all bearers or only GBR bearers are released. E-UTRAN does not initiate re-establishment of the released unicast bearers upon alleviation of the congestion.

The UE shall submit the MBMSInterestIndication message to lower layers for transmission.

17.4.7.3	Test description

17.4.7.3.1	Pre-test conditions

System Simulator:

-	2 cells on the same E-UTRA frequency and with the same PLMN.

-	The 2 cells are part of the same MBSFN area.

-	MBSFNAreaConfiguration message as defined in TS 36.508 [18] Table 4.6.1-4A is transmitted on Cell 1 and Cell 2.

-	System information combination 15 as defined in TS 36.508[18] clause 4.4.3.1 is used in Cell 1 and Cell 2.

UE:

-	The UE is configured to receive MBMS services

Preamble:

-	The UE is in state Registered, Idle Mode (state 2) on Cell 1 according to [18].

17.4.7.3.2	Test procedure sequence

Table 17.4.7.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while the row marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the text of the column "Procedure" in Table 17.4.7.3.2-2.

Table 17.4.7.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -115 |
| T1 | Cell-specific RS EPRE | dBm/15kHz | “Off” | -85 |
| Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |

Table 17.4.7.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | The SS transmits a Paging message including a systemInfoModification for Cell1 and Cell 2. | <-- | Paging | - | - |
| 0B | From the beginning of the next modification period the SS starts broadcast of SystemInformationBlockType15 according to System information combination 19 as defined in TS 36.508[18] clause 4.4.3.1 on Cell 1 and Cell 2 including mbms-SAI-IntraFreq-r11 list indicating MBMS SAI=1. | <-- | SystemInformationBlockType15 | - | - |
| 0C | Wait 13 s for UE to receive modified system information (Note). | - | - | - | - |
| 0D | The UE is made interested in receiving a MBMS service with MBMS Service ID=0 associated with the MBMS SAI (1) broadcasted in SIB15 mbms-SAI-InterFreq list on Cell 1 and Cell 2. | - | - | - | - |
| 0E | The UE is made aware that the MBMS service is active. | - | - | - | - |
| 1-1D | Steps 1 to 5 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 1E, the steps specified in Table 17.4.7.3.2-3 take place. | - | - | - | - |
| 1E | Wait 2s until UE transmits MBMSInterestIndication | - | - | - | - |
| 1F-1J | Steps 6 to 9 of the generic radio bearer establishment procedure TS 36.508, section 4.5.3.3-1 are executed. | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 17.4.7.3.2-1. | - | - | - | - |
| - | The following messages are to be observed on Cell 2 unless explicitly stated otherwise. | - | - | - | - |
| 4 | UE transmits RRCConnectionReestablishmentRequest message. | --> | RRCConnectionReestablishmentRequest | - | - |
| 5 | SS transmits RRCConnectionReestablishment message. | <-- | RRCConnectionReestablishment | - | - |
| 6 | UE transmits RRCConnectionReestablishmentComplete message. | --> | RRCConnectionReestablishmentComplete | - | - |
| - | EXCEPTION: the behaviour in table 17.4.7.3.2-4 runs in parallel with step 9 below. | - | - | - | - |
| 7 | Void | - | - | - | - |
| 8 | Void | - | - | - | - |
| 9 | Check: Does the UE transmit MBMSInterestIndication message? | --> | MBMSInterestIndication | 1 | P |
| Note: Minimum delay 12,8 sec. = 2.5 * BCCH modification period (512 rf) to ensure UE detected SIB update. |  |  |  |  |  |

Table 17.4.7.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a describes a behaviour which depends on the UE implementation | - | - | - | - |
| 1a | UE transmits a MBMSInterestIndication message. | --> | MBMSInterestIndication | - | - |

Table 17.4.7.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationtComplete | - | - |

17.4.7.3.3	Specific message contents

Table 17.4.7.3.3-1: SystemInformationBlockType2 for Cells 1 and 2 (preamble and all steps, Table 17.4.1.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-1, condition MBMS.

Table 17.4.7.3.3-1A: SystemInformationBlockType3 for Cells 1 and 2 (Preamble and all steps)

| Derivation Path: 36.508, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| neighCellConfig | '10'B | The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell on this frequency, if configured, and of that in the PCell otherwise |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.7.3.3-2: SystemInformationBlockType15 for Cells 1 and 2 (Step 0B and subsequent steps, Table 17.4.2.3.2-2)

Derivation Path: 36.508 table 4.4.3.3-14, condition MBMS_intraFreq.

Table 17.4.7.3.3-3: MBMSInterestIndication (steps 1a, Table 17.4.7.3.2-3 and 9, Table 17.4.7.3.2-2)

| Derivation Path: 36.508 Table 4.6.1-4AC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| interestIndication-r11 OF SEQUENCE { |  |  |  |
| mbms-FreqList-r11[1] SEQUENCE (SIZE (1..maxFreqMBMS-r11)) OF { INTEGER (0..maxEARFCN2) } | EARFCN of Cell 1 | 1 entry |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.7.3.3-4: SystemInformationBlockType2 for Cell 1 (preamble and all steps, Table 17.4.7.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t310 | ms200 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 17.4.7.3.3-5: RRCConnectionReconfiguration (step 1, Table 17.4.7.3.2-2; Step 8, 36.508 Table 4.5.3.3-1 and step 1, Table 17.4.7.3.2-4)

| Derivation Path: 36.508 table 4.6.1-8, condition SRB2-DRB(1, 0) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| RadioResourceConfigDedicated SEQUENCE { |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| drx-Config | Not present |  | NOT pc_FeatrGrp_5 |
| drx-Config CHOICE { |  |  | pc_FeatrGrp_5 |
| setup SEQUENCE { |  |  |  |
| onDurationTimer | Psf2 |  |  |
| drx-InactivityTimer | Psf100 |  |  |
| drx-RetransmissionTimer | sf16 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |
| sf40 | 4 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of changed section>