3GPP TSG-RAN WG5 #79 	R5-182462

# Busan, Kerea, 21st – 25th May, 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4181 | rev |  | Current version: | 15.1.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTE_B48_Reference sensitivity level correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test |  |  |  |  |  |  | Date: |  | 2018-05-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | LTE B48 Reference sensitivity level in test requirement table is incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 7.3.5-1 and Table 7.3B.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect B48 Reference sensitivity requirement in TS 36.521-1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.5 and 7.3B.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[Start of Changes]

## 7.3	Reference sensitivity level

<Unchanged section skipped>

### 7.3.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Tables 7.3.5-1 and Table 7.3.5-2.

Table 7.3.5-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 2 | -102.0 | -99.0 | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
| 3 | -101.0 | -98.0 | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
| 4 | -104.0 | -101 | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 5 | -102.5 | -99.5 | -97.3 | -94.3 |  |  | FDD |
| 6 | - | - | -99.3 | -96.3 |  |  | FDD |
| 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
| 8 | -101.5 | -98.5 | -96.3 | -93.3 |  |  | FDD |
| 9 | - | - | -98.3 | -95.3 | -93.5 | -92.3 | FDD |
| 10 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 11 | - | - | -99.3 | -96.3 |  |  | FDD |
| 12 | -101.0 | -98.0 | -96.3 | -93.3 |  |  | FDD |
| 13 |  |  | -96.3 | -93.3 |  |  | FDD |
| 14 |  | - | -96.3 | -93.3 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 | - | - | -96.3 | -93.3 |  |  | FDD |
| 18 | - | - | -99.37 | -96.37 | -94.57 | - | FDD |
| 19 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
| 20 |  |  | -96.3 | -93.3 | -90.5 | -89.3 | FDD |
| 21 |  |  | -99.3 | -96.3 | -94.5 |  | FDD |
| 22 |  |  | -96.0 | -93.0 | -91.2 | -90.0 | FDD |
| 23 | -104.0 | -101 | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 24 |  |  | -99.3 | -96.3 |  |  | FDD |
| 25 | -100.5 | -97.5 | -95.8 | -92.8 | -91.0 | -89.8 | FDD |
| 26 | -102 | -99 | -96.86 | -93.86 | -926 |  | FDD |
| 27 | -102.5 | -99.5 | -97.3 | -94.3 |  |  | FDD |
| 28 |  | -99.5 | -97.8 | -94.8 | -93.0 | -90.3 | FDD |
| 30 | - | - | -98.3 | -95.3 | - | - | FDD |
| 31 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 33 | - | - | -99,3 | -96.3 | -94.5 | -93.3 | TDD |
| 34 | - | - | -99.3 | -96.3 | -94.5 | - | TDD |
| 35 | -105.5 | -101.5 | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 36 | -105.5 | -101.5 | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 37 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 38 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 39 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 40 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
| 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| 43 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| 44 |  | [-99.5] | [-97.3] | [-94.3] | [-92.5] | [-91.3] | TDD |
| 45 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| … |  |  |  |  |  |  |  |
| 48 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| … |  |  |  |  |  |  |  |
| 65 | -103.5 | -100.5 | -99.5 | -96.5 | -94 | -92.8 | FDD |
| 66 | -103.5 | -100.5 | -98.8 | -95.8 | -94 | -92.8 | FDD |
| … |  |  |  |  |  |  |  |
| 68 |  |  | -97.8 | -94.8 | -93.0 |  | FDD |
| … |  |  |  |  |  |  |  |
| 70 |  |  | -99.3 | -96.3 | -94.5 |  | FDD |
| 71 |  |  | -96.5 | -93.5 | -91.3 | -86.8 | FDD |
| 72 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 74 | -104.08 | -101.08 | -98.88 | -95.88 | -94.08 | -92.88 | FDD |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3.5-2) Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. Note 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. Note 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 7: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. Note 8: 8 indicates that the requirement is modified by -0.5 dB when the assigned E-UTRA channel bandwidth is confined within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

<Unchanged section skipped>

## 7.3B	Reference sensitivity level for UL-MIMO

<Unchanged section skipped>

### 7.3B.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with parameters specified in Tables 7.3B.5-1 and Table 7.3B.5-2.

Table 7.3B.5-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 2 | -102.0 | -99.0 | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
| 3 | -101.0 | -98.0 | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
| 4 | -104.0 | -101.0 | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 5 | -102.5 | -99.5 | -97.3 | -94.3 |  |  | FDD |
| 6 | - | - | -99.3 | -96.3 |  |  | FDD |
| 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
| 8 | -101.5 | -98.5 | -96.3 | -93.3 |  |  | FDD |
| 9 | - | - | -98.3 | -95.3 | -93.5 | -92.3 | FDD |
| 10 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 11 | - | - | -99.3 | -96.3 |  |  | FDD |
| 12 | -101.0 | -98.0 | -96.3 | -93.3 |  |  | FDD |
| 13 |  |  | -96.3 | -93.3 |  |  | FDD |
| 14 |  | - | -96.3 | -93.3 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 | - | - | -96.3 | -93.3 |  |  | FDD |
| 18 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
| 19 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
| 20 |  |  | -96.3 | -93.3 | -90.5 | -89.3 | FDD |
| 21 |  |  | -99.3 | -96.3 | -94.5 |  | FDD |
| 22 |  |  | -96.0 | -93.0 | -91.2 | -90.0 | FDD |
| 23 | -104.0 | -101.0 | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| 24 |  |  | -99.3 | -96.3 |  |  | FDD |
| 25 | -100.5 | -97.5 | -95.8 | -92.8 | -91.0 | -89.8 | FDD |
| 26 | -102 | -99 | -96.86 | -93.86 | -926 |  | FDD |
| 27 | -102.5 | -99.5 | -97.3 | -94.3 |  |  | FDD |
| 28 |  | -99.5 | -97.8 | -94.8 | -93.0 | -90.3 | FDD |
| 30 | - | - | -98.3 | -95.3 | - | - | FDD |
| 31 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 33 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 34 | - | - | -99.3 | -96.3 | -94.5 | - | TDD |
| 35 | -105.5 | -101.5 | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 36 | -105.5 | -101.5 | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 37 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 38 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 39 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 40 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
| 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| 43 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| 45 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| … |  |  |  |  |  |  |  |
| 48 |  |  | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
| … |  |  |  |  |  |  |  |
| 65 | -103.5 | -100.5 | -99.5 | -96.5 | -94 | -92.8 | FDD |
| 66 | -103.5 | -100.5 | -98.8 | -95.8 | -94 | -92.8 | FDD |
| … |  |  |  |  |  |  |  |
| 68 |  |  | -97.8 | -94.8 | -93.0 |  | FDD |
| … |  |  |  |  |  |  |  |
| 72 | -98.3 | -95.0 | -92.8 |  |  |  | FDD |
| Note 1: The transmitter shall be set to PUMAX as defined in clause 6.2.5 Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. Note 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. Note 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. |  |  |  |  |  |  |  |

<Unchanged section skipped>

[End of Changes]