3GPP TSG-RAN WG5 #79	R5-182465

Busan, Republic of Korea, 21st – 25th May 2018

Source:	MVG Industries

Title:	Impact of Phase variation on UE antenna arrays beam pattern at mmWave

Agenda item:	5.3.14.14

Document for:	Discussion

# Introduction

During 3GPP RAN4 #85, and #86, two contributions [1-2] with simulation results for measurement distance uncertainty were presented. Specifically, a pointing error was observed when testing an UE antenna array offset with respect to the centre of the system setup at measurement distance shorter than 2D^2/lambda. Due to the fact that MU is now a RAN5 topic, this contribution summarizes the simulations performed and results obtained for the measurement uncertainty associated with the measurement distance – range length. This uncertainty is included in the MU for DFF, and NFTF setups so that a way to estimate it is required.

# Background

In [3] MU format and assessment have been defined for DFF. This setup must be capable of emulating a plane wave at FF distance. If the FF criteria is given by the 2D^2/lambda formula, where D is the whole DUT dimension, the associated measurement distances and free space path losses are reported in Table 1:

| DUT Size [m] | Freq [GHz] | FF distance [m] | System Path Loss [dB] |
| --- | --- | --- | --- |
| 0.15 | 30 | 4.5 | -75 |
| 0.15 | 40 | 6 | -80 |

Table 1. FF measurement distance and path loss

Due to the very high path loss (system dynamic range) and the need of large anechoic chambers, RAN4 has been studying a new FF criteria.

Before estimating what happens to the UE RF requirements such as, for example EIRP and EVM when those will be measured at a shorter distance than the 2D^2/lambda, it is a requirement for the measurement setup to emulate a plane wave with a certain amplitude taper and phase taper at the DUT location [4]. From [5], it is known that 22.5deg is the phase curvature of the plane wave in the QZ when elementary source is used as probe when at 2D^2/lambda (FF distance). The effect of phase variation is that the nulls of the pattern are partially filled, and the amplitudes of the side lobes are changed [5] as it is shown in Figure 1:

Figure 1. Calculated Radiation Patterns Illustrating the Effect of Quadratic Phase Errors Encountered in Measuring Patterns at the Ranges Indicated. A 30 dB Taylor Distribution Is Assumed

Anyway, how to translate this phase variation [deg] in measurement uncertainty [dB] is not defined in literature.

This can be estimated by using the so called “Transmission formula” [6] given below:

wA,χ,θ,ϕ=12smnσμνQsmn3ejmϕdμmϕejμχCσμνsn(3)kARσμνp

where wA,χ,θ,ϕ is the measured signal, Qsmn3 are the AUT (Antenna under test) Spherical Wave Coefficients (SWC), and Rσμνp are the probe SWC. This formulation allows to emulate the signal radiated by an AUT and sampled by a receiving probe all over the measurement sphere. From this formulation, the measurement uncertainty due to the measurement distance can potentially be estimated since the phase curvature in the QZ can be seen as Rσμνp while the UE antenna array pattern as Qsmn3. It shall be noted that the measurement uncertainty due to the measurement distance is seen as UE antenna dependent.

This contribution provides simulations for this approach when using two type of antennas at 30GHz. Estimation of the uncertainty on the EIRP beam pattern and peak EIRP is also provided.

# Simulation assumptions

Specifically, the following antenna types have been simulated and analysed:

- 4x1 UE antenna array ONSET -> Dir=9dBi

  - 4x1 array is aligned with the centre of the system setup

- 4x1 UE antenna array OFFSET

  - The array is offset 40mm in X and 75mm in Y

- 8x1 UE antenna array ONSET -> Dir=12dBi

  - 8x1 Array is aligned with the centre of the system setup

- 8x1 UE antenna array OFFSET

  - The array is offset 40mm in X and 75mm in Y

In figure 2, the 8x1 ONSET, and OFFSET test cases are represented:

80mm

80mm

150mm

150mm

(b)

Figure 2. 8x1 array – (a) ONSET, (b) OFFSET

It has to be noted that the two arrays have been selected based on the directivity and HPBW being representative of an UE antenna array at mmWave. The impact of directivity of the array on the pointing error is also analysed.

In all test cases the FF radiation pattern (infinite distance between Measurement Antenna and AUT) was used as a baseline for the radiation pattern comparison.

To address the impact of the phase curvature of the field in the QZ to the UE antenna beam pattern, the radiation pattern of the six antenna types were reconstructed when using a dipole, a SGH and an artificially generated QZ with 0.4dB amplitude ripple, and 7deg phase ripple to represent a generic QZ from a IFF where used as measurement antenna. For the cases where the dipole and SGH are used, the simulated range lengths were 45cm, 62.5cm, and 72.5cm while for the artificial CATR QZ, the simulated range lengths were 62.5cm and 72.5cm.

# Simulation Results

In section 4.1 through 4.4 the simulation results for the four test cases in terms of pattern comparison at 28GHz are shown.

  - 4x1 array ONSET

In Figures 3 to 6, the simulation results are shown when the range length is set to 45cm, 62.5cm, and 72.5cm for both SGH and dipole as sources. The simulations for the Simulated QZ are shown just for the case where range length is equal to 72.5cm:

- (b)

Figure 3. Pattern Comparison – 45cm range length – (a) Phi=90deg, (b) Theta=90deg

Figure 4. Zoom in Pattern Phi=90deg – 45cm range length

- (b)

Figure 6. Pattern Comparison – 72.5cm range length – (a) Phi=90deg,  (b) Theta=90deg

It can be observed that there is good agreement between the reference and the pattern measured with SGH, dipole and simulated QZ at distance 72.5cm which does correspond to 2*D^2/lambda but considering the antenna aligned with the system setup.

  - 4x1 array offset

In figure 7 through figure 10, results are shown:

Figure 7. Pattern Comparison – 45cm range length – Phi=90deg,

Figure 8. Pattern Comparison – 45cm range length – Theta=90deg,

Figure 9. Pattern Comparison – 72.5cm range length – Phi=90deg

Figure 10. Pattern Comparison – 72.5cm range length – Theta=90deg,

It can be noted that the pointing error is around 6deg at 72.5cm distance regardless the probe, either SGH or dipole. This would translate in around 0.5dB uncertainty for the peak directivity for the 4x1 array. Simulated QZ is in good agreement with reference. No pointing error is expected when using a CATR system setup.

  - 8x1 array ONSET

In figure 11 through figure 14, results are shown:

Figure 11. Pattern Comparison – 45cm range length –Phi=90deg

Figure 12. Pattern Comparison – 45cm range length –Theta=90deg

Figure 13. Pattern Comparison – 72.5cm range length –Phi=90deg

Figure 14. Pattern Comparison – 72.5cm range length –Theta=90deg

It can be observed that there is good agreement between the reference and the pattern measured with SGH, dipole and simulated QZ at distance 72.5cm which does correspond to 2*D^2/lambda but considering the antenna aligned with the system setup.

  - 8x1 array OFFSET

In figure 15 through figure 14, results are shown:

Figure 15. Pattern Comparison – 45cm range length –Phi=90deg

Figure 16. Pattern Comparison – 45cm range length –Theta=90deg

Figure 17. Pattern Comparison – 72.5cm range length –Phi=90deg

Figure 18. Pattern Comparison – 72.5cm range length –Theta=90deg

It can be noted that the pointing error is around 6deg at 72.5cm distance regardless the probe, either SGH or dipole. This would translate in around 0.5dB uncertainty for the peak directivity for the 8x1 array. Simulated QZ is in good agreement with reference. No pointing error is expected when using a CATR system setup.

# Conclusion

A numerical approach for estimating the measurement uncertainty associated with the measurement distance has been proposed in [1]. Further simulation results have been provided for two antenna types, 4x1 and 8x1 arrays when ONSET and OFFSET..

It was observed that in case of the array offset, there is pointing error of around 6deg which would translate in an uncertainty of around 0.5dB for the directivity. In case of array onset as expected there is no pointing error even at 45cm which is less than the 2*D^2/lambda.

It was observed that in case of array offset, the pointing error is 0deg for the simulated QZ of a CATR. The latter is not affected by the pointing error effect due to the phase variation.

Proposal: The impact of phase variation shall be considered in the MU uncertainty for EIRP/TRP type of measurements. Agree on the method for addressing this.

# Reference

[1] R4-1713659, “Estimating the Measurement Distance Uncertainty for the measurement baseline setup at mmWave

”, October 2017

[2] R4-1802907, “Impact of Phase variation on UE antenna arrays beam pattern at mmWave”, Feb2018

[3] R5-181836, Draft TR 38.903 v.0.1.0, April 2018

[4] 3GPP TR 38.810, v2.1.0, April 2018

[5] IEEE Std 149-1979 (Rev2008), “IEEE Standard Test Procedures for Antennas”

[6] J. E. Hansen (ed.), Spherical Near-Field Antenna Measurements, Peter Peregrinus Ltd., on behalf of IEE, London, United Kingdom, 1988