3GPP TSG RAN WG5 Meeting #79	R5-183000

Busan, Republic of Korea, 21st – 25th May 2018

Source:		Rohde & Schwarz

Title:	Discussion on test point selection for transmit modulation quality test cases in FR1

Agenda Item:		5.3.14.16

Document for:	Endorsement

# Introduction

At RAN#78, it was decided that for each NR test case a discussion of test point selection regarding frequency, channel BW, SCS, RB allocation and waveform is required and that the analysis is documented in TR 38.905.

In this contribution the selection of frequency, channel BW, SCS, RB allocation and waveform for the Error Vector Magnitude test case [1] is analyzed.

# Error Vector Magnitude

## Discussion

The Error Vector Magnitude (EVM) is a figure of merit for the transmit modulation quality and is defined in section 6.4.2.1 and Annex F Transmit Modulation of TS 38.101-1 [2].

The minimum requirement is specified as follows:

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage [] shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. For DFT-s-OFDM waveforms, the EVM result is defined after the front-end FFT and IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %. For CP-OFDM waveforms, the EVM result is defined after the front-end FFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH and is one slot for the PUCCH and PUSCH in the time domain. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the EVM measurement interval is reduced by one symbol, accordingly. The PUSCH or PUCCH EVM measurement interval is also reduced when the mean power, modulation or allocation between slots is expected to change. In the case of PUSCH transmission, the measurement interval is reduced by a time interval equal to the sum of 5 μs and the applicable exclusion period defined in subclause 6.3.3, adjacent to the boundary where the power change is expected to occur. For DFT-spread PUSCH, the exclusion period is applied to the signal obtained after the front-end IDFT. In the case of PUCCH transmission with power change, the PUCCH EVM measurement interval is reduced by one symbol adjacent to the boundary where the power change is expected to occur.

The RMS average of the basic EVM measurements for [10 sub-frames excluding any transient period for the average EVM case, and 60 sub-frames excluding any transient period for the reference signal EVM case], for the different modulations schemes shall not exceed the values specified in Table 6.4.2.1-1 for the parameters defined in Table 6.4.2.1-2. For EVM evaluation purposes, [all PRACH preamble formats 0-4 and] all PUCCH formats 1, 1a, 1b, 2, 2a and 2b are considered to have the same EVM requirement as QPSK modulated.

Table 6.4.2.1-1: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

Table 6.4.2.1-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | Table 6.3.1-1 |
| UE Output Power for 256 QAM | dBm | [TBD] |
| Operating conditions |  | Normal conditions |

## Test Environment

In LTE, EVM is tested applying normal condition as test environment. Since the EVM test methodology for NR FR1 is very similar to LTE, using the same test environment is a reasonable approach.

Proposal 1: Define test environment as Normal for EVM in FR1.

## Test Subcarrier Spacing

In this section the selection of the subcarrier spacing to be tested is analyzed. The subcarrier spacing determines the spectrum efficiency as well as the symbol duration. On the one hand, the maximum transmission bandwidth, i.e. largest spectrum efficiency, is achieved with the lowest subcarrier spacing setting a tighter requirement on the modulation performance in the frequency domain. On the other hand, a large subcarrier spacing results in short symbols which need to be well defined in the time domain to achieve a high signal quality. For a mid-valued SCS neither the largest transmission bandwidth nor the shortest symbols occur. However, in the case of lowest SCS the requirement in the time domain is most relaxed and for the highest SCS the requirement for the frequency domain is lowest. For a mid SCS, requirements for the frequency and time domain behavior are both at an intermediate level which may pose similar demands on the modulation quality. Therefore, also this case should be covered by the test.

Proposal 2: Select lowest, mid and highest SCS for EVM in FR1.

## Test Frequencies

In LTE, EVM is tested for Low range, Mid range, and High range. Due to the per band filters present in the RF frontend of FR1 NR UEs, the same test coverage as for LTE is needed to ensure that the EVM is not deteriorated by transmission properties of the filters in combination with those of the PAs.

Proposal 3: Test EVM for Low range, Mid range, and High range frequencies in FR1.

## Test Channel Bandwidth

In LTE, EVM is tested for Lowest, 5 MHz and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviors of the UE transmitter related to different frequency selectivity with respect to different channel bandwidth at first sight. However, in order to achieve test time reduction, we believe that mid channel bandwidth can be omitted as it does not pose an extreme case.

Proposal 4: Test EVM for Lowest, and Highest channel bandwidth in FR1.

## Test Modulations and waveforms

The minimum requirements are specified for Pi/2-BPSK, QPSK, 16 QAM, 64 QAM, and 256 QAM. In order to ensure that the UE meets all these requirements the test has to cover these. Due to the in differences in the modulation scheme (with and without precoding) and due to the different CREST factors for dft-s-OFDM and CP-OFDM both waveforms should be tested.

Proposal 5: Test EVM for the modulation waveform combinations stated in Table 2.6-1.

Table 2.6-1: Modulation and waveform combinations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

DFT-s-OFDM 256 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

CP-OFDM 256 QAM

## RB allocation

In LTE, EVM is measured with full allocation and the highest partial allocation where no MPR occurs. As for 64 QAM and 256 QAM always the same MPR is applied, it is sufficient to test these modulations with outer_full only. The lower modulations should be tested with outer_full and inner_full allocation to consider the different MPR since EVM depends on the power level.

Proposal 6: Test EVM with the modulation, waveform and RB allocation combinations comprised in Table 2.7-1.

Table 2.7-1: Modulation, waveform and RB combinations

| Test ID | Uplink Configuration |  |
| --- | --- | --- |
|  | Modulation | RB allocation (NOTE 1) |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 3 | DFT-s-OFDM QPSK | Inner Full |
| 4 | DFT-s-OFDM QPSK | Outer Full |
| 5 | DFT-s-OFDM 16 QAM | Inner Full |
| 6 | DFT-s-OFDM 16 QAM | Outer Full |
| 7 | DFT-s-OFDM 64 QAM | Outer Full |
| 8 | DFT-s-OFDM 256 QAM | Outer Full |
| 9 | CP-OFDM QPSK | Inner Full |
| 10 | CP-OFDM QPSK | Outer Full |
| 11 | CP-OFDM 16 QAM | Inner Full |
| 12 | CP-OFDM 16 QAM | Outer Full |
| 13 | CP-OFDM 64 QAM | Outer Full |
| 14 | CP-OFDM 256 QAM | Outer Full |

# Conclusion

To sum up, the following selection of test points is proposed for EVM in FR1:

Proposal 1: Define test environment as Normal for EVM in FR1.

Proposal 2: Select lowest, mid and highest SCS for EVM in FR1.

Proposal 3: Test EVM for Low range, Mid range, and High range frequencies in FR1.

Proposal 4: Test EVM for Lowest and Highest channel bandwidth in FR1.

Proposal 5: Test EVM for the modulation waveform combinations stated in Table 2.6-1.

Table 2.6-1: Modulation and waveform combinations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

DFT-s-OFDM 256 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

CP-OFDM 256 QAM

Proposal 6: Test EVM with the modulation, waveform and RB allocation combinations comprised in Table 2.7-1.

Table 2.7-1: Modulation, waveform and RB combinations

| Test ID | Uplink Configuration |  |
| --- | --- | --- |
|  | Modulation | RB allocation (NOTE 1) |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 3 | DFT-s-OFDM QPSK | Inner Full |
| 4 | DFT-s-OFDM QPSK | Outer Full |
| 5 | DFT-s-OFDM 16 QAM | Inner Full |
| 6 | DFT-s-OFDM 16 QAM | Outer Full |
| 7 | DFT-s-OFDM 64 QAM | Outer Full |
| 8 | DFT-s-OFDM 256 QAM | Outer Full |
| 9 | CP-OFDM QPSK | Inner Full |
| 10 | CP-OFDM QPSK | Outer Full |
| 11 | CP-OFDM 16 QAM | Inner Full |
| 12 | CP-OFDM 16 QAM | Outer Full |
| 13 | CP-OFDM 64 QAM | Outer Full |
| 14 | CP-OFDM 256 QAM | Outer Full |

# Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR1.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 3 | 2 | 3 | 14 | 189 |

# References

- R5-183922, TP for transmit modulation quality test cases, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #79, May 2018

- R4-1806026, Draft CR to TS 38.101-1: Implementation of endorsed draft CRs from RAN4 #86bis, 3GPP TSG RAN WG4, May 2018