3GPP TSG-RAN5 Meeting #80	R5-184939

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4489 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cat1bis new RF test case: Spurious emission for UE category 1bis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Austria |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UE_cat_1RX-UEConTest |  |  |  |  |  | Date: |  |  | 2018-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Cat1bis workplan identifies the need to add spurious emission test case for UE category 1bis which is currently missing in the 36.521-1 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 6.6.3EB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cat 1bis WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3EB (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 6.6.3EA.1.5	Test requirement

This clause specifies the requirements for the specified E-UTRA band for Transmitter Spurious emissions requirement with frequency range as indicated in table. 6.6.3EA.1.5-1.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.6.3EA.1.5-1.

The spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) from the edge of the channel bandwidth shown in Table 6.6.3.1.3-1.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3EA.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |

<start changes>

### 6.6.3EB	Spurious emission for UE category 1bis

#### 6.6.3EB.1	Transmitter Spurious emissions for UE category 1bis

##### 6.6.3EB.1.1	Test purpose

Same test purpose as in clause 6.6.3.1.1.

##### 6.6.3EB.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

##### 6.6.3EB.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.3.1.3.

##### 6.6.3EB.1.4	Test description

6.6.3EB.1.4.1	Initial conditions

Same initial conditions as in clause 6.6.3.1.4.1

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.6.3EB.1.4.2	Test procedure

Same test procedure as in clause 6.6.3.1.4.2

6.6.3EB.1.4.3	Message contents

Message contents same clause 6.6.3.1.4.3.

##### 6.6.3EB.1.5	Test requirement

This clause specifies the requirements for the specified E-UTRA band for Transmitter Spurious emissions requirement with frequency range as indicated in table. 6.6.3EB.1.5-1.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.6.3EB.1.5-1.

The spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) from the edge of the channel bandwidth shown in Table 6.6.3.1.3-1.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3EB.1.5-1: General spurious emissions test requirements

| Frequency Range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |

#### 6.6.3EB.2	Spurious emission band UE co-existence for UE category 1bis

##### 6.6.3EB.2.1	Test purpose

Same test purpose as in clause 6.6.3.2.1.

##### 6.6.3EB.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

##### 6.6.3EB.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.3.2.3

##### 6.6.3EB.2.4	Test description

6.6.3EB.2.4.1	Initial conditions

Same initial conditions as in clause 6.6.3.2.4.1

-	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.6.3EB.2.4.2	Test procedure

Same test procedure as in clause 6.6.3.2.4.2

6.6.3EB.2.4.3	Message contents

Message contents same as 6.6.3.2.4.3

##### 6.6.3EB.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

#### 6.6.3EB.3	Additional spurious emissions for UE category 1bis

##### 6.6.3EB.3.1	Test purpose

Same test purpose as in clause 6.6.3.3.1.

##### 6.6.3EB.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category 1bis.

##### 6.6.3EB.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.3.3.3.

##### 6.6.3EB.3.4	Test description

6.6.3EB.3.4.1	Initial conditions

Same initial conditions as in clause 6.2.3.4.1.

-	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

6.6.3EB.3.4.2	Test procedure

Same test procedure as in clause 6.2.3.4.2

6.6.3EB.3.4.3	Message contents

Same message contents as in clause 6.6.3.3.4.3.

##### 6.6.3E.3.5	Test requirement

This clause specifies the requirements for the specified E-UTRA band for an additional spectrum emission requirement with protected bands as indicated from table 6.6.3.1.5-1 to table 6.6.3.2.5-1.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.6.3E.3.5.1	Test requirement (network signalled value "NS_07")

When "NS_07" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-7 as appropriate,

and

-	the measured average power of spurious emission, derived in step 4, shall not exceed the described value in Table 6.6.3EB.3.5.1-1. These requirements also apply for the frequency ranges that are less than ΔfOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EB.3.5.1-1: Additional requirements (network signalled value "NS_07")

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz |  |
| 769 ≤ f ≤ 775 | -55.5 | 6.25 kHz |
| Note: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

<end of changes>