A-180263

Title:	LS from 5GAA for rel-15 2 RX vehicle mounted UE

Source:	5GAA WG2

To:	3GPP RAN 4

Cc:	3GPP RAN 2, 3GPP RAN 5, 3GPP RAN

Contact Person:	Visvesh Saravanan

E-mail Address:	visvesh.saravanan@harman.com

Send any reply LS to:	5GAA Liaison

5gaa-liaison@5GAA.ORG

# 1. Overall Description:

5GAA Working Group 2 (WG 2) would like to thank the 3GPP for considering usage of 2 RX antennas in vehicle mounted UE, as an exception, by starting a new Study Item [1], [2].

5GAA would like to provide collective car OEM inputs on the system impacts (coverage link budget and throughput) of 2 RX vehicle mounted NR rel-15 UE. 5GAA kindly requests 3GPP to consider following baseline (see also input in Annex) antenna gain values which actually impact the performance (and the simulations).

- Baseline antenna gain including antenna cable loss of vehicle mounted UE with 2 RX antennas: - 3 dBi to – 4 dBi.

Some automotive implementations can actually perform better than baseline values mentioned above depending on implementation mechanisms. Section 5 (appendix) contain further details.

Some companies in 5GAA observe that antenna gain values of handheld devices, considering measurements reported in section 6 (appendix), may differ from values which are commonly used in RAN4 simulations like 0 dBi.

5GAA kindly requests 3GPP to use realistic values of antenna gains offered by UEs in live network.

# 2. Actions

To 3GPP RAN 4 WG: 5GAA kindly requests 3GPP RAN4 to consider the antenna gain including antenna cable loss mentioned in section 1 & 5 of this document within the scope of the new Study Item.

# 3. Date of Next 5GAA Meetings:

| 5GAA F2F meeting #08 (WG & GA) | October 22-25, 2018 | Japan (Tokyo) |
| --- | --- | --- |
| 5GAA F2F meeting #09 (WG) | January 2019 | Korea (Seoul) |

# 4. References

- RP-181468, LS reply on minimum number of RX antennae for NR vehicular UEs

- RP-181467, New SID: Evaluation for 2 RX exception in Rel-15 vehicle mounted UE

# 5. Appendix (Antenna Gain Values): Vehicle mounted UEs

The following baseline antenna gain values are recommended by various automotive OEMs.

## 5.1 Baseline Antenna Gain Values: OEM A

The cable loss depends on the chosen package and mounting concept of the telematics unit and the antenna. OEM A has chosen a concept which allows to eliminate the cable loss completely in nearly all cars.

- The average gain of the 2x2 MIMO antenna is about -1 dBi at 2.6 GHz.

- The average loss of a coax cable is about 1 dB/m.

This is an example for better performance than baseline antenna values including cable loss, mentioned in section 1 of this document.

## 5.2 Baseline Antenna Gain Values: OEM B

The following antenna gain (dBi) values are expected minimum in the frequency ranges until 3.8GHz:

2 RX scenarios:

- Antenna 1: Average Antenna gain (theta=60°-90°, phi=0° to 359°, Car implementation on PEC) including cable loss: 1 dBi, based on a rooftop antenna implementation

- Antenna 2: Average Antenna gain (theta=60°-90°, phi=0° to 359°, Car implementation on PEC) including cable loss: 0 dBi, based on a second radiator in  rooftop antenna implementation

This is an example for better performance than baseline antenna values including cable loss, mentioned in section 1 of this document.

Scenarios with more than 2 RX antenna positions result in lower gains due to shadowing and cable losses.

- Antenna 3: Average Antenna gain (theta=60°-90°, phi=0° to 359°, Car implementation on PEC) including cable loss: -12 dBi, based on bumper antenna implementation in the front

- Antenna 4: Average Antenna gain (theta=60°-90°, phi=0° to 359°, Car implementation on PEC) including cable loss: -6 dBi, based on bumper antenna implementation in the back

## 5.3 Baseline Antenna Gain Values: OEM C

The cable loss depends on the chosen package and mounting concept of the telematics unit and the antenna.

- The average gain of the 2x2MIMO antennas is about -2/-3 dBi at 2.5 GHz.

- The average loss of a coax cable is about 1 dB/m  at 2.5 GHz

Window antenna:

- Antenna Gain: -2 dBi

- Cable length: 2 m

- Overall gain: -4 dBi

This is an example for performance which is closer to baseline antenna values including cable loss, mentioned in section 1 of this document.

# 6. Appendix (Antenna Gain Values): Commercial handset maker A

The following findings in figure 1 from a commercial mobile phone provider (HH UE) showcase the average antenna gain values (passive) for Multi-band & Multi-mode supporting Antenna in the targeted frequency band.

Fig. 1 Antenna performance of commercial HH UE at high frequency