3GPP TSG-RAN WG5 Meeting #80	R5-185067

# Gothenburg, Sweden, 20-24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0018 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GS RLC Test case 7.1.2.3.10 AM RLC / Re-transmission of RLC PDU with and without re-segmentation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections to test case 7.1.2.3.10 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated 7.1.2.3.10.3.1 pre-test conditions by removing AM SN default sizes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Pre-test conditions will not be updated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | R5-185067 is the final versio of R5-184341 |  |  |  |  |  |  |  |  |

#### <Start of changes>

#### 7.1.2.3.10	AM RLC / Re-transmission of RLC PDU with and without re-segmentation

7.1.2.3.10.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a STATUS PDU including a NACK_SN for missing AMD PDUs and missing AMD PDUs can be transmitted as indicated by lower layer at the particular transmission opportunity }

then { UE successfully retransmits missing AMD PDUs without re-segmentation }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { NACK received for missing AMD PDUs and RETX_COUNT < maxRetxThreshold }

then { UE retransmits AMD PDUs }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { AMD PDU to be retransmitted does not fit in new allocated TBS }

then { UE segments AMD PDU }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { AMD PDU segment to be retransmitted does not fit in new allocated TBS }

then { UE re-segments AMD PDU segment to fit TBS }

}

7.1.2.3.10.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.322, clauses 5.3.2, 6.2.2.5, 6.2.3.3, 6.2.3.4, 6.2.3.5, 6.2.3.7, 6.2.3.10, 6.2.3.12, 6.2.3.14 and 6.2.3.15. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.2]

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

-	if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < TX_Next:

-	consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

-	if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

-	set the RETX_COUNT associated with the RLC SDU to zero.

-	else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

-	increment the RETX_COUNT.

-	if RETX_COUNT = maxRetxThreshold:

-	indicate to upper layers that max retransmission has been reached.

When retransmitting an RLC SDU or an RLC SDU segment, the transmitting side of an AM RLC entity shall:

-	if needed, segment the RLC SDU or the RLC SDU segment;

-	form a new AMD PDU which will fit within the total size of AMD PDU(s) indicated by lower layer at the particular transmission opportunity;

-	submit the new AMD PDU to lower layer.

When forming a new AMD PDU, the transmitting side of an AM RLC entity shall:

-	only map the original RLC SDU or RLC SDU segment to the Data field of the new AMD PDU;

-	modify the header of the new AMD PDU in accordance with the description in sub clause 6.2.2.4;

-	set the P field according to sub clause 5.3.3.

[TS 38.322, clause 6.2.2.4]

AMD PDU consists of a Data field and an AMD PDU header. The AMD PDU header is byte aligned.

An AM RLC entity is configured by RRC to use either a 12 bit SN or a 18 bit SN. The length of the AMD PDU header is two and three bytes respectively.

An AMD PDU header contains a D/C, a P, a SI, and a SN. An AMD PDU header contains the SO field only when the Data field consists of an RLC SDU segment which is not the first segment, in which case a 16 bit SO is present.

Figure 6.2.2.4-1: AMD PDU with 12 bit SN (No SO)

Figure 6.2.2.4-2: AMD PDU with 18 bit SN (No SO)

Figure 6.2.2.4-3: AMD PDU with 12 bit SN with SO

Figure 6.2.2.4-4: AMD PDU with 18 bit SN with SO

[TS 38.322, clause 6.2.2.5]

STATUS PDU consists of a STATUS PDU payload and an RLC control PDU header.

RLC control PDU header consists of a D/C and a CPT field.

The STATUS PDU payload starts from the first bit following the RLC control PDU header, and it consists of one ACK_SN and one E1, zero or more sets of a NACK_SN, an E1, an E2 and an E3, and possibly a pair of a SOstart and a SOend or a NACK range field for each NACK_SN.

Figure 6.2.2.5-1: STATUS PDU with 12 bit SN

Figure 6.2.2.5-2: STATUS PDU with 18 bit SN

[TS 38.322, clause 6.2.3.3]

Length: 12 bits or 18 bits (configurable) for AMD PDU. 6 bits or 12 bits (configurable) for UMD PDU.

The SN field indicates the sequence number of the corresponding RLC SDU. For RLC AM, the sequence number is incremented by one for every RLC SDU. For RLC UM, the sequence number is incremented by one for every segmented RLC SDU.

[TS 38.322, clause 6.2.3.4]

Length: 2 bits.

The SI field indicates whether an RLC PDU contains a complete RLC SDU or the first, middle, last segment of an RLC SDU.

Table 6.2.3.4-1: SI field interpretation

| Value | Description |
| --- | --- |
| 00 | Data field contains all bytes of an RLC SDU |
| 01 | Data field contains the first segment of an RLC SDU |
| 10 | Data field contains the last segment of an RLC SDU |
| 11 | Data field contains neither the first nor last segment of an RLC SDU |

[TS 38.322, clause 6.2.3.5]

Length: 16 bits

The SO field indicates the position of the RLC SDU segment in bytes within the original RLC SDU. Specifically, the SO field indicates the position within the original RLC SDU to which the first byte of the RLC SDU segment in the Data field corresponds. The first byte of the original RLC SDU is referred by the SO field value "0000000000000000", i.e., numbering starts at zero.

[TS 38.322, clause 6.2.3.7]

Length: 1 bit.

The P field indicates whether or not the transmitting side of an AM RLC entity requests a STATUS report from its peer AM RLC entity. The interpretation of the P field is provided in Table 6.2.3.7-1.

Table 6.2.3.7-1: P field interpretation

| Value | Description |
| --- | --- |
| 0 | Status report not requested |
| 1 | Status report is requested |

[TS 38.322, clause 6.2.3.10]

Length: 12 bits or 18 bits (configurable).

The ACK_SN field indicates the SN of the next not received RLC SDU which is not reported as missing in the STATUS PDU. When the transmitting side of an AM RLC entity receives a STATUS PDU, it interprets that all RLC SDUs up to but not including the RLC SDU with SN = ACK_SN have been received by its peer AM RLC entity, excluding those RLC SDUs indicated in the STATUS PDU with NACK_SN, portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, SOstart and SOend, RLC SDUs indicated in the STATUS PDU with NACK_SN and NACK_range, and portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, NACK range, SOstart and SOend.

[TS 38.322, clause 6.2.3.12]

Length: 12 bits or 18 bits (configurable).

The NACK_SN field indicates the SN of the RLC SDU (or RLC SDU segment) that has been detected as lost at the receiving side of the AM RLC entity.

[TS 38.322, clause 6.2.3.14]

Length: 16 bits.

The SOstart field (together with the SOend field) indicates the portion of the RLC SDU with SN = NACK_SN (the NACK_SN for which the SOstart is related to) that has been detected as lost at the receiving side of the AM RLC entity. Specifically, the SOstart field indicates the position of the first byte of the portion of the RLC SDU in bytes within the original RLC SDU. The first byte of the original RLC SDU is referred by the SOstart field value "0000000000000000", i.e., numbering starts at zero.

[TS 38.322, clause 6.2.3.15]

Length: 16 bits.

When E3 is 0, the SOend field (together with the SOstart field) indicates the portion of the RLC SDU with SN = NACK_SN (the NACK_SN for which the SOend is related to) that has been detected as lost at the receiving side of the AM RLC entity. Specifically, the SOend field indicates the position of the last byte of the portion of the RLC SDU in bytes within the original RLC SDU. The first byte of the original RLC SDU is referred by the SOend field value "0000000000000000", i.e., numbering starts at zero. The special SOend value "1111111111111111" is used to indicate that the missing portion of the RLC SDU includes all bytes to the last byte of the RLC SDU.

When E3 is 1, the SOend field indicates the portion of the RLC SDU with SN = NACK_SN + NACK range - 1 that has been detected as lost at the receiving side of the AM RLC entity. Specifically, the SOend field indicates the position of the last byte of the portion of the RLC SDU in bytes within the original RLC SDU. The first byte of the original RLC SDU is referred by the SOend field value "0000000000000000", i.e., numbering starts at zero. The special SOend value "1111111111111111" is used to indicate that the missing portion of the RLC SDU includes all bytes to the last byte of the RLC SDU.

7.1.2.3.10.3	Test description

7.1.2.3.10.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception for the AM DRB is configured according to Table 7.1.2.3.10.3.1-1.

Table 7.1.2.3.10.3.1-1: PDCP-Config-DRB-AM

| Parameter | Value |
| --- | --- |
| statusReportRequired | FALSE |

Table 7.1.2.3.10.3.1-2: RLC settings

| Parameter | Value |
| --- | --- |
| t-PollRetransmit | ms150 |

7.1.2.3.10.3.2	Test procedure sequence

Table 7.1.2.3.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one AMD PDU containing SDU#1 (100 bytes) in its data field. | <-- | AMD PDU#1 | - | - |
| 2 | The UE transmits one AMD PDU containing SDU#1 in its data field. | --> | AMD PDU#1 (SN=0) | - | - |
| 3 | The SS transmits one AMD PDU containing SDU#2 (100 bytes) in its data field. | <-- | AMD PDU#2 | - | - |
| 4 | The UE transmits one AMD PDU containing SDU#2 in its data field. | --> | AMD PDU#2 (SN=1) | - | - |
| 5 | The SS transmits a RLC STATUS PDU. ACK_SN=2, NACK_SN=0. | <-- | STATUS PDU | - | - |
| 6 | Check: Does the UE transmit the AMD PDU not yet acknowledged? | --> | AMD PDU#1 (SN=0) | 1 | P |
| 7 | The SS transmits a RLC STATUS PDU. ACK_SN=2. | <-- | STATUS PDU | - | - |
| 8 | The SS transmits one AMD PDU containing SDU#3 (100 bytes) in its data field. | <-- | AMD PDU#3 | - | - |
| 9 | The UE transmits an AMD PDU containing SDU#3 in its data field. | --> | AMD PDU#3 (SN=2) | - | - |
| - | EXCEPTION: Steps 10 to 11 shall be repeated until RETX_COUNT < maxRetxThreshold. | - | - | - | - |
| 10 | The SS transmits a RLC STATUS PDU. ACK_SN =3 and NACK_SN =2. | <-- | STATUS PDU | - | - |
| 11 | Check: Does the UE retransmit the AMD PDU not yet acknowledged? | --> | AMD PDU#3 (SN=2) | 2 | P |
| 12 | The SS transmits a RLC STATUS PDU. ACK_SN =3. | <-- | STATUS PDU | - | - |
| 13 | The SS stops the UL grant transmission. | - | - | - | - |
| 14 | The SS transmits one AMD PDU containing SDU#4 (100 bytes) in its data field. | <-- | AMD PDU#4 (SN=3) | - | - |
| 15 | The SS waits for 60ms and allocates UL grant of size 872 bits. (Note 1) | <-- | (UL grant, 872 bits) | - | - |
| 16 | The UE transmits an AMD PDU with the same data contents as received in the corresponding part of SDU#4? | --> | AMD PDU#4 (SN=3) | - | - |
| 17 | The SS transmits a STATUS PDU. This PDU nacks the AMD PDU with SN=3. ACK_SN=4 and NACK_SN=3. | <-- | STATUS PDU | - | - |
| 18 | The SS waits for 20 ms and then allocates 2 UL grants of size 472 bits such that there is 20 ms gap between UL grants (Note 2, Note 5) | <-- | (UL grants, 472 bits) | - | - |
| 19 | Check: Does the UE transmit an SDU segment with SI=01 and the <x-1> data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 1 (SN=3) | 3 | P |
| 20 | Check: Does the UE transmit an SDU segment with SI=10 and SO=<x>and the same data contents at the received positions as in the original SDU#4? (Note 4) | --> | SDU#4 segment 2 (SN=3) | 3 | P |
| 21 | After 100 ms SS transmits a STATUS PDU. This PDU nacks the SDU with SN=3. NACK_SN=3, SOStart=0, SOEnd=<x-1> and ACK_SN=4 (Note 4, Note 6) | <-- | STATUS PDU | - | - |
| 22 | The SS waits for 20 ms and then allocates 2 UL grants (UL grant allocation type 2) of size 328 bits such that there is 20 ms gap between UL grants (Note 3) (Note 5) | <-- | (UL grants, 328 bits) | - | - |
| 23 | Check: Does the UE transmit an AMD PDU segment with SI=01 and the <y-1> data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 1, first part (SN=3) | 4 | P |
| 24 | Check: Does the UE transmit an AMD PDU segment with SI=10, SO=<y> and the same data contents at the received positions as in the original SDU#4? (Note 6) | --> | SDU#4 segment 1, second part (SN=3) | 4 | P |
| 25 | The SS transmits a STATUS PDU. This PDU acks the AMD PDUs with SN=3. ACK_SN=4. | <-- | STATUS PDU | - | - |
| Note 1: UL grant of 872 bits=109 bytes (index 51 of TS 38.214 [15] Table 5.1.3.2-2) is chosen to allow the UE to transmit one PDU at a time. Note 2: UL grant of 472 bits (index 38 of TS 38.214 [15] Table 5.1.3.2-2) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 472 bits=59 bytes fit an AMD PDU payload of >= 50 bytes + 3 bytes for the first segment of the AMD PDU headr OR 5 bytes for the second segment of the AMD PDU header +? bytes spare for MAC header and possible RLC STATUS PDU and BSR report. Note 3: UL grant of 328 bits (index 31 of TS 38.214 [15] Table 5.1.3.2-2) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 328 bits=41 bytes fit an AMD PDU payload of >= 25 bytes + 3 bytes for the first segment of the AMD PDU headr OR 5 bytes for the second segment of the AMD PDU header +? bytes spare for MAC header and possible RLC STATUS PDU and BSR report. Note 4: The values x and y depend upon the need of the UE to add RLC STATUS PDU and BSR report. The TBS has been chosen to ensure that the PDUs to be resegmented can be carried in 2 segments. Note 5: 20 ms gap between transmissions both in DL and UL respectively allows TTCN to tolerate one HARQ retransmission (FDD/TDD) per transport block, if such happen (TS 38.523-3 [3]). Note 6: As <x> becomes available in step 20 only the transmission in step 21 can be scheduled afterwards. This requires a 100 ms activation time. Editor’s Note: Note 5 needs to be defined in 38.523-3 which is FFS. UL grant parameters should be updated with IMCS and nPRB instead of Index value once all the parameters related TB size determination are defined in TS 38.523-3. |  |  |  |  |  |

7.1.2.3.10.3.3	Specific message contents

None.

<END of changes>