3GPP TSG-RAN5 Meeting #82	R5-19xxxx

# Athens, Greece, 25th Feb 2019 - 1th March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | yyyy | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction to demodulation tests for 4Rx CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_4Rx_AP_DL_CA-UEConTest |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Many incorrect table references |  |  |  |  |  |  |  |  |
| Summary of change: |  | Incorrect table references are corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect references remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.13.1.1.3.5, 8.13.1.1.4.5, 8.13.1.2.2.5, 8.13.1.2.3.5, 8.13.1.4.1.5, 8.13.2.1.3.5, 8.13.2.2.2.5, 8.13.2.2.3.5, 8.13.3.7.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.13	Demodulation of PDSCH CA and DC (4 receiver antenna ports)

### 8.13.1	FDD (CA and DC)

#### 8.13.1.1	Closed Loop Multi Layer Spatial Multiplexing

##### 8.13.1.1.1	Closed Loop Multi Layer Spatial Multiplexing 4x4 for CA

##### 8.13.1.1.1.1	Minimum Requirement

The purpose of these tests is to verify the closed loop rank-two performance with frequency selective precoding.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.1.1.1.1-3, based on single carrier requirement specified in Table 8.13.1.1.1.1-2, with the addition of the parameters in Table 8.13.1.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are specified in Table 8.13.1.1.1.1-4, based on single carrier requirement specified in Table 8.13.1.1.1.1-2, with the addition of the parameters in Table 8.13.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.13.1.1.1.1-5, based on single carrier requirement specified in Table 8.13.1.1.1.1-2, with the addition of the parameters in Table 8.13.1.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.13.1.1.1.1-6, based on single carrier requirement specified in Table 8.13.1.1.1.1-2, with the addition of the parameters in Table 8.13.1.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.1.1.1.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for Tests in Table 8.13.1.1.1.1-3, and with PUCCH format 3 for Tests in Table 8.13.1.1.1.1-4, Table 8.13.1.1.1.1-5 and Table 8.13.1.1.1.1-6. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.1.1.1.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 7.5 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.1 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.2 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.7 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.6 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.3 | 5.6 |

Table 8.13.1.1.1.1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.1.1.1-4: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |  |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.1.1.1.1-5: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.1.1.1-6: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.13.1.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.1.1.1.

##### 8.13.1.1.1.2	FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (2DL CA)

8.13.1.1.1.2.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on four antenna ports using closed-loop spatial multiplexing.

8.13.1.1.1.2.2	Test applicability

This test applies to E-UTRA FDD release10 and forward UE of category ≥ 5 which supports inter-band OR intra-band contiguous DL CA and 4 Rx antenna ports.

This test also applies to E-UTRA FDD release 11 and forward UE of category ≥ 5 that supports intra-band non-contiguous DL CA and 4 Rx antenna ports.

8.13.1.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.1.1.1.1.

8.13.1.1.1.2.4	Test description

8.13.1.1.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Tables 8.2.1-1, 8.13.1.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.1.1.2.4.3.

8.13.1.1.1.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents for PDSCH-ConfigDedicated-DEFAULT is defined in Table 8.13.1.1.1.2.4.3-1, PhysicalConfigDedicated-DEFAULT  is defined in Table 8.13.1.1.1.2.4.3-2, PhysicalConfigDedicatedSCell-r10-DEFAULT is defined in Table 8.13.1.1.1.2.4.3-2A, CQI-ReportConfig-r10-DEFAULT is defined in Table 8.13.1.1.1.2.4.3-4, CQI-ReportAperiodic-r10-DEFAULT is defined in Table 8.13.1.1.1.2.4.3-5, CQI-ReportConfigSCell-r10-DEFAULT is defined in Table 8.13.1.1.1.2.4.3-6, PUSCH-ConfigDedicated-DEFAULT is defined in Table 8.13.1.1.1.2.4.3-7.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Table 8.13.1.1.1.2.5-2. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.1.1.1.2.5-1 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

8.13.1.1.1.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 with the following exceptions:

Table 8.13.1.1.1.2.4.3-1: PDSCH-ConfigDedicated-DEFAULT: Additional  FDD PDSCH closed loop spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.1.1.1.2.4.3-2: PhysicalConfigDedicated-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 5.5.1.2, Table 5.5.1.2-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.1.1.1.2.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.1.1.2.4.3-3: CQI-ReportConfig-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  | RBC |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.1.2.4.3-4: CQI-ReportConfig-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.1.2.4.3-5: CQI-ReportAperiodic-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.1.1.2.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.1.2.4.3-7: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.1.1.1.2.5	Test requirement

Table 8.13.1.1.1.1-1 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.1.1.1.2.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.1.1.2.5-1: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.1.1.1.2.5-2: Test requirements (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.1.1.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.1.1.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.1.1.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.1.1.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.1.1.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.1.1.1.3	FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (3DL CA)

8.13.1.1.1.3.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on two antenna ports using closed-loop spatial multiplexing.

8.13.1.1.1.3.2	Test applicability

This test case applies to all types of E-UTRA FDD release 10 and forward UE of category ≥ 5 that support 3DL with CA configurations in Table 7.1-2a and 4 Rx antenna ports.

This test case also applies to all types of E-UTRA FDD release 11 and forward UE of category ≥ 5 that support 3DL with CA configurations in Table 7.1-2a and 4 Rx antenna ports.

8.13.1.1.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.1.1.1.1.

8.13.1.1.1.3.4	Test description

8.13.1.1.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.2.1-1 and 8.13.1.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.1.1.3.4.3.

8.13.1.1.1.3.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.1.1.1.3.4.3.

3.	The SS activates SCCs by sending the MAC-CE according to TS 36.321 [13] clauses 5.13 and 6.1.3.8. Wait for at least 2 seconds as per TS 36.133 [4] clause 8.3.3.2.

4.	The SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Table8.13.1.1.1.3.5-2. The SS sends downlink MAC padding bits on the DL RMC.

5.	The SS schedules the UL transmission to carry the PUSCH CQI feedback via PDCCH DCI format 0 with CQI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR on each of the component carriers according to Table 8.13.1.1.1.3.5-1 as appropriate.

7.	Measure the average throughput per component carrier for duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on each of the component carriers.

8.13.1.1.1.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 8.13.1.1.1.3.4.3-1: PDSCH-ConfigDedicated-DEFAULT: Additional FDD PDSCH closed loop spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.1.1.1.3.4.3-2: PhysicalConfigDedicated-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 5.5.1.2, Table 5.5.1.2-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.1.1.1.3.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.1.1.3.4.3-3: CQI-ReportConfig-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  | RBC |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.1.3.4.3-4: CQI-ReportConfig-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.1.3.4.3-5: CQI-ReportAperiodic-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11100000 | P-Cell, S-Cells report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.1.1.3.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.1.3.4.3-7: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.1.1.1.3.5	Test requirement

Table 8.13.1.1.1.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.1.1.1.3.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.1.1.3.5-1: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.1.1.1.3.5-2: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |  |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.13.1.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.1.1.1.4	FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (4DL CA)

8.13.1.1.1.4.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on two antenna ports using closed-loop spatial multiplexing.

8.13.1.1.1.4.2	Test applicability

This test case applies to all types of E-UTRA FDD release 11 and forward UE of category ≥ 8 that support 4DL with CA configurations in Table 7.1-2b or 4DL with Intra-band non-contiguous and Intra-band non-contiguous CA and 4 Rx antenna ports.

8.13.1.1.1.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.1.1.1.1.

8.13.1.1.1.4.4	Test description

8.13.1.1.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.2.1-1 and 8.13.1.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.1.1.4.4.3.

8.13.1.1.1.4.4.2	Test procedure

Same test procedure as in clause 8.13.1.1.1.2.4.2 with the following exceptions:

-	Instead of Table 8.13.1.1.1.2.5-1  use Table 8.13.1.1.1.4.5-1

8.13.1.1.1.4.4.3	Message contents

Same message contents as in clause 8.13.1.1.1.2.4.3 with the following exceptions:

Table 8.13.1.1.1.4.4.3-1: CQI-ReportAperiodic-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11110000 | P-Cell, S-Cells report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.1.1.1.4.5	Test requirement

Table 8.13.1.1.1.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.1.1.1.4.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.1.1.4.5-1: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.1.1.1.4.5-2: Test requirements (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.13.1.1.1.4.5-1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.13.1.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.1.1.1.5	FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (5DL CA)

8.13.1.1.1.5.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by th9.e SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on two antenna ports using closed-loop spatial multiplexing.

8.13.1.1.1.5.2	Test applicability

This test case applies to all types of E-UTRA FDD release 11 and forward UE of category 8, 11 or higher that support 5DL with CA configurations in Table 7.1-2c and 4 Rx antenna ports

This test case also applies to all types of E-UTRA FDD release 12 and forward UE of category 8, 11 or higher that support 5DL with CA configurations in Table 7.1-2c and 4 Rx antenna ports.

8.13.1.1.1.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.1.1.1.1.

8.13.1.1.1.5.4	Test description

8.13.1.1.1.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95

2.	The parameter settings for the cell are set up according to Table 8.2.1-1 and 8.13.1.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.1.1.5.4.3.

8.13.1.1.1.5.4.2	Test procedure

Same test procedure as in clause 8.13.1.1.1.2.4.2 with the following exceptions:

-	Instead of Table 8.13.1.1.1.2.5-1  use Table 8.13.1.1.1.5.5-1

8.13.1.1.1.5.4.3	Message contents

Same message contents as in clause 8.13.1.1.1.2.4.3 with the following exceptions:

Table 8.13.1.1.1.5.4.3-1: CQI-ReportAperiodic-r10-DEFAULT: Additional FDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11111000 | P-Cell, S-Cells report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.1.1.1.5.5	Test requirement

Table 8.13.1.1.1.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.1.1.1.5.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.1.1.5.5-1: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.1.1.1.5.5-2: Test requirements (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.1.1.1.5.5-1 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.1.1.1.5.5-1 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.13.1.1.1.5.5-1 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.13.1.1.1.5.5-1 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.13.1.1.1.5.5-1 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.13.1.1.1.5.5-1 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.13.1.1.1.5.5-1 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.1.1.2

##### 8.13.1.1.3	FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM (2DL CA)

8.13.1.1.3.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with frequency selective precoding with 256QAM under CA.

8.13.1.1.3.2	Test applicability

This test applies to E-UTRA FDD release10 and forward UE of category ≥ 5 which supports inter-band OR intra-band contiguous DL CA and 4 Rx antenna ports and 256QAM in DL.

This test also applies to E-UTRA FDD release 11 and forward UE of category ≥ 5 that supports intra-band non-contiguous DL CA and 4 Rx antenna ports and 256QAM in DL.

8.13.1.1.3.3	Minimum conformance requirements

For CA with 2 DL CCs, the requirements are specified in Table 8.13.1.1.3.3-3, based on single carrier requirement specified in Table 8.13.1.1.3.3-2, with the addition of the parameters in Table 8.13.1.1.3.3-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.1.1.3.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for Tests in Table 8.13.1.1.3-3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.1.1.3.3-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 FDD | R.72-1 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 23.4 | 19.3 |
| 10 MHz | R.72 FDD | R.72 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 18.0 |
| 15MHz | R.72-2 FDD | R.72-2 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.7 | 17.4 |
| 20MHz | R.72-3 FDD | R.72-3 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.8 | 17.5 |

Table 8.13.1.1.3.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 20MHz+15MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.1.1.3.

8.13.1.1.3.4	Test description

8.13.1.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.13.1.1.3.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.1.3.4.3.

8.13.1.1.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.1.2.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.  For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.1.1.3.3-2 and 8.13.1.1.3.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CQI feedback via PDCCH DCI format 0 with CQI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.1.1.3.5-1 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3A.5 and G.3A.6 in Annex G clause G.3.

8.13.1.1.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 and 5.5 with the following exceptions.

Table 8.13.1.1.3.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.1.1.3.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| cqi-ReportConfigPCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
| } |  |  |  |

Table 8.13.1.1.3.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| cqi-ReportConfigSCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
| } |  |  |  |

Table 8.13.1.1.3.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.3.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.3.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.1.3.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.3.4.3-7: CQI-ReportConfig-v1250-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-v1250 ::= SEQUENCE { |  |  |  |
| csi-SubframePatternConfig-r12 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| cqiCQI-ReportBoth-v1250 | Not present |  |  |
| cqi-ReportAperiodic-v1250 | Not present |  |  |
| altCQI-Table-r12 | allSubframes |  | DL 256QAM |
| } |  |  |  |

Table 8.13.1.1.3.4.3-8: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.1.1.3.5	Test requirement

Table 8.13.1.1.3.3-1 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Tables 8.13.1.1.3.5-1 and 8.13.1.1.3.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.1.3.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 FDD | R.72-1 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 24.3 | 20.2 |
| 10 MHz | R.72 FDD | R.72 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.5 | 18.9 |
| 15MHz | R.72-2 FDD | R.72-2 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.6 | 18.3 |
| 20MHz | R.72-3 FDD | R.72-3 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.7 | 18.4 |

Table 8.13.1.1.3.5-2: Test requirement (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 20MHz+15MHz | As specified in Table 8.13.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.1.1.4	FDD PDSCH Closed Loop Four-Layer Spatial Multiplexing for CA (2DL CA)

8.13.1.1.4.1	Test purpose

To verify the UE’s rank-four performance and ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for four-layer transmission on 4Tx and 4Rx under CA.

8.13.1.1.4.2	Test applicability

This test applies to all types of E-UTRA FDD release11 and forward UE of categories 5 or higher that support 4 Rx antenna ports and 4-layer spatial multiplexing for PDSCH transmission mode 4 and E-UTRA FDD 2DL CA.

8.13.1.1.4.3	Minimum conformance requirements

For CA with 2 DL CCs, the requirements are specified in Table 8.13.1.1.4.3-3, based on single carrier requirement specified in Table 8.13.1.1.4.3-2, with the addition of the parameters in Table 8.13.1.1.4.3-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.1.1.4.3-1: Test Parameters for Four-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 |
| Precoding granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 |
| Reporting interval |  | ms | 1 |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.1.1.4.3-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.1 |
| 10MHz | R.74 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.9 |
| 15MHz | R.74-2 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.3 |
| 20MHz | R.74-3 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.4 |

Table 8.13.1.1.4.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.1.4-2 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.1.1.4.

8.13.1.1.4.4	Test description

8.13.1.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.13.1.1.4.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.1.3.4.3.

8.13.1.1.4.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.1.2.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.1.1.4.3-2 and 8.13.1.1.4.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CQI feedback via PDCCH DCI format 0 with CQI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.1.1.4.5-1 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3A.5 and G.3A.6 in Annex G clause G.3.

8.13.1.1.4.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 4.6 and 5.5 with the following exceptions.

Table 8.13.1.1.4.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.1.1.4.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0xFFFF000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.1.1.4.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0xFFFF000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.1.4.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm31 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.4.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.4.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm31 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.1.4.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm31 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.1.1.4.4.3-7: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  |  |  |
| lateNonCriticalExtension | Not present |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.1.1.4.4.3-8: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.1.1.4.5	Test requirement

Table 8.13.1.1.4.3-1 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Tables 8.13.1.1.3.5-1 and 8.13.1.1.3.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.1.4.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.0 |
| 10MHz | R.74 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.8 |
| 15MHz | R.74-2 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.2 |
| 20MHz | R.74-3 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.3 |

Table 8.13.1.1.4.5-2: Test requirement (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.1.4.5-1 per CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.1.4.5-1 per CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.1.4.5-1 per CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.1.4.5-1 per CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.1.4.5-1 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.1.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)

8.13.1.2.1	Minimum Requirement

For CA with 2 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.1.2.1-3, based on single carrier requirement specified in Table 8.13.1.2.1-2, with the addition of the parameters in Table 8.13.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.1.2.1-4, based on single carrier requirement specified in Table 8.13.1.2.1-2, with the addition of the parameters in Table 8.13.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.1.2.1-5, based on single carrier requirement specified in Table 8.13.1.2.1-2, with the addition of the parameters in Table 8.13.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.1.2.1-6, based on single carrier requirement specified in Table 8.13.1.2.1-2, with the addition of the parameters in Table 8.13.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.1.2.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
|  |  |  | Reference Value in Table 8.13.1.2.1-2 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.13.1.2.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 FDD | R.51-2 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.2 | 9.0 |
| 10 MHz | R.51 FDD | R.51 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.7 | 8.8 |
| 15MHz | R.51-3 FDD | R.51-3 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.6 | 8.7 |
| 20MHz | R.51-4 FDD | R.51-4 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.3 | 8.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.1.2.1-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 20MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.2.1-4: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |  |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.1.2.1-5: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.1.2.1-6: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.1.2.1.

##### 8.13.1.2.2	FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (2DL CA)

###### 8.13.1.2.2.1	Test purpose

To verify the UE’s rank-2 performance and ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for dual-layer transmission on antenna ports 7 and 8 using DM-RS with full RB allocation with multiple CSI reference symbol configurations with non-zero and zero transmission power.

###### 8.13.1.2.2.2	Test applicability

This test applies to E-UTRA FDD release10 and forward UE of category ≥ 5 which supports inter-band OR intra-band contiguous DL CA and 4 Rx antenna ports.

This test also applies to E-UTRA FDD release 11 and forward UE of category ≥ 5 that supports intra-band non-contiguous DL CA and 4 Rx antenna ports.

###### 8.13.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.1.2.1.

###### 8.13.1.2.2.4	Test description

8.13.1.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.13.1.2.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.2.2.4.3.

8.13.1.2.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.1.2.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2C for C_RNTI to transmit the DL RMC according to Tables 8.13.1.2.1-2 and 8.13.1.2.1-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table 8.13.1.2.2.5-1 as appropriate.  BCH/CRS/PDCCH/PCFICH are sent on antenna ports 0 and 1 using two Tx antennas, while DRS/ PDSCH for the test UE are sent on antenna ports 7 and 8 using two Tx antennas with beam-forming model as specified in Annex B.4.2 and precoder update granularity specified in Table 8.3.1-1. CSI-RS are sent on antenna ports 15 and 16 using two Tx antennas with mapping according to beam-forming model as specified in Annex B.4.2.

6.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

8.13.1.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 with the following exceptions:

Table 8.13.1.2.2.4.3-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.6 Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo | Not present |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10 |  |  |
| } |  |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| } |  |  |  |

Table 8.13.1.2.2.4.3-2: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo | Not present |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10 |  |  |
| } |  |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| } |  |  |  |

Table 8.13.1.2.2.4.3-3: AntennaInfoDedicated-r10

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AntennaInfoDedicated-r10 ::= SEQUENCE { |  |  |  |  |
| transmissionMode-r10 |  | tm9-v1020 |  |  |
| codebookSubsetRestriction-r10 |  | Not present | If the UE is configured with transmissionMode tm9, E-UTRAN only configures the field codebookSubsetRestriction if PMI/RI reporting is configured |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |  |
| Release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.1.2.2.4.3-4: CSI-RS-Config-r10

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-Config-r10 ::= SEQUENCE { |  |  |  |
| csi-RS-r10 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| antennaPortsCount-r10 | an2 | Parameter represents the number of antenna ports used for transmission of CSI reference signals |  |
| resourceConfig-r10 | 8 | Parameter: CSI reference signal configuration |  |
| subframeConfig-r10 | 2 | = when CSI-RS SubframeConfig is from 0-4; Parameter: |  |
| p-C-r10 | 0 | Parameter: which is the assumed ratio of PDSCH EPRE to CSI-RS EPRE when UE derives CSI feedback |  |
| } |  |  |  |
| } |  |  |  |
| zeroTxPowerCSI-RS-r10 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| zeroTxPowerResourceConfigList-r10 | 2 | Parameter: ZeroPowerCSI-RS |  |
| zeroTxPowerSubframeConfig-r10 | 3 | Parameter: |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.13.1.2.2.5	Test requirement

Table 8.13.1.2.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.1.2.2.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.2.2.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 FDD | R.51-2 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.1 | 9.9 |
| 10 MHz | R.51 FDD | R.51 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.6 | 9.7 |
| 15MHz | R.51-3 FDD | R.51-3 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.5 | 9.6 |
| 20MHz | R.51-4 FDD | R.51-4 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.2 | 9.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.1.2.2.5-2: Test requirement (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.2.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.2.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.2.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 20MHz+5MHz | As specified in Table 8.13.1.2.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.2.2.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.1.2.3	FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (3DL CA)

###### 8.13.1.2.3.1	Test purpose

Same test purpose as 8.13.1.2.2.1

###### 8.13.1.2.3.2	Test applicability

This test case applies to all types of E-UTRA FDD release 10 and forward UE of category ≥ 5 that support 3DL with CA configurations in Table 7.1-2a and 4 Rx antenna ports

This test case also applies to all types of E-UTRA FDD release 11 and forward UE of category ≥ 5 that support 3DL with CA configurations in Table 7.1-2a and 4 Rx antenna ports.

###### 8.13.1.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.1.2.1.

###### 8.13.1.2.3.4	Test description

8.13.1.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.13.1.2.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.2.3.4.3.

8.13.1.2.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.1.2.3.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2C for C_RNTI to transmit the DL RMC according to Tables 8.13.1.2.1-2 and 8.13.1.2.1-3 on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table 8.13.1.2.3.5-1 as appropriate.  BCH/CRS/PDCCH/PCFICH are sent on antenna ports 0 and 1 using two Tx antennas, while DRS/ PDSCH for the test UE are sent on antenna ports 7 and 8 using two Tx antennas with beam-forming model as specified in Annex B.4.2 and precoder update granularity specified in Table 8.3.1-1. CSI-RS are sent on antenna ports 15 and 16 using two Tx antennas with mapping according to beam-forming model as specified in Annex B.4.2.

6.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

8.13.1.2.3.4.3	Message contents

Same message contents as 8.13.1.2.2.4.3

###### 8.13.1.2.3.5	Test requirement

Table 8.13.1.2.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.1.2.3.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.2.3.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 FDD | R.51-2 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.1 | 9.9 |
| 10 MHz | R.51 FDD | R.51 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.6 | 9.7 |
| 15MHz | R.51-3 FDD | R.51-3 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.5 | 9.6 |
| 20MHz | R.51-4 FDD | R.51-4 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.2 | 9.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.1.2.3.5-2: Test requirement (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |  |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 3 | 20MHz+20MHz+10MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 4 | 20MHz+15MHz+15MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 5 | 20MHz+15MHz+10MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 6 | 20MHz+10MHz+10MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 7 | 15MHz+15MHz+10MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 8 | 20MHz+10MHz+5MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 9 | 20MHz+15MHz+5MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 10 | 10MHz+10MHz+5MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |  |
| 11 | 5MHz+5MHz+20MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 12 | 3x10MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 13 | 5MHz+5MHz+10MHz | As specified in Table 8.13.1.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.1.2.4	FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (4DL CA)

###### 8.13.1.2.4.1	Test purpose

Same test purpose as 8.13.1.2.2.1

###### 8.13.1.2.4.2	Test applicability

This test case applies to all types of E-UTRA FDD release 11 and forward UE of category ≥ 5 that support 4DL with CA configurations in Table 7.1-2b or 4DL with Intra-band non-contiguous and Intra-band non-contiguous CA and 4 Rx antenna ports.

###### 8.13.1.2.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.1.2.1.

###### 8.13.1.2.4.4	Test description

8.13.1.2.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.13.1.2.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.2.4.4.3.

8.13.1.2.4.4.2	Test procedure

Same test procedure as in clause 8.13.1.2.3.4.2 with the following exceptions:

-	Instead of Table 8.13.1.2.3.5-1  use Table 8.13.1.2.4.5-1

8.13.1.2.4.4.3	Message contents

Same message contents as 8.13.1.2.2.4.3

###### 8.13.1.2.4.5	Test requirement

Table 8.13.1.2.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.1.2.4.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.2.4.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 FDD | R.51-2 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.1 | 9.9 |
| 10 MHz | R.51 FDD | R.51 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.6 | 9.7 |
| 15MHz | R.51-3 FDD | R.51-3 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.5 | 9.6 |
| 20MHz | R.51-4 FDD | R.51-4 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.2 | 9.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.1.2.4.5-2: Test requirement (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 10MHz+20MHz+20MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 3 | 10MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 4 | 5MHz+10MHz+20MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 5 | 5MHz+10MHz+10MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 6 | 15+3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 7 | 2x15+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 8 | 10+15+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 9 | 3x10+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 10 | 2x5+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 11 | 2x5+10+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 12 | 4x10MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.1.2.5	FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (5DL CA)

###### 8.13.1.2.5.1	Test purpose

Same test purpose as 8.13.1.2.2.1

###### 8.13.1.2.5.2	Test applicability

This test case applies to all types of E-UTRA FDD release 11 and forward UE of category 8, 11 or higher that support 5DL with CA configurations in Table 7.1-2c and 4 Rx antenna ports.

This test case also applies to all types of E-UTRA FDD release 12 and forward UE of category 8, 11 or higher that support 5DL with CA configurations in Table 7.1-2c and 4 Rx antenna ports.

###### 8.13.1.2.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.1.2.1.

###### 8.13.1.2.5.4	Test description

8.13.1.2.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.13.1.2.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.2.5.4.3.

8.13.1.2.5.4.2	Test procedure

Same test procedure as in clause 8.13.1.2.3.4.2 with the following exceptions:

-	Instead of Table 8.13.1.2.3.5-1  use Table 8.13.1.2.5.5-1

8.13.1.2.5.4.3	Message contents

Same message contents as 8.13.1.2.2.4.3

###### 8.13.1.2.5.5	Test requirement

Table 8.13.1.2.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.1.2.5.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.1.2.5.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 FDD | R.51-2 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.1 | 9.9 |
| 10 MHz | R.51 FDD | R.51 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.6 | 9.7 |
| 15MHz | R.51-3 FDD | R.51-3 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.5 | 9.6 |
| 20MHz | R.51-4 FDD | R.51-4 FDD | OP.1 FDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.2 | 9.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.1.2.5.5-2: Test requirement (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 3 | 10MHz+4x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 4 | 2x10MHz+3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 5 | 5MHz+10MHz+3x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 6 | 3x10MHz+2x20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 7 | 4x10MHz+20MHz | As specified in Table 8.13.1.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| NOTE 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.1.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing

##### 8.13.1.3.1	FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA)

8.13.1.3.1.1	Test purpose

The purpose of these tests is to verify the closed loop rank one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3.

8.13.1.3.1.2	Test applicability

This test applies to all types of E-UTRA FDD release11 and forward UE of categories 5 or higher that support 4 Rx antenna ports and enhanced receiver Type A and E-UTRA FDD 2DL CA.

8.13.1.3.1.3	Minimum conformance requirements

For CA with 2 DL CCs, the requirements are specified in Table 8.13.1.3.1.3-3, based on single carrier requirement specified in Table 8.13.1.3.1.3-2, with the addition of the parameters in Table 8.13.1.3.1.3-1 and the downlink physical channel setup according to Annex C.3.2. In Table 8.13.1.3.1.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell.

Table 8.13.1.3.1.3-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  |  | Unit |  |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  |  | dB |  | N/A |  | -1.73 |  |
| Cell Id |  |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  |  | 6 |  | 4 |  |
| Interference model |  |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  |  | PRB |  | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |  | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |  |
| PMI delay (Note 4) |  |  |  |  | ms |  | 8 |  | N/A |  |
| Reporting interval |  |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  |  | 1111 |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  |  |  | PUSCH (Note 7) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  | FDD PCC |  |  |  |  | 2 |  | N/A |  |
|  |  | FDD SCC |  |  |  |  | 3 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. Note 7: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, SF#2, SF#6 and #7 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5 SF#6 SF#0 and SF#1. |  |  |  |  |  |  |  |  |  |  |

Table 8.13.1.3.1.3-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 FDD | R.47-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -3.0 |
| 10 MHz | R.47 FDD | R.47 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 15MHz | R.47-2 FDD | R.47-2 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.9 |
| 20MHz | R.47-3 FDD | R.47-3 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.1 | -2.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.1.3.1.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.3.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.1.3.1.

8.13.1.3.1.4	Test description

8.13.1.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.94.

2.	The parameter settings for the cell 1 are set up according to Table 8.13.1.3.1.5-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.3.1.4.3.

8.13.1.3.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.1.3.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1B for C_RNTI to transmit the DL RMC according to Tables 8.13.1.3.1.5-1 and 8.13.1.1.3.5-2 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC. The SS sends uplink scheduling information via PDCCH DCI format 0 with CQI request bit set to 0 to schedule UL RMC in subframe  #0, #1, #5 and #6 (Table A.2.2.1.2-1).

5.	SS sets up parameters for interfering cells (cell 2 for PCC and SCC) according to table 8.13.1.3.1.5-1 as appropriate.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SINR according to Tables 8.13.1.3.1.5-2 and 8.13.1.3.1.5-3 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3A.5 and G.3A.6 in Annex G clause G.3.

8.13.1.3.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 and 5.5 with the following exceptions.

Table 8.13.1.3.1.4.3-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 5.5.1.2, Table 5.5.1.2-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfig | CQI-ReportConfig-DEFAULT |  |  |
| antennaInfo CHOICE { |  |  |  |
| antennaInfoDedicated ::= SEQUENCE { |  |  |  |
| transmissionMode | tm6 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n2TxAntenna-tm4 | 1111 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| Release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.1.3.1.4.3-2: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm6 |  |  |
| codebookSubsetRestriction-r10 | 1111 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.3.1.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 2 | (see Table 7.2.2-1A in TS 36.213) |  |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL | (see Table 7.2.2-1B in TS 36.213) |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.3.1.4.3-4: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 |  |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  | CQI_PERIODIC |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.1.3.1.4.3-5: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportPeriodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 3 |  |  |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.1.3.1.5	Test requirement

Table 8.13.1.3.1.5-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.1 for throughput test shall meet or exceed the specified value in Tables 8.13.1.3.1.5-2 and 8.13.1.3.1.5-3 for the specified SINR including test tolerances.

Table 8.13.1.3.1.5-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  |  | Unit |  |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  |  | dB |  | N/A |  | -1.73 |  |
| Cell Id |  |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  |  | 6 |  | 4 |  |
| Interference model |  |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  |  | PRB |  | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |  | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |  |
| PMI delay (Note 4) |  |  |  |  | ms |  | 8 |  | N/A |  |
| Reporting interval |  |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  |  | 1111 |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  |  |  | PUSCH (Note 7) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  | FDD PCC |  |  |  |  | 2 |  | N/A |  |
|  |  | FDD SCC |  |  |  |  | 3 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. Note 7: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, SF#2, SF#6 and #7 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5 SF#6 SF#0 and SF#1. |  |  |  |  |  |  |  |  |  |  |

Table 8.13.1.3.1.5-2: Test requirement Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 FDD | R.47-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8+TT | -3.0+TT |
| 10 MHz | R.47 FDD | R.47 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0+TT | -2.7+TT |
| 15MHz | R.47-2 FDD | R.47-2 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8+TT | -2.9+TT |
| 20MHz | R.47-3 FDD | R.47-3 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.1+TT | -2.8+TT |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.1.3.1.5-3: Test requirement (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.3.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.3.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.3.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.3.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.3.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.1.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.1.4.1	FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA)

8.13.1.4.1.1	Test purpose

To verify the UE’s ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SINR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for single-layer transmission on antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4.

8.13.1.4.1.2	Test applicability

This test applies to all types of E-UTRA FDD release11 and forward UE of categories 5 or higher that support 4 Rx antenna ports and enhanced receiver Type A and E-UTRA FDD 2DL CA. Applicability requires support for FGI bit 103.

8.13.1.4.1.3	Minimum conformance requirements

The requirements are specified in Table 8.13.1.4.1.3-2, with the addition of the parameters in Table 8.13.1.4.1.3-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.13.1.4.1.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

Table 8.13.1.4.1.3-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 |
|  |  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 2 | N/A |
| CSI reference signal configuration |  |  |  | 0 | N/A |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.73 |
| BWChannel |  |  | MHz | 10 | 10 |
| Cell Id |  |  |  | 0 | 126 |
| PDSCH transmission mode |  |  |  | 9 | 9 |
| Beamforming model |  |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | N/A | 70 |
|  |  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 5) |  |  | Ms | 8 | N/A |
| Reporting interval |  |  | Ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | FDD PCC |  |  | 4 | N/A |
|  | FDD SCC |  |  | 5 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3, SF#4, SF#8 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7,SF#8, SF#2 and #3. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |

Table 8.13.1.4.1.3-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A, CDM-multiplexed DM RS

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 FDD | R.76-4 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.0 |
| 10 MHz | R.76 FDD | R.76-5 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.1 | -1.8 |
| 15MHz | R.76-2 FDD | R.76-6 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -1.9 |
| 20MHz | R.76-3 FDD | R.76-7 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -1.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.1.4.1.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.4.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.1.4.1.

8.13.1.4.1.4	Test description

8.13.1.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell 1 are set up according to Table 8.13.1.4.1.5-1 as appropriate.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.4.1.4.3.

8.13.1.4.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.1.4.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2C for C_RNTI to transmit the DL RMC according to Tables 8.13.1.4.1.5-2, 8.13.1.4.1.5-3. The SS sends downlink MAC padding bits on the DL RMC. The SS sends uplink scheduling information via PDCCH DCI format 0 with CQI request bit set to 0 to schedule UL RMC in subframe #2, #3, #7 and #8 (Table A.2.2.1.1-1).

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 8.13.1.4.1.5-1 as appropriate.
BCH/CRS/PDCCH/PCFICH are sent on antenna ports 0 and 1 using two Tx antennas, while DRS/PDSCH for the test UE are sent on antenna port 7 (or 8) using two Tx antennas with beam-forming model as specified in Annex B.4.3 and precoder update granularity specified in Table 8.13.1.4.1.5-1. CSI-RS are sent on antenna ports 15 and 16 using two Tx antennas with mapping according to beam-forming model as specified in Annex B.4.3.

6.	Set the interfering cells (cell 2 for PCC and SCC) –aggressor cell– as defined in Tables 8.13.1.4.1.5-1, 8.13.1.4.1.5-2 and according to Annex B.5.4.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5A.5 and G.3.6A.6 in Annex G clause G.3.

8.13.1.4.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 and 5.5 with the following exceptions:

Table 8.13.1.4.1.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB0 |  |  |
| } |  |  |  |

Table 8.13.1.4.1.4.3-2: AntennaInfoDedicated-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AntennaInfoDedicated-r10 ::= SEQUENCE { |  |  |  |  |
| transmissionMode-r10 |  | tm9-v1020 |  |  |
| codebookSubsetRestriction-r10 |  | 001111 |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.1.4.1.4.3-3: CSI-RS-Config-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-Config-r10 ::= SEQUENCE { |  |  |  |
| csi-RS-r10 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| antennaPortsCount-r10 | an2 | Parameter represents the number of antenna ports used for transmission of CSI reference signals |  |
| resourceConfig-r10 | 0 | Parameter: CSI reference signal configuration |  |
| subframeConfig-r10 | 2 | = when CSI-RS SubframeConfig is from 0-4; = 5; Parameter: |  |
| p-C-r10 | -3 dB | Parameter: which is the assumed ratio of PDSCH EPRE to CSI-RS EPRE when UE derives CSI feedback |  |
| } |  |  |  |
| } |  |  |  |
| zeroTxPowerCSI-RS-r10 CHOICE{ |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.4.1.4.3-4: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2AC CQI-ReportPeriodic-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportPeriodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 4 | FDD |  |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ri-ConfigIndex | 483 | FDD |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.4.1.4.3-5: PhysicalConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.6 Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfig | Not present |  | RBC |
| antennaInfo | Not present |  | RBC |
| antennaInfo-r10 CHOICE{ |  |  | RBC |
| explicitValue-r10 | AntennaInfoDedicated-r10 |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10- DEFAULT using condition CQI_PERIODIC |  | RBC |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  | RBC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |

Table 8.13.1.4.1.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2AB CQI-ReportConfigSCell-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cqi-ReportPeriodicSCell-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 5 | FDD |  |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ri-ConfigIndex | 483 | FDD |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.1.4.1.4.3-7: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo | Not present |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10 |  |  |
| } |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| } |  |  |  |

Table 8.13.1.4.1.4.3-8: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportAperiodic-r10 | Not Present |  |  |
| pmi-RI-Report-r9 | setup |  |  |
| } |  |  |  |

Table 8.13.1.4.1.4.3-9: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| pmi-RI-Report-r10 | setup |  |  |
| } |  |  |  |

8.13.1.4.1.5	Test requirement

Table 8.13.1.4.1.5-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.3.1 for throughput test shall meet or exceed the specified value in Tables 8.13.1.4.1.5-2 and 8.13.1.4.1.5-3 for the specified SINR including test tolerances.

Table 8.13.1.4.1.5-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 |
|  |  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 2 | N/A |
| CSI reference signal configuration |  |  |  | 0 | N/A |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.73 |
| BWChannel |  |  | MHz | 10 | 10 |
| Cell Id |  |  |  | 0 | 126 |
| PDSCH transmission mode |  |  |  | 9 | 9 |
| Beamforming model |  |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | N/A | 70 |
|  |  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 5) |  |  | Ms | 8 | N/A |
| Reporting interval |  |  | Ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | FDD PCC |  |  | 4 | N/A |
|  | FDD SCC |  |  | 5 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#3, SF#4, SF#8 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7, SF#8, SF#2 and #3. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |

Table 8.13.1.4.1.5-2: Test requirement single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A, CDM-multiplexed DM RS

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 FDD | R.76-4 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4+TT | -2.0+TT |
| 10 MHz | R.76 FDD | R.76-5 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.1+TT | -1.8+TT |
| 15MHz | R.76-2 FDD | R.76-6 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2+TT | -1.9+TT |
| 20MHz | R.76-3 FDD | R.76-7 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4+TT | -1.7+TT |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.1.4.1.5-3: Test requirement (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x10 MHz | As specified in Table 8.13.1.4.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 2x20 MHz | As specified in Table 8.13.1.4.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 3 | 2x5 MHz | As specified in Table 8.13.1.4.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 4 | 15MHz+5MHz | As specified in Table 8.13.1.4.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 5 | 10MHz+5MHz | As specified in Table 8.13.1.4.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

### 8.13.2	TDD (CA and DC)

#### 8.13.2.1	Closed Loop Multi Layer Spatial Multiplexing

##### 8.13.2.1.1	Closed Loop Multi Layer Spatial Multiplexing 4x4 for CA

##### 8.13.2.1.1.1	Minimum Requirement

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.1.1.1-2A, based on single carrier requirement specified in Table 8.13.2.1.1.1-2, with the addition of the parameters in Table 8.13.2.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, the requirements are specified in Table 8.13.2.1.1.1-3, based on single carrier requirement specified in Table 8.13.2.1.1.1-2, with the addition of the parameters in Table 8.13.2.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, the requirements are specified in Table 8.13.2.1.1.1-4, based on single carrier requirement specified in Table 8.13.2.1.1.1-2, with the addition of the parameters in Table 8.13.2.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, the requirements are specified in Table 8.13.2.1.1.1-5, based on single carrier requirement specified in Table 8.13.2.1.1.1-2, with the addition of the parameters in Table 8.13.2.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.2.1.1.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 8 |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests with 2CCs; PUCCH format 3 for Tests with more than 2 CCs |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 4) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Multiple CC-s under test are configured as the 1st set of serving cells by high layers. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.2.1.1.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 8.1 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.8 | 5.4 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.0 | 5.9 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.5 | 6.3 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.6 | 6.2 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.4 |

Table 8.13.2.1.1.1-2A: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.2.1.1.1-3: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.2.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.2.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.2.1.1.1-4: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.2.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.13.2.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.2.1.1.1-5: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.2.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.2.1.1.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6 |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.2.1.1.

##### 8.13.2.1.1.2	TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for CA (2DL CA)

8.13.2.1.1.2.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on four antenna ports using closed-loop spatial multiplexing.

8.13.2.1.1.2.2	Test applicability

E-UTRA TDD release 10 and forward UE of category 5 or higher that supports inter-band OR intra-band contiguous DL CA and 4 Rx antenna ports.

This test also applies to E-UTRA TDD release 11 and forward UE of category 5 or higher that supports intra-band non-contiguous DL CA and 4 Rx antenna ports.

8.13.2.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.2.1.1.1.

8.13.2.1.1.2.4	Test description

8.13.2.1.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Tables 8.2.2 -1, 8.13.2.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.1.1.2.4.3.

8.13.2.1.1.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents for PDSCH-ConfigDedicated-DEFAULT is defined in Table 8.13.2.1.1.2.4.3-1, PhysicalConfigDedicatedSCell-r10-DEFAULT is defined in Table 8.13.2.1.1.2.4.3-2A , PhysicalConfigDedicated-DEFAULT  is defined in Table 8.13.2.1.1.2.4.3-2, CQI-ReportConfigSCell-DEFAULT is defined in Table 8.13.2.1.1.2.4.3-4, CQI-ReportAperiodic-r10-DEFAULT is defined in Table 8.13.2.1.1.2.4.3-5, CQI-ReportConfigSCell-r10-DEFAULT is defined in Table 8.13.2.1.1.2.4.3-6.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Table 8.13.2.1.1.2.5-2. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table 8.13.2.1.1.2.5-1 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3A.5 and G.3A.6 in Annex G clause G.3.

8.13.2.1.1.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 with the following exceptions:

Table 8.13.2.1.1.2.4.3-1: PDSCH-ConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.2.1.1.2.4.3-2: PhysicalConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.1.2.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.2.1.1.2.4.3-3: CQI-ReportConfig-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.2.4.3-4: CQI-ReportConfig-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.2.4.3-5: CQI-ReportAperiodic-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.1.2.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.2.4.3-7: PUSCH-ConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.2.1.1.2.5	Test requirement

Table 8.13.2.1.1.1-1 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.2.1.1.2.5-1 or 8.13.2.1.1.2.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.1.1.2.5-1: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.2 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.2.1.1.2.5-2: Test requirements (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.1.1.2.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.1.1.2.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.2.1.1.3	TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for CA (3DL CA)

8.13.2.1.1.3.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on four antenna ports using closed-loop spatial multiplexing.

8.13.2.1.1.3.2	Test applicability

This test case applies to all types of E-UTRA TDD release 10 and forward UE of category 5 or higher that support 3DL with CA configurations in Table 7.1-2a and 4 Rx antenna ports.

This test case also applies to all types of E-UTRA TDD release 11 and forward UE of category 5 or higher that support 3DL with CA configurations in Table 7.1-2a and 4 Rx antenna ports.

8.13.2.1.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.2.1.1.1.

8.13.2.1.1.3.4	Test description

8.13.2.1.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.2.2-1 and 8.13.2.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.1.1.3.4.3.

8.13.2.1.1.3.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.2.1.1.3.4.3.

3.	The SS activates SCCs by sending the MAC-CE according to TS 36.321 [13] clauses 5.13 and 6.1.3.8. Wait for at least 2 seconds as per TS 36.133 [4] clause 8.3.3.2.

4.	The SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Table 8.13.2.1.1.3.5-2. The SS sends downlink MAC padding bits on the DL RMC.

5.	The SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR on each of the component carriers according to Tables 8.13.2.1.1.3.5-1 as appropriate.

7.	Measure the average throughput per component carrier for duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on each of the component carriers.

8.13.2.1.1.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 8.13.2.1.1.3.4.3-1: PDSCH-ConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.2.1.1.3.4.3-2: PhysicalConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 5.5.1.2, Table 5.5.1.2-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.2.1.1.3.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 [7] clause 4.6.3, Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.1.3.4.3-3: CQI-ReportConfig-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 [7] clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  | RBC |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.3.4.3-4: CQI-ReportConfig-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 [7] clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.3.4.3-5: CQI-ReportAperiodic-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11100000 | P-Cell, S-Cells report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.1.3.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.3.4.3-7: PUSCH-ConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.2.1.1.3.5	Test requirement

Table 8.13.2.1.1.3.3-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.4.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.2.1.1.3.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.1.1.3.5-1: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.2 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.2.1.1.3.5-2: Test requirements (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.2.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.2.1.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.2.1.1.4	TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for CA (4DL CA)

8.13.2.1.1.4.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on four antenna ports using closed-loop spatial multiplexing.

8.13.2.1.1.4.2	Test applicability

This test case applies to all types of E-UTRA TDD release 11 and forward UE of category 8 or higher that support 4DL with CA configurations in Table 7.1-2b and 4 Rx antenna ports.

8.13.2.1.1.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.2.1.1.1.

8.13.2.1.1.4.4	Test description

8.13.2.1.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.2.2-1 and 8.13.2.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.1.1.4.4.3.

8.13.2.1.1.4.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.2.1.1.4.4.3.

3.	The SS activates SCCs by sending the MAC-CE according to TS 36.321 [13] clauses 5.13 and 6.1.3.8. Wait for at least 2 seconds as per TS 36.133 [4] clause 8.3.3.2.

4.	The SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Table 8.13.2.1.1.4.5-2. The SS sends downlink MAC padding bits on the DL RMC.

5.	The SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR on each of the component carriers according to Table 8.13.2.1.1.4.5-1 as appropriate.

7.	Measure the average throughput per component carrier for duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on each of the component carriers.

8.13.2.1.1.4.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 8.13.2.1.1.4.4.3-1: PDSCH-ConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.2.1.1.4.4.3-2: PhysicalConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 5.5.1.2, Table 5.5.1.2-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.2.1.1.4.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 [7] clause 4.6.3, Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.1.4.4.3-3: CQI-ReportConfig-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 [7] clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  | RBC |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.4.4.3-4: CQI-ReportConfig-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 [7] clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.4.4.3-5: CQI-ReportAperiodic-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11110000 | P-Cell, S-Cells report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.1.4.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.4.4.3-7: PUSCH-ConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.2.1.1.4.5	Test requirement

Table 8.13.2.1.1.4.3-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.4.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.2.1.1.4.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.1.1.4.5-1: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.2 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.2.1.1.4.5-2: Test requirements (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.2.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.13.2.1.1.4.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.2.1.1.5	TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for CA (5DL CA)

8.13.2.1.1.5.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on four antenna ports using closed-loop spatial multiplexing.

8.13.2.1.1.5.2	Test applicability

This test case applies to all types of E-UTRA TDD release 11 and forward UE of category 8, 11 or higher that support 5DL with CA configurations in Table 7.1-2c and 4 Rx antenna ports

This test case also applies to all types of E-UTRA TDD release 12 and forward UE of category 8, 11 or higher that support 5DL with CA configurations in Table 7.1-2c and 4 Rx antenna ports.

8.13.2.1.1.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.2.1.1.1.

8.13.2.1.1.5.4	Test description

8.13.2.1.1.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.2.2-1 and 8.13.2.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.1.1.5.4.3.

8.13.2.1.1.5.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.2.1.1.5.4.3.

3.	The SS activates SCCs by sending the MAC-CE according to TS 36.321 [13] clauses 5.13 and 6.1.3.8. Wait for at least 2 seconds as per TS 36.133 [4] clause 8.3.3.2.

4.	The SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Table 8.13.2.1.1.5.5-2. The SS sends downlink MAC padding bits on the DL RMC.

5.	The SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR on each of the component carriers according to Table 8.13.2.1.1.5.5-1 as appropriate.

7.	Measure the average throughput per component carrier for duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on each of the component carriers.

8.13.2.1.1.5.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the following exceptions:

Table 8.13.2.1.1.5.4.3-1: PDSCH-ConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.2.1.1.5.4.3-2: PhysicalConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: TS 36.508 [7] clause 5.5.1.2, Table 5.5.1.2-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.2.1.1.5.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 [7] clause 4.6.3, Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.1.5.4.3-3: CQI-ReportConfig-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 [7] clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  | RBC |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.5.4.3-4: CQI-ReportConfig-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 [7] clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.5.4.3-5: CQI-ReportAperiodic-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11111000 | P-Cell, S-Cells report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.1.5.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.1.5.4.3-7: PUSCH-ConfigDedicated-DEFAULT: Additional TDD PDSCH closed loop multi-layer spatial multiplexing performance downlink power allocation

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.2.1.1.5.5	Test requirement

Table 8.13.2.1.1.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.4.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.2.1.1.5.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.1.1.5.5-1: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.2 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.2.1.1.5.5-2: Test requirements (FRC) based on single carrier performance for CA with 5 DL CCs

| 1 | 5x20MHz | As specified in Table 8.13.2.1.1.5.5-1 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| --- | --- | --- | --- |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.2.1.1.5.5-1 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6 |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.2.1.3	TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM (2DL CA)

###### 8.13.2.1.3.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with frequency selective precoding with 256QAM under CA.

###### 8.13.2.1.3.2	Test applicability

This test applies to E-UTRA TDD release10 and forward UE of category ≥ 5 which supports inter-band OR intra-band contiguous DL CA and 4 Rx antenna ports and 256QAM in DL.

This test also applies to E-UTRA TDD release 11 and forward UE of category ≥ 5 that supports intra-band non-contiguous DL CA and 4 Rx antenna ports and 256QAM in DL.

###### 8.13.2.1.3.3	Minimum conformance requirements

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.1.3.3-3, based on single carrier requirement specified in Table 8.13.2.1.3.3-2, with the addition of the parameters in Table 8.13.2.1.3.3-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.2.1.3.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests with 2CCs |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 4) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Multiple CC-s under test are configured as the 1st set of serving cells by high layers. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.2.1.3.3-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 TDD | R.72-1 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.4 | 18.0 |
| 10 MHz | R.72 TDD | R.72 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.4 | 17.5 |
| 15MHz | R.72-2 TDD | R.72-2 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.5 | 17.1 |
| 20MHz | R.72-3 TDD | R.72-3 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 17.2 |

Table 8.13.2.1.3.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.1.3-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.1.3-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.2.1.3.

###### 8.13.2.1.3.4	Test description

8.13.2.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.13.2.1.3.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.1.3.4.3.

8.13.2.1.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.2.1.3.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.  For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.2.1.3.3-2 and 8.13.2.1.3.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CQI feedback via PDCCH DCI format 0 with CQI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.2.1.3.5-1 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3A.5 and G.3A.6 in Annex G clause G.3.

8.13.2.1.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 and 5.5 with the following exceptions.

Table 8.13.2.1.3.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.2.1.3.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| cqi-ReportConfigPCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
| } |  |  |  |

Table 8.13.2.1.3.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| cqi-ReportConfigSCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
| } |  |  |  |

Table 8.13.2.1.3.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.3.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.3.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.3.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.3.4.3-7: CQI-ReportConfig-v1250-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-v1250 ::= SEQUENCE { |  |  |  |
| csi-SubframePatternConfig-r12 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| cqiCQI-ReportBoth-v1250 | Not present |  |  |
| cqi-ReportAperiodic-v1250 | Not present |  |  |
| altCQI-Table-r12 | allSubframes |  | DL 256QAM |
| } |  |  |  |

Table 8.13.2.1.3.4.3-8: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

###### 8.13.2.1.3.5	Test requirement

Table 8.13.2.1.3.3-1 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Tables 8.13.2.1.3.5-1 and 8.13.2.1.3.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.1.3.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 TDD | R.72-1 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 23.3 | 18.9 |
| 10 MHz | R.72 TDD | R.72 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.3 | 18.4 |
| 15MHz | R.72-2 TDD | R.72-2 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.4 | 18.0 |
| 20MHz | R.72-3 TDD | R.72-3 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.5 | 18.1 |

Table 8.13.2.1.3.5-2: Test requirement (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.1.3.5-1 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.2.1.4	TDD PDSCH Closed Loop Four-Layer Spatial Multiplexing for CA (2DL CA)

###### 8.13.2.1.4.1	Test purpose

To verify the UE’s rank-four performance and ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for four-layer transmission on 4Tx and 4Rx under CA.

###### 8.13.2.1.4.2	Test applicability

This test applies to all types of E-UTRA TDD release11 and forward UE of categories 5 or higher that support 4 Rx antenna ports and 4-layer spatial multiplexing for PDSCH transmission mode 4 and E-UTRA TDD 2DL CA.

###### 8.13.2.1.4.3	Minimum conformance requirements

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.1.4.3-3, based on single carrier requirement specified in Table 8.13.2.1.4.3-2, with the addition of the parameters in Table 8.13.2.1.4.3-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.2.1.4.3-1: Test Parameters for Four-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Precoding granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |
| PMI delay |  | ms | 10 or 11 |
| Reporting interval |  | ms | 1 or 4 |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| ACK/NACK feedback mode |  |  | PUCCH format 1b with channel selection for Tests with 2CCs |
| CSI request field (Note 4) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Uplink-Downlink Configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4) Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: Multiple CC-s under test are configured as the 1st set of serving cells by high layers. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.2.1.4.3-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.5 |
| 10MHz | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.4 |
| 15MHz | R.74-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.6 |
| 20MHz | R.74-3 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.8 |

Table 8.13.2.1.4.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.1.4-2 per CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.1.4-2 per CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.2.1.4.

###### 8.13.2.1.4.4	Test description

8.13.2.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.13.2.1.4.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.1.3.4.3.

8.13.2.1.4.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.1.2.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.2.1.4.3-2 and 8.13.2.1.4.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CQI feedback via PDCCH DCI format 0 with CQI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.2.1.4.5-1 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3A.5 and G.3A.6 in Annex G clause G.3.

8.13.2.1.4.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 4.6 and 5.5 with the following exceptions.

Table 8.13.2.1.4.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.2.1.4.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0xFFFF000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.2.1.4.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0xFFFF000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.4.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm31 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.4.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.4.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm31 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.1.4.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm31 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.1.4.4.3-7: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  |  |  |
| lateNonCriticalExtension | Not present |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.2.1.4.4.3-8: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

###### 8.13.2.1.4.5	Test requirement

Table 8.13.2.1.4.3-1 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Tables 8.13.2.1.4.5-1 and 8.13.2.1.4.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.1.4.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.4 |
| 10MHz | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.3 |
| 15MHz | R.74-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.5 |
| 20MHz | R.74-3 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.7 |

Table 8.13.2.1.4.5-2: Test requirement (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.1.4.5-1 per CC | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.1.4.5-1 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.2.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.2.2.1	Minimum Requirement

For CA with 2 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.2.2.1-2A, based on single carrier requirement specified in Table 8.13.2.2.1-2, with the addition of the parameters in Table 8.13.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 3 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.2.2.1-3, based on single carrier requirement specified in Table 8.13.2.2.1-2, with the addition of the parameters in Table 8.13.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 4 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.2.2.1-4, based on single carrier requirement specified in Table 8.13.2.2.1-2, with the addition of the parameters in Table 8.13.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For CA with 5 DL CCs, for dual-layer transmission on antenna ports 7 and 8 upon detection of a PDCCH with DCI format 2C, the requirements are specified in Table 8.13.2.2.1-5, based on single carrier requirement specified in Table 8.13.2.2.1-2, with the addition of the parameters in Table 8.13.2.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.2.2.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 |
| Cell ID |  |  | 0 |
| CSI reference signals |  |  | Antenna ports 15,16 |
| Beamforming model |  |  | Annex B.4.2 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 |
| CSI reference signal configuration |  |  | 8 |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 4 / 0010000000000000 |
| at antenna port |  | dBm/15kHz | -98 |
|  |  |  | Reference Value in Table 8.13.2.2.1-2 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 |
| Simultaneous transmission |  |  | No |
| PDSCH transmission mode |  |  | 9 |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |

Table 8.13.2.2.1-2: Single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 TDD | R.51-2 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.0 | 9.0 |
| 10 MHz | R.51 TDD | R.51 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.9 | 9.1 |
| 15MHz | R.51-3 TDD | R.51-3 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 13.9 | 9.2 |
| 20MHz | R.51-4 TDD | R.51-4 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.1 | 9.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.2.2.1-2A: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.2.2.1-3: Minimum performance (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Table 8.13.2.2.1-4: Minimum performance (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Table 8.13.2.1.1-5: Minimum performance (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6 |  |  |  |

##### 8.13.2.2.2	TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (2DL CA)

###### 8.13.1.2.2.1	Test purpose

To verify the UE’s rank-2 performance and ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for dual-layer transmission on antenna ports 7 and 8 using DM-RS with full RB allocation with multiple CSI reference symbol configurations with non-zero and zero transmission power.

###### 8.13.2.2.2.2	Test applicability

This test applies to E-UTRA TDD release10 and forward UE of category ≥ 5 which supports inter-band OR intra-band contiguous DL CA and 4 Rx antenna ports.

This test also applies to E-UTRA TDD release 11 and forward UE of category ≥ 5 that supports intra-band non-contiguous DL CA and 4 Rx antenna ports.

###### 8.13.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.2.2.1.

###### 8.13.2.2.2.4	Test description

8.13.2.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.13.1.2.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.2.2.4.3.

8.13.2.2.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.2.2.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2C for C_RNTI to transmit the DL RMC according to Tables 8.13.1.2.1-2 and 8.13.1.2.1-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table 8.13.2.2.2.5-1 as appropriate.  BCH/CRS/PDCCH/PCFICH are sent on antenna ports 0 and 1 using two Tx antennas, while DRS/ PDSCH for the test UE are sent on antenna ports 7 and 8 using two Tx antennas with beam-forming model as specified in Annex B.4.2 and precoder update granularity specified in Table 8.3.1-1. CSI-RS are sent on antenna ports 15 and 16 using two Tx antennas with mapping according to beam-forming model as specified in Annex B.4.2.

6.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

8.13.2.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 with the following exceptions:

Table 8.13.2.2.2.4.3-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.6 Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo | Not present |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10 |  |  |
| } |  |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| } |  |  |  |

Table 8.13.2.2.2.4.3-2: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo | Not present |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10 |  |  |
| } |  |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| } |  |  |  |

Table 8.13.2.2.2.4.3-3: AntennaInfoDedicated-r10

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AntennaInfoDedicated-r10 ::= SEQUENCE { |  |  |  |  |
| transmissionMode-r10 |  | tm9-v1020 |  |  |
| codebookSubsetRestriction-r10 |  | Not present | If the UE is configured with transmissionMode tm9, E-UTRAN only configures the field codebookSubsetRestriction if PMI/RI reporting is configured |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |  |
| Release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.2.2.2.4.3-4: CSI-RS-Config-r10

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-Config-r10 ::= SEQUENCE { |  |  |  |
| csi-RS-r10 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| antennaPortsCount-r10 | an2 | Parameter represents the number of antenna ports used for transmission of CSI reference signals |  |
| resourceConfig-r10 | 8 | Parameter: CSI reference signal configuration |  |
| subframeConfig-r10 | 4 | = when CSI-RS SubframeConfig is from 0-4; Parameter: |  |
| p-C-r10 | 0 | Parameter: which is the assumed ratio of PDSCH EPRE to CSI-RS EPRE when UE derives CSI feedback |  |
| } |  |  |  |
| } |  |  |  |
| zeroTxPowerCSI-RS-r10 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| zeroTxPowerResourceConfigList-r10 | 2 | Parameter: ZeroPowerCSI-RS |  |
| zeroTxPowerSubframeConfig-r10 | 4 | Parameter: |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.13.2.2.2.5	Test requirement

Table 8.13.1.2.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.2.2.2.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.2.2.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 TDD | R.51-2 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.9 | 9.9 |
| 10 MHz | R.51 TDD | R.51 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.8 | 10.0 |
| 15MHz | R.51-3 TDD | R.51-3 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.8 | 10.1 |
| 20MHz | R.51-4 TDD | R.51-4 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.0 | 10.2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.2.2.2.5-2: Test requirement (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.2.2.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.2.2.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.2.2.3	TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (3DL CA)

###### 8.13.2.2.3.1	Test purpose

Same test purpose as 8.13.2.2.2.1

###### 8.13.2.2.3.2	Test applicability

This test case applies to all types of E-UTRA TDD release 10 and forward UE of category ≥ 5 that support 3DL with CA configurations in Table 7.1-2a and 4 Rx antenna ports

This test case also applies to all types of E-UTRA TDD release 11 and forward UE of category ≥ 5 that support 3DL with CA configurations in Table 7.1-2a and 4 Rx antenna ports.

###### 8.13.2.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.2.2.1.

###### 8.13.2.2.3.4	Test description

8.13.2.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.13.2.2.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.2.3.4.3.

8.13.2.2.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.2.2.3.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2C for C_RNTI to transmit the DL RMC according to Tables 8.13.2.2.1-2 and 8.13.2.2.1-3 on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Table 8.13.2.2.3.5-1 as appropriate.  BCH/CRS/PDCCH/PCFICH are sent on antenna ports 0 and 1 using two Tx antennas, while DRS/ PDSCH for the test UE are sent on antenna ports 7 and 8 using two Tx antennas with beam-forming model as specified in Annex B.4.2 and precoder update granularity specified in Table 8.3.1-1. CSI-RS are sent on antenna ports 15 and 16 using two Tx antennas with mapping according to beam-forming model as specified in Annex B.4.2.

6.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

8.13.2.2.3.4.3	Message contents

Same message contents as 8.13.2.2.2.4.3

###### 8.13.2.2.3.5	Test requirement

Table 8.13.2.2.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.2.2.3.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.2.3.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 TDD | R.51-2 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.9 | 9.9 |
| 10 MHz | R.51 TDD | R.51 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.8 | 10.0 |
| 15MHz | R.51-3 TDD | R.51-3 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.8 | 10.1 |
| 20MHz | R.51-4 TDD | R.51-4 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.0 | 10.2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.2.2.3.5-2: Test requirement (FRC) based on single carrier performance for CA with 3 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 3x20MHz | As specified in Table 8.13.2.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+20MHz+15MHz | As specified in Table 8.13.2.2.3.5-1 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.2.2.4	TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (4DL CA)

###### 8.13.2.2.4.1	Test purpose

Same test purpose as 8.13.2.2.2.1

###### 8.13.2.2.4.2	Test applicability

This test case applies to all types of E-UTRA TDD release 11 and forward UE of category ≥ 5 that support 4DL with CA configurations in Table 7.1-2b and 4 Rx antenna ports.

###### 8.13.2.2.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.2.2.1.

###### 8.13.2.2.4.4	Test description

8.13.2.2.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.13.2.2.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.2.4.4.3.

8.13.2.2.4.4.2	Test procedure

Same test procedure as in clause 8.13.2.2.3.4.2 with the following exceptions:

-	Instead of Table 8.13.2.2.3.5-1  use Table 8.13.2.2.4.5-1

8.13.2.2.4.4.3	Message contents

Same message contents as 8.13.2.2.2.4.3

###### 8.13.2.2.4.5	Test requirement

Table 8.13.2.2.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.2.2.4.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.2.4.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 TDD | R.51-2 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.9 | 9.9 |
| 10 MHz | R.51 TDD | R.51 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.8 | 10.0 |
| 15MHz | R.51-3 TDD | R.51-3 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.8 | 10.1 |
| 20MHz | R.51-4 TDD | R.51-4 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.0 | 10.2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.2.2.4.5-2: Test requirement (FRC) based on single carrier performance for CA with 4 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 4x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| 2 | 20MHz+20MHz+20MHz+15MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.2.2.5	TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (5DL CA)

###### 8.13.2.2.5.1	Test purpose

Same test purpose as 8.13.2.2.2.1

###### 8.13.2.2.5.2	Test applicability

This test case applies to all types of E-UTRA TDD release 11 and forward UE of category 8, 11 or higher that support 5DL with CA configurations in Table 7.1-2c and 4 Rx antenna ports.

This test case also applies to all types of E-UTRA TDD release 12 and forward UE of category 8, 11 or higher that support 5DL with CA configurations in Table 7.1-2c and 4 Rx antenna ports.

###### 8.13.2.2.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.2.2.1.

###### 8.13.2.2.5.4	Test description

8.13.2.2.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.13.2.2.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.2.5.4.3.

8.13.2.2.5.4.2	Test procedure

Same test procedure as in clause 8.13.2.2.3.4.2 with the following exceptions:

-	Instead of Table 8.13.2.2.3.5-1  use Table 8.13.2.2.5.5-1

8.13.2.2.5.4.3	Message contents

Same message contents as 8.13.2.2.2.4.3

###### 8.13.2.2.5.5	Test requirement

Table 8.13.2.2.1-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Table 8.13.2.2.5.5-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.2.2.5.5-1: Test requirement single carrier performance for multiple CA configurations

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.51-2 TDD | R.51-2 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.9 | 9.9 |
| 10 MHz | R.51 TDD | R.51 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.8 | 10.0 |
| 15MHz | R.51-3 TDD | R.51-3 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 14.8 | 10.1 |
| 20MHz | R.51-4 TDD | R.51-4 TDD | OP.1 TDD | ETU5 | 2x2 Low | 2x4 Low | 70 | 15.0 | 10.2 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. Note 3: SNR corresponds to of Cell 1. |  |  |  |  |  |  |  |  |  |

Table 8.13.2.2.5.5-2: Test requirement (FRC) based on single carrier performance for CA with 5 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 5x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| 2 | 15MHz+4x20MHz | As specified in Table 8.13.2.2.1-2 per CC depending on either 2Rx CC or 4Rx CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6 |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.2.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing

##### 8.13.2.3.1	TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA)

8.13.2.3.1.1	Test purpose

To verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model.

8.13.2.3.1.2	Test applicability

This test applies to all types of E-UTRA TDD release11 and forward UE of categories 5 or higher that support 4 Rx antenna ports and enhanced receiver Type A and E-UTRA TDD 2DL CA.

8.13.2.3.1.3	Minimum conformance requirements

The purpose of these tests is to verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3. In Table 8.13.2.3.1.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.3.1.3-3, based on single carrier requirement specified in Table 8.13.2.3.1.3-2, with the addition of the parameters in Table 8.13.2.3.1.3-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.2.3.1.3-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  |  | Unit |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  | dB |  | N/A |  | -1.73 |  |
| BWChannel |  |  |  | MHz |  | 10 |  | 10 |  |
| Cell Id |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  | 6 |  | N/A |  |
| Interference model |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  | PRB |  | 50 |  | 6 |  |
| PMI delay (Note 4) |  |  |  | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  | 1111 |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  |  | PUSCH (Note 7) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  | TDD PCC |  |  |  | 3 |  | N/A |  |
|  |  | TDD SCC |  |  |  | 4 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 4: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. Note 7: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, SF#4, SF#6 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 SF#8 SF#2 and SF#3. |  |  |  |  |  |  |  |  |  |

Table 8.13.2.3.1.3-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 TDD | R.47-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.8 |
| 10 MHz | R.47 TDD | R.47 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.5 |
| 15MHz | R.47-2 TDD | R.47-2 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 20MHz | R.47-3 TDD | R.47-3 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.2.3.1.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.3.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.3.1.3-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.2.3.

8.13.2.3.1.4	Test description

8.13.2.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.94.

2.	The parameter settings for cell 1 are set up according to Tables 8.13.2.3.1.5-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.3.1.4.3.

8.13.2.3.1.4.2	Test procedure

1. 	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.2.3.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1B for C_RNTI to transmit the DL RMC according to Tables 8.13.2.3.1.5-1 and 8.13.2.3.1.5-2 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC. The SS sends uplink scheduling information via PDCCH DCI format 0 with CQI request bit set to 0 to schedule UL RMC in subframe #2, #3, #7 and #8 (Table A.2.3.1.2-1).

5.	SS sets up parameters for interfering cells (cell 2 for PCC and SCC) according to table 8.13.2.3.1.5-1 as appropriate.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SINR according to Tables 8.13.2.3.1.5-2 and 8.13.2.3.1.5-3 as appropriate.

7.	Measure the average throughput per component carrier for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5, on both PCC and SCC. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.10.1.2.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions.

Table 8.13.2.3.1.4.3-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| antennaInfoDedicated ::= SEQUENCE { |  |  |  |
| transmissionMode | tm6 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n2TxAntenna-tm6 | 1111 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| Release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.2.3.1.4.3-2: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm6 |  |  |
| codebookSubsetRestriction-r10 | 1111 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.3.1.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 3 | (see Table7.2.2-1C in TS 36.213) |  |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL | (see Table 7.2.2-1B in TS 36.213) |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.3.1.4.3-4: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 |  |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  | CQI_PERIODIC |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.2.3.1.4.3-5: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportPeriodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 4 |  |  |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.2.3.1.5	Test requirement

Table 8.13.2.3.1.5-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.4.2.1 for throughput test shall meet or exceed the specified value in Tables 8.13.2.3.1.5-2 and 8.13.2.3.1.5-3 for the specified SINR including test tolerances.

Table 8.13.2.3.1.5-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  |  | Unit |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  | dB |  | N/A |  | -1.73 |  |
| BWChannel |  |  |  | MHz |  | 10 |  | 10 |  |
| Cell Id |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  | 6 |  | N/A |  |
| Interference model |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  | PRB |  | 50 |  | 6 |  |
| PMI delay (Note 4) |  |  |  | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  | 1111 |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  |  | PUSCH (Note 7) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  | TDD PCC |  |  |  | 3 |  | N/A |  |
|  |  | TDD SCC |  |  |  | 4 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2, 3 are the interfering cells. Note 4: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. Note 7: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, SF#4, SF#6 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 SF#8 SF#2 and SF#3. |  |  |  |  |  |  |  |  |  |

Table 8.13.2.3.1.5-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 TDD | R.47-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8+TT | 2.8+TT |
| 10 MHz | R.47 TDD | R.47 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3+TT | 2.5+TT |
| 15MHz | R.47-2 TDD | R.47-2 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0+TT | 2.7+TT |
| 20MHz | R.47-3 TDD | R.47-3 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3+TT | 2.3+TT |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.2.3.1.5-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.3.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.3.1.5-2 per CC depending on either 2Rx CC or 4Rx CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.2.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.2.4.1	TDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA)

8.13.2.4.1.1	Test purpose

To verify the UE’s ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for single-layer transmission on antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4.

8.13.2.4.1.2	Test applicability

This test applies to all types of E-UTRA TDD release11 and forward UE of categories 5 or higher that support 4 Rx antenna ports and enhanced receiver Type A and E-UTRA TDD 2DL CA. Applicability requires support for FGI bit 103.

8.13.2.4.1.3	Minimum conformance requirements

The purpose of these tests is to verify closed loop rank one performance on one of the antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4. In 8.13.2.4.1.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For CA with 2 DL CCs, the requirements are specified in Table 8.13.2.4.1.3-3, based on single carrier requirement specified in Table 8.13.2.4.1.3-2, with the addition of the parameters in Table 8.13.2.4.1.3-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.2.4.1.3-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 |
|  |  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 4 | N/A |
| CSI reference signal configuration |  |  |  | 0 | N/A |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.73 |
| BWChannel |  |  | MHz | 10 | 10 |
| Cell Id |  |  |  | 0 | 126 |
| PDSCH transmission mode |  |  |  | 9 | 9 |
| Beamforming model |  |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | N/A | 70 |
|  |  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  |  | PRB | 50 | 6 |
| PMI delay (Note 5) |  |  | ms | 10 or 11 | N/A |
| Reporting interval |  |  | ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  |  | No simultaneous transmission on the other antenna port in (7 or 8) not used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | TDD PCC |  |  | 3 | N/A |
|  | TDD SCC |  |  | 4 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, SF#4, SF#6 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7, SF#8, SF#2 and #3. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |

Table 8.13.2.4.1.3-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A, CDM-multiplexed DM RS

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 TDD | R.76-4 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.6 | -2.4 |
| 10 MHz | R.76 TDD | R.76-5 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.1 | -2.1 |
| 15MHz | R.76-2 TDD | R.76-6 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -2.1 |
| 20MHz | R.76-3 TDD | R.76-7 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.2.4.1.3-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.4.1.3-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.4.1.3-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.2.4.

8.13.2.4.1.4	Test description

8.13.2.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.13.2.4.1.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.2.4.1.4.3.

8.13.2.4.1.4.2	Test procedure

1. 	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.2.4.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2C for C_RNTI to transmit the DL RMC according to Tables 8.13.2.4.1.3-1, 8.13.2.4.1.3-2 and 8.13.2.4.1.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC. The SS sends uplink scheduling information via PDCCH DCI format 0 with CQI request bit set to 0 to schedule UL RMC in subframe#2, #3,  #7 and #8 (Table A.2.2.1.2-1).

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to  Tables 8.13.2.4.1.5-2 and 8.13.2.4.1.5-3 as appropriate.
BCH/CRS/PDCCH/PCFICH are sent on antenna ports 0 and 1 using two Tx antennas, while DRS/PDSCH for the test UE are sent on antenna port 7 (or 8) using two Tx antennas with beam-forming model as specified in Annex B.4.3 and precoder update granularity specified in Table 8.13.2.4.1.5-1. CSI-RS are sent on antenna ports 15 and 16 using two Tx antennas with mapping according to beam-forming model as specified in Annex B.4.3.

6.	Set the interfering cells (cell 2 for PCC and SCC) –aggressor cell– as defined in Tables 8.13.2.4.1.5-1, 8.13.2.4.1.5-2 and according to Annex B.5.4.

7.	Measure the average throughput per component carrier for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5, on both PCC and SCC. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.13.2.4.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions.

Table 8.13.2.4.1.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB0 |  |  |
| } |  |  |  |

Table 8.13.2.4.1.4.3-2: AntennaInfoDedicated-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AntennaInfoDedicated-r10 ::= SEQUENCE { |  |  |  |  |
| transmissionMode-r10 |  | tm9-v1020 |  |  |
| codebookSubsetRestriction-r10 |  | 001111 |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.2.4.1.4.3-3: CSI-RS-Config-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-Config-r10 ::= SEQUENCE { |  |  |  |
| csi-RS-r10 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| antennaPortsCount-r10 | an2 | Parameter represents the number of antenna ports used for transmission of CSI reference signals |  |
| resourceConfig-r10 | 0 | Parameter: CSI reference signal configuration |  |
| subframeConfig-r10 | 4 | = when CSI-RS SubframeConfig is from 0-4; = 5; Parameter: |  |
| p-C-r10 | -3 dB | Parameter: which is the assumed ratio of PDSCH EPRE to CSI-RS EPRE when UE derives CSI feedback |  |
| } |  |  |  |
| } |  |  |  |
| zeroTxPowerCSI-RS-r10 CHOICE{ |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.4.1.4.3-4: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2AC CQI-ReportPeriodic-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportPeriodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 3 | TDD |  |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ri-ConfigIndex | 483 | TDD |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.4.1.4.3-5: PhysicalConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.6 Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfig | Not present |  | RBC |
| antennaInfo | Not present |  | RBC |
| antennaInfo-r10 CHOICE{ |  |  | RBC |
| explicitValue-r10 | AntennaInfoDedicated-r10 |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10- DEFAULT using condition CQI_PERIODIC |  | RBC |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  | RBC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |

Table 8.13.2.4.1.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2AB CQI-ReportConfigSCell-r10-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cqi-ReportPeriodicSCell-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 4 | TDD |  |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ri-ConfigIndex | 483 | TDD |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.2.4.1.4.3-7: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo | Not present |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10 |  |  |
| } |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| } |  |  |  |

Table 8.13.2.4.1.4.3-8: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportAperiodic-r10 | Not Present |  |  |
| pmi-RI-Report-r9 | setup |  |  |
| } |  |  |  |

Table 8.13.2.4.1.4.3-9: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| pmi-RI-Report-r10 | setup |  |  |
| } |  |  |  |

8.13.2.4.1.5	Test requirement

Table 8.13.2.4.1.5-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.4.3.3 for throughput test shall meet or exceed the specified value in Tables 8.13.2.4.1.5-2 and 8.13.2.4.1.5-3 for the specified SINR including test tolerances.

Table 8.13.2.4.1.5-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 |
|  |  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes | 5 / 4 | N/A |
| CSI reference signal configuration |  |  |  | 0 | N/A |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.73 |
| BWChannel |  |  | MHz | 10 | 10 |
| Cell Id |  |  |  | 0 | 126 |
| PDSCH transmission mode |  |  |  | 9 | 9 |
| Beamforming model |  |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | N/A | 70 |
|  |  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  |  | PRB | 50 | 6 |
| PMI delay (Note 5) |  |  | ms | 10 or 11 | N/A |
| Reporting interval |  |  | ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  |  | No simultaneous transmission on the other antenna port in (7 or 8) not used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | TDD PCC |  |  | 3 | N/A |
|  | TDD SCC |  |  | 4 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1, SF#4, SF#6 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7, SF#8, SF#2 and #3. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |

Table 8.13.2.4.1.5-2: Single carrier performance for multiple CA configurations Enhanced Performance Requirement Type A, CDM-multiplexed DM RS

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 TDD | R.76-4 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.6+TT | 2.4+TT |
| 10 MHz | R.76 TDD | R.76-5 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.1+TT | 2.1+TT |
| 15MHz | R.76-2 TDD | R.76-6 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.2+TT | 2.1+TT |
| 20MHz | R.76-3 TDD | R.76-7 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.4+TT | 2.1+TT |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.2.4.1.5-3: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test num. | CA Band-width combination |  | Requirement | UE category |
| --- | --- | --- | --- | --- |
| 1 | 2x20MHz | As specified in Table 8.13.2.4.1.5-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| 2 | 20MHz+15MHz | As specified in Table 8.13.2.4.1.5-2 per CC depending on either 2Rx CC or 4Rx CC |  | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

### 8.13.3	TDD-FDD (CA and DC)

#### 8.13.3.1	Closed Loop Multi Layer Spatial Multiplexing

##### 8.13.3.1.1	Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell

###### 8.13.3.1.1.1	Minimum Requirement

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.1.1.1-4 based on single carrier requirement specified in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3, with the addition of the parameters in Table 8.13.3.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.13.3.1.1.1-5 based on single carrier requirement specified in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3, with the addition of the parameters in Table 8.13.3.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.13.3.1.1.1-6 based on single carrier requirement specified in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3, with the addition of the parameters in Table 8.13.3.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.13.3.1.1.1-7 based on single carrier requirement specified in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3, with the addition of the parameters in Table 8.13.3.1.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.1.1.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband precoding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.1.1.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 7.5 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.1 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.2 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.7 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.6 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.3 | 5.6 |

Table 8.13.3.1.1.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 8.1 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.8 | 5.4 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.0 | 5.9 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.5 | 6.3 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.6 | 6.2 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.4 |

Table 8.13.3.1.1.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.1.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.1.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.13.3.1.1.1-2 and Table 8.13.3.1.1.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.1.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.13.3.1.1.2-2 and Table 8.13.3.1.1.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

###### 8.13.3.1.1.2	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (2DL CA)

8.13.3.1.1.2.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.1.1.2.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support E-UTRA FDD and TDD and 2DL CA with FDD as PCell and 4 Rx antenna ports.

8.13.3.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.3.1.1.1.

8.13.3.1.1.2.4	Test description

8.13.3.1.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables 8.2.3-1, 8.13.3.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.1.1.2.4.3.

8.13.3.1.1.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.1.1.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4.	For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.3.1.1.1-1 and 8.13.3.1.1.1-2 or 8.13.3.1.1.1-3. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.3.1.1.2.5-1 or 8.13.3.1.1.2.5-2 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

8.13.3.1.1.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 with the following exceptions:

Table 8.13.3.1.1.2.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.3.1.1.2.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.3.1.1.2.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.1.1.2.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.3.1.1.2.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.1.1.2.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.1.1.2.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.1.1.2.4.3-7: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.3.1.1.2.5	Test requirement

Table 8.13.3.1.1.1-1 define the primary level settings.

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.1.1.2.5-3 based on single carrier requirement specified in Table  8.13.3.1.1.2.5-1 and Table  8.13.3.1.1.2.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.1.1.2.5-1 or 8.13.3.1.1.2.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.1.1.2.5-1: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.3.1.1.2.5-2: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.4 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.3.1.1.2.5-3: Test requirement for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.1.1.2.5-1 and Table 8.13.3.1.1.2.5-2 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.1.1.2.5-1 and Table 8.13.3.1.1.2.5-2 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.1.1.2.5-1 and Table 8.13.3.1.1.2.5-2 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

###### 8.13.3.1.1.3	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (3DL CA)

8.13.3.1.1.3.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.1.1.3.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support E-UTRA FDD and TDD and 3DL CA with FDD as PCell and 4 Rx antenna ports.

8.13.3.1.1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.3.1.1.1.

8.13.3.1.1.3.4	Test description

8.13.3.1.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables 8.2.3-1, 8.13.3.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.1.1.3.4.3.

8.13.3.1.1.3.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.1.1.3.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4.	For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.3.1.1.1-1 and 8.13.3.1.1.1-2 or 8.13.3.1.1.1-3. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.3.1.1.3.5-1 or 8.13.3.1.1.3.5-2 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3.5 and G.3.6 in Annex G clause G.3.

8.13.3.1.1.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 with the following exceptions:

Table 8.13.3.1.1.3.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.3.1.1.3.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.3.1.1.3.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.1.1.3.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.3.1.1.3.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.1.1.3.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11100000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.1.1.3.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.1.1.3.4.3-7: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.3.1.1.3.5	Test requirement

Table 8.13.3.1.1.1-1 define the primary level settings.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.13.3.1.1.3.5-3 based on single carrier requirement specified in Table  8.13.3.1.1.3.5-1 and Table  8.13.3.1.1.3.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.1.1.3.5-1 or 8.13.3.1.1.3.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.1.1.3.5-1: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.3.1.1.3.5-2: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.4 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.3.1.1.3.5-3: Test requirement for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.13.3.1.1.3.5-1 and Table 8.13.3.1.1.3.5-2 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.13.3.1.1.3.5-1 and Table 8.13.3.1.1.3.5-2 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.13.3.1.1.3.5-1 and Table 8.13.3.1.1.3.5-2 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.13.3.1.1.3.5-1 and Table 8.13.3.1.1.3.5-2 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.13.3.1.1.3.5-1 and Table 8.13.3.1.1.3.5-2 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.13.3.1.1.3.5-1 and Table 8.13.3.1.1.3.5-2 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.13.3.1.1.3.5-1 and Table 8.13.3.1.1.3.5-2 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.13.3.1.1.3.5-1 and Table 8.13.3.1.1.3.5-2 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.13.3.1.1.3.5-1 and Table 8.13.3.1.1.3.5-2 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

###### 8.13.3.1.1.4	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (4DL CA)

8.13.3.1.1.4.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.1.1.4.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 8 or higher that support E-UTRA FDD and TDD and 4DL CA with FDD as PCell and 4 Rx antenna ports.

8.13.3.1.1.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.3.1.1.1.

8.13.3.1.1.4.4	Test description

8.13.3.1.1.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables 8.2.3-1, 8.13.3.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.1.1.4.4.3.

8.13.3.1.1.4.4.2	Test procedure

Same test procedure as in clause 8.13.3.1.1.3.4.2 with the following exceptions:

-	Instead of Table 8.13.3.1.1.3.5-1  use Table 8.13.3.1.1.4.5-1

8.13.3.1.1.4.4.3	Message contents

Same message contents as in clause 8.13.3.1.1.2.4.3 with the following exceptions:

Table 8.13.3.1.1.4.4.3-1: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11110000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.3.1.1.4.5	Test requirement

Table 8.13.3.1.1.1-1 define the primary level settings.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.13.3.1.1.4.5-3 based on single carrier requirement specified in Table  8.13.3.1.1.4.5-1 and Table  8.13.3.1.1.4.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.1.1.4.5-1 or 8.13.3.1.1.4.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.1.1.4.5-1: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.3.1.1.4.5-2: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.4 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.3.1.1.4.5-3: Test requirement for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.13.3.1.1.4.5.1-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.13.3.1.1.4.5-1 and Table 8.13.3.1.1.4.5-2 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

###### 8.13.3.1.1.5	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (5DL CA)

8.13.3.1.1.5.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.1.1.5.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 8, 11 or higher that support E-UTRA FDD and TDD and 5DL CA with FDD as PCell and 4 Rx antenna ports.

8.13.3.1.1.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.3.1.1.1.

8.13.3.1.1.5.4	Test description

8.13.3.1.1.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables 8.2.3-1, 8.13.3.1.1.1-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.1.1.5.4.3.

8.13.3.1.1.5.4.2	Test procedure

Same test procedure as in clause 8.13.3.1.1.3.4.2 with the following exceptions:

-	Instead of Table 8.13.3.1.1.3.5-1  use Table 8.13.3.1.1.5.5-1

8.13.3.1.1.5.4.3	Message contents

Same message contents as in clause 8.13.3.1.1.2.4.3 with the following exceptions:

Table 8.13.3.1.1.5.4.3-1: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11111000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.3.1.1.5.5	Test requirement

Table 8.13.3.1.1.1-1 define the primary level settings.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.13.3.1.1.5.5-3 based on single carrier requirement specified in Table  8.13.3.1.1.5.5-1 and Table  8.13.3.1.1.5.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.1.1.5.5-1 or 8.13.3.1.1.5.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.1.1.5.5-1: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.3.1.1.5.5-2: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.4 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.3.1.1.5.5-3: Test requirement for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.13.3.1.1.5.5-1 and Table 8.13.3.1.1.5.5-2 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.3.1.2	Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell

###### 8.13.3.1.2.1	Minimum Requirement

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.1.2.1-4 based on single carrier requirement specified in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3, with the addition of the parameters in Table 8.13.3.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell and 3DL CCs, the requirements are specified in Table 8.13.3.1.2.1-5 based on single carrier requirement specified in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3, with the addition of the parameters in Table 8.13.3.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell and 4DL CCs, the requirements are specified in Table 8.13.3.1.2.1-6 based on single carrier requirement specified in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3, with the addition of the parameters in Table 8.13.3.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell and 5DL CCs, the requirements are specified in Table 8.13.3.1.2.1-7 based on single carrier requirement specified in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3, with the addition of the parameters in Table 8.13.3.1.2.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.1.2.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband pre-coding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | TM4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.1.2.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 7.5 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.1 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.5 | 5.2 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.7 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.1 | 5.6 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.3 | 5.6 |

Table 8.13.3.1.2.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 8.1 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 9.8 | 5.4 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.0 | 5.9 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.5 | 6.3 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.6 | 6.2 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.4 |

Table 8.13.3.1.2.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.1.1-2 and Table 8.13.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.2.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Table 8.13.3.1.2.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC |  | TDD CC |  |  |
| 1 | 4x20 |  | 20 | 3x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 2 | 4x20 |  | 2×20 | 2×20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 3 | 3x20+15 |  | 20+15 | 2×20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 |  | 2×15 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 5 | 3x20+15 |  | 2×20+15 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 |  | 2x15+20 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 7 | 3x20+10 |  | 2x20+10 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 8 | 4x20 |  | 3x20 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 9 | 10+3x20 |  | 10 | 3x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 10 | 2x10+2x20 |  | 2x10 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 11 | 2x10+20+15 |  | 2x10 | 20+15 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 12 | 10+3x20 |  | 10+20 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 13 | 10+15+2x20 |  | 10+15 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| 14 | 10+15+2x20 |  | 10+15+20 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |  |

Table 8.13.3.1.2.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

###### 8.13.3.1.2.2	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (2DL CA)

8.13.3.1.2.2.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.1.2.2.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support E-UTRA FDD and TDD and 2DL CA with TDD as PCell and 4 Rx antenna ports.

8.13.3.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.3.1.2.1.

8.13.3.1.2.2.4	Test description

8.13.3.1.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables 8.2.3-1, 8.13.3.2.1.1-1as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.1.2.2.4.3.

8.13.3.1.2.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.1.2.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.3.1.2.2.3-1 and 8.13.3.1.2.2.3-2 or 8.13.3.1.2.2.3-3. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.3.1.2.2.5-1 or 8.13.3.1.2.2.5-2 as appropriate.

7.	Measure the average throughput per component carrier for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on both PCC and SCC. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.13.3.1.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 with the following exceptions:

Table 8.13.3.1.2.2.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.3.1.2.2.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.1.2.2.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.3.1.2.2.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.3.1.2.2.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.1.2.2.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.1.2.2.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.1.2.2.4.3-7: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.3.1.2.2.5	Test requirement

Table 8.13.3.1.2.1-1 define the primary level settings.

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.1.2.2.5-3 based on single carrier requirement specified in Table  8.13.3.1.2.2.5-1 and Table  8.13.3.1.2.2.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.1.2.2.5-1 or 8.13.3.1.2.2.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.1.2.2.5-1: Single carrier test requirement with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.3.1.2.2.5-2: Single carrier test requirement with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.2 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.3.1.2.2.5-3: Test requirement for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.1.2.2.5-1 and Table 8.13.3.1.2.2.5-2 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.1.2.2.5-1 and Table 8.13.3.1.2.2.5-2 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.1.2.2.5-1 and Table 8.13.3.1.2.2.5-2 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

###### 8.13.3.1.2.3	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (3DL CA)

8.13.3.1.2.3.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.1.2.3.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support E-UTRA FDD and TDD and 3DL CA with TDD as PCell and 4 Rx antenna ports.

8.13.3.1.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.3.1.2.1.

8.13.3.1.2.3.4	Test description

8.13.3.1.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables 8.2.3-1, 8.13.3.2.1.1-1as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.1.2.3.4.3.

8.13.3.1.2.3.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.1.2.3.4.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.3.1.2.3.3-1 and 8.13.3.1.2.3.3-2 or 8.13.3.1.2.3.3-3. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.3.1.2.3.5-1 or 8.13.3.1.2.3.5-2 as appropriate.

7.	Measure the average throughput per component carrier for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5 on both PCC and SCC. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.13.3.1.2.3.4.3	Message contents

Same message contents as in clause 8.13.3.1.2.2.4.3 with the following exceptions:

Table 8.13.3.1.2.3.4.3-1: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11100000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.3.1.2.3.5	Test requirement

Table 8.13.3.1.2.1-1 define the primary level settings.

For TDD FDD CA with TDD PCell and 3DL CCs, the requirements are specified in Table 8.13.3.1.2.3.5-3 based on single carrier requirement specified in Table  8.13.3.1.2.3.5-1 and Table  8.13.3.1.2.3.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.1.2.3.5-1 or 8.13.3.1.2.3.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.1.2.3.5-1: Single carrier test requirement with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.3.1.2.3.5-2: Single carrier test requirement with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.2 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.3.1.2.3.5-3: Test requirement for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.13.3.1.2.1-2 and Table 8.13.3.1.2.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

###### 8.13.3.1.2.4	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (4DL CA)

8.13.3.1.2.4.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.1.2.4.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 8 or higher that support E-UTRA FDD and TDD and 4DL CA with TDD as PCell and 4 Rx antenna ports.

8.13.3.1.2.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.3.1.2.1.

8.13.3.1.2.4.4	Test description

8.13.3.1.2.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables 8.2.3-1, 8.13.3.2.1.1-1as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.1.2.4.4.3.

8.13.3.1.2.4.4.2	Test procedure

Same test procedure as in clause 8.13.3.1.2.3.4.2 with the following exceptions:

-	Instead of Table 8.13.3.1.2.3.5-1  use Table 8.13.3.1.2.4.5-1

8.13.3.1.2.4.4.3	Message contents

Same message contents as in clause 8.13.3.1.2.2.4.3 with the following exceptions:

Table 8.13.3.1.2.4.4.3-1: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11110000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.3.1.2.4.5	Test requirement

Table 8.13.3.1.2.1-1 define the primary level settings.

For TDD FDD CA with TDD PCell and 4DL CCs, the requirements are specified in Table 8.13.3.1.2.4.5-3 based on single carrier requirement specified in Table  8.13.3.1.2.4.5-1 and Table  8.13.3.1.2.4.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.1.2.4.5-1 or 8.13.3.1.2.4.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.1.2.4.5-1: Single carrier test requirement with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.3.1.2.4.5-2: Single carrier test requirement with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.2 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.3.1.2.4.5-3: Test requirement for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  |  | Requirement | UE Category |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC |  | TDD CC |  |  |
| 1 | 4x20 |  | 20 | 3x20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 2 | 4x20 |  | 2×20 | 2×20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 3 | 3x20+15 |  | 20+15 | 2×20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 4 | 2×15+2x20 |  | 2×15 | 2x20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 5 | 3x20+15 |  | 2×20+15 | 20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 6 | 2×15+2x20 |  | 2x15+20 | 20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 7 | 3x20+10 |  | 2x20+10 | 20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 8 | 4x20 |  | 3x20 | 20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 9 | 10+3x20 |  | 10 | 3x20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 10 | 2x10+2x20 |  | 2x10 | 2x20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 11 | 2x10+20+15 |  | 2x10 | 20+15 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 12 | 10+3x20 |  | 10+20 | 2x20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 13 | 10+15+2x20 |  | 10+15 | 2x20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| 14 | 10+15+2x20 |  | 10+15+20 | 20 | As defined in Table 8.13.3.1.2.4.5-1 and Table 8.13.3.1.2.4.5-2 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

###### 8.13.3.1.2.5	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (5DL CA)

8.13.3.1.2.5.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.1.2.5.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 8, 11 or higher that support E-UTRA FDD and TDD and 5DL CA with TDD as PCell and 4 Rx antenna ports.

8.13.3.1.2.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 8.13.3.1.2.1.

8.13.3.1.2.5.4	Test description

8.13.3.1.2.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables 8.2.3-1, 8.13.3.2.1.1-1as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.1.2.5.4.3.

8.13.3.1.2.5.4.2	Test procedure

Same test procedure as in clause 8.13.3.1.2.3.4.2 with the following exceptions:

-	Instead of Table 8.13.3.1.2.3.5-1  use Table 8.13.3.1.2.5.5-1

8.13.3.1.2.5.4.3	Message contents

Same message contents as in clause 8.13.3.1.2.2.4.3 with the following exceptions:

Table 8.13.3.1.2.5.4.3-1: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11111000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.3.1.2.5.5	Test requirement

Table 8.13.3.1.2.1-1 define the primary level settings.

For TDD FDD CA with TDD PCell and 5DL CCs, the requirements are specified in Table 8.13.3.1.2.5.5-3 based on single carrier requirement specified in Table  8.13.3.1.2.5.5-1 and Table  8.13.3.1.2.5.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.1.2.5.5-1 or 8.13.3.1.2.5.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.1.2.5.5-1: Single carrier test requirement with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.14-4 FDD | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.3 | 8.4 |
| 3MHz | R.14-5 FDD | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.0 |
| 5MHz | R.14-6 FDD | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.4 | 6.1 |
| 10 MHz | R.14 FDD | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.6 |
| 15MHz | R.14-7 FDD | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.0 | 6.5 |
| 20MHz | R.14-3 FDD | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.2 | 6.5 |

Table 8.13.3.1.2.5.5-2: Single carrier test requirement with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 1.4MHz | R.43-1 TDD | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.9 | 9.0 |
| 3MHz | R.43-2 TDD | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.7 | 6.3 |
| 5MHz | R.43-3 TDD | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 10.9 | 6.8 |
| 10 MHz | R.43-4 TDD | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.4 | 7.2 |
| 15MHz | R.43-5 TDD | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.5 | 7.1 |
| 20MHz | R.43 TDD | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 4x4 Low | 70 | 11.6 | 7.3 |

Table 8.13.3.1.2.5.5-3: Test requirement for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.13.3.1.2.5.5-1 and 8.13.3.1.2.5.5-2 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.3.2

#### 8.13.3.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing

##### 8.13.3.3.1	TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA)

8.13.3.3.1.1	Test purpose

To verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3.

8.13.3.3.1.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support 4 Rx antenna ports and enhanced receiver Type A and E-UTRA FDD and TDD and 2DL CA with FDD as PCELL in any FDD or TDD RF band.

8.13.3.3.1.3	Minimum conformance requirements

In Table 8.13.3.3.1.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For TDD FDD CA with FDD PCell and with 2 DL CCs, the requirements are specified in Table 8.13.3.3.1.3-4, based on single carrier requirement specified in Table 8.13.3.3.1.3-2 and Table 8.13.3.3.1.3-3, with the addition of the parameters in Table 8.13.3.3.1.3-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.3.3.1.3-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  |  | Unit |  |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  |  | dB |  | N/A |  | -1.73 |  |
| Cell Id |  |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  |  | 6 |  | 4 |  |
| Interference model |  |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  |  | PRB |  | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |  | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |  |
| PMI delay (Note 4) |  |  |  | FDD CC | ms |  | 8 |  | N/A |  |
|  |  |  |  | TDD CC | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  |  | 1111 |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  |  |  | PUSCH (Note 7) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  | FDD PCC |  |  |  |  | 4 |  | N/A |  |
|  |  | TDD SCC |  |  |  |  | 5 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. Note 7: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3 for TDD CCs, and PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2 for FDD CCs. |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.1.3-2: Single carrier performance for Enhanced Performance Requirement Type A for FDD PCell and SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 FDD | R.47-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -3.0 |
| 10 MHz | R.47 FDD | R.47 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 15MHz | R.47-2 FDD | R.47-2 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.9 |
| 20MHz | R.47-3 FDD | R.47-3 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.1 | -2.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.1.3-3: Single carrier performance for Enhanced Performance Requirement Type A for TDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 TDD | R.47-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.8 |
| 10 MHz | R.47 TDD | R.47 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.5 |
| 15MHz | R.47-2 TDD | R.47-2 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 20MHz | R.47-3 TDD | R.47-3 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.1.3-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.3.1.3-2 and Table 8.13.3.3.1.3-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.3.1.3-2 and Table 8.13.3.3.1.3-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.3.1.3-2 and Table 8.13.3.3.1.3-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.3.3.

8.13.3.3.1.4	Test description

8.13.3.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: Select one according to Table 8.13.3.3.1.4.1-1.

CA Capability to be tested: Select one according to Table 8.13.3.3.1.4.1-1.

Table 8.13.3.3.1.4.1-1: Test point selection for TM4 TDD FDD 2DL CA for FDD PCell

| CA Capability | Bandwidth Combination |  |  |
| --- | --- | --- | --- |
|  | 20+20 | 10+20 | 15+20 |
| Inter-band CA (two bands) (CA2_A2) | Test 1 | Test 2 | Test3 |
| Note 1: One test point per UE supported CA capability is tested. For each UE supported CA capability, select the largest aggregated CA bandwidth combination supported by the UE within the chosen CA capability. |  |  |  |

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.2.3-1 and 8.13.3.3.1.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.3.1.4.3.

8.13.3.3.1.4.2	Test procedure

1. 	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.3.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4	SS transmits PDSCH via PDCCH DCI format 1B for C_RNTI to transmit the DL RMC according to Tables 8.13.3.3.1.3-1, 8.13.3.3.1.3-2 and 8.13.3.3.1.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC. The SS sends uplink scheduling information via PDCCH DCI format 0 with CQI request bit set to 0 to schedule UL RMC in subframe #2, #3, #7 and #8 (Table A.2.2.1.2-1).

5.	Set the parameters of the reference channel, MCS, the propagation condition, the correlation matrix and the SNR according to Tables 8.13.3.3.1.5-2 and 8.13.3.3.1.5-3 as appropriate.

6.	Measure the average throughput per component carrier for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5, on both PCC and SCC. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.13.3.3.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions.

Table 8.13.3.3.1.4.3-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| antennaInfoDedicated ::= SEQUENCE { |  |  |  |
| transmissionMode | tm6 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n2TxAntenna-tm6 | 1111 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| Release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.3.3.1.4.3-2: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm6 |  |  |
| codebookSubsetRestriction-r10 | 1111 |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.3.1.4.3-3: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 |  |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.3.1.4.3-4: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 4 |  | Pcell |
|  | 5 |  | Scell |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  | Pcell, Scell |
| simultaneousAckNackAndCQI | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.3.3.1.5	Test requirement

Tables 8.13.3.3.1.5-2 and 8.13.3.3.1.5-3 define the primary level settings.

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.3.1.5-4 based on single carrier requirement specified in Table 8.13.3.3.1.5-2 and Table 8.13.3.3.1.5-3.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.3.1.5-2 and 8.13.3.3.1.5-3 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.3.1.5-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  |  | Unit |  |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  |  | dB |  | N/A |  | -1.45 |  |
| Cell Id |  |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  |  | 6 |  | 4 |  |
| Interference model |  |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  |  | PRB |  | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |  | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |  |
| PMI delay (Note 4) |  |  |  | FDD CC | ms |  | 8 |  | N/A |  |
|  |  |  |  | TDD CC | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  |  | 1111 |  | N/A |  |
| Physical channel for CQI reporting |  |  |  |  |  |  | PUSCH (Note 7) |  | N/A |  |
| cqi-pmi-ConfigurationIndex |  | FDD PCC |  |  |  |  | 4 |  | N/A |  |
|  |  | TDD SCC |  |  |  |  | 5 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. Note 7: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3 for TDD CCs, and PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2 for FDD CCs. |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.1.5-2: Single carrier performance for Enhanced Performance Requirement Type A for FDD PCell and SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 FDD | R.47-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.78 | -2.02 |
| 10 MHz | R.47 FDD | R.47 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.98 | -1.72 |
| 15MHz | R.47-2 FDD | R.47-2 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.78 | -1.92 |
| 20MHz | R.47-3 FDD | R.47-3 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 2.08 | -1.82 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.1.5-3: Single carrier performance for Enhanced Performance Requirement Type A for TDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 TDD | R.47-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.78 | -1.82 |
| 10 MHz | R.47 TDD | R.47 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 2.28 | -1.52 |
| 15MHz | R.47-2 TDD | R.47-2 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.98 | -1.72 |
| 20MHz | R.47-3 TDD | R.47-3 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 2.28 | -1.32 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.1.5-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.3.1.5-2 and Table 8.13.3.3.1.5-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.3.1.5-2 and Table 8.13.3.3.1.5-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.3.1.5-2 and Table 8.13.3.3.1.5-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.3.3.2	TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA)

8.13.3.3.2.1	Test purpose

To verify the closed loop rank-one performance with wideband precoding with two transmit antennas when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 4 interference model defined in clause B.5.3.

8.13.3.3.2.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support 4 Rx antenna ports and enhanced receiver Type A and E-UTRA FDD and TDD and 2DL CA with TDD as PCELL in any FDD or TDD RF band.

8.13.3.3.2.3	Minimum conformance requirements

In Table 8.13.3.3.2.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For TDD FDD CA with TDD PCell and with 2 DL CCs, the requirements are specified in Table 8.13.3.3.2.3-4, based on single carrier requirement specified in Table 8.13.3.3.2.3-2 and Table 8.13.3.3.2.3-3, with the addition of the parameters in Table 8.13.3.3.2.3-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.3.3.2.3-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  | Unit |  |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  | dB |  | N/A |  | -1.73 |  |
| Cell Id |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  | 6 |  | 4 |  |
| Interference model |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  | PRB |  | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |  | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |  |
| PMI delay (Note 4) |  |  | FDD CC | ms |  | 8 |  | N/A |  |
|  |  |  | TDD CC | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  | 1111 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.2.3-2: Single carrier performance for Enhanced Performance Requirement Type A for FDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 FDD | R.47-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -3.0 |
| 10 MHz | R.47 FDD | R.47 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 15MHz | R.47-2 FDD | R.47-2 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.9 |
| 20MHz | R.47-3 FDD | R.47-3 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.1 | -2.8 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.2.3-3: Single carrier performance for Enhanced Performance Requirement Type A for TDD PCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 TDD | R.47-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.8 | -2.8 |
| 10 MHz | R.47 TDD | R.47 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.5 |
| 15MHz | R.47-2 TDD | R.47-2 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.0 | -2.7 |
| 20MHz | R.47-3 TDD | R.47-3 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.3 | -2.3 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.2.3-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.3.2.3-2 and Table 8.13.3.3.2.3-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.3.2.3-2 and Table 8.13.3.3.2.3-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.3.2.3-2 and Table 8.13.3.3.2.3-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.3.3.

8.13.3.3.2.4	Test description

8.13.3.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: Select one according to Table 8.13.3.3.2.4.1-1.

CA Capability to be tested: Select one according to Table 8.13.3.3.2.4.1-1.

Table 8.13.3.3.2.4.1-1: Test point selection for TM4 TDD FDD 2DL CA for TDD PCell

| CA Capability | Bandwidth Combination |  |  |
| --- | --- | --- | --- |
|  | 20+20 | 10+20 | 15+20 |
| Inter-band CA (two bands) (CA2_A2) | Test 1 | Test 2 | Test3 |
| Note 1: One test point per UE supported CA capability is tested. For each UE supported CA capability, select the largest aggregated CA bandwidth combination supported by the UE within the chosen CA capability. |  |  |  |

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.89.

2.	The parameter settings for the cell are set up according to Table 8.2.3-1 and 8.13.3.3.2.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.3.2.4.3.

8.13.3.3.2.4.2	Test procedure

1. 	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.3.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4	SS transmits PDSCH via PDCCH DCI format 1B for C_RNTI to transmit the DL RMC according to Tables 8.13.3.3.2.3-1, 8.13.3.3.2.3-2 and 8.13.3.3.1.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC. The SS sends uplink scheduling information via PDCCH DCI format 0 with CQI request bit set to 0 to schedule UL RMC in subframe #1, #4, #6 and #9 (Table A.2.3.1.2-1).

5.	Set the parameters of the reference channel, MCS, the propagation condition, the correlation matrix and the SNR according to Tables 8.13.3.3.2.5-2 and 8.13.3.3.2.5-3 as appropriate.

6.	Measure the average throughput per component carrier for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5, on both PCC and SCC. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.13.3.3.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions.

Table 8.13.3.3.2.4.3-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| antennaInfoDedicated ::= SEQUENCE { |  |  |  |
| transmissionMode | tm6 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n2TxAntenna-tm6 | 1111 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| Release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.3.3.2.4.3-2: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm6 |  |  |
| codebookSubsetRestriction-r10 | 1111 |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.3.2.4.3-3: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 |  |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.3.2.4.3-4: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 4 |  | Pcell |
|  | 3 |  | Scell |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.3.3.2.5	Test requirement

Tables 8.13.3.3.2.5-2 and 8.13.3.3.2.5-3 define the primary level settings.

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.3.2.5-4 based on single carrier requirement specified in Table 8.13.3.3.2.5-2 and Table 8.13.3.3.2.5-3.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.3.2.5-2 and 8.13.3.3.2.5-3 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.3.2.5-1: Test Parameters for Single-Layer Spatial Multiplexing (FRC) with TM4 interference model for CA

| Parameter |  |  | Unit |  |  | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  |  | dB |  | -3 |  | -3 |  |
|  |  |  |  | dB |  | -3 (Note 1) |  | -3 |  |
|  |  |  |  | dB |  | 0 |  | 0 |  |
| Cell-specific reference signals |  |  |  |  |  | Antenna ports 0,1 |  | Antenna ports 0,1 |  |
| at antenna port |  |  |  | dBm/15kHz |  | -98 |  | N/A |  |
| DIP (Note 2) |  |  |  | dB |  | N/A |  | -1.45 |  |
| Cell Id |  |  |  |  |  | 0 |  | 1 |  |
| PDSCH transmission mode |  |  |  |  |  | 6 |  | 4 |  |
| Interference model |  |  |  |  |  | N/A |  | As specified in clause B.5.3 |  |
| Probability of occurrence of transmission rank in interfering cells |  |  | Rank 1 | % |  | N/A |  | 80 |  |
|  |  |  | Rank 2 | % |  | N/A |  | 20 |  |
| Precoding granularity |  |  |  | PRB |  | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |  | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |  |
| PMI delay (Note 4) |  |  | FDD CC | ms |  | 8 |  | N/A |  |
|  |  |  | TDD CC | ms |  | 10 or 11 |  | N/A |  |
| Reporting interval |  |  |  | ms |  | 5 |  | N/A |  |
| Reporting mode |  |  |  |  |  | PUCCH 1-1 |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  |  | 1111 |  | N/A |  |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 5: All cells are time-synchronous. Note 6: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.2.5-2: Single carrier performance for Enhanced Performance Requirement Type A for FDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 FDD | R.47-1 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.78 | -2.02 |
| 10 MHz | R.47 FDD | R.47 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.98 | -1.72 |
| 15MHz | R.47-2 FDD | R.47-2 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.78 | -1.92 |
| 20MHz | R.47-3 FDD | R.47-3 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 2.08 | -1.82 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.2.5-3: Single carrier performance for Enhanced Performance Requirement Type A for TDD PCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.47-1 TDD | R.47-1 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.78 | -1.82 |
| 10 MHz | R.47 TDD | R.47 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | [2.28 | -1.52 |
| 15MHz | R.47-2 TDD | R.47-2 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.98 | -1.72 |
| 20MHz | R.47-3 TDD | R.47-3 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 2.28 | -1.32 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.3.2.5-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.3.2.5-2 and Table 8.13.3.3.2.5-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.3.2.5-2 and Table 8.13.3.3.2.5-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.3.2.5-2 and Table 8.13.3.3.2.5-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.3.4	Enhanced Performance Requirements Type A – Single-layer Spatial Multiplexing (User-Specific Reference Symbols)

##### 8.13.3.4.1	TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA)

8.13.3.4.1.1	Test purpose

To verify the UE’s ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for single-layer transmission on antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4.

8.13.3.4.1.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support 4 Rx antenna ports and enhanced receiver Type A and E-UTRA FDD and TDD and 2DL CA with FDD as PCELL in any FDD or TDD RF band. Applicability requires support for FGI bit 103.

8.13.3.4.1.3	Minimum conformance requirements

In Table 8.13.3.4.1.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For TDD FDD CA with FDD PCell with 2 DL CCs, the requirements are specified in Table 8.13.3.4.1.3-4, based on single carrier requirement specified in Table 8.13.3.4.1.3-2 and Table 8.13.3.4.1.3-3, with the addition of the parameters in Table 8.13.3.4.1.3-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.3.4.1.3-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 |
|  |  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS | FDD CC |  | Subframes | 5 / 2 | N/A |
|  | TDD CC |  |  | 5 / 4 | N/A |
| CSI reference signal configuration |  |  |  | 0 | N/A |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.73 |
| Cell Id |  |  |  | 0 | 126 |
| PDSCH transmission mode |  |  |  | 9 | 9 |
| Beamforming model |  |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | N/A | 70 |
|  |  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 5) | FDD CC |  | Ms | 8 | N/A |
|  | TDD CC |  |  | 10 or 11 | N/A |
| Reporting interval |  |  | Ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | FDD PCC |  |  | 4 | N/A |
|  | TDDS CC |  |  | 5 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3 for TDD CCs, and PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2 for FDD CCs. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |

Table 8.13.3.4.1.3-2: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for FDD PCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 FDD | R.76-4 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | 0-2. |
| 10 MHz | R.76 FDD | R.76-5 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.1 | -1.8 |
| 15MHz | R.76-2 FDD | R.76-6 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -1.9 |
| 20MHz | R.76-3 FDD | R.76-7 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -1.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.1.3-3: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for TDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 TDD | R.76-4 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.6 | -2.4 |
| 10 MHz | R.76 TDD | R.76-5 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.1 | -2.1 |
| 15MHz | R.76-2 TDD | R.76-6 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -2.1 |
| 20MHz | R.76-3 TDD | R.76-7 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.1.3-4: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.4.1.3-2 and Table 8.13.3.4.1.3-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.4.1.3-2 and Table 8.13.3.4.1.3-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.4.1.3-2 and Table 8.13.3.4.1.3-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.3.4.

8.13.3.4.1.4	Test description

8.13.3.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: Select one according to Table 8.13.3.4.1.4.1-1.

CA Capability to be tested: Select one according to Table 8.13.3.4.1.4.1-1.

Table 8.13.3.4.1.4.1-1: Test point selection for TM9 TDD FDD 2DL CA for FDD PCell

| CA Capability | Bandwidth Combination |  |  |
| --- | --- | --- | --- |
|  | 20+20 | 10+20 | 15+20 |
| Inter-band CA (two bands) (CA2_A2) | Test 1 | Test 2 | Test3 |
| Note 1: One test point per UE supported CA capability is tested. For each UE supported CA capability, select the largest aggregated CA bandwidth combination supported by the UE within the chosen CA capability. |  |  |  |

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.94.

2.	The parameter settings for the cell are set up according to Table 8.2.3-1] and 8.13.3.4.1.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.4.1.4.3.

8.13.3.4.1.4.2	Test procedure

1. 	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.4.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4	SS transmits PDSCH via PDCCH DCI format 2C for C_RNTI to transmit the DL RMC according to Tables 8.13.3.4.1.3-1, 8.13.3.4.1.3-2 and 8.13.3.4.1.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC. The SS sends uplink scheduling information via PDCCH DCI format 0 with CQI request bit set to 0 to schedule UL RMC in subframe #2, #3, #7 and #8 (Table A.2.2.1.2-1).

5.	Set the parameters of the reference channel, MCS, the propagation condition, the correlation matrix and the SNR according to Tables 8.13.3.4.1.5-2 and 8.13.3.4.1.5-3 as appropriate. 
BCH/CRS/PDCCH/PCFICH are sent on antenna ports 0 and 1 using two Tx antennas, while DRS/PDSCH for the test UE are sent on antenna port 7 (or 8) using two Tx antennas with beam-forming model as specified in Annex B.4.3 and precoder update granularity specified in Table 8.13.3.4.1.5-1. CSI-RS are sent on antenna ports 15 and 16 using two Tx antennas with mapping according to beam-forming model as specified in Annex B.4.3.

6.	Measure the average throughput per component carrier for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5, on both PCC and SCC. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.13.3.4.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions.

Table 8.13.3.4.1.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB0 |  |  |
| } |  |  |  |

Table 8.13.3.4.1.4.3-2: AntennaInfoDedicated-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AntennaInfoDedicated-r10 ::= SEQUENCE { |  |  |  |  |
| transmissionMode-r10 |  | tm9-v1020 |  |  |
| codebookSubsetRestriction-r10 |  | 001111 |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.3.4.1.4.3-3: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo |  |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10-DEFAULT |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| } |  |  |  |

Table 8.13.3.4.1.4.3-4: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.4.1.4.3-5: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 |  |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.4.1.4.3-6: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 4 |  | Pcell |
|  | 5 |  | Scell |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | 483 |  | Pcell, Scell |
| simultaneousAckNackAndCQI | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.4.1.4.3-7: CSI-RS-Config-r10

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-Config-r10 ::= SEQUENCE { |  |  |  |
| csi-RS-r10 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| antennaPortsCount-r10 | an2 | Parameter represents the number of antenna ports used for transmission of CSI reference signals |  |
| resourceConfig-r10 | 0 | Parameter: CSI reference signal configuration |  |
| subframeConfig-r10 | 2 | = when CSI-RS SubframeConfig is from 0-4; = 5; Parameter: | PCell |
|  | 4 |  | SCell |
| p-C-r10 | -3 dB | Parameter: which is the assumed ratio of PDSCH EPRE to CSI-RS EPRE when UE derives CSI feedback |  |
| } |  |  |  |
| } |  |  |  |
| zeroTxPowerCSI-RS-r10 CHOICE{ |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.3.4.1.5	Test requirement

Tables 8.13.3.4.1.5-2 and 8.13.3.4.1.5-3 define the primary level settings.

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.4.1.5-4 based on single carrier requirement specified in Table 8.13.3.4.1.5-2 and Table 8.13.3.4.1.5-3.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.4.1.5-2 and 8.13.3.4.1.5-3 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.4.1.5-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 |
|  |  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS | FDD CC |  | Subframes | 5 / 2 | N/A |
|  | TDD CC |  |  | 5 / 4 | N/A |
| CSI reference signal configuration |  |  |  | 0 | N/A |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.45 |
| Cell Id |  |  |  | 0 | 126 |
| PDSCH transmission mode |  |  |  | 9 | 9 |
| Beamforming model |  |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | N/A | 70 |
|  |  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 5) | FDD CC |  | Ms | 8 | N/A |
|  | TDD CC |  |  | 10 or 11 | N/A |
| Reporting interval |  |  | Ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | FDD PCC |  |  | 4 | N/A |
|  | TDDS CC |  |  | 5 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3 for TDD CCs, and PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2 for FDD CCs. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |

Table 8.13.3.4.1.5-2: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for FDD PCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 FDD | R.76-4 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.58 | -1.02 |
| 10 MHz | R.76 FDD | R.76-5 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.08 | -0.82 |
| 15MHz | R.76-2 FDD | R.76-6 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.78 | -0.92 |
| 20MHz | R.76-3 FDD | R.76-7 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.58 | -0.72 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.1.5-3: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for TDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 TDD | R.76-4 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.38 | -1.42 |
| 10 MHz | R.76 TDD | R.76-5 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.88 | -1.12 |
| 15MHz | R.76-2 TDD | R.76-6 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.78 | -1.12 |
| 20MHz | R.76-3 TDD | R.76-7 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.58 | -1.12 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.1.5-4: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.4.1.5-2 and Table 8.13.3.4.1.5-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.4.1.5-2 and Table 8.13.3.4.1.5-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.4.1.5-2 and Table 8.13.3.4.1.5-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.3.4.2	TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA)

8.13.3.4.2.1	Test purpose

To verify the UE’s ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for single-layer transmission on antenna ports 7 or 8 without a simultaneous transmission on the other antenna port in the serving cell when the PDSCH transmission in the serving cell is interfered by PDSCH of one dominant interfering cell applying transmission mode 9 interference model defined in clause B.5.4.

8.13.3.4.2.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support 4 Rx antenna ports and enhanced receiver Type A and E-UTRA FDD and TDD and 2DL CA with TDD as PCELL in any FDD or TDD RF band. Applicability requires support for FGI bit 103.

8.13.3.4.2.3	Minimum conformance requirements

In Table 8.13.3.4.2.3-1, Cell 1 is the serving cell, and Cell 2 is the interfering cell. The downlink physical channel setup is according to Annex C.3.2 for each of Cell 1 and Cell 2, respectively.

For TDD FDD CA with TDD PCell with 2 DL CCs, the requirements are specified in Table 8.13.3.4.2.3-4, based on single carrier requirement specified in Table 8.13.3.4.2.3-2 and Table 8.13.3.4.2.3-3, with the addition of the parameters in Table 8.13.3.4.2.3-1 and the downlink physical channel setup according to Annex C.3.2.

Table 8.13.3.4.2.3-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 |
|  |  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS | FDD CC |  | Subframes | 5 / 2 | N/A |
|  | TDD CC |  |  | 5 / 4 | N/A |
| CSI reference signal configuration |  |  |  | 0 | N/A |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.73 |
| Cell Id |  |  |  | 0 | 126 |
| PDSCH transmission mode |  |  |  | 9 | 9 |
| Beamforming model |  |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | N/A | 70 |
|  |  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 5) | FDD CC |  | Ms | 8 | N/A |
|  | TDD CC |  |  | 10 or 11 | N/A |
| Reporting interval |  |  | Ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | FDD CC |  |  | 3 | N/A |
|  | TDD CC |  |  | 4 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3 for TDD CCs, and PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2 for FDD CCs. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |

Table 8.13.3.4.2.3-2: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for FDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 FDD | R.76-4 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -0.2 |
| 10 MHz | R.76 FDD | R.76-5 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.1 | -1.8 |
| 15MHz | R.76-2 FDD | R.76-6 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -1.9 |
| 20MHz | R.76-3 FDD | R.76-7 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -1.7 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.2.3-3: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for TDD PCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 TDD | R.76-4 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.6 | -2.4 |
| 10 MHz | R.76 TDD | R.76-5 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.1 | -2.1 |
| 15MHz | R.76-2 TDD | R.76-6 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.2 | -2.1 |
| 20MHz | R.76-3 TDD | R.76-7 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | -0.4 | -2.1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.2.3-4: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.4.2.3-2 and Table 8.13.3.4.2.3-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.4.2.3-2 and Table 8.13.3.4.2.3-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.4.2.3-2 and Table 8.13.3.4.2.3-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.3.4.

8.13.3.4.2.4	Test description

8.13.3.4.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum WGap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: Select one according to Table 8.13.3.4.2.4.1-1.

CA Capability to be tested: Select one according to Table 8.13.3.4.2.4.1-1.

Table 8.13.3.4.2.4.1-1: Test point selection for TM9 TDD FDD 2DL CA for TDD PCell

| CA Capability | Bandwidth Combination |  |  |
| --- | --- | --- | --- |
|  | 20+20 | 10+20 | 15+20 |
| Inter-band CA (two bands) (CA2_A2) | Test 1 | Test 2 | Test3 |
| Note 1: One test point per UE supported CA capability is tested. For each UE supported CA capability, select the largest aggregated CA bandwidth combination supported by the UE within the chosen CA capability. |  |  |  |

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.89.

2.	The parameter settings for the cell are set up according to Table 8.2.3-1 and 8.13.3.4.2.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.4.1.4.3.

8.13.3.4.2.4.2	Test procedure

1. 	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.4.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4	SS transmits PDSCH via PDCCH DCI format 2C for C_RNTI to transmit the DL RMC according to Tables 8.13.3.4.2.3-1, 8.13.3.4.2.3-2 and 8.13.3.4.2.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC. The SS sends uplink scheduling information via PDCCH DCI format 0 with CQI request bit set to 0 to schedule UL RMC in subframe #1, #4, #6 and #9 (Table A.2.3.1.2-1).

5.	Set the parameters of the reference channel, MCS, the propagation condition, the correlation matrix and the SNR according to Tables 8.13.3.4.2.5-2 and 8.13.3.4.2.5-3 as appropriate. 
BCH/CRS/PDCCH/PCFICH are sent on antenna ports 0 and 1 using two Tx antennas, while DRS/PDSCH for the test UE are sent on antenna port 7 (or 8) using two Tx antennas with beam-forming model as specified in Annex B.4.3 and precoder update granularity specified in Table 8.13.3.4.2.5-1. CSI-RS are sent on antenna ports 15 and 16 using two Tx antennas with mapping according to beam-forming model as specified in Annex B.4.3.

6.	Measure the average throughput per component carrier for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5, on both PCC and SCC. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.13.3.4.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions.

Table 8.13.3.4.2.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB0 |  |  |
| } |  |  |  |

Table 8.13.3.4.2.4.3-2: AntennaInfoDedicated-r10-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| AntennaInfoDedicated-r10 ::= SEQUENCE { |  |  |  |  |
| transmissionMode-r10 |  | tm9-v1020 |  |  |
| codebookSubsetRestriction-r10 |  | 001111 |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |  |
| release |  | NULL |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.3.4.2.4.3-3: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo |  |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10-DEFAULT |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| } |  |  |  |

Table 8.13.3.4.2.4.3-4: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 CHOICE{ |  |  |  |
| explicitValue-r10 | AntennaInfoDedicated-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-RS-Config-r10 | CSI-RS-Config-r10 |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.4.2.4.3-5: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 |  |  |  |
|  | CQI-ReportPeriodic-r10-DEFAULT |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.4.2.4.3-6: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 3 |  | Pcell |
|  | 4 |  | Scell |
| cqi-FormatIndicatorPeriodic CHOICE { |  |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.4.2.4.3-7: CSI-RS-Config-r10

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-Config-r10 ::= SEQUENCE { |  |  |  |
| csi-RS-r10 CHOICE{ |  |  |  |
| setup SEQUENCE { |  |  |  |
| antennaPortsCount-r10 | an2 | Parameter represents the number of antenna ports used for transmission of CSI reference signals |  |
| resourceConfig-r10 | 0 | Parameter: CSI reference signal configuration |  |
| subframeConfig-r10 | 4 | = when CSI-RS SubframeConfig is from 0-4; = 5; Parameter: | PCell |
|  | 2 |  | SCell |
| p-C-r10 | -3 dB | Parameter: which is the assumed ratio of PDSCH EPRE to CSI-RS EPRE when UE derives CSI feedback |  |
| } |  |  |  |
| } |  |  |  |
| zeroTxPowerCSI-RS-r10 CHOICE{ |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

8.13.3.4.2.5	Test requirement

Tables 8.13.3.4.2.5-2 and 8.13.3.4.2.5-3 define the primary level settings.

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.4.2.5-4 based on single carrier requirement specified in Table 8.13.3.4.2.5-2 and Table 8.13.3.4.2.5-3.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.4.2.5-2 and 8.13.3.4.2.5-3 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.4.2.5-1: Test Parameters for Testing CDM-multiplexed DM RS (single layer) with TM9 interference model for CA

| parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | 0 | 0 |
|  |  |  | dB | 0 (Note 1) | 0 |
|  |  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  |  | Antenna ports 15,16 | N/A |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS | FDD CC |  | Subframes | 5 / 2 | N/A |
|  | TDD CC |  |  | 5 / 4 | N/A |
| CSI reference signal configuration |  |  |  | 0 | N/A |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  |  | dB | N/A | -1.45 |
| Cell Id |  |  |  | 0 | 126 |
| PDSCH transmission mode |  |  |  | 9 | 9 |
| Beamforming model |  |  |  | As specified in clause B.4.3 (Note 4, 5) | N/A |
| Interference model |  |  |  | N/A | As specified in clause B.5.4 |
| Probability of occurrence of transmission rank in interfering cells |  | Rank 1 |  | N/A | 70 |
|  |  | Rank 2 |  | N/A | 30 |
| Precoder update granularity |  |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs | 4 for 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 5) | FDD CC |  | Ms | 8 | N/A |
|  | TDD CC |  |  | 10 or 11 | N/A |
| Reporting interval |  |  | Ms | 5 | N/A |
| Reporting mode |  |  |  | PUCCH 1-1 | N/A |
| CodeBookSubsetRestriction bitmap |  |  |  | 001111 | N/A |
| Symbols for unused PRBs |  |  |  | OCNG (Note 6) | N/A |
| Simultaneous transmission |  |  |  | No simultaneous transmission on the other antenna port in (7 or 8) used for the input signal under test | N/A |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 8) | N/A |
| cqi-pmi-ConfigurationIndex | FDD CC |  |  | 3 | N/A |
|  | TDD CC |  |  | 4 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: The modulation symbols of the signal under test in Cell 1 are mapped onto antenna port 7 or 8. Note 4: The precoder in clause B.4.3 follows UE recommended PMI. Note 5: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 6: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 7: All cells are time-synchronous. Note 8: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3 for TDD CCs, and PDCCH DCI format 0 shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2 for FDD CCs. Note 9: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection configured for tests with 2 CCs. |  |  |  |  |  |

Table 8.13.3.4.2.5-2: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for FDD SCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 FDD | R.76-4 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.58 | -1.02 |
| 10 MHz | R.76 FDD | R.76-5 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 1.08 | -0.82 |
| 15MHz | R.76-2 FDD | R.76-6 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.78 | -0.92 |
| 20MHz | R.76-3 FDD | R.76-7 FDD | OP.1 FDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.58 | -0.72 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.2.5-3: Single carrier performance for Enhanced Performance Requirement Type A, CDM-multiplexed DM RS for TDD PCell (FRC)

| Band-width | Reference Channel |  | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) |  | Reference Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2Rx CC | 4Rx CC | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
|  |  |  |  |  |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.76-1 TDD | R.76-4 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.38 | -1.42 |
| 10 MHz | R.76 TDD | R.76-5 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.88 | -1.12 |
| 15MHz | R.76-2 TDD | R.76-6 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.78 | -1.12 |
| 20MHz | R.76-3 TDD | R.76-7 TDD | OP.1 TDD | N/A | EVA5 | EVA5 | 2x2 Low | 2x4 Low | 70 | 0.58 | -1.12 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |  |

Table 8.13.3.4.2.5-4: Minimum performance (FRC) based on single carrier performance for CA with 2 DL CCs

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.4.2.5-2 and Table 8.13.3.4.2.5-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.4.2.5-2 and Table 8.13.3.4.2.5-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.4.2.5-2 and Table 8.13.3.4.2.5-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.3.5

#### 8.13.3.6	Closed-loop spatial multiplexing performance 4Tx Antenna Port with 256QAM

##### 8.13.3.6.1	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for FDD PCell (2DL CA)

8.13.3.6.1.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.6.1.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support 4 Rx antenna ports and E-UTRA FDD and TDD and 2DL CA and 256QAM in DL with FDD as PCell in any FDD or TDD RF band.

8.13.3.6.1.3	Minimum conformance requirements

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.6.1.3-4 based on single carrier requirement specified in Table 8.13.3.6.1.3-2 and Table 8.13.3.6.1.3-3, with the addition of the parameters in Table 8.13.3.6.1.3-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.6.1.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband precoding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.6.1.3-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 FDD | R.72-1 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 23.4 | 19.3 |
| 10 MHz | R.72 FDD | R.72 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 18.0 |
| 15MHz | R.72-2 FDD | R.72-2 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.7 | 17.4 |
| 20MHz | R.72-3 FDD | R.72-3 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.8 | 17.5 |

Table 8.13.3.6.1.3-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 TDD | R.72-1 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.4 | 18.0 |
| 10 MHz | R.72 TDD | R.72 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.4 | 17.5 |
| 15MHz | R.72-2 TDD | R.72-2 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.5 | 17.1 |
| 20MHz | R.72-3 TDD | R.72-3 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 17.2 |

Table 8.13.3.6.1.3-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.6.1.3-2 and Table 8.13.3.6.1.3-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.6.1.3-2 and Table 8.13.3.6.1.3-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.6.1.3-2 and Table 8.13.3.6.1.3-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.3.6.

8.13.3.6.1.4	Test description

8.13.3.6.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables [8.13.3-1] and 8.13.3.6.1.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.6.1.4.3.

8.13.3.6.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.6.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4.	For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.3.6.1.3-1 and 8.13.3.6.1.3-2 or 8.13.3.6.1.3-3. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.3.6.1.5-1 or 8.13.3.6.1.5-2 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.13.3.6.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 with the following exceptions:

Table 8.13.3.6.1.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.3.6.1.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| cqi-ReportConfigPCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
| } |  |  |  |

Table 8.13.3.6.1.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| cqi-ReportConfigSCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
| } |  |  |  |

Table 8.13.3.6.1.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.3.6.1.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.6.1.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.6.1.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.6.1.4.3-7: CQI-ReportConfig-v1250-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-v1250 ::= SEQUENCE { |  |  |  |
| csi-SubframePatternConfig-r12 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| cqiCQI-ReportBoth-v1250 | Not present |  |  |
| cqi-ReportAperiodic-v1250 | Not present |  |  |
| altCQI-Table-r12 | allSubframes |  | DL 256QAM |
| } |  |  |  |

Table 8.13.3.6.1.4.3-8: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.3.6.1.5	Test requirement

Tables 8.13.3.6.1.5-1 and 8.13.3.6.1.5-2 define the primary level settings.

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.6.1.5-3 based on single carrier requirement specified in Table 8.13.3.6.1.5-1 and Table 8.13.3.6.1.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.6.1.5-1 or 8.13.3.6.1.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.6.1.5-1: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 FDD | R.72-1 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 24.3 | 20.2 |
| 10 MHz | R.72 FDD | R.72 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.5 | 18.9 |
| 15MHz | R.72-2 FDD | R.72-2 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.6 | 18.3 |
| 20MHz | R.72-3 FDD | R.72-3 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.7 | 18.4 |

Table 8.13.3.6.1.5-2: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 TDD | R.72-1 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 23.3 | 18.9 |
| 10 MHz | R.72 TDD | R.72 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.3 | 18.4 |
| 15MHz | R.72-2 TDD | R.72-2 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.4 | 18 |
| 20MHz | R.72-3 TDD | R.72-3 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.5 | 18.1 |

Table 8.13.3.6.1.5-3: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.6.1.5-1 and Table 8.13.3.6.1.5-2 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.6.1.5-1 and Table 8.13.3.6.1.5-2 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.6.1.5-1 and Table 8.13.3.6.1.5-2 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.3.6.2	TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for TDD PCell (2DL CA)

8.13.3.6.2.1	Test purpose

The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

8.13.3.6.2.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support 4 Rx antenna ports and E-UTRA FDD and TDD and 2DL CA and 256QAM in DL with TDD as PCell in any FDD or TDD RF band.

8.13.3.6.2.3	Minimum conformance requirements

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.6.2.3-4 based on single carrier requirement specified in Table 8.13.3.6.2.3-2 and Table 8.13.3.6.2.3-3, with the addition of the parameters in Table 8.13.3.6.2.3-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.6.2.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband pre-coding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.6.2.3-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 FDD | R.72-1 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 23.4 | 19.3 |
| 10 MHz | R.72 FDD | R.72 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 18.0 |
| 15MHz | R.72-2 FDD | R.72-2 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.7 | 17.4 |
| 20MHz | R.72-3 FDD | R.72-3 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.8 | 17.5 |

Table 8.13.3.6.2.3-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 TDD | R.72-1 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.4 | 18.0 |
| 10 MHz | R.72 TDD | R.72 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.4 | 17.5 |
| 15MHz | R.72-2 TDD | R.72-2 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.5 | 17.1 |
| 20MHz | R.72-3 TDD | R.72-3 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 21.6 | 17.2 |

Table 8.13.3.6.2.3-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.6.2.3-2 and Table 8.13.3.6.2.3-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.6.2.3-2 and Table 8.13.3.6.2.3-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.6.2.3-2 and Table 8.13.3.6.2.3-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.3.6.

8.13.3.6.2.4	Test description

8.13.3.6.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for Pcell are set up according to Tables [8.13.3-1] and 8.13.3.6.2.3-1as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.3.6.2.4.3.

8.13.3.6.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause  8.13.3.6.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133 [4], clauses 8.3.3.2).

4.	For multi-layer spatial multiplexing, SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.3.6.2.3-1 and 8.13.3.6.2.3-2 or 8.13.3.6.2.3-3. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0 with CSI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.3.6.2.5-1 or 8.13.3.6.2.5-2 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3A.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest on both component carriers.

8.13.3.6.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 5.5 and 4.6 with the following exceptions:

Table 8.13.3.6.2.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.3.6.2.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0000000000000000000000000000000011111111111111110000000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| cqi-ReportConfigPCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
| } |  |  |  |

Table 8.13.3.6.2.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0000000000000000000000000000000011111111111111110000000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| cqi-ReportConfigSCell-v1250 | CQI-ReportConfig-v1250-DEFAULT |  | DL 256QAM |
| } |  |  |  |

Table 8.13.3.6.2.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.3.6.2.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.6.2.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.6.2.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm12 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.6.2.4.3-7: CQI-ReportConfig-v1250-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-v1250 ::= SEQUENCE { |  |  |  |
| csi-SubframePatternConfig-r12 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| cqiCQI-ReportBoth-v1250 | Not present |  |  |
| cqi-ReportAperiodic-v1250 | Not present |  |  |
| altCQI-Table-r12 | allSubframes |  | DL 256QAM |
| } |  |  |  |

Table 8.13.3.6.2.4.3-8: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

8.13.3.6.2.5	Test requirement

Tables 8.13.3.6.2.5-1 and 8.13.3.6.2.5-2 define the primary level settings.

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.6.2.5-3 based on single carrier requirement specified in Table 8.13.3.6.2.5-1 and Table 8.13.3.6.2.5-2.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2 and A.3.4.2 for each throughput test shall meet or exceed the specified value in Tables 8.13.3.6.2.5-1 or 8.13.3.6.2.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.6.2.5-1: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 FDD | R.72-1 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 24.3 | 20.2 |
| 10 MHz | R.72 FDD | R.72 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.5 | 18.9 |
| 15MHz | R.72-2 FDD | R.72-2 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.6 | 18.3 |
| 20MHz | R.72-3 FDD | R.72-3 FDD | OP.1 FDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.7 | 18.4 |

Table 8.13.3.6.2.5-2: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference channel |  | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  | 2Rx CC | 4Rx CC |  |  | 2Rx CC | 4Rx CC |  | 2Rx CC | 4Rx CC |
| 5MHz | R.72-1 TDD | R.72-1 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 23.3 | 18.9 |
| 10 MHz | R.72 TDD | R.72 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.3 | 18.4 |
| 15MHz | R.72-2 TDD | R.72-2 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.4 | 18 |
| 20MHz | R.72-3 TDD | R.72-3 TDD | OP.1 TDD | EPA5 | 4x2 Low | 4x4 Low | 70 | 22.5 | 18.1 |

Table 8.13.3.6.2.5-3: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.6.2.5-1 and Table 8.13.3.6.2.5-2 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.6.2.5-1 and Table 8.13.3.6.2.5-2 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.6.2.5-1 and Table 8.13.3.6.2.5-2 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

#### 8.13.3.7	Closed-loop spatial multiplexing performance 4Tx Antenna Port with Four layers

##### 8.13.3.7.1	TDD-FDD CA PDSCH Closed Loop Four-Layer Spatial Multiplexing for FDD PCell (2DL CA)

###### 8.13.3.7.1.1	Test purpose

To verify the UE’s rank-four performance and ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for four-layer transmission on 4Tx and 4Rx under CA.

###### 8.13.3.7.1.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support 4 Rx antenna ports and E-UTRA FDD and TDD and 2DL CA with FDD as PCell in any FDD or TDD RF band.

###### 8.13.3.7.1.3	Minimum conformance requirements

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.7.1.3-4, based on single carrier requirement specified in Table 8.13.3.7.1.3-2 and Table 8.13.3.7.1.3-3, with the addition of the parameters in Table 8.13.3.7.1.3-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.7.1.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 |
| Precoding granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.7.1.3-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel | OCNG pattern |  | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 FDD | OP.1 FDD | EPA5 |  | 4x4 Low | 70 | 14.1 |
| 10MHz | R.74 FDD | OP.1 FDD | EPA5 |  | 4x4 Low | 70 | 14.9 |
| 15MHz | R.74-2 FDD | OP.1 FDD | EPA5 |  | 4x4 Low | 70 | 14.3 |
| 20MHz | R.74-3 FDD | OP.1 FDD | EPA5 |  | 4x4 Low | 70 | 14.4 |

Table 8.13.3.7.1.3-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.5 |
| 10MHz | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.4 |
| 15MHz | R.74-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.6 |
| 20MHz | R.74-3 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.8 |

Table 8.13.3.7.1.3-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.7.1-2 and Table 8.13.3.7.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.7.1-2 and Table 8.13.3.7.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.7.1-2 and Table 8.13.3.7.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.3.7.1.

###### 8.13.3.7.1.4	Test description

8.13.3.7.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.13.3.7.1.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.1.3.4.3.

8.13.3.7.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.1.2.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.3.7.1.3-1 and 8.13.3.7.1.3-2 and 8.13.3.7.1.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CQI feedback via PDCCH DCI format 0 with CQI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.3.7.1.5-1 or 8.13.3.7.1.5-2 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3A.5 and G.3A.6 in Annex G clause G.3.

8.13.3.7.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 4.6 and 5.5 with the following exceptions.

Table 8.13.3.7.1.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.3.7.1.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0xFFFF000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.3.7.1.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0xFFFF000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.7.1.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm31 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.3.7.1.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.7.1.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm31 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.7.1.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm31 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.7.1.4.3-7: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  |  |  |
| lateNonCriticalExtension | Not present |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.3.7.1.4.3-8: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

###### 8.13.3.7.1.5	Test requirement

Table 8.13.3.7.1.3-1 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Tables 8.13.3.7.1.5-1, 8.13.3.7.1.5-2 and 8.13.3.7.1.5-3 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.7.1.5-1: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.0 |
| 10MHz | R.74 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.8 |
| 15MHz | R.74-2 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.2 |
| 20MHz | R.74-3 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.3 |

Table 8.13.3.7.1.5-2: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.4 |
| 10MHz | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.3 |
| 15MHz | R.74-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.5 |
| 20MHz | R.74-3 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.7 |

Table 8.13.3.7.1.5-3 Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.7.1.5-1 and Table 8.13.3.7.1.5-2 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.7.1.5-1 and Table 8.13.3.7.1.5-2 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.7.1.5-1 and Table 8.13.3.7.1.5-2 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.

##### 8.13.3.7.2	TDD-FDD CA PDSCH Closed Loop Four-Layer Spatial Multiplexing for TDD PCell (2DL CA)

###### 8.13.3.7.2.1	Test purpose

To verify the UE’s rank-four performance and ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for four-layer transmission on 4Tx and 4Rx under CA.

###### 8.13.3.7.2.2	Test applicability

This test applies to all types of E-UTRA FDD and TDD Release 12 and forward UE of categories 5 or higher that support 4 Rx antenna ports and E-UTRA FDD and TDD and 2DL CA with TDD as PCell in any FDD or TDD RF band.

###### 8.13.3.7.2.3	Minimum conformance requirements

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.13.3.7.2.3-4, based on single carrier requirement specified in Table 8.13.3.7.2.3-2 and Table 8.13.3.7.2.3-3, with the addition of the parameters in Table 8.13.3.7.2.3-1 and the downlink physical channel setup according to Annex C.3.2. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.13.3.7.2.3-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 |
| Precoding granularity |  | PRB | 25 for 5MHz CCs, 50 for 10MHz CCs, 75 for 15MHz and 100 for 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0xFFFF000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| PDSCH rank |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subframe SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 1b with channel selection Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.13.3.7.2.3-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell and SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.1 |
| 10MHz | R.74 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.9 |
| 15MHz | R.74-2 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.3 |
| 20MHz | R.74-3 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 14.4 |

Table 8.13.3.7.2.3-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.5 |
| 10MHz | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.4 |
| 15MHz | R.74-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.6 |
| 20MHz | R.74-3 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 14.8 |

Table 8.13.3.7.2.3-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.7.2-2 and Table 8.13.3.7.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.7.2-2 and Table 8.13.3.7.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.7.2-2 and Table 8.13.3.7.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.13.3.7.2.

###### 8.13.3.7.2.4	Test description

8.13.3.7.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: The maximum aggregated bandwidth in the selected CA configuration, as defined in clause 8.1.2.6.5.

CA Capability to be tested: The CA capability containing the maximum number of 4Rx supported CCs, as defined in clause 8.1.2.6.5.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.95.

2.	The parameter settings for the cell are set up according to Table 8.13.3.7.2.3-1 as appropriate.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 8.13.1.1.3.4.3.

8.13.3.7.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 8.13.3.7.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 2 for C_RNTI to transmit the DL RMC according to Tables 8.13.3.7.2.3-1 and 8.13.3.7.2.3-2 and 8.13.3.7.2.3-3 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS schedules the UL transmission to carry the PUSCH CQI feedback via PDCCH DCI format 0 with CQI request bit set to ‘10’ and I_MCS=29 and N_PRB allocated to be less or equal to 20.

6.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix, antenna configuration and the SNR according to Tables 8.13.3.7.2.5-1 or 8.13.3.7.2.5-2 as appropriate.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.3 on each of the component carriers. Count the number of NACKs, ACKs and statDTXs on the UL during the test interval and decide pass or fail according to Tables G.3A.5 and G.3A.6 in Annex G clause G.3.

8.13.3.7.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clauses 4.6 and 5.5 with the following exceptions.

Table 8.13.3.7.2.4.3-1: PDSCH-ConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-6 PDSCH-ConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p-a | dB-6 |  |  |
| } |  |  |  |

Table 8.13.3.7.2.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm4 |  |  |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n4TxAntenna-tm4 | 0xFFFF000000000000 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cqi-ReportConfig-r10 | CQI-ReportConfig-r10-DEFAULT |  |  |
| } |  |  |  |

Table 8.13.3.7.2.4.3-2A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated SCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 SEQUENCE { |  |  |  |
| transmissionMode-r10 | tm4 |  |  |
| codebookSubsetRestriction-r10 | 0xFFFF000000000000 | BIT STRING |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.7.2.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm31 |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic | Not present |  |  |
| } |  |  |  |

Table 8.13.3.7.2.4.3-4: CQI-ReportConfig-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-r10 ::= SEQUENCE { |  |  | RBC |
| cqi-ReportAperiodic-r10 | CQI-ReportAperiodic-r10-DEFAULT |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic-r10 | Not present |  |  |
| pmi-RI-Report-r9 | Not present |  |  |
| csi-SubframePatternConfig-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.7.2.4.3-5: CQI-ReportAperiodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportAperiodic-r10 ::=CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm31 |  |  |
| aperiodicCSI-Trigger-r10 ::= SEQUENCE { |  |  |  |
| trigger1-r10 | 11000000 | P-Cell, S-Cell report |  |
| trigger2-r10 | 00000000 | No report |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.13.3.7.2.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-2AB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | rm31 |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | Not present |  |  |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 8.13.3.7.2.4.3-7: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  |  |  |
| lateNonCriticalExtension | Not present |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| maxLayersMIMO-r10 | fourLayers |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.13.3.7.2.4.3-8: PUSCH-ConfigDedicated-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| betaOffset-ACK-Index | 5 |  |  |
| betaOffset-RI-Index | 6 |  |  |
| betaOffset-CQI-Index | 5 |  |  |
| } |  |  |  |

###### 8.13.3.7.2.5	Test requirement

Table 8.13.3.7.2.3-1 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A clause A.3.3.2.2 for the throughput test shall meet or exceed the specified value in Tables 8.13.3.7.2.5-1, 8.13.3.7.2.5-2 and 8.13.3.7.2.5-3 for the specified SNR including test tolerances for all throughput tests.

Table 8.13.3.7.2.5-1: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell and SCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.0 |
| 10MHz | R.74 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.8 |
| 15MHz | R.74-2 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.2 |
| 20MHz | R.74-3 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | 15.3 |

Table 8.13.3.7.2.5-2: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell (FRC)

| Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5MHz | R.74-1 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.4 |
| 10MHz | R.74 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.3 |
| 15MHz | R.74-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.5 |
| 20MHz | R.74-3 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | 15.7 |

Table 8.13.3.7.2.5-3: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.13.3.7.2.5-1 and Table 8.13.3.7.2-2 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.13.3.7.2.5-1 and Table 8.13.3.7.2-2 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.13.3.7.2.5-1 and Table 8.13.3.7.2-2 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.6. |  |  |  |  |  |

Decide pass or fail for each subtest according to Annex G.3A.4. Decide the entire test pass or fail according to Annex G.3A.6.