3GPP TSG-RAN WG4 Meeting #90bis 	R4-1903065

# Xi’an, China, April 8-12, 2019

Title:	Draft LS on the applicability of UL RMCs to MOP test cases

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN4

To:	RAN5

CC:

Contact Person:

#### Name:	Anatoliy Ioffe

E-mail Address:	aioffe@apple.com

Attachments:	none

1. Overall Description:

RAN4 has corrected the FR2 MPR definition in TS38.101-2 for all power classes in Clauses 6.2.2.1, 6.2.2.2, 6.2.2.3, and 6.2.2.4 in order to ensure UEs comply with regulatory requirements on occupied bandwidth.  As an outcome of this change, the specification defines regions of RB allocations for DFT-s-OFDM pi/2 BPSK and DFT-s-OFDM QPSK modulations where MPR = 0 dB.  For clarity, the reference waveform used for the power class definition is captured in Clause 6.2.2.0 as the following:

For all power classes, the waveform defined by BW = 100 MHz, SCS = 120 KHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

The UL reference measurement channel corresponding to this waveform can be found in Table A.2.3.2-2 of the specification.

RAN4 respectfully asks RAN5 to take the above information into account in their work related to conformance test specification development.

2. Actions:

None

3. Date of Next RAN4 Meetings:

3GPP RAN4 #91 		25 – 29 May 2019, 		Reno, USA

3GPP RAN4 #92 		26 – 30 August 2019, 		Ljubljana, Slovenia