3GPP TSG-RAN5 Meeting #83	R5-194064

# Reno, Nevada, United States, 13th May 2019 - 17th May 2019

Title:		Channel spacing for FR2 intra-band contiguous CA

Source:					NTT DOCOMO, INC.

Agenda Item:			5.3.5.17

Document for:		Discussion and Endorsement

Introduction

Test frequency and related parameters for FR2 UL CA (intra-band contiguous) are not defined in TS 38.508-1 [1] yet. The purpose of this contribution is to propose channel spacing value for conformance test.

Summary

Proposal 1	: Regarding intra-band contiguous CA with Band n257, to set test channel spacing as 99.36 MHz.

- Observation 1	: Channel spacing during test should be set as less than or equal to NCS.

- Observation 2	: Test channel spacing for CA_n257X should be set as “less than or equal to 99.84 MHz”

- Observation 3	: Test channel spacing for CA_n257X should be set as “95.04 + 0.72, 1.44 x N”.

Discussion

Scope of this contribution

All discussions, observations, and proposals in this contribution refer to only following CA combinations related to Japanese FR2 Band n257.

Table 5.5A.1-1: NR CA configurations, bandwidth combination sets and fallback group defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set / Fallback group |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated BW (MHz) | BCS | Fallback group |
|  |  | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) | CBW (MHz) |  |  |  |
| CA_n257G | CA_n257G | 100 | 100 |  |  |  |  |  |  | 200 | 0 | 3 |
| CA_n257H | CA_n257H | 100 | 100 | 100 |  |  |  |  |  | 300 | 0 |  |
| CA_n257I | CA_n257I | 100 | 100 | 100 | 100 |  |  |  |  | 400 | 0 |  |
| CA_n257J | CA_n257J | 100 | 100 | 100 | 100 | 100 |  |  |  | 500 | 0 |  |
| CA_n257K | CA_n257K | 100 | 100 | 100 | 100 | 100 | 100 |  |  | 600 | 0 |  |
| CA_n257L | CA_n257L | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  | 700 | 0 |  |
| CA_n257M | CA_n257M | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 800 | 0 |  |

Nominal Channel Spacing

Following section is definition of Nominal Channel Spacing (NCS) in TS 38.101-2 [2]. In FR2 operation, NCS value depends on 1) Channel bandwidth, 2) Guard Band, and 3) SCS of each component carrier. In addition as described in blue highlighted part, channel spacing can be defined as any value which is lower than or equal to NCS.

Observation 1	: Channel spacing during test should be set as less than or equal to NCS.

- 5.4A.1	Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent NR component carriers is defined as the following unless stated otherwise:

For NR operating bands with 60kHz channel raster:

with

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR component carriers according to Table 5.3.2-1 with values in MHz. and the GBChannel(i) is the minimum guard band defined in sub-clause 5.3.3, while µ1 and µ2 are the subcarrier spacing configurations of the component carriers as defined in TS 38.211 [9]. The channel spacing for intra-band contiguous carrier aggregation can be adjusted to any multiple of sub-carrier spacing less than the nominal channel spacing to optimize performance in a particular deployment scenario.

In the recent meetings, definition of NCS has been re-discussed in RAN4, and following two options are considered as RAN4’s assumption according to WF [3]. With simple formula (Option 1, (3)), NCS becomes 99.84 MHz regardless of SCSs.

The equation to calculate the nominal channel spacing:

- Option 1		The formulas in R4-1902870 apply for the calculation of the nominal channel spacing for CA

- Option 2		Keep the current specs unchanged by defining a reference SCS mapping to the reference bandwidth

- Nominal channel spacing=BWChannel(1)+BWChannel(2)0.480.24 MHz                   (3)

Channel spacing for CA_n257X

This section describes proposed test channel spacing for FR2 intra-band contiguous CA with Band n257.

First, as described in section 3.1, channel spacing should be less than or equal to NCS. Regarding CA_n257G (100 MHz x 2 CCs) cases, NCS can be calculated as below. In symmetric case (BWChannel(1) = BWChannel(2)), NCS depends on only SCS value. From maintainability point of view, it is beneficial for conformance test to define common test channel spacing among SCS parameters. Considering observation 1, lower NCS can be selected.

Observation 2	: Test channel spacing for CA_n257X should be set as “less than or equal to 99.84 MHz”.

Table 3.3-1 NCS for CA_n257G

| BWChannel (1),(2) [MHz] | GBChannel(1),(2) [kHz] | SCS [kHz] | n | NCS [MHz] |
| --- | --- | --- | --- | --- |
| 100 | 2450 | 60 | 0 | 99.96 |
| 100 | 2420 | 120 | 1 | 99.84 |

In addition, future scalability should be considered when defining test channel spacing. Following figure 5.3.1-1 describes relationship between channel bandwidth and the transmission bandwidth configuration. In this example of n257 CA, transmission bandwidth can be calculated as 95.04 MHz as described in Table 3.3-2.

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

Table 3.3-2 Transmission BW

| SCS [kHz] | BW / 1RB [MHz] | NRB [RB] | Transmission BW [MHz] |
| --- | --- | --- | --- |
| 60 | 0.72 | 132 | 95.04 |
| 120 | 1.44 | 66 | 95.04 |

Following figure summarizes channel spacing in intra-band contiguous CA, and channel spacing can be calculated as “95.04 (47.52 x 2) MHz + Guard band”. In Rel-16 NR core specification, guard band can be optimized (reduced) in order to enhance transmission efficiency. Considering this future scalability, guard band should be set as integral multiple of RB (0.72, 1.44 MHz).

Observation 3	: Test channel spacing for CA_n257X should be set as “95.04 + 0.72, 1.44 x N”.

Figure 3.3-1: Channel spacing in intra-band contiguous CA

Table 3.3-3 Channel spacing considering guard band

| SCS | Channel spacing | Calculated value [MHz] |
| --- | --- | --- |
| 60 kHz | 95.04 + 0 x RB (0.72) | 95.04 |
|  | 95.04 + 1 x RB (0.72) | 95.76 |
|  | 95.04 + 2 x RB (0.72) | 96.48 |
|  | 95.04 + 3 x RB (0.72) | 97.20 |
|  | 95.04 + 4 x RB (0.72) | 97.92 |
|  | 95.04 + 5 x RB (0.72) | 98.64 |
|  | 95.04 + 6 x RB (0.72) | 99.36 |
|  | 95.04 + 7 x RB (0.72) | 100.08 (> 99.84 MHz) |
| 120 kHz | 95.04 + 0 x RB (1.44) | 95.04 |
|  | 95.04 + 1 x RB (1.44) | 96.48 |
|  | 95.04 + 2 x RB (1.44) | 97.92 |
|  | 95.04 + 3 x RB (1.44) | 99.36 |
|  | 95.04 + 4 x RB (1.44) | 100.80 (> 99.84 MHz) |

Considering observation 3, channel spacing can be calculated as above and 99.36 MHz is suitable for both SCS 60 kHz and 120 kHz.

Proposal 1	: Regarding intra-band contiguous CA with Band n257, to set test channel spacing as 99.36 MHz.

Proposal

Proposal 1	: Regarding intra-band contiguous CA with Band n257, to set test channel spacing as 99.36 MHz.

- Reference

[2] TS 38.508-1		: “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Common test environment (Release 15) (v15.3.0)”

[2] TS 38.101-2		: “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 15) (v15.5.0)”

[3] R4-1905240		: “WF on nominal channel spacing”, ZTE, RAN4#90bis