3GPP TSG-RAN5 Meeting #83	R5-194592

# Reno, Nevada, 13 – 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4848 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maximum Test System Uncertainty for receiver tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SPRINT Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel13-UEConTest |  |  |  |  |  | Date: |  |  | May 3, 2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CA_25A-26A needs to be defined in RAN5 specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Uncertainty of the test system is defined for receiver test 7.4A.3_H Maximum input level for CA (inter-band DL CA without UL CA) for 256QAM in DL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information of the CA configuration remains undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex F (normative):
Measurement uncertainties and Test Tolerances

<<< unchanged sections removed >>>

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
|  | <<< unchanged rows removed >>> |  |
| 7.4 Maximum input level | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 7.4_1 Maximum input level with 4 Rx antenna ports | Same as 7.4 |  |
| 7.4A.1 Maximum input level for CA (intra band contiguous DL CA and UL CA) | Same as 7.4 for each CC | Uncertainties apply for each CC |
| 7.4A.1 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA and UL CA) | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.2 Maximum input level for CA (intra band contiguous DL CA without UL CA) | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.2 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA without UL CA) | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.3 Maximum input level for CA (inter-band DL CA without UL CA) | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.3_H Maximum input level for CA (inter-band DL CA without UL CA) for 256QAM in DL | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.4 Maximum input level for CA (intra band non-contiguous DL CA without UL CA) | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.5 Maximum input level for 3DL CA | Downlink power same as 7.4A.3 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.5_H Maximum input level for SCE-L1 for 3DL CA | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.7 Maximum input level for 4DL CA | Downlink power same as 7.4A.3 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.7_H Maximum input level for 4DL CA for 256QAM in DL | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
| 7.4A.8_H Maximum input level for 5DL CA for 256QAM in DL | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 | Uncertainties apply for each CC |
|  | <<< unchanged rows removed >>> |  |
| Note 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |

## F.1.4	Measurement of performance requirements