3GPP TSG-RAN5 Meeting #84	R5-195762

# Ljubljana, Slovenia,  26th Aug – 30th Aug

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4919 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding points 25 25 QPSK 25+25- - 75 25 QPSK 75+25 on Table 6.3.5A.3.1.4.1-1: Test Configuration Table: PUCCH sub-test and points 25 25 QPSK 50 P_25@0 S_25@0 -- 75 25 QPSK 100 P_75@0 P_25@0 on Table 6.3.5A.3.1.4.1-2: Test Configuration Table: PUSCH sub-test for UL-CA_66B. Correction on Table 6.3.5A.3.1.4.1-2 changing the point 2575 into 75 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dekra Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2019-06-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing points 25 25 QPSK 25+25- - 75 25 QPSK 75+25 on Table Table 6.3.5A.3.1.4.1-1 for UL-CA_66B and points 25 25 QPSK 50 P_25@0 S_25@0 -- 75 25 QPSK 100 P_75@0 P_25@0 on Table 6.3.5A.3.1.4.1-2: for UL-CA_66B. Correction on Table 6.3.5A.3.1.4.1-2 changing the point 2575 into 75 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding points 25 25 QPSK 25+25- - 75 25 QPSK 75+25 on Table Table 6.3.5A.3.1.4.1-1 for UL-CA_66B and points 25 25 QPSK 50 P_25@0 S_25@0 -- 75 25 QPSK 100 P_75@0 P_25@0 on Table 6.3.5A.3.1.4.1-2 for UL-CA_66B. Correction on Table 6.3.5A.3.1.4.1-2 changing the point 2575 into 75 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 2DL/2UL: DL-CA_66B / UL-CA_66B would not be available in clause 6.3.5A.3.1 Table 6.3.5A.3.1.4.1-2 will have wrong point declaration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

##### 6.3.5A.3.1	Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA)

6.3.5A.3.1.1	Test purpose

To verify the ability of a UE to maintain its power in non-contiguous transmission within 21 ms in response to 0 dB TPC commands with respect to the first UE transmission, when the power control parameters specified in [TS 36.213] are constant on all active component carriers.

6.3.5A.3.1.2	Test applicability

This test applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and UL CA.

6.3.5A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3.5A.3.0

6.3.5A.3.1.4	Test description

6.3.5A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.3.5A.3.1.4.1-1and table 6.3.5A.3.1.4.1-2. The details of the uplink reference measurement channel (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.5A.3.1.4.1-1: Test Configuration Table: PUCCH sub-test

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | Normal |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Mid range |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Lowest NRB_agg Highest NRB_agg |
| Test Parameters for CA Configurations |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation |  | Uplink Configuration |  |
| PCC NRB | SCCs NRB | CC Mod | PCC & SCC RB allocation | FDD: PUCCH format = Format 1b with channel selection/Format 3 TDD: PUCCH format = Format 1b with channel selection/Format 3 |  |
| 25 | 25 | QSPK | 25+25 |  |  |
| 25 | 50 | QPSK | 25+50 |  |  |
| 50 | 50 | QPSK | 50+50 |  |  |
| 75 | 25 | QPSK | 75+25 |  |  |
| 75 | 75 | QPSK | 75+75 |  |  |
| 100 | 25 | QPSK | 100+25 |  |  |
| 100 | 50 | QPSK | 100+50 |  |  |
| 100 | 75 | QPSK | 100+75 |  |  |
| 100 | 100 | QPSK | 100+100 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 6.3.5A.3.1.4.1-2: Test Configuration Table: PUSCH sub-test

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] clause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] clause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in clause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB_agg Highest NRB_agg |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 |  |  |
| 75 | 25 |  | QPSK | 100 | P_75@0 | P_25@0 |  |  |
| 75 | 75 | N/A | QPSK | 150 | P_75@0 | S_75@0 |  |  |
| 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 |  |  |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 |  |  |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 |  |  |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

<< End of changes>>