3GPP RAN TSG|WG5 Meeting #84		R5-196361

Ljubljana, Slovenia, Aug 23-27

Agenda item:	5.3.5.17

Source:	Apple Inc.

Title:	AP#83.22 Maximum difference in direction between the beam peak at NTC when compared to ETC

Release:	Rel-15

Document for:	Discussion and Endorsement

# 1	Introduction

The purpose of this discussion paper is to provide maximum difference in direction between the beam peak at NTC when compared to ETC to the following action point [1]:

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#83.22 | RF | To provide feedback on the maximum difference in direction between the beam peak at NTC when compared to ETC to determine the Test procedure and framework for peak EIRP and peak EIS test cases | RAN5 | R5-194596 | RAN5#84 | Open |

# 2	Discussion

In previous meetings, there is assumption to reuse beam peak from NTC to ETC, but there was no data to prove that assumption. To make a progress on mm-wave test procedure and framework for test cases that need to verify at beam peak direction, we present the impact of temperature on beam forming in FR2 based on simulation results from AP#83.22 [1]. Two simulation cases are provided for DUT with two 2 panels for 9 beam codebooks (15-degree step) and 21 beam code books (7.5-degree step). Mutual coupling, form factor integration, and implementation losses are not modelled. Reference coverage CDF pattern for 9 beam code books and 21 code books with no impairment are shown in Figure 1 and Figure 2 below.

Figure 1: Spherical Coverage CDF for 9 beam code books

Figure 2: Spherical Coverage CDF for 21 beam code books

As we mentioned in [3], the current REFSENS requirement is derived at 20C as it is using -174 dBm(kT). Therefore, the difference in the noise floor between 20C and either of two extreme temperatures i.e. -10C and +55C is about 0.5 dB using the equation below.

∆NF=10log10273.15+55273.15+20≈0.5dB

Observation 1: The REFSENS requirement is derived under normal temperature condition (NTC) assumption and testing it in extreme temperature condition (ETC) will result in 0.5dB difference in noise floor.

The extreme temperature not only changes the thermal noise floor, but also changes much more inside the ICs. For example, usually the gain changes, where a lower gain at higher temperatures results in a higher cascaded noise figure. Further, ETC will also increase the phase noise of the LO which decreases the SNR. Furthermore, ETC also changes the phase and amplitude balance of the IQ down-conversion thus reducing the performance. The modeled impairments to evaluate the impact of beam peak direction are LNA noise figure, phase shifter, and RSRP error. To represent the impact of beam peak direction from ETC, an ISSCC publication [2] is used to derive more practical parameters for magnitude error and phase error.

Figure 3: Temperature impact on Tx/Rx parameters

Observation 2: In addition to impact on thermal noise floor which impacts magnitude error, TX/RX VGA gain control and phase shifter loss variation also impact magnitude error. It is expected to observe 0.1 dB under NTC and 1.0 dB under ETC.

Observation 3: Phase error from TX/RX VGA phase variation which impacts beam forming is 1.5° under NTC and 2.0° under ETC.

Both Peak EIRP and Peak EIS measurement requires Rx beam sweep to have best beam forming in each direction. Having RSRP error could cause the DUT to select the wrong beam. In addition, in order to find the beam peak direction in ETC, the test vendor should increase/decrease the temperature around the DUT by, for example, putting a bubble around DUT which increases the insertion loss. It is also worth noting that by putting the bubble around the DUT, the beam peak direction may change and it is thus needed to find the beam peak direction again in ETC.

Observation 4: RSRP measurement accuracy for RRM specification in FR2 is derived from TS 38.133 clauses 10.1.3 [4] is 6 dB for NTC and 9 dB for ETC. The RRM requirement is defined as low SNR. The beam correspondence tolerance requirement is defined over the link angles “corresponding to the top 50% of the EIRP2 measurement over the whole sphere.” [6] This, the BC requirement is defined as high SNR, so RSRP error is expected to be less than 6 dB for NTC and less than 9 dB for ETC.

Based on observations given above, we use magnitude error, phase error, and RSRP error to simulate

impact of beam peak direction between NTC and ETC. ETC Spherical coverage CDF pattern for 9 beam code books with impairments is shown in Figure 3

Figure 3: ETC Spherical Coverage CDF for 9 beam code books

Observation 5: Based on the simulations with all impairment models, maximum difference magnitude and angle in beam peak direction between ETC and NTC for worst case of 9 beam code books and 21 beam code books is 0.9 dB and 14°

We look further into our impairment models, and we can improve the impairment by proposing additional test procedure in the following:

- Find the beam peak under NTC following TS38.810 clause 5.2.13.7 [5]

- Lock the beam peak such that UE doesn’t perform DL beam sweeping

- Lock UE position relative to measurement antenna

- Apply ETC

Observation 6: With proposed procedure by locking beam peak in NTC, then apply ETC. RSRP error

between ETC and NTC shall be negligible; however, the impact of magnitude error and phase error is

expected. The maximum difference in beam peak direction between ETC and NTC with magnitude error and

phase error for worst case of 9 beam code books and 21 beam code books is 0.7 dB and 12°.

Based on observations given above, we think it makes sense to add additional test procedure for ETC test

Cases. We therefore propose to have conical region spanning in ETC based on beam peak in NTC.

Proposal 1: Perform a beam peak search refinement over conical region spanning +/- 12° around beam

peak direction which was found under NTC.

In case that the chamber isn’t able to move positioner in conical region spanning +/- 12° inside an ETC

Bubble, we propose to increase test tolerance by 0.9 dB to account for beam peak direction change under

ETC

Proposal 2: In case that the chamber isn’t able to move positioner in conical region spanning +/- 12° inside

a temperature control bubble, we propose to increase test tolerance for ETC by 0.9 dB for MOP and REFSENs test cases.

FR2 ETC discussions have been discussed many times in RAN4 meetings; however, no conclusion is made, so the decision is to continue to discuss ETC testability aspects in Rel-16 based on draft RAN4#91 meeting report.

Proposal 3: To align with RAN4 objective, RAN5 shall keep all MOPs and REFSENs test cases under normal temperature conditions until RAN4 finishes work study item in Rel-16.

# 3	Conclusions

In this paper, we explained the impacts of impairments from magnitude error, phase error, and RSRP error in extreme temperature conditions.

Based on the discussions, the following observations and proposal are made:

Observation 1: The REFSENS requirement is derived under normal temperature condition (NTC) assumption and testing it in extreme temperature condition (ETC) will result in 0.5dB difference in noise floor.

Observation 2: In addition to impact on thermal noise floor which impacts magnitude error, TX/RX VGA gain control and phase shifter loss variation also impact magnitude error. It is expected to observe 0.1 dB under NTC and 1.0 dB under ETC.

Observation 3: Phase error from TX/RX VGA phase variation which impacts beam forming is 1.5° under NTC and 2.0° under ETC.

Observation 4: RSRP measurement accuracy in FR2 is derived from TS 38.133 clauses 10.1.3 [4] from the same cell between two measurements is 6 dB for NTC and 9 dB for ETC.

Observation 5: Based on the simulations with all impairment models, maximum difference magnitude and angle in beam peak direction between ETC and NTC for worst case of 9 beam code books and 21 beam code books is 0.9 dB and 14°

Observation 6: With proposed procedure by locking beam peak in NTC, then apply ETC. RSRP error

between ETC and NTC shall be negligible; however, the impact of magnitude error and phase error is

expected. The maximum difference in beam peak direction between ETC and NTC with magnitude error and

phase error for worst case of 9 beam code books and 21 beam code books is 0.7 dB and 12°.

Proposal 1: Perform a beam peak search refinement over conical region spanning +/- 12° around beam

peak direction which was found under NTC.

Proposal 2: In case that the chamber isn’t able to move positioner in conical region spanning +/- 12° inside

a temperature control bubble, we propose to increase test tolerance for ETC by 0.9 dB for MOP and REFSENs test cases.

Proposal 3: To align with RAN4 objective, RAN5 shall keep all MOPs and REFSENs test cases under normal temperature conditions until RAN4 finishes work study item in Rel-16.

# 4	References

R5-193516_Action_Points_RAN5#83_RF

“A 28GHz 32-Element Phased-Array Transceiver IC with Concurrent Dual Polarized Beams and 1.4 Degree Beam-Steering Resolution for 5G Communication”, B. Sadhu, et al., ISSCC 2017

R4-1812388, “Further Views On EIS Spherical Coverage for FR2 UEs”, Apple Inc, 3GPP RAN4 #88bis, October 2018

TS38.133 v16.0.0, “Requirements for support of radio resource management,” 3GPP, June 2019

TS38.810 v16.3.0, “Study on test methods,” 3GPP, July 2019

[6]   3GPP TS38.101-2, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” V15.5.0, 3GPP RAN #83, March 2019