3GPP TSG-RAN5 #84 Meeting	R5-196572

# Ljubljana, Slovenia, 13th – 17th May. 2019

Title:							Impact of noise for FR2 ACLR

Source:			Anritsu

Agenda Item:			5.3.5.17

Document for:			Endorsement

# 1.	Introduction

# In this paper, we provide the analysis for the impact from noise for FR2 ACLR test case. As per the offline discussion during and after RAN5#83 meeting, we provide the SNRs for below assumptions.

- Assumption 1 :

  - Assumption for min TRP power value on wanted channel shall be:  min peak EIRP – 16 dB (UE antenna (8x2) directivity) – MBR - MPR

  - Assumption for min TRP power value on adjacent channel shall be:  min peak EIRP –  16 dB (UE antenna (8x2) directivity) – MBR - MPR – ACLR requirement

- Assumption 2 :

  - Assumption for min TRP power value on wanted channel shall be:  min peak EIRP – 10 dB (UE antenna (1x4) directivity) – MBR - MPR

  - Assumption for min TRP power value on adjacent channel shall be:  min peak EIRP –  10 dB (UE antenna (1x4) directivity) – MBR - MPR – ACLR requirement

# Where, MBR is multi-band relaxation and we assume the worst case value. MPR depends on RB allocation and waveforms, modulation orders. According to the current test point, MPR=2 to 6.5dB is specified. As a consequence, we obtain minimum TRP assumptions as shown in Table 1.

Table 1 Minimum TRP assumptions

|  | Min TRP [dBm/CBW] (Assumption 1) |  |  |  | Min TRP [dBm/CBW] (Assumption 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FR2a |  | FR2b |  | FR2a |  | FR2b |  |
| MPR[dB] | CP | ACP | CP | ACP | CP | ACP | CP | ACP |
| 0 | 4.7 | -12.3 | 2.9 | -13.1 | 10.7 | -6.3 | 8.9 | -7.1 |
| 2 | 2.7 | -14.3 | 0.9 | -15.1 | 8.7 | -8.3 | 6.9 | -9.1 |
| 2.5 | 2.2 | -14.8 | 0.4 | -15.6 | 8.2 | -8.8 | 6.4 | -9.6 |
| 3 | 1.7 | -15.3 | -0.1 | -16.1 | 7.7 | -9.3 | 5.9 | -10.1 |
| 3.5 | 1.2 | -15.8 | -0.6 | -16.6 | 7.2 | -9.8 | 5.4 | -10.6 |
| 4 | 0.7 | -16.3 | -1.1 | -17.1 | 6.7 | -10.3 | 4.9 | -11.1 |
| 4.5 | 0.2 | -16.8 | -1.6 | -17.6 | 6.2 | -10.8 | 4.4 | -11.6 |
| 5 | -0.3 | -17.3 | -2.1 | -18.1 | 5.7 | -11.3 | 3.9 | -12.1 |
| 5.5 | -0.8 | -17.8 | -2.6 | -18.6 | 5.2 | -11.8 | 3.4 | -12.6 |
| 6.5 | -1.8 | -18.8 | -3.6 | -19.6 | 4.2 | -12.8 | 2.4 | -13.6 |

# 2.	Discussion

We’ve already corrected the estimated SNRs from some TE vendors for Assumption 1, but with MPR=0 in such as R5-192984, R5-194133 and R5-193104.

R5-192984 proposing single values irrespective of EIRPmax , while R5-194133 and R5-193104 proposed EIRP max depending values. With some re-consideration and studies, we think we can remove the dependency for EIRPmax and is updated as shown in 3rd column of Table 1. The proposed values from each vendors for Assumption 1, MPR=0, CBW=400MHz are listed below.

Table 1 Estimated SNR values for ACLR, Assumption 1, MPR=0, CBW=400MHz (SNRA1,MPR=0,400MHz)

| Frequency Range | Channel | Estimated SNR (Update) | R5-192984 | R5-194133 |
| --- | --- | --- | --- | --- |
| 23.45 GHz ≤ f ≤ 32.125 GHz | CP | 12.5 | 7 |  (EIRPMAX > 33dBm) |
|  |  |  |  | 15.0 (EIRPMAX ≤ 33dBm) |
|  | ACP | -4.5 | -10 |  (EIRPMAX > 33dBm) |
|  |  |  |  | -2 (EIRPMAX ≤ 33dBm) |
| 32.125 GHz < f ≤ 40.8 GHz | CP | 8.4 | 2 | 1.0 (EIRPMAX > 33dBm) |
|  |  |  |  | 8.0 (EIRPMAX ≤ 33dBm) |
|  | ACP | -7.6 | -14 | .0 (EIRPMAX > 33dBm) |
|  |  |  |  | -8.0 (EIRPMAX ≤ 33dBm) |

With our views, SNR is proportional to MPR, ChBW and dir, i.e. SNR = SNRA1,MPR=0,400MHz – MPR + ChBW  +Dir, where,

- Dir = difference of UE directivity assumption from 2x8 array, i.e. Dir=0[dB] for Assumption 1 and  Dir = 6[dB] for  Assumption 2.

- ChBW = 10log10(400MHz/ChBW).

Seeing the values in Table 1, at least we can conclude that it is infeasible to test ACLR as it is defined in core spec with the currently defined test methods.

Observation 1 : With assumption 1, FR2 ACLR is not feasible to test as is defined in core spec with the currently defined test methods.

Proposal 1 : With assumption 1, conclude that FR2 ACLR is not feasible to test as is defined in core spec with the currently defined test methods.

With assumption 2, there may be a chance that test can be done without relaxation only for lower MPR and narrower CBW case.

Observation 2 : With assumption 2, there may be a chance that test can be done without relaxation only for lower MPR and narrower CBW case.

We propose to adopt following values as a common SNR assumptions for ACP used to determine SNR and relaxation amount.

- FR2a : -4.5dB – MPR + ChBW  +Dir [dB]

- FR2b : -8.0dB – MPR + ChBW  +Dir [dB]

Proposal 2 : Agree on common SNR assumption for PC3 ACLR FR2a ACP : -4.5dB – MPR + ChBW  +Dir

Proposal 3 : Agree on common SNR assumption for PC3 ACLR FR2b ACP : -8.0dB – MPR + ChBW  +Dir

It is encouraged to decide treatment of Assumption 1 and Assumption 2 (Dir ), i.e. choose one of them or make it up to UE declaration, etc.

Proposal 4 : Decide treatment of Assumption 1 and Assumption 2 (Dir ), i.e. choose one of them or make it up to UE declaration, etc.

# 4.	Conclusion

Observation 1 : With assumption 1, FR2 ACLR is not feasible to test as is defined in core spec with the currently defined test methods.

Proposal 1 : With assumption 1, conclude that FR2 ACLR is not feasible to test as is defined in core spec with the currently defined test methods.

Observation 2 : With assumption 2, there may be a chance that test can be done without relaxation only for lower MPR and narrower CBW case.

Proposal 2 : Agree on common SNR assumption for PC3 ACLR FR2a ACP : -4.5dB – MPR + ChBW  +Dir

Proposal 3 : Agree on common SNR assumption for PC3 ACLR FR2b ACP : -8.0dB – MPR + ChBW  +Dir

Proposal 4 : Decide treatment of Assumption 1 and Assumption 2 (Dir ), i.e. choose one of them or make it up to UE declaration, etc.

# 5.	References

[1] R5-192663, “WF on Noise Impact on RF Measurements for FR2”, Keysight/Anritsu/R&S, RAN5#82

[2] R5-193104, “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5 NR#5 AdHoc

[3] R5-192984, “On the SNR for FR2 TRx test cases”, Rohde & Schwarz, RAN5 NR#5 AdHoc

[4] R5-193187, “On noise impact”, Keysight Technology, RAN5 NR#5 AdHoc

[5] R5-194352, “On the minimum TRP assumption for FR2 ACLR MU”, Anritsu, RAN5#83

# Annex A

Table A.1 shows SNR values for ACP based on common SNR assumptions in Proposal 2 and Proposal 3. Green background cell indicates SNR is better than 6dB, then testable with SNR=1.0[dB].

Table A.1. SNR values for ACP based on Proposal 2 and Proposal 3

| BW | MPR | FR2a |  | FR2b |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Assumption 1 | Assumption 2 | Assumption 1 | Assumption 2 |
|  |  | ACP | ACP | ACP | ACP |
| 50MHz | 0 | 4.53 | 10.53 | 1.03 | 7.03 |
|  | 2 | 2.53 | 8.53 | -0.97 | 5.03 |
|  | 2.5 | 2.03 | 8.03 | -1.47 | 4.53 |
|  | 3 | 1.53 | 7.53 | -1.97 | 4.03 |
|  | 3.5 | 1.03 | 7.03 | -2.47 | 3.53 |
|  | 4 | 0.53 | 6.53 | -2.97 | 3.03 |
|  | 4.5 | 0.03 | 6.03 | -3.47 | 2.53 |
|  | 5 | -0.47 | 5.53 | -3.97 | 2.03 |
|  | 5.5 | -0.97 | 5.03 | -4.47 | 1.53 |
|  | 6.5 | -1.97 | 4.03 | -5.47 | 0.53 |
| 100MHz | 0 | 1.52 | 7.52 | -1.98 | 4.02 |
|  | 2 | -0.48 | 5.52 | -3.98 | 2.02 |
|  | 2.5 | -0.98 | 5.02 | -4.48 | 1.52 |
|  | 3 | -1.48 | 4.52 | -4.98 | 1.02 |
|  | 3.5 | -1.98 | 4.02 | -5.48 | 0.52 |
|  | 4 | -2.48 | 3.52 | -5.98 | 0.02 |
|  | 4.5 | -2.98 | 3.02 | -6.48 | -0.48 |
|  | 5 | -3.48 | 2.52 | -6.98 | -0.98 |
|  | 5.5 | -3.98 | 2.02 | -7.48 | -1.48 |
|  | 6.5 | -4.98 | 1.02 | -8.48 | -2.48 |
| 200MHz | 0 | -1.49 | 4.51 | -4.99 | 1.01 |
|  | 2 | -3.49 | 2.51 | -6.99 | -0.99 |
|  | 2.5 | -3.99 | 2.01 | -7.49 | -1.49 |
|  | 3 | -4.49 | 1.51 | -7.99 | -1.99 |
|  | 3.5 | -4.99 | 1.01 | -8.49 | -2.49 |
|  | 4 | -5.49 | 0.51 | -8.99 | -2.99 |
|  | 4.5 | -5.99 | 0.01 | -9.49 | -3.49 |
|  | 5 | -6.49 | -0.49 | -9.99 | -3.99 |
|  | 5.5 | -6.99 | -0.99 | -10.49 | -4.49 |
|  | 6.5 | -7.99 | -1.99 | -11.49 | -5.49 |
| 400MHz | 0 | -4.50 | 1.50 | -8.00 | -2.00 |
|  | 2 | -6.50 | -0.50 | -10.00 | -4.00 |
|  | 2.5 | -7.00 | -1.00 | -10.50 | -4.50 |
|  | 3 | -7.50 | -1.50 | -11.00 | -5.00 |
|  | 3.5 | -8.00 | -2.00 | -11.50 | -5.50 |
|  | 4 | -8.50 | -2.50 | -12.00 | -6.00 |
|  | 4.5 | -9.00 | -3.00 | -12.50 | -6.50 |
|  | 5 | -9.50 | -3.50 | -13.00 | -7.00 |
|  | 5.5 | -10.00 | -4.00 | -13.50 | -7.50 |
|  | 6.5 | -11.00 | -5.00 | -14.50 | -8.50 |

# Annex A

# Table A-1. MPR and Minimum TRP for each TP

|  |  |  |  |  |  |  | Min TRP (Assumption 1) |  |  |  | Min TRP (Assumption 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | FR2a |  | FR2b |  | FR2a |  | FR2b |  |
| Test ID | Modulation | RB allocation | BW | MPRnarrow | MPRWT | MPR max(MPRnarrow, MPRWT) | CP | ACP | CP | ACP | CP | ACP | CP | ACP |
| 1 | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left | BW ≤ 200MHz | 2.5 | 2 | 2.5 | 2.2 | -14.8 | 0.4 | -15.6 | 8.2 | -8.8 | 6.4 | -9.6 |
|  |  |  | BW = 400MHz | 2.5 | 3 | 3 | 1.7 | -15.3 | -0.1 | -16.1 | 7.7 | -9.3 | 5.9 | -10.1 |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right | BW ≤ 200MHz | 2.5 | 2 | 2.5 | 2.2 | -14.8 | 0.4 | -15.6 | 8.2 | -8.8 | 6.4 | -9.6 |
|  |  |  | BW = 400MHz | 2.5 | 3 | 3 | 1.7 | -15.3 | -0.1 | -16.1 | 7.7 | -9.3 | 5.9 | -10.1 |
| 3 | DFT-s-OFDM PI/2 BPSK | Outer_Full | BW ≤ 200MHz | 0 | 2 | 2 | 2.7 | -14.3 | 0.9 | -15.1 | 8.7 | -8.3 | 6.9 | -9.1 |
|  |  |  | BW = 400MHz | 0 | 3 | 3 | 1.7 | -15.3 | -0.1 | -16.1 | 7.7 | -9.3 | 5.9 | -10.1 |
| 4 | DFT-s-OFDM QPSK | Outer_1RB_Left | BW ≤ 200MHz | 2.5 | 2 | 2.5 | 2.2 | -14.8 | 0.4 | -15.6 | 8.2 | -8.8 | 6.4 | -9.6 |
|  |  |  | BW = 400MHz | 2.5 | 3 | 3 | 1.7 | -15.3 | -0.1 | -16.1 | 7.7 | -9.3 | 5.9 | -10.1 |
| 5 | DFT-s-OFDM QPSK | Outer_1RB_Right | BW ≤ 200MHz | 2.5 | 2 | 2.5 | 2.2 | -14.8 | 0.4 | -15.6 | 8.2 | -8.8 | 6.4 | -9.6 |
|  |  |  | BW = 400MHz | 2.5 | 3 | 3 | 1.7 | -15.3 | -0.1 | -16.1 | 7.7 | -9.3 | 5.9 | -10.1 |
| 6 | DFT-s-OFDM QPSK | Outer_Full | BW ≤ 200MHz | 0 | 2 | 2 | 2.7 | -14.3 | 0.9 | -15.1 | 8.7 | -8.3 | 6.9 | -9.1 |
|  |  |  | BW = 400MHz | 0 | 3 | 3 | 1.7 | -15.3 | -0.1 | -16.1 | 7.7 | -9.3 | 5.9 | -10.1 |
| 7 | DFT-s-OFDM 16 QAM | Outer_1RB_Left | BW ≤ 200MHz | 2.5 | 3.5 | 3.5 | 1.2 | -15.8 | -0.6 | -16.6 | 7.2 | -9.8 | 5.4 | -10.6 |
|  |  |  | BW = 400MHz | 2.5 | 4.5 | 4.5 | 0.2 | -16.8 | -1.6 | -17.6 | 6.2 | -10.8 | 4.4 | -11.6 |
| 8 | DFT-s-OFDM 16 QAM | Outer_1RB_Right | BW ≤ 200MHz | 2.5 | 3.5 | 3.5 | 1.2 | -15.8 | -0.6 | -16.6 | 7.2 | -9.8 | 5.4 | -10.6 |
|  |  |  | BW = 400MHz | 2.5 | 4.5 | 4.5 | 0.2 | -16.8 | -1.6 | -17.6 | 6.2 | -10.8 | 4.4 | -11.6 |
| 9 | DFT-s-OFDM 16 QAM | Outer_Full | BW ≤ 200MHz | 0 | 3.5 | 3.5 | 1.2 | -15.8 | -0.6 | -16.6 | 7.2 | -9.8 | 5.4 | -10.6 |
|  |  |  | BW = 400MHz | 0 | 4.5 | 4.5 | 0.2 | -16.8 | -1.6 | -17.6 | 6.2 | -10.8 | 4.4 | -11.6 |
| 10 | DFT-s-OFDM 64 QAM | Outer_Full | BW ≤ 200MHz | 0 | 5.5 | 5.5 | -0.8 | -17.8 | -2.6 | -18.6 | 5.2 | -11.8 | 3.4 | -12.6 |
|  |  |  | BW = 400MHz | 0 | 6.5 | 6.5 | -1.8 | -18.8 | -3.6 | -19.6 | 4.2 | -12.8 | 2.4 | -13.6 |
| 11 | CP-OFDM QPSK | Outer_1RB_Left | BW ≤ 200MHz | 2.5 | 4 | 4 | 0.7 | -16.3 | -1.1 | -17.1 | 6.7 | -10.3 | 4.9 | -11.1 |
|  |  |  | BW = 400MHz | 2.5 | 5 | 5 | -0.3 | -17.3 | -2.1 | -18.1 | 5.7 | -11.3 | 3.9 | -12.1 |
| 12 | CP-OFDM QPSK | Outer_1RB_Right | BW ≤ 200MHz | 0 | 4 | 4 | 0.7 | -16.3 | -1.1 | -17.1 | 6.7 | -10.3 | 4.9 | -11.1 |
|  |  |  | BW = 400MHz | 0 | 5 | 5 | -0.3 | -17.3 | -2.1 | -18.1 | 5.7 | -11.3 | 3.9 | -12.1 |
| 13 | CP-OFDM QPSK | Outer_Full | BW ≤ 200MHz | 0 | 4 | 4 | 0.7 | -16.3 | -1.1 | -17.1 | 6.7 | -10.3 | 4.9 | -11.1 |
|  |  |  | BW = 400MHz | 0 | 5 | 5 | -0.3 | -17.3 | -2.1 | -18.1 | 5.7 | -11.3 | 3.9 | -12.1 |