3GPP TSG-RAN5 #84 Meeting	R5-19673

# Ljubljana, Slovenia, 13th – 17th May. 2019

Title:							Impact of noise for FR2 OBW

Source:			Anritsu

Agenda Item:			5.3.5.17

Document for:			Endorsement

# 1.	Introduction

OBW MUs have been discussed in [1-5] but no conclusion was reached as the discussion about discrepancy of OBW and ACLR core requirement were held in RAN4. That discussion is closed in RAN4 with the conclusion that we additionally apply MPR for some combination of waveform and RB allocations, while keeping OBW requirement of 99% ChBW. By applying the MPRs, the actual ACLR will be improved from those specified in ACLR core requirement, then OBW core requirement could be met. In this paper, we propose impact of noise MU for FR2 OBW measurement based on the updated information.

# 2.	Discussion

Impact from noise is analysed in [5] for some assumed values. We provide analysis of measured OBW depending on ACLR of the DUT and SNR of test equipment based on more realistic assumptions.

# Assumptions

- OBW is calculated as the frequency width containing the power of 1% of total power in 2*BW.

- Shape of the spectrum as depicted in Figure 1.

Figure 1 Assumed spectrum

Fig 2. shows the impact for measured OBW for various SNR and ACLR assumptions for BW=400MHz (TxBW=380.16MHz).

Figure 2 Measured OBW for various SNR and ALCR assumptions

We observe :

# Observation 1 : UE ACLR and TE noise jointly affect the measured OBW

In spite of observation 1, as the impact from ACLR is emitted from DUT and is target of measurement, then no need to count it as measurement uncertainty.

# Observation 2 : No need to count impact from ACLR of DUT to MU

# Fig 3. shows the impact from noise (as Error [%ChBW]) which is derive assuming OBW with SNR=+inf  is the true value. Note we confirmed that we obtain almost the same result regardless of ChBW then Fig 3 is applicable regardless of ChBW.

Figure 3 Impact from noise [%ChBW] for various SNR and ALCR assumptions

From Fig 3, we can say SNR 40dB would achieve ignorable impact from noise. However, considering the SNR estimations ever provided by TE vendors, it is unreachable in mmWave OTA test environment, then is not ignorable.

# Observation 3 : In FR2 OBW OTA test environment, impact from noise is not ignorable.

According to the some offline discussion, there was information from some UE/chip vendors that the actual ACLR will be 24dBc. From Fig2, if we still leave ~7MHz margin from test requirement for ACLR = 24dBc, SNR of 26dB will be required.

According to the latest core specification, For PC3, BW=400MHz, MPR of 5.0dB is the worst case value.

Table 6.2.2.3-2 MPRWT for power class 3, BWchannel = 400 MHz

| Modulation |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations, Region 1 | Edge RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | ≤ 3.0 |
|  | QPSK | 0.0 | ≤ 3.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 5.0 |
|  | 16 QAM | ≤ 6.5 | ≤ 6.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 9.0 |

Hence, the worst case assumed power for FR2a and FR2b channel powers are calculated with min peak EIRP – multi-band-relaxation – MPR as below :

- FR2a : 22.4-1.7-5=15.7dBm/400MHz

- FR2b : 20.6-0.4-5=15.2dBm/400MHz

TE needs to achieve SNR >= 26dB for these levels. In the some already provided SNRs in[6] and [7], the SNR is estimated as 18dB for FR2a and 13dB for FR2b ( if simply subtract 5dB from SNR for MOP peak EIRP ).

With our latest view, 26dB SNR is borderline in FR2a for ChBW <= 400MHz, but not likely to be achievable in FR2b. Roughly says, FR2b SNR will be -5 ~ -8dB of FR2a, then we get SNR < ~ 20dB which is obviously not applicable for OBW measurement according to Fig 2 and Fig 3.

# Proposal 1 : For PC3 FR2a ChBW <= 400MHz, impact from noise for OBW is 2.0% of ChBW

# 2.1. Ideas to deal with testability issue

For FR2 ChBW > 400MHz(i.e. contiguous UL CA) and FR2b, we need to take this testability issue into account. Several ideas to deal with this issue are listed below.

# Option 1 : Narrower OBW measurement span

# Historically, we applied 2 times or wider of channel bandwidth as OBW measurement span. If we narrower it e.g. 1.5 times of channel bandwidth, then we got less impact from noise. It can be noted that taking the OBW measurement span of 2 times or more is not specified in core specification. Some existing test methodology such as ITU-R SM 443-4[8] applies 1.5 times of channel BW. Fig 4 shows the difference of OBW measurement error[%ChBW] for OBW measurement span = 2.0*ChBW and 1.5*ChBW.

Figure 4 Effect of narrower OBW measurement span (2.0*ChBW vs 1.5*ChBW)

# Option 2 : Limit the tested channel bandwidth

SNR will be improved (+3dB for 1/2 BW) then less impact from noise is expected.

# Option 3 : Test with non-full RB allocation

# This would reduce the effective ACLR, and then testability would be improved.

# Option 4: Others not precluded

# Proposal 2 : For FR2a ChBW > 400MHz  and FR2b, RAN5 to study how to deal with testability issue considering Option 1 – 4.

# 3.	Conclusion

# Observation 1 : UE ACLR and TE noise jointly affect the measured OBW

# Observation 2 : No need to count impact from ACLR of DUT to MU

# Observation 3 : In FR2 OBW OTA test environment, impact from noise is not ignorable.

# Proposal 1 : For PC3 FR2a ChBW <= 400MHz, impact from noise for OBW is 2.0% of ChBW

# Proposal 2 : For FR2a ChBW > 400MHz  and FR2b, RAN5 to study how to deal with testability issue considering Option 1 – 4.

# Option 1 : Narrower OBW measurement span

# Option 2 : Limit the tested channel bandwidth

# Option 3 : Test with non-full RB allocation

# Option 4: Others not precluded

# 4.	References

[1] R5-188189, “On the FR2 MU for occupied BW and ACLR”, Anritsu, RAN5#81

[2] R5-190390, “On QoQZ MU Impact on OBW”, Keysight, RAN5 NR#4 AdHoc

[3] R5-192660, “On the FR2 OBW MU”, Anritsu, RAN5#82

[4] R5-191617, “On OBW measurement uncertainty”, Keysight, RAN5#82

[5] R5-194353, “On the noise impact to FR2 OBW”, Anritsu, RAN5#83

[6] R5-192984 , “On the SNR for FR2 TRx test cases”, R&S, Rohde & Schwarz, RAN5 NR#5 AdHoc

[7] R5-193187, “On noise impact”, Keysight, RAN5 NR#5 AdHoc

[8] ITU-R SM 443-4, “SM.443 : Bandwidth measurement at monitoring stations”, https://www.itu.int/rec/R-REC-SM.443-4-200702-I/en