3GPP TSG RAN WG5 #84 Meeting	R5-196852

Ljubljana, Slovenia, 26th – 30th August 2019

Source:		Rohde & Schwarz

Title:	On the SNR for FR2 TRx test cases

Agenda Item:		5.3.5.17

Document for:	Endorsement

# Introduction

At RAN5 #83, estimations for the SNR have been discussed [1,2,3] and some terms have been agreed among TE vendors, e.g., relaxation for OFF power in FR2a and impact of noise. In this document, we present the results of our analysis based on the common assumptions for the not yet decided figures.

# Discussion

A detailed description of the total SNRtotal based on the measurement of total component power for both polarizations can be found in reference [4].

In Table 1, we present our results for SNRtotal under the assumption that the UE is allowed to transmit a maximum peak EIRP of 43 dBm.

Regarding the MOP spherical coverage, it has to be emphasized that the uncertainty of the test system has to be based on the test requirement as the latter decides whether an UE passes or fails the test.

Observation 1: The test requirement is the decision criterion whether an UE passes or fails the test and thus the MU should be derived for this level.

Proposal 1: Determine the impact of noise for the spherical coverage based on the test requirement, i.e., the reduction by the test tolerance has to be considered.

Unfortunately, the test tolerance discussion cannot come to an conclusion without the value for the MU. This resembles an chicken and egg problem. Proposals for the spherical coverage TT were TT = 2dB [5] and a factor based approach TT = 0.65*MU dB [6]. A rigorous analysis of the factor approach would require a self-consistent solution when taking the form of a typical CDF into account.

The expanded system uncertainty without the impact of noise is 4.19 dB.

For simplicity reasons, we assume as worst case a TT = 0.65*5.0 dB = 3.25 dB. This may be updated during RAN5#84 depending on the TT discussion.

Among TE vendors it has been decided to study the SNR for ACLR based on the following two assumptions:

ACLR Assumption 1

- Assumption for min TRP power value on wanted channel shall be: min EIRP – 16 dB (UE antenna (8x2) directivity) - MPR

- Assumption for min TRP power value on adjacent channel shall be: min EIRP – 16 dB (UE antenna (8x2) directivity) – MPR - ACLR requirement

ACLR Assumption 2

- Assumption for min TRP power value on wanted channel shall be:  min EIRP – 10 dB (UE antenna (1x4) directivity) - MPR

- Assumption for min TRP power value on adjacent channel shall be:  min EIRP – 10 dB (UE antenna (1x4) directivity) – MPR - ACLR requirement

The MPR depends on waveform, modulation and RB allocation. For the currently defined test points the highest MPR occurs for 400 MHz channel bandwidth and 64QAM DFT-s-OFDM with a value of 6.5 dB. It is noted that the MU and SNR analysis has to be updated once CP-OFDM 64QAM were applied as test point for ACLR.

Observation 2: With ACLR assumption 1 the impact of noise is already very high for the measurement of the wanted signal. This could only be avoided by selecting test points with low MPR.

Observation 3: Independent of the ACLR assumption ACLR needs considerable amount of relaxation in order to be testable.

Table 1: SNRtotal

| Test case | Frequency (GHz) | Estimated SNRtotal(dB) | Requirement (dBm) | Impact of Noise w/o relaxation (dB) | Required Relaxation |
| --- | --- | --- | --- | --- | --- |
| MOP Spherical coverage | 23.45 ≤ f ≤ 32.125 | [14.1]1 | 6.5 (= 11.5 – 1.75 – 3.25) | [0.17]1 | no |
|  | 32.125 < f ≤ 40.8 | [9.8]1 | 4.35 (= 8 – 0.4 – 3.25) | [0.44]1 | no |
| Transmit OFF Power (TRP) | 32.125 < f ≤ 40.8 | -29.5 | -35 | 29.5 | do not test if tested relax by 39.5 dB with SNR impact = 0.42 dB |
| ACLR AC (TRP) Assumption 1 | 23.45 ≤ f ≤ 32.125 | -11.2 | -18.8 (= 20.7 – 16 – 17 – 6.5) | 11.52 | yes |
|  | 32.125 < f ≤ 40.8 | -14.1 | -19.6 (= 18.9 – 16 – 16 – 6.5) | 14.27 | yes |
| ACLR wanted signal (TRP) Assumption 1 | 23.45 ≤ f ≤ 32.125 | 5.8 | -1.8 (= 20.7 – 16 – 6.5) | 1.02 | yes |
|  | 32.125 < f ≤ 40.8 | 1.9 | -3.6 (= 18.9 – 16 – 6.5) | 2.17 | yes |
| ACLR AC (TRP) Assumption 2 | 23.45 ≤ f ≤ 32.125 | -5.2 | -12.8 (= 20.7 – 10 – 17 – 6.5) | 6.35 | yes |
|  | 32.125 < f ≤ 40.8 | -8.1 | -13.6 (= 18.9 – 10 – 16 – 6.5) | 8.73 | yes |
| ACLR wanted signal (TRP) Assumption 2 | 23.45 ≤ f ≤ 32.125 | 11.8 | 4.2 (= 20.7 – 10 – 6.5) | 0.28 | no |
|  | 32.125 < f ≤ 40.8 | 7.9 | 2.4 (= 18.9 – 10 – 6.5) | 0.66 | no |
| Spurious Emissions for UE co-existence | work in progress |  |  |  |  |
| Transmitter Spurious Emissions | work in progress |  |  |  |  |
| Receiver Spurious Emissions | work in progress |  |  |  |  |
| NOTE 1: Rough estimate based on the 50%-tile power level requirement. A more rigorous analysis will be based on an acceptable CDF. |  |  |  |  |  |

Proposal 2: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements 6 GHz – 80 GHz for spurious emissions measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |
| #6 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, SNR estimations for low PSD test cases have been provided and the impact on the systematic uncertainty has been analyzed.

Observation 1: The test requirement is the decision criterion whether an UE passes or fails the test and thus the MU should be derived for this level.

Proposal 1: Determine the impact of noise for the spherical coverage based on the test requirement, i.e., the reduction by the test tolerance has to be considered.

Observation 2: With ACLR assumption 1 the impact of noise is already very high for the measurement of the wanted signal. This could only be avoided by selecting test points with low MPR.

Observation 3: Independent of the ACLR assumption ACLR needs considerable amount of relaxation in order to be testable.

Proposal 2: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise.

# References

- R5-194351, On the noise impact for FR2 priority 1 and 2 TRx TCs, Anritsu, 3GPP TSG RAN WG5 Meeting #83, May 2019

- R5-194133, On noise impact, Keysight Technologies UK Ltd, 3GPP TSG RAN WG5 Meeting #83, May 2019

- R5-194353, On the noise impact to FR2 OBW, Anritsu, 3GPP TSG RAN WG5 Meeting #83, May 2019

- R5-192125, On the SNR for FR2 TRx test cases, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #82, February 2019

- R5-192657, Spherical Coverage TT Proposal, Telecom Italia, Orange, AT&T, Vodafone, CMCC, China Unicom, 3GPP TSG RAN WG5 Meeting #82, February 2019

- R5-191836, Discussion on FR2 Min EIRP and EIS spherical coverage TT for PC3 UE, Qualcomm Inc., 3GPP TSG RAN WG5 Meeting #82, February 2019