3GPP TSG-RAN WG5 #85		R5-198261
Reno, NV, USA, 18 – 22 November 2019

Agenda item:	5.3.5.17

Source:	Keysight Technologies

Title:	On 15cm Quiet Zone

Document for:	Discussion and Endorsement

# Introduction

This contribution outlines ways forward regarding the 15cm Quiet Zone which is insufficient to test PC3 handheld devices like smartphones.

# Discussion

Parts of this contribution were initially presented and discussed in RAN4 [1] and are now presented in RAN5 following the advice from the RAN4 chair to present quiet zone size related contributions in RAN5 instead of RAN4.

Various quiet zone/test zone sizes have been introduced for OTA test plans and tabulated in Table 1.

Table 1: Overview of currently defined OTA quiet zones and test zone sizes.

| Quiet Zone/ Test Zone Size | Additional QZ Information | Applicability | Reference |
| --- | --- | --- | --- |
| 15cm | Spherical QZ with diameter of 15cm | UE (RF, DEMOD, RRM) NR FR2 testing for smartphone UEs | [2][3] |
| 20cm | Spherical QZ with diameter of 20cm | NR FR1 and FR2 MIMO OTA for handset devices | [4] |
| 30cm | Cylindrical QZ with diameter and height of 30cm | 2G/3G/4G SISO and LTE MIMO OTA testing | [5][6] |
| 30cm | Spherical QZ with diameter of 30cm | UE (RF, DEMOD, RRM) NR FR2 testing | [2][3] |
| 50cm | Cylindrical QZ with diameter of 50cm and height of 36cm | 2G/3G/4G SISO OTA testing of notebooks and tablets | [5] |

The 15cm spherical QZ size for NR FR2 was defined in 2017 by 3GPP RAN4 as part of the FS_NR_testMethods Study Item in [7] and seemed rather arbitrary

The MU assumptions of this SI are based on a smartphone DUT [RAN4 #82bis R4-1704396] with max quiet zone size of 15cm in diameter

The 20cm test zone size was introduced for handheld UEs in 2019 as part of the NR_MIMO_OTA_TestMethod Study Item in [8]. This definition was based on a thorough review of dimensions for 135 different LTE capable handsets certified in CAICT’s lab in 2018. The conclusion of the CDF analysis was that a 20cm test zone size covers (almost) all smartphones; it also shows none of the tested LTE smartphones could fit into the 15cm quiet zone size.

Figure 1: The CDF curve of 135 wide LTE handsets (width >72mm) in 2018 CTA data pool from CAICT OTA lab (from [8])

To determine the applicability of current, commercially available smartphones to a 15cm quiet zone, a brief survey of smartphone dimension was conducted to study the required quiet zone sizes for these devices. All of these models support LTE and were listed on at least one of the US operator’s websites in September 2019. The results of the survey are tabulated in Table 2.

Table 2: Survey of commercially available smartphones on US operator websites (September 2019)

| Models | Typical Dimensions [cm] |  |  | Min. Spherical Quiet Zone Diameter [cm] |
| --- | --- | --- | --- | --- |
|  | Height | Width | Depth |  |
| Apple iPhone 11 Pro Max | 15.8 | 7.8 | 0.8 | 17.6 |
| Apple iPhone 11 Pro | 14.4 | 7.1 | 0.8 | 16.1 |
| Apple iPhone 11 | 15.1 | 7.6 | 0.8 | 16.9 |
| Samsung Galaxy Note 10 | 15.1 | 7.2 | 0.8 | 16.7 |
| Samsung Galaxy Note 10+ | 16.2 | 7.7 | 0.8 | 18.0 |
| Samsung Galaxy S10+ | 15.7 | 7.4 | 0.8 | 17.4 |
| Samsung Galaxy S10e | 14.2 | 7.0 | 0.8 | 15.9 |
| Samsung Galaxy S10 | 15.0 | 7.0 | 0.8 | 16.6 |
| Samsung Fold | 16.1 | 11.8 | 0.7 | 19.9 |
| LG G8 ThinQ | 15.2 | 7.2 | 0.8 | 16.9 |
| LG Stylo 4+ | 16.0 | 7.8 | 0.8 | 17.8 |
| Kyocera DuraForce Pro 2 | 15.0 | 7.3 | 1.4 | 16.8 |
| Razer Phone 2 | 15.8 | 7.9 | 0.8 | 17.7 |
| LG V40 ThinQ | 15.9 | 7.6 | 0.8 | 17.6 |
| Motorola moto z4 | 15.8 | 7.5 | 0.7 | 17.5 |
| Google Pixel 3 | 14.6 | 6.8 | 0.8 | 16.1 |
| Google Pixel 3 XL | 15.8 | 7.7 | 0.8 | 17.6 |
| Samsung Galaxy A50 | 15.7 | 7.4 | 0.8 | 17.4 |
| LG Stylo 5 | 16.1 | 7.8 | 0.9 | 17.9 |
| OnePlus 7 Pro 5G | 16.3 | 7.6 | 0.9 | 18.0 |
| moto e 6th gen | 15.0 | 7.2 | 0.9 | 16.6 |

Clearly, none of the devices fit inside a 15cm quiet zone.

Observation 1: The 15cm quiet zone is insufficient for common LTE smartphones.

The RAN5 technical specification [9] already adopted the 15cm diameter (7.5cm radius) and the 30cm diameter (15cm radius) spherical quiet zones, as outlined in Clause O.2.4.

### O.2.4 Size of the quiet zone The size of the quiet zone within which the variations of measurements are evaluated depends on the size of the DUT. For smartphones, the quiet zone shall be considered a sphere with radius of R=7.5cm. For larger smartphones and tablet type devices, the quiet zone shall be considered a sphere with radius of R=15cm. Alternate quiet zone sizes can be defined for even larger DUTs. The quality of quiet zone procedure for systems supporting larger quiet zone sizes can be performed for the largest quiet zone radius only and the results can be applied to the smaller quiet zone radius. Performing separate sets of quality of quiet zone measurements for different radii is not precluded.

Sample quality of quiet zone MU values have been presented for both previously agreed quiet zones [10][11] and been included [12] The MUs agreed for the two different quiet zones are tabulated in Table 3.

Table 3: Quality of Quiet Zone MU Values for MU spreadsheet to determine MTSU

|  | 15cm QZ MU | 30cm QZ MU |
| --- | --- | --- |
| Quality of QZ – TRP, EIRP, EIS, Spherical Coverage | 0.6 dB | 0.5 dB |
| Quality of QZ for Calibration Process | 0.4 dB | 0.2 dB |

Observation 2: The 15cm Quality of Quiet Zone MUs (in DUT measurement and range reference stage) are greater than the 30cm MUs.

In the last meeting, RAN5 agreed to track just a single MU template/table taking only the largest MU contributors into account for quiet zones less or equal to 30cm [13][14]. It should be noted that RAN5 did not endorse any arbitrary quiet zone definition less or equal to 30cm but merely to use the worst-case MU values of the 15cm quiet zone.

Proposal 1: Use only one measurement uncertainty in the specification for all quiet zone sizes less or equal to 30 cm based on the values for 15 cm quiet zone size (worst case).

While all FR2 requirements so far have been defined for the free-space test condition, it is well known that the free space test condition does not reflect real-world conditions which are emulated more appropriately with phantoms. Most 3GPP technical specifications related to OTA test cases include various phantoms [16][17] with phantoms adopted from the CTIA test plan [5]. CTIA is currently actively developing phantoms for mm-wave [18] and RAN4 has already expressed interest to CTIA in taking phantoms into account to develop corresponding FR2 requirements [19].

Observation 3: OTA testing with phantoms reflects more realistic conditions and RAN4 has already expressed interest in defining requirements using phantoms

Since the 15cm quiet zone is too small to test smartphone UEs, various way forwards have been discussed in CTIA [15], 3GPP RAN4 [1][20] and offline between interested parties. The potential way forwards are summarized in Table 4.

Table 4: Way forward options to handle quiet zones in RAN5

| Option | “Small QZ” | “Largest” QZ | Advantage | Disadvantage |
| --- | --- | --- | --- | --- |
| Keep 15cm and 30cm QZs | 15cm | 30cm | - Smaller footprint for 15cm QZ systems possible - Sample MUs were provided for both QZs | - Smartphones do not fit into the 15cm QZ - MTSU is increased unnecessarily - Very large portion of phantoms would be outside of the 15cm QZ |
| Replace 15cm QZ with 20cm QZ | 20cm | 30cm | - Smaller footprint for 20cm QZ systems possible | - No MU for 20cm QoQZ available to finalize MTSU - MTSU is likely increased unnecessarily - Large portion of phantoms would be outside of the 20cm QZ |
| Allow any arbitrary QZ sizes less or equal to 30cm | Any QZ<30cm | 30cm | - Test systems can be optimized in size for smaller QZ sizes | - No clear approach how MTSU is determined for QZ sizes ≤ 30cm - MTSU is likely increased unnecessarily - Large portion of phantoms would be outside of QZs less than 30cm - Confusion caused with multiple arbitrary QZ |
| Abandon the 15cm QZ and focus only on the 30cm QZ | N/A | 30cm | - Minimal changes in 38.521-2 - MTSU is minimized - No confusion about different QZ sizes - Alignment with other OTA test plans - Forward compatibility with phantoms | - no opportunity for size reduced systems due to smaller quiet zone. |

Weighing the pros and cons of the various options, the approach to abandon the 15cm quiet zone is proposed and to focus on the 30cm quiet zone in RAN5.

Proposal 1: Abandon the 15cm quiet zone for PC3 smartphones and handsets and focus only on the 30cm quiet zone for smartphone UEs and handhelds.

# Conclusion

The following observations and proposals were made in this contribution

Observation 1: The 15cm quiet zone is insufficient for common LTE smartphones.

Observation 2: The 15cm Quality of Quiet Zone MUs (in DUT measurement and range reference stage) are greater than the 30cm MUs.

Observation 3: OTA testing with phantoms reflects more realistic conditions and RAN4 has already expressed interest in defining requirements using phantoms

Proposal 1: Abandon the 15cm quiet zone for PC3 smartphones and handsets and focus only on the 30cm quiet zone for smartphone UEs and handhelds.

# References

- R4-1912097, On Quiet Zone Sizes for PC3 Devices, Keysight Technologies, 3GPP TSG-RAN WG4 #92bis, October 2019

- TR38.810, Study on test methods, V16.3.0 (2019-06)

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases, V16.0.0 (2019-06)

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE), V1.0.0 (2019-05)

- Test Plan for Wireless Device Over-the-Air Performance, CTIA, Version 3.8.2, April 2019

- Test Plan for 2x2 Downlink MIMO and Transmit Diversity Over-the-Air Performance, CTIA, Version 1.2, April 2018

- R4-1710077, WF on NR MU and test tolerance, CATR, Intel Corporation, Keysight Technologies, Rohde & Schwarz, Anritsu, 3GPP TSG-RAN WG4 Meeting NR#3, September 2017

- R4-1904183, Proposals on NR MIMO OTA, CAICT, SAICT, 3GPP TSG-RAN WG4 Meeting #90bis, April 2019

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.10 (2019-09)

- R5-185955, Quality of Quiet Zone Results for IFF and 15cm Quiet Zone, Keysight Technologies, 3GPPRAN5#3-5G-NR Adhoc, October 2018

- R5-185956, Quality of Quiet Zone Results for IFF and 30cm Quiet Zone, Keysight Technologies, 3GPPRAN5#3-5G-NR Adhoc, October 2018

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases, V16.0.0 (2019-06)

- R5-197498, Discussion on Quiet Zone sizes and measurement uncertainties, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #84, August 2019

- R5-197501, Update of FR2 MUs in TS 38.521-2, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #84, August 2019

- 5GmmW190903_R1*, On 15cm Quiet Zone, Keysight Technologies

- TS 34.114, User Equipment (UE) / Mobile Station (MS) Over The Air (OTA) antenna performance; Conformance testing, V12.2.0 (2016-09)

- TS 37.544, Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) Over The Air (OTA) performance; Conformance testing, V14.7.0 (2018-12)

- NFP191003*, Optimization of Performance as a Function of Head Weight & Size vs Performance of the mmWave CTIA/IEC Head Phantom, IT’IS Foundation

- R4-1808515, draft Response LS to CTIA on FR2 OTA Testing, Rohde&Schwarz, 3GPP TSG RAN WG4 Meeting #87, May 2018

- R4-1911805, Draft CR to TR 38.810: Size of the Quiet Zone, Rohde & Schwarz, 3GPP TSG-RAN4 Meeting #92-Bis, October 2019

* This is a CTIA Sub-Working Group Document available to CTIA Sub-Working Group members