3GPP TSG-RAN5 #85 Meeting	R5-198447

# Reno, USA, 18th – 22nd Nov. 2019

Title:							On FR2 spherical coverage MU

Source:			Anritsu

Agenda Item:			5.3.5.17

Document for:			Endorsement

# 1.	Introduction

Measurement uncertainty for spherical coverage test cases have been discussed in [1]-[6], but not yet finalized. This paper analyses and propose the MU values for spherical coverage test cases based on the WF[2]. There are 3 open points in the WF. The Points #1 and #2 are mentioned in this document. Point #3 is mentioned in the separated paper.

- Before concluding the noise impact for Tx spherical coverage, run simulations using realistic CDF curves provided by different UE vendors ([6], [7]), similar to the ones in [1].

- To be clarified whether noise impact can be considered as a constant systematic offset or if it includes some randomness

- To be clarified whether the determination of the impact of noise shall be based on the minimum requirement or on the test requirement

# 2.	Discussion

In [6], it is shown from us that the MU at 50%-tile EIRP does not depends on the shape of CDF curve based on the assumption of100%-correlation of MU elements among measurement grids, and such assumption was agreed in [1]. However, some requests from the group in RAN5#84 were to simulate the MUs using the provided reference CDF curves in [7] and [8]. In this section, we present the simulation results for the reference CDF curves.

# 2.1	Simulation method

EIRP MU can be classified to following components.

- Random MU

(a-1) Those with 100% non-correlated.

(a-2) Those with 100% correlated.

(a-3) Impact from grid

- Systematic error

(b-1) Impact from noise

Evaluation for variation of 50%-tile EIRP can be carried out with following steps. Here we divide the step into 2 methods, which was labelled as Noise Assumption A and Noise Assumption B. The difference of these 2 methods is the position to add noise for the CDF curve. Noise Assumption A applies the noise finally after applying other MUs, while Noise Assumption B applies the noise firstly before applying other MUs, which corresponds to the equation (1) and (2) in [6], respectively. The real situation is considered somewhere among these 2 cases. In the real equipment, the noise will distributed over the RF measurement path and subject to different amount of uncertainty hence not feasible to be evaluated exactly.

# Observation 1 : Different simulation method is possible depending on the place addition of noise. However, it is practically infeasible to assess which stage the noise is added and which uncertainty is applied, as the noise is distributed in the whole measurement path and subject to different amount of uncertainty.

SNC=<RSS of (a-1) elements>

SC=<RSS of (a-2) elements>

SG=<RSS of (a-3) elements>

- ----

- Noise Assumption A :

Add a single random error that follows the normal distribution with sigma = SC to reference CDF curve

Add independent random errors that follows the normal distribution with sigma = SNC to each sample of CDF curve obtained in step 4

Add noise to the obtained CDF curve in step 5. Noise power PNoise[dBm] = PNoiseMean [dBm] +  PNoiseVar[dB], where  PNoiseVar follows the normal distribution with variance Snoise dB

- Noise Assumption B :

- Add noise to the CDF curve. Noise power PNoise[dBm] = PNoiseMean [dBm] +  PNoise[dB], where  PNoiseVar follows the normal distribution with variance Snoise dB

Add a single random error that follows the normal distribution with sigma = SC to reference CDF curve

Add independent random errors that follows the normal distribution with sigma = SNC to each sample of CDF curve obtained in step 4

- ----

Step 4 – 6 are repeated for N realizations and derive the STD and Mean of 50%-tile value (write it as S’ and M’, respectively).

S50%-tile = RSS(S’, SG)

E50%-tile= 1.96* S50%-tile + abs(M’-M), where M is 50%-tile value in the original reference CDF curve,

The noise power PNoise is modelled with constant power having random variation, i.e. PNoise[dBm] = PNoiseMean [dBm] +  PNoiseVar [dB]. The random variation of measured noise power is already analyzed in [2].  When only the noise is measured, the random variation depends on mean time and MBW and can be modelled as the normal distribution having variance (α sigma) of 10log101+αMBW*MeanTime. Table 1 shows the example of variance for some combination of MBW and mean time. PNoiseVar is generated for each realization which follows the normal distribution with variance Snoise dB.

Table 1 Variance of mean power (1 sigma) (Sited from [2], adding 50MHz column)

|  |  | MBW [MHz] |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0.18 | 1 | 5 | 10 | 20 | 40 | 50 | 80 | 100 | 200 | 400 |
| Mean Time[ms] | 10 | 0.1012 | 0.0432 | 0.0194 | 0.0137 | 0.0097 | 0.0069 | 0.0061 | 0.0049 | 0.0043 | 0.0031 | 0.0022 |
|  | 1 | 0.3122 | 0.1352 | 0.0610 | 0.0432 | 0.0306 | 0.0217 | 0.0194 | 0.0153 | 0.0137 | 0.0097 | 0.0069 |
|  | 0.5 | 0.4352 | 0.1900 | 0.0860 | 0.0610 | 0.0432 | 0.0306 | 0.0274 | 0.0217 | 0.0194 | 0.0137 | 0.0097 |
|  | 0.25 | 0.6035 | 0.2663 | 0.1211 | 0.0860 | 0.0610 | 0.0432 | 0.0387 | 0.0306 | 0.0274 | 0.0194 | 0.0137 |
|  | 0.125 | 0.8308 | 0.3720 | 0.1703 | 0.1211 | 0.0860 | 0.0610 | 0.0546 | 0.0432 | 0.0387 | 0.0274 | 0.0194 |

# 2.2	Simulation parameters

Table 2 shows the simulation parameters used for the simulation. SNC, SC and SG are same as those used in [1] based on our analysis on the correlation of MU elements among grids one by one. For Snoise , the worst case value for FR2 spherical coverage is the minimum BW=50MHz and min measurement time 1ms(1subframe) which gives Snoise = 0.019[dB] according to Table 1. For simplicity and with even safer assumption, Snoise =0.1dB is simulated.

Table 2 Simulation parameters

|  | # 100% correlated | # Assumption 1 | # Assumption 2 |
| --- | --- | --- | --- |
| # SNC = <RSS of (a-1) elements> | # 0 | # 0.25 | # 1.19 |
| # SC = <RSS of (a-2) elements> | # 2.14 | # 2.12 | # 1.78 |
| # SG = <RSS of (a-3) elements> | # 0.12 | # 0.12 | # 0.12 |
| # PNoiseMean [dBm] | # Corresponds to SNR=0.0, 0.5 and 1.0 for 11.5dBm # (Core requirement level) |  |  |
| # Snoise[dB] | # 0.0 [dB] and 0.1[dB] |  |  |
| # Number of realizations N | # 1,000,000 |  |  |

Following CDF curves are simulated.

- Linear form CDF curve with 50%-tile on 11.5dBm and 100%-tile on 22.3 dBm (same as used in [6])

- Reference CDF curve provided in [7] from Qualcomm

- Reference CDF curve provided in [8] from Apple

- Reference CDF curve provided in [7] from Qualcomm but the curve is shifted so that 50%-tile value equals to 11.5dBm

- Reference CDF curve provided in [8] from Apple but the curve is shifted so that 50%-tile value equals to 11.5dBm

Figure 1 Simulated CDF curves

Note that the original (b) and (c) are not on the borderline of spherical coverage core requirement at 50%-tile. Because MU and TT need to be associated with the PASS/FAIL rate of borderline UEs [11], and because our concern is impact from relative relation of the CDF samples (shape of CDF curve) to the 50%-tile EIRP MU, we simulate the curve (d) and (e) which hold the shape of original curve but shifted so that 50%-tile becomes 11.5dBm(PC3 core requirement level). Note that (d) and (e) does not meet peak EIRP requirement, but we think it is not the matter for defining MU/TT for spherical coverage assuming they are independent requirements.

# 2.2	Simulation results

Figure 2 shows the expanded random uncertainty (1.96* S50%-tile) and Mean Error(M-M’) (shown as stacked bar graph) for Snoise=0.0.

Figure 2 Simulation result

# As seen Noise Assumption A and B gives different MU, and Noise Assumption B gives bigger total MU than A.

# Observation 2 : Place of noise to be added will give different MU values for 50%-tile EIRP.

# In case of Noise Assumption A, the mean error becomes slightly bigger than 10log101+SNR50%-tile -1 even for 100%-correlation case. Actually, this is generic mathematical nature that mean of  10log1010x/10+10C/10, where x follows 0 mean normal distribution, is bigger than C, and is nothing to do with CDF curves. Anyway the random uncertainty is less than Noise Assumption B and total MU is less than Noise Assumption B.

# Observation 3 : In Noise Assumption A, the offset from noise will be  slightly increased from 10log101+SNR-1. However, the other random uncertainty is smaller, and then the total MU is less than Noise Assumption B.

# Seeing all the cases, the Noise Assumption B with 100%-correlation of MU assumption gives the worst case MU and is equal to EIRP random uncertainty MU + 10log101+SNR50%-tile-1.

# Observation 4 : Noise Assumption B with 100% correlation of MUs among grids give the worst case MU assessment for all the assumed EIRP CDF curves(linear, QC[8], Apple[9]), and is equals to EIRP random uncertainty + 10log101+SNR50%-tile-1.

Noise Random Variation impact

Figure 3 and 4 show the expanded random uncertainty (1.96* S50%-tile) and Mean Error(M-M’) (shown as stacked bar graph) comparing the Snoise = 0.0 and Snoise = 0.1.

Figure 3 Impact from noise power variation (Noise Assumption A)

Figure 4 Impact from noise power variation (Noise Assumption B)

# We do not see any significant difference for Snoise=0.0 and Snoise=0.1 case. This conclusion matches with the analysis in [2]. Impact from noise power variation is ignorable level in spherical coverage tests.

# Observation 5 : The impact from noise power variation to the final 50%-tile EIRP is ignorable.

# 3.	Conclusion

We obtain following observations from the analysis in section 2.

# Observation 1 : Different simulation method is possible depending on the place addition of noise. However, it is practically infeasible to assess which stage the noise is added and which uncertainty is applied, as the noise is distributed in the whole measurement path and subject to different amount of uncertainty.

# Observation 2 : Place of noise to be added will give different MU values for 50%-tile EIRP.

# Observation 3 : In Noise Assumption A, the offset from noise will be  slightly increased from 10log101+SNR-1. However, the other random uncertainty is smaller, and then the total MU is less than Noise Assumption B.

# Observation 4 : Noise Assumption B with 100% correlation of MUs among grids give the worst case MU assessment for all the assumed EIRP CDF curves(linear, QC[8], Apple[9]), and is equals to EIRP random uncertainty + 10log101+SNR50%-tile-1.

# Observation 5 : The impact from noise power variation to the final 50%-tile EIRP is ignorable.

# RAN5 is asked to endorse following proposals.

Proposal 1 : Define the spherical coverage MU as (single point) EIRP MU with ∆SNR=10log101+SNR50%-tile-1, where SNR(linear) is defined for 50%-tile core requirement taking MBR and TE noise from 2 polarization into account.

# 4.	References

[1] R5-193834, “On the MU for EIRP spherical coverage”, Anritsu, RAN5#83

[2] R5-190059, “Impact from noise for FR2 TRx MU”, Anritsu, RAN5 NR#4 AdHoc

[3] R5-192984, “On the SNR for FR2 TRx test cases”, Rohde & Schwartz, RAN5 NR#5 AdHoc

[4] R5-194133, “On noise impact”, Keysight, RAN5#83

[5] R5-193104, “SNR estimation update for FR2 priority 1 and 2 TRx TCs “, Anritsu, RAN5 NR#5 AdHoc

[6] R5-195688, “On FR2 spherical coverage MU”, Anritsu, RAN5#84

[7] R5-197588, “WF on TX spherical coverage noise impact”, Keysight Technologies, Anritsu, Rohde&Schwarz, Ericsson, RAN5#84

[8] R5-194361, “AP#82.22 Acceptable assumption for CDF curve”, Qualcomm, RAN5#83

[9] R5-197495, “AP#82.22 Assumption for the CDF curve”, Apple, RAN5#84

[10] R5-196789, “Discussion on MU, TT and UE fail rate for EIRP CDF”, Ericsson, RAN5#84

[11] R5-183909: “Discussion on TT calculation from MU in FR2, 3GPP TSG-RAN WG5 Meeting #79, KDDI

# Annex A

Following figures show the distorted CDF curves for first 1000 random realizations.

# Curve (a), Noise Assumption A, Snoise=0.0

# Curve (d), Noise Assumption A, Snoise=0.0

Curve (e), Noise Assumption A, Snoise=0.0

# Curve (a), Noise Assumption B, Snoise=0.0

# Curve (d), Noise Assumption B, Snoise=0.0

# Curve (e), Noise Assumption B, Snoise=0.0