3GPP TSG RAN WG5 #85 Meeting	R5-198733

Reno, Nevada, USA, 18th – 22nd November 2019

Source:		Rohde & Schwarz

Title:	On the SNR for FR2 TRx test cases

Agenda Item:		5.3.5.17

Document for:	Endorsement

# Introduction

At RAN5 #84, estimations for the SNR have been discussed [1 – 5]. In this document, we present the results of our analysis based on the common assumptions for the not yet decided figures.

# Discussion

A detailed description of the total SNRtotal based on the measurement of total component power for both polarizations can be found in reference [6].

In Table 1, we present our results for SNRtotal under the assumption that the UE is allowed to transmit a maximum peak EIRP of 43 dBm.

Among TE vendors it has been decided to study the SNR for ACLR based on the following two assumptions:

ACLR Assumption 1

- Assumption for min TRP power value on wanted channel shall be: min EIRP – 16 dB (UE antenna (8x2) directivity) - MPR

- Assumption for min TRP power value on adjacent channel shall be: min EIRP – 16 dB (UE antenna (8x2) directivity) – MPR - ACLR requirement

ACLR Assumption 2

- Assumption for min TRP power value on wanted channel shall be:  min EIRP – 10 dB (UE antenna (1x4) directivity) - MPR

- Assumption for min TRP power value on adjacent channel shall be:  min EIRP – 10 dB (UE antenna (1x4) directivity) – MPR - ACLR requirement

The MPR depends on waveform, modulation and RB allocation. For the currently defined test points the highest MPR occurs for 400 MHz channel bandwidth and 64QAM DFT-s-OFDM with a value of 6.5 dB. It is noted that the MU and SNR analysis has to be updated once CP-OFDM 64QAM were applied as test point for ACLR.

Observation 1: With ACLR assumption 1 the impact of noise is already very high for the measurement of the wanted signal. This could only be avoided by selecting test points with low MPR.

Observation 2: Independent of the ACLR assumption ACLR needs considerable amount of relaxation in order to be testable.

Table 1: SNRtotal,core based on core requirement

| Test case | Frequency (GHz) | Estimated SNRtotal(dB) | Requirement (dBm) | Impact of Noise w/o relaxation (dB) | Required Relaxation |
| --- | --- | --- | --- | --- | --- |
| MOP Spherical coverage | 23.45 ≤ f ≤ 32.125 | [17.35]1 | 9.75 (= 11.5 – 1.75) | [0.08]1 | no |
|  | 32.125 < f ≤ 40.8 | [13.05]1 | 7.6 (= 8 – 0.4) | [0.21]1 | no |
| Transmit OFF Power (TRP) | 32.125 < f ≤ 40.8 | -29.5 | -35 | 29.5 | do not test if tested relax by 39.5 dB with SNR impact = 0.42 dB |
| ACLR AC (TRP) Assumption 1 | 23.45 ≤ f ≤ 32.125 | -11.2 | -18.8 (= 20.7 – 16 – 17 – 6.5) | 11.52 | yes |
|  | 32.125 < f ≤ 40.8 | -14.1 | -19.6 (= 18.9 – 16 – 16 – 6.5) | 14.27 | yes |
| ACLR wanted signal (TRP) Assumption 1 | 23.45 ≤ f ≤ 32.125 | 5.8 | -1.8 (= 20.7 – 16 – 6.5) | 1.02 | yes |
|  | 32.125 < f ≤ 40.8 | 1.9 | -3.6 (= 18.9 – 16 – 6.5) | 2.17 | yes |
| ACLR AC (TRP) Assumption 2 | 23.45 ≤ f ≤ 32.125 | -5.2 | -12.8 (= 20.7 – 10 – 17 – 6.5) | 6.35 | yes |
|  | 32.125 < f ≤ 40.8 | -8.1 | -13.6 (= 18.9 – 10 – 16 – 6.5) | 8.73 | yes |
| ACLR wanted signal (TRP) Assumption 2 | 23.45 ≤ f ≤ 32.125 | 11.8 | 4.2 (= 20.7 – 10 – 6.5) | 0.28 | no |
|  | 32.125 < f ≤ 40.8 | 7.9 | 2.4 (= 18.9 – 10 – 6.5) | 0.66 | no |
| Transmitter Spurious Emissions | 6 ≤ f < 12.75 | 10 | -30 | 0.41 | no |
|  | 12.75 ≤ f ≤ 50 | 10 | -13 | 0.41 | no |
|  | 50 < f ≤ 80 | work in progress |  |  |  |
| Spurious Emissions for UE co-existence | work in progress |  |  |  |  |
| Receiver Spurious Emissions | work in progress |  |  |  |  |
| NOTE 1: Rough estimate based on the 50%-tile power level requirement. A more rigorous analysis will be based on an acceptable CDF. |  |  |  |  |  |

Proposal 1: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise for the TT analysis.

In Ref. [5], we proposed for spherical coverage to base the uncertainty of the test system on the test requirement and received the valid feedback that it would be required to apply this for all test cases and that the impact on the already specified MTSU and TT has to be analyzed.

For TT, the MU is required to judge the pass and fail rates of an UE with respect to the core requirement. Consequently this MU should be based on the core requirement level.

The MTSU, however, should be based on the test requirement since the measurement value is compared to the latter in order to determine whether an UE passes or fails the test.

Observation 3: The specified TT values remain unchanged when updating the MTSU with respect to the test requirement level.

When basing the MTSU on the test requirement instead of the core requirement, the only MU contributor requiring an update is the influence of noise. Whether the MTSU based on the test requirement is smaller or larger than the one based on the core requirements depends on the sign of TT, i.e., whether the limit is an upper or lower one.

In the following, we derive the change of the influence of noise when moving from conformance requirement to test requirement as a basis.

In order to allow for a notation without distinction of cases, we define

TT=Test requirement-Core requirement .

Thus, negative values represent the case of a lower limit and positive values apply for an upper limit.

SNRcore= -10log10(10ΔSNRcore10-1)

SNRtest=SNRcore+TT

ΔSNRtest=10log101+10- SNRtest10

Here, the subscripts, core and test, denote that the core requirement or test requirement have been used as basis for SNR or ΔSNR, respectively.

The MTSU based on the test requirement is calculated as follows from the MTSU based on the core requirement:

MTSUtest=MTSUcore+ΔSNRtest-ΔSNRcore

Table 2 comprises the values for MTSU based on the test requirement.

Table 2: MTSU based on the test requirement

| Test case | FR | MTSUcore | ΔSNRcore | TT | ΔSNRtest | MTSUtest |
| --- | --- | --- | --- | --- | --- | --- |
| MOP Min Peak EIRP | FR2a | 4.89 | 0.1 | -3.18 | 0.21 | 5.01 |
|  | FR2b | 5.09 | 0.3 | -3.31 | 0.62 | 5.41 |
| MOP Max EIRP | FR2a | 4.89 | 0.1 | 0 | 0.1 | 4.89 |
|  | FR2b | 5.09 | 0.3 | 0 | 0.3 | 5.09 |
| MOP Max TRP | FR2a | 4.42 | 0.1 | 2.87 | 0.052 | 4.37 |
|  | FR2b | 4.62 | 0.3 | 3.00 | 0.15 | 4.47 |
| SEM | FR2a | 4.94 | 0.62 | 3.21 | 0.31 | 4.63 |
|  | FR2b | 5.32 | 1.0 | 3.46 | 0.48 | 4.80 |

Proposal 2: Specify MTSU based on the test requirement and apply the values of MTSUtest in Table 2.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurements 6 GHz – 80 GHz for spurious emissions measurements |
| #2 | Size of QZ for IFF | 30 cm |
| #3 | UE power class | PC3 |
| #5 | Temperature range of the test equipment | 20°C – 35°C |
| #6 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, SNR estimations for low PSD test cases have been provided and the influence of noise has been analyzed.

Observation 1: With ACLR assumption 1 the impact of noise is already very high for the measurement of the wanted signal. This could only be avoided by selecting test points with low MPR.

Observation 2: Independent of the ACLR assumption ACLR needs considerable amount of relaxation in order to be testable.

Proposal 1: Apply the SNR values in Table 1 as basis for determining the relaxation amount and for specifying the MU impact due to noise for the TT analysis.

Observation 3: The specified TT values remain unchanged when updating the MTSU with respect to the test requirement level.

Proposal 2: Specify MTSU based on the test requirement and apply the values of MTSUtest in Table 2.

# References

- R5-195688, On FR2 spherical coverage MU, Anritsu, 3GPP TSG RAN WG5 Meeting #84, August 2019

- R5-196572, Impact of noise for FR2 ACLR, Anritsu, 3GPP TSG RAN WG5 Meeting #84, August 2019

- R5-197626, Impact of noise for FR2 OBW, Anritsu, 3GPP TSG RAN WG5 Meeting #84, August 2019

- R5-197499, On noise impact, Keysight Technologies UK Ltd, 3GPP TSG RAN WG5 Meeting #84, August 2019

- R5-196852, On the SNR for FR2 TRx test cases, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #84, August 2019

- R5-192125, On the SNR for FR2 TRx test cases, Rohde & Schwarz, 3GPP TSG RAN WG5 Meeting #82, February 2019