3GPP TSG-RAN5 #85	R5-198366

Reno, USA, 18th Nov – 22nd Nov 2019

Title:	Benefit of 2 AoA RRM test setup with IFF TRxPs for FR2

Source:			Anritsu

Agenda Item:			5.3.5.17

Document for:			Discusson and approval

# 1.	Introduction

This contribution is a follow-up paper of the one [1] presented in RAN4 which proposed to allow a two- angle of arrival (AoA) RRM test setup with the indirect far field (IFF) TRxPs. One of the reasons this decision was postponed was due to a comment which requested to clarify a benefit of this test system. In this paper we introduce the benefit of this test system again. And also considering the current situation of feasibility study with RRM test system configuration, we propose a way forward to proceed a feasibility study and measurement uncertainty estimation for RRM test cases from now.

# 2.	Discussion

2.1 Comparison of FR2 test systems (RF, RRM and demodulation)

For information, as excerpted below, a current UE RRM measurement setup for 2 AoAs is configured only with a direct far field (DFF) TRxPs while TRx RF test system can be configured by IFF, DFF and near field to far field transform (NFTF).

Excerpt from TR 38.810 [2] section 6.2.1.1 - For the scope of Rel-15 testing NMAX_AoAs = 2. - For UE RRM baseline measurement setup based on DFF, the supported NMAX_AoAs = 2. - For UE RRM baseline measurement setup based on simplified DFF, the supported NMAX_AoAs = 1. - For UE RRM baseline measurement setup based on IFF, the supported NMAX_AoAs = 1.

Also for UE demodulation and CSI test, one AoA test setup is considered as follows.

Excerpt from TR 38.810 [2] section 7.2.1.1 - The following permitted test setups are considered - DFF test setup as described in Clause 5. - Simplified DFF test setup as described in Clause 5. - IFF test setup as described in Clause 5. - Direct near field (DNF) test setup. - Note: No final conclusions on the feasibility of DNF setup were made.

The observations which were introduced in [1] are excerpted again below.

Excerpted from [1] Observation 1: Current definition of RRM test setup for 2 AoA(NMAX_AoAs = 2) is excluding the use of IFF TRxP as one (or more) of the TRxPs. Observation 2: By following the description in TR 38.810, UE RF and RRM measurement system need to be separately configured, i.e. we need two chambers to run FR2 RF and RRM conformance tests. Observation 3: IFF TRxP(s) should be allowed as one (or more) of the TRxPs in RRM test setup for 2 AoA (NMAX_AoAs = 2). Observation 4: There is a chance that RF, RRM and Demodulation test systems can be configured in one chamber if IFF TRxP is allowed in RRM test setup for 2AoA (NMAX_AoAs = 2).

As can be seen above, RF, demodulation and CSI test setup can already be configured by a common test setup. However as already mentioned by the observation in the previous paper [1], by following the description in TR 38.810, UE RF and RRM measurement system need to be separately configured, i.e. we need two chambers to run FR2 RF and RRM conformance tests. Therefore by allowing the IFF TRxP(s) as one (or more) of the TRxPs in the RRM test setup for 2 AoA, we can gain one of the biggest benefit that we can carry out all RF, RRM, demodulation and CSI tests by one chamber

Observation 1: By allowing the IFF TRxP(s) as one (or more) of the TRxPs in the RRM test setup for 2 AoA, there is  a possibility that we can carry out all the RF, RRM, demodulation and CSI tests in FR2 by one test system in a chamber, which may significantly decrease an investment of FR2 test system in a site.

2.2 Consideration on the feasibility studies for RRM test system

Of course an influence of the additional measurement structure due to the IFF TRxP(s) needs to be verified such as quality of quiet zone, unwanted reflections of DFF signals to a DUT via the IFF reflector, achievable SNR, angular error between UE and measurement antenna due to the black box approach, etc. However as can be seen in TR 38.810, these kinds of feasibility / measurement uncertainty estimations have not been completed even with the DFF TRxP only or IFF TRxP only setup. And there is no evidence shown these already defined setups are feasible to carry out all the existing RRM test cases. Therefore it is premature to exclude any possible test methods for RRM at this moment.

Observation 2: None of the permitted test methods for RRM test cases have completed a feasibility study and measurement uncertainty estimation yet. Therefore it is premature to exclude any possible test methods for RRM at this moment.

Proposal 1: Allow the IFF TRxP(s) as one (or more) of the TRxPs in the RRM test setup for 2 AoA (NMAX_AoAs = 2).

Proposal 2: Prioritization of RRM DFF / IFF / mixture of DFF and IFF test system shall not be made, and interested proponents carry out the feasibility study and measurement uncertainty estimation in parallel.

Proposal3: Other test methods for RRM shall not be excluded

- Conclusion

In this contribution we introduced our ideas of allowing IFF TRxPs in the RRM test systems. We also proposed the way forward on the development of

Observation 1: By allowing the IFF TRxP(s) as one (or more) of the TRxPs in the RRM test setup for 2 AoA, there is  a possibility that we can carry out all the RF, RRM, demodulation and CSI tests in FR2 by one test system in a chamber, which may significantly decrease an investment of FR2 test system in a site.

Observation 2: None of the permitted test methods for RRM test cases have completed a feasibility study and measurement uncertainty estimation yet. Therefore it is premature to exclude any possible test methods for RRM at this moment.

Proposal 1: Allow the IFF TRxP(s) as one (or more) of the TRxPs in the RRM test setup for 2 AoA (NMAX_AoAs = 2).

Proposal 2: Prioritization of RRM DFF / IFF / mixture of DFF and IFF test system shall not be made, and interested proponents carry out the feasibility study and measurement uncertainty estimation in parallel.

Proposal3: Other test methods for RRM shall not be excluded

# 4.	References

[1] R4-1911340, “Introduction of 2 AoA RRM test setup with IFF TRxPs for FR2”, Anritsu corp, RAN4 #92bis, Chongqing, China

[2] TR 38.810, “Study on test methods”, V16.4.0, 2019-9