3GPP TSG-RAN5 Meeting #86-e	R5-200468

# Electronic Meeting, 17th – 28th February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1153 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of a few R16s inter-band EN-DC FR1 test configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test configurations for a few R16s inter-band EN-DC FR1 need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test configurations for following R16s inter-band EN-DC configurations: DC_2A-7C-66A_n78A DC_2A-7A-7A-66A_n78A DC_2A-66A_n5A DC_13A_n66A DC_12A_n78A DC_8A_n41A DC_7C_n66A DC_7A_n66A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test for inter-band EN-DC configurations will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.4.1.2, 4.3.1.4.1.3, 4.3.1.4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### <<< START OF CHANGES >>>

4.3.1.4.1	Inter-band EN-DC configurations within FR1

4.3.1.4.1.1	General

For inter-band EN-DC configurations as listed in this subclause, the following apply:

For the E-UTRA band and E-UTRA CA configurations, test frequencies as specified in TS 36.508 [2], clause 4.3.1 are used.

For the NR band and NR CA configurations, test frequencies as specified in clause 4.3.1.1 are used.

For the EN-DC inter-band configuration that includes an EN-DC contiguous configuration (e.g. DC_2A-(n)71AA) the EN-DC contiguous configuration is listed in the NR configuration column and the test frequencies as specified in clause 4.3.1.4.2 are used.

For the secondary NR band in inter-band signalling test cases, the band selected is based on the subset of NR bands supported within the EN-DC configurations specified in Table 4.3.1.4.1.2-1 for NR FR1 and 4.3.1.5.1.2-1 for NR FR2.

4.3.1.4.1.2	Inter-band EN-DC configurations within FR1 (two bands)

Table 4.3.1.4.1.2-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC Configuration | EN-DC E-UTRA downlink configuration | EN-DC NR downlink configuration | EN-DC E-UTRA uplink configuration | EN-DC NR uplink configuration |
| --- | --- | --- | --- | --- | --- |
| DC_1A_n28A | DC_1A_n28A | 1A | n28A | 1A | n28A |
| DC_1A_n40A | DC_1A_n40A | 1A | n40A | 1A | n40A |
| DC_1A_n51A | DC_1A_n51A | 1A | n51A | 1A | n51A |
| DC_1A_n77A | DC_1A_n77A | 1A | n77A | 1A | n77A |
| DC_1A_n77C | DC_1A_n77A | 1A | CA_n77C | 1A | n77A |
| DC_1A_n78A | DC_1A_n78A | 1A | n78A | 1A | n78A |
| DC_1A_n78C | DC_1A_n78A | 1A | CA_n78C | 1A | n78A |
| DC_1A_n79A | DC_1A_n79A | 1A | n79A | 1A | n79A |
| DC_1A_n79C | DC_1A_n79A | 1A | CA_n79C | 1A | n79A |
| DC_7A-7A_n78A | DC_7A_n78A | CA_7A-7A | n78A | 7A | n78A |
| <<< Unchanged Sections Skipped>>> |  |  |  |  |  |
| DC_7A_n28A | DC_7A_n28A | 7A | n28A | 7A | n28A |
| DC_7A_n51A | DC_7A_n51A | 7A | n51A | 7A | n51A |
| DC_7A_n66A | DC_7A_n66A | 7A | n66A | 7A | n66A |
| DC_7A_n78A | DC_7A_n78A | 7A | n78A | 7A | n78A |
| DC_7C_n66A | DC_7A_n66A | CA_7C | n66A | 7A | n66A |
| DC_7C_n78A | DC_7A_n78A | CA_7C | n78A | 7A | n78A |
| DC_8A_n40A | DC_8A_n40A | 8A | n40A | 8A | n40A |
| DC_8A_n41A | DC_8A_n41A | 8A | n41A | 8A | n41A |
| DC_8A_n77A | DC_8A_n77A | 8A | n77A | 8A | n77A |
| DC_8A_n78A | DC_8A_n78A | 8A | n78A | 8A | n78A |
| DC_8A_n79A | DC_8A_n79A | 8A | n79A | 8A | n79A |
| DC_11A_n77A | DC_11A_n77A | 11A | n77A | 11A | n77A |
| DC_11A_n78A | DC_11A_n78A | 11A | n78A | 11A | n78A |
| DC_11A_n79A | DC_11A_n79A | 11A | n79A | 11A | n79A |
| DC_12A_n5A | DC_12A_n5A | 12A | n5A | 12A | n5A |
| DC_12A_n66A | DC_12A_n66A | 12A | n66A | 12A | n66A |
| DC_12A_n78A | DC_12A_n78A | 12A | n78A | 12A | n78A |
| DC_13A_n66A | DC_13A_n66A | 13A | n66A | 13A | n66A |
| DC_18A_n77A | DC_18A_n77A | 18A | n77A | 18A | n77A |
| DC_18A_n78A | DC_18A_n78A | 18A | n78A | 18A | n78A |
| DC_18A_n79A | DC_18A_n79A | 18A | n79A | 18A | n79A |
| <<< Unchanged Sections Skipped>>> |  |  |  |  |  |

4.3.1.4.1.3	Inter-band EN-DC configurations within FR1 (three bands)

Table 4.3.1.4.1.3-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC Configuration | EN-DC E-UTRA downlink configuration | EN-DC NR downlink configuration | EN-DC E-UTRA uplink configuration | EN-DC NR uplink configuration |
| --- | --- | --- | --- | --- | --- |
| <<< Unchanged Sections Skipped>>> |  |  |  |  |  |
| DC_2A-66A_n5A | DC_2A_n5A | CA_2A-66A | n5A | 2A | n5A |
|  | DC_66A_n5A | CA_2A-66A | n5A | 66A | n5A |
| DC_2A-66A_n71A | DC_2A_n71A | CA_2A-66A | n71A | 2A | n71A |
|  | DC_66A_n71A | CA_2A-66A | n71A | 66A | n71A |
| DC_2A-(n)71AA | DC_2A_n71A | 2A | DC_(n)71AA | 2A | n71A |
|  | DC_(n)71AA | 2A | DC_(n)71AA | - | DC_(n)71AA |
| DC_3A-19A_n78A | DC_3A_n78A | CA_3A-19A | n78A | 3A | n78A |
|  | DC_19A_n78A | CA_3A-19A | n78A | 19A | n78A |
| DC_3A-19A_n79A | DC_3A_n79A | CA_3A-19A | n79A | 3A | n79A |
|  | DC_19A_n79A | CA_3A-19A | n79A | 19A | n79A |
| DC_3A-21A_n78A | DC_3A_n78A | CA_3A-21A | n78A | 3A | n78A |
|  | DC_21A_n78A | CA_3A-21A | n78A | 21A | n78A |
| DC_3A-21A_n79A | DC_3A_n79A | CA_3A-21A | n79A | 3A | n79A |
|  | DC_21A_n79A | CA_3A-21A | n79A | 21A | n79A |
| DC_3A_n28A-n78A | DC_3A_n28A | 3A | CA_n28A-n78A | 3A | n28A |
|  | DC_3A_n78A | 3A | CA_n28A-n78A | 3A | n78A |
| <<< Unchanged Sections Skipped>>> |  |  |  |  |  |

4.3.1.4.1.4	Inter-band EN-DC configurations within FR1 (four bands)

Table 4.3.1.4.1.4-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC Configuration | EN-DC E-UTRA downlink configuration | EN-DC NR downlink configuration | EN-DC E-UTRA uplink configuration | EN-DC NR uplink configuration |
| --- | --- | --- | --- | --- | --- |
| DC_1A-3A-7A_n28A | DC_1A_n28A | CA_1A-3A-7A | n28A | 1A | n28A |
|  | DC_3A_n28A | CA_1A-3A-7A | n28A | 3A | n28A |
|  | DC_7A_n28A | CA_1A-3A-7A | n28A | 7A | n28A |
| DC_1A-3A-7A_n78A | DC_1A_n78A | CA_1A-3A-7A | n78A | 1A | n78A |
|  | DC_3A_n78A | CA_1A-3A-7A | n78A | 3A | n78A |
|  | DC_7A_n78A | CA_1A-3A-7A | n78A | 7A | n78A |
| DC_1A-3A-19A_n78A | DC_1A_n78A | CA_1A-3A-19A | n78A | 1A | n78A |
|  | DC_3A_n78A | CA_1A-3A-19A | n78A | 3A | n78A |
|  | DC_19A_n78A | CA_1A-3A-19A | n78A | 19A | n78A |
| DC_1A-3A-19A_n79A | DC_1A_n79A | CA_1A-3A-19A | n79A | 1A | n79A |
|  | DC_3A_n79A | CA_1A-3A-19A | n79A | 3A | n79A |
|  | DC_19A_n79A | CA_1A-3A-19A | n79A | 19A | n79A |
| DC_1A-3A-21A_n78A | DC_1A_n78A | CA_1A-3A-21A | n78A | 1A | n78A |
|  | DC_3A_n78A | CA_1A-3A-21A | n78A | 3A | n78A |
|  | DC_21A_n78A | CA_1A-3A-21A | n78A | 21A | n78A |
| DC_1A-3A-21A_n79A | DC_1A_n79A | CA_1A-3A-21A | n79A | 1A | n79A |
|  | DC_3A_n79A | CA_1A-3A-21A | n79A | 3A | n79A |
|  | DC_21A_n79A | CA_1A-3A-21A | n79A | 21A | n79A |
| DC_1A-3A_n28A-n78A | DC_1A_n28A | CA_1A-3A | CA_n28A-n78A | 1A | n28A |
|  | DC_1A_n78A | CA_1A-3A | CA_n28A-n78A | 1A | n78A |
|  | DC_3A_n28A | CA_1A-3A | CA_n28A-n78A | 3A | n28A |
|  | DC_3A_n78A | CA_1A-3A | CA_n28A-n78A | 3A | n78A |
| <<< Unchanged Sections Skipped>>> |  |  |  |  |  |
| DC_2A-7A-7A-13A_n66A | DC_2A_n66A | CA_2A-7A-7A-13A | n66A | 2A | n66A |
|  | DC_7A_n66A | CA_2A-7A-7A-13A | n66A | 7A | n66A |
|  | DC_13A_n66A | CA_2A-7A-7A-13A | n66A | 13A | n66A |
| DC_2A-7A-7A-66A_n66A | DC_2A_n66A | CA_2A-7A-7A-66A | n66A | 2A | n66A |
|  | DC_7A_n66A | CA_2A-7A-7A-66A | n66A | 7A | n66A |
|  | DC_66A_n66A | CA_2A-7A-7A-66A | n66A | 66A | n66A |
| DC_2A-7A-7A-66A_n78A | DC_2A_n78A | CA_2A-7A-7A-66A | n78A | 2A | n78A |
|  | DC_7A_n78A | CA_2A-7A-7A-66A | n78A | 7A | n78A |
|  | DC_66A_n78A | CA_2A-7A-7A-66A | n78A | 66A | n78A |
| DC_2A-7C-13A_n66A | DC_2A_n66A | CA_2A-7C-13A | n66A | 2A | n66A |
|  | DC_7A_n66A | CA_2A-7C-13A | n66A | 7A | n66A |
|  | DC_13A_n66A | CA_2A-7C-13A | n66A | 13A | n66A |
| DC_2A-7C-66A_n66A | DC_2A_n66A | CA_2A-7C-66A | n66A | 2A | n66A |
|  | DC_7A_n66A | CA_2A-7C-66A | n66A | 7A | n66A |
|  | DC_66A_n66A | CA_2A-7C-66A | n66A | 66A | n66A |
| DC_2A-7C-66A_n78A | DC_2A_n78A | CA_2A-7C-66A | n78A | 2A | n78A |
|  | DC_7A_n78A | CA_2A-7C-66A | n78A | 7A | n78A |
|  | DC_66A_n78A | CA_2A-7C-66A | n78A | 66A | n78A |
| DC_2A-66A-(n)71AA | DC_2A_n71A | CA_2A-66A | DC-(n)71AA | 2A | n71A |
|  | DC_66A_n71A | CA_2A-66A | DC-(n)71AA | 66A | n71A |
|  | DC_(n)71AA | CA_2A-66A | DC-(n)71AA | - | (n)71AA |
| DC_3A-7A_n28A-n78A | DC_3A_n28A | CA_3A-7A | CA_n28A-n78A | 3A | n28A |
|  | DC_3A_n78A | CA_3A-7A | CA_n28A-n78A | 3A | n78A |
|  | DC_7A_n28A | CA_3A-7A | CA_n28A-n78A | 7A | n28A |
|  | DC_7A_n78A | CA_3A-7A | CA_n28A-n78A | 7A | n78A |
| <<< Unchanged Sections Skipped>>> |  |  |  |  |  |

##### << End of changes >>