Title:	Test point selection for Out-of-band blocking for intra-band contiguous EN-DC in FR1

Source:		CAICT

Introduction

In this contribution the selection of test environment, frequency, channel BW, SCS, RB allocation and modulation for Out-of-band blocking for intra-band contiguous EN-DC in FR1 is analyzed.

Discussion

The Out-of-band blocking for intra-band contiguous EN-DC in FR1 is defined in section 7.6B.3.1 of TS 38.101-3.

7.6B.3.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC out-of-band requirement and parameters are defined in Table 7.6B.3.1-1.

Table 7.6B.3.1-1: Out-of-band blocking

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [4]. For inter-band combinations where the intra-band requirements are applicable, in which the E-UTRA band is a subset of an NR-only band, the NR band interferer values and offsets specified from Table 7.6A.3-2 in TS 38.101-1 [2] apply to both E-UTRA and NR carriers. NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink and downlink modulations and RB configurations.

  - Test Environment

In LTE and NR single carrier testing, the Out-of-band blocking is tested under Normal condition. The same condition can be selected for intra-band contiguous EN-DC in FR1 testing.

Proposal 1: Define test environment as Normal for Out-of-band blocking for intra-band contiguous EN-DC in FR1.

Test Subcarrier Spacing

In NR single carrier testing, the Out-of-band blocking is tested using lowest subcarrier spacing. This can be followed in intra-band contiguous EN-DC in FR1 testing.

Proposal 2: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, test Lowest subcarrier spacing.

Test Frequencies

In LTE CA, the Test Frequency of Out-of-band blocking for intra-band contiguous CA is Mid range. The same approach can be followed in intra-band contiguous EN-DC in FR1 testing.

Proposal 3: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, the Mid range is tested.

Test Channel Bandwidth

In LTE CA, the test channel bandwidth of Out-of-band blocking is Highest NRB_agg. The same approach can be followed in intra-band contiguous EN-DC in FR1 testing.

Proposal 4: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, the Highest NRB_agg is tested.

Modulation

In LTE single carrier testing, the Out-of-band blocking is tested using QPSK for downlink and uplink. This can be followed in intra-band contiguous EN-DC in FR1 testing for E-UTRA CC.

In NR single carrier testing, the Out-of-band blocking is tested using CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink. This can be followed in intra-band contiguous EN-DC in FR1 testing for NR CC.

Proposal 5: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, test QPSK for downlink and uplink for E-UTRA CC, test CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink for NR CC.

RB allocation

In LTE and NR single carrier testing, the Out-of-band blocking is tested using the same downlink and uplink RB allocations as Reference sensitivity level. This can be followed in intra-band contiguous EN-DC in FR1 testing.

Proposal 6: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, test the same downlink and uplink RB allocations as the LTE and NR single carrier Reference sensitivity level TC for each E-UTRA CC and NR CC.

Conclusion

To sum up, the following selection of test points is proposed for Out-of-band blocking for intra-band contiguous EN-DC in FR1:

Proposal 1: Define test environment as Normal for Out-of-band blocking for intra-band contiguous EN-DC in FR1.

Proposal 2: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, test Lowest subcarrier spacing.

Proposal 3: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, the Mid range is tested.

Proposal 4: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, the Highest NRB_agg is tested.

Proposal 5: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, test QPSK for downlink and uplink for E-UTRA CC, test CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink for NR CC.

Proposal 6: For Out-of-band blocking for intra-band contiguous EN-DC in FR1, test the same downlink and uplink RB allocations as the LTE and NR single carrier Reference sensitivity level TC for each E-UTRA CC and NR CC.

Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for intra-band contiguous EN-DC in FR1.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |