Title:	Test point selection for Out-of-band blocking for intra-band non-contiguous EN-DC in FR1

Source:		CAICT

Introduction

In this contribution the selection of test environment, frequency, channel BW, SCS, RB allocation and modulation for Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 is analyzed.

Discussion

The Out-of-band blocking for intra-band non-contiguous EN-DC in FR1 is defined in section 7.6B.3.2 of TS 38.101-3.

7.6B.3.2	Intra-band non-contiguous EN-DC in FR1

For the E-UTRA sub-block containing one or multiple CC's, the requirement is dfined in clause 7.6.2.1 for single carrier operation and in clause 7.6.2.1A for CA in TS 36.101 [4].

For the NR sub-block, the requirement is defined in clause 7.6.3 is [2].

Observation 1: According to 7.6B.3.2 in TS 38.101-3, there is no extra requirement compared with the SA NR FR1 requirements defined in TS 38.101-1 Clause 7.6.3 and E-UTRA requirements defined in TS 36.101 Clauses 7.6.2.1 and 7.6.2.1A.

Proposal 1: LTE agnostic approach is applied for Out-of-band blocking test for Intra-band non-contiguous EN-DC in FR1, ensuring LTE agnostic resources do not interfere NR CG measurement. Single E-UTRA component carrier is configured regardless the number of E-UTRA CCs in the Intra-band non-contiguous EN-DC configuration.TP analysis for SCG operating on NR FR1 leverages from the TP analysis for SA NR FR1 Out-of-band blocking test with the exception in proposal 2.

Test frequency and Test Channel Bandwidth

High with maxWGap and Highest NRB_agg is suggested for test frequency and test channel bandwidth selection according to R5-191451[1]

Proposal 2: Test High with maxWGap and Highest NRB_agg for Out-of-band blocking for intra-band non-contiguous EN-DC in FR1.

Conclusion

To sum up, the following selection of test points is proposed for Out-of-band blocking for intra-band non-contiguous EN-DC in FR1:

Proposal 1: LTE agnostic approach is applied for Out-of-band blocking test for Intra-band non-contiguous EN-DC in FR1, ensuring LTE agnostic resources do not interfere NR CG measurement. Single E-UTRA component carrier is configured regardless the number of E-UTRA CCs in the Intra-band non-contiguous EN-DC configuration.TP analysis for SCG operating on NR FR1 leverages from the TP analysis for SA NR FR1 Out-of-band blocking test with the exception in proposal 2.

Proposal 2: Test High with maxWGap and Highest NRB_agg for Out-of-band blocking for intra-band non-contiguous EN-DC in FR1.

Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for intra-band non-contiguous EN-DC in FR1.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

References

[1] R5-191451” “Resolving ambiguity in LTE anchor agnostic approach”, Anritsu, RAN5#82, Athens, Nov. 2018.