Title:	Test point selection for Out-of-band blocking for inter-band EN-DC within FR1

Source:		CAICT

Introduction

In this contribution the selection of test environment, frequency, channel BW, SCS, RB allocation and modulation for Out-of-band blocking for inter-band EN-DC within FR1 is analyzed.

Discussion

The Out-of-band blocking for inter-band EN-DC within FR1 is defined in section 7.6B.3.3 of TS 38.101-3.

7.6B.3.3	Inter-band EN-DC within FR1

Out-of band blocking requirements for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (two bands) in clause 5.2.B.4.1 with following conditions

one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L and the NR band whose downlink is being tested has its uplink carrier output power set to minimum output power as defined in clause 6.3.1 of TS 38.101-1 [2]

one NR uplink carrier with the output power set to 4 dB below PCMAX_L on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to minimum output power as defined in clause 6.3.2.1 of TS 36.101 [4].

If CW interferer falls in a gap between FDL_high of the E-UTRA or NR band and FDL_low of the NR or EUTRA band, where the corresponding OOB ranges 1 and 2 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

If FDL_high of the lower E-UTRA or NR band is greater than or equal to the FDL_low of the upper NR or E-UTRA band as in overlapping RX frequency ranges, then the OOB range shall start from the FDL_low of the lower E-UTRA or NR band, and from the FDL_high of the upper NR or E-UTRA band.

For EN-DC combination listed in Table 7.6B.3.3-1 under the first test condition above, exceptions to the requirement specified in Table 7.6B.3.3-2 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

Table 7.6B.3.3-1: EN-DC combination with exceptions allowed

EN-DC combination

DC_5_n78

DC_8_n77

DC_8_n78

DC_8_n79

DC_11_n77

DC_18_n77

DC_18_n78

DC_18_n79

DC_19_n77

DC_19_n78

DC_19_n79

DC_20_n77

DC_20_n78

DC_21_n77

DC_26_n77

DC_26_n78

DC_26_n79

DC_28_n77

DC_28_n78

DC_28_n79

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when fInterferer± fULLB- fDLHB≤(BWULLB+ BWDLHB)/2, where fULLB and fDLHB are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. BWULLB and BWDLHB are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

For each of the two test cases in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] for all interferer frequency ranges a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink and downlink modulations and RB configurations.

  - Test Environment

In LTE and NR single carrier testing, the Out-of-band blocking is tested under Normal condition. The same condition can be selected for inter-band EN-DC within FR1 testing.

Proposal 1: Define test environment as Normal for Out-of-band blocking for inter-band EN-DC within FR1.

Test Subcarrier Spacing

In NR single carrier testing, the Out-of-band blocking is tested using lowest subcarrier spacing. This can be followed in inter-band EN-DC within FR1 testing.

Proposal 2: For Out-of-band blocking for inter-band EN-DC within FR1, test Lowest subcarrier spacing.

Test Frequencies

In LTE CA, the Test Frequency of Out-of-band blocking for inter-band CA is Mid range for all PCC/SCCs. The same approach can be followed in inter-band EN-DC within FR1 testing.

Proposal 3: For Out-of-band blocking for inter-band EN-DC within FR1, Mid range for E-UTRA and Mid range for NR is tested.

Test Channel Bandwidth

In LTE CA, the test channel bandwidth of Out-of-band blocking is Highest NRB_agg. The same approach can be followed in inter-band EN-DC within FR1 testing.

Proposal 4: For Out-of-band blocking for inter-band EN-DC within FR1, the Highest NRB_agg is tested.

Modulation

In LTE single carrier testing, the Out-of-band blocking is tested using QPSK for downlink and uplink. This can be followed in inter-band EN-DC within FR1 testing for E-UTRA CC.

In NR single carrier testing, the Out-of-band blocking is tested using CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink. This can be followed in inter-band EN-DC within FR1 testing for NR CC.

Proposal 5: For Out-of-band blocking for inter-band EN-DC within FR1, test QPSK for downlink and uplink for E-UTRA CC, test CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink for NR CC.

RB allocation

In LTE and NR single carrier testing, the Out-of-band blocking is tested using the same downlink and uplink RB allocations as Reference sensitivity level. This can be followed in inter-band EN-DC within FR1 testing.

Proposal 6: For Out-of-band blocking for inter-band EN-DC within FR1, test the same downlink and uplink RB allocations as the corresponding Reference sensitivity level TC for each E-UTRA CC and NR CC.

Conclusion

To sum up, the following selection of test points is proposed for Out-of-band blocking for inter-band EN-DC within FR1:

Proposal 1: Define test environment as Normal for Out-of-band blocking for inter-band EN-DC within FR1.

Proposal 2: For Out-of-band blocking for inter-band EN-DC within FR1, test Lowest subcarrier spacing.

Proposal 3: For Out-of-band blocking for inter-band EN-DC within FR1, Mid range for E-UTRA and Mid range for NR is tested.

Proposal 4: For Out-of-band blocking for inter-band EN-DC within FR1, the Highest NRB_agg is tested.

Proposal 5: For Out-of-band blocking for inter-band EN-DC within FR1, test QPSK for downlink and uplink for E-UTRA CC, test CP-OFDM QPSK for downlink and DFT-s-OFDM QPSK for uplink for NR CC.

Proposal 6: For Out-of-band blocking for inter-band EN-DC within FR1, test the same downlink and uplink RB allocations as the corresponding Reference sensitivity level TC for each E-UTRA CC and NR CC.

Number of test points

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for inter-band EN-DC within FR1.

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |