3GPP TSG-RAN5 Meeting #88-e	R5-203595

# Electronic Meeting, 17th August 2020 – 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4943 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification and editorial changes to TC 22.4.26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-UEConTest, TEI13_Test |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Modification: In the original CR R5-202067, si-WindowLength was set to spare to check for proper error handling behavior when a spare field that the UE does not understand is encountered in SI. However, setting this to spare may result in undefined behavior. Therefore, we have decided to align the NB-IoT TC with the LTE TC in R5-202060, and test spare field handling by using spare bits in MIB-NB. Editorials: Test case refers to “Cell1”, under Pre-test conditions and in the test procedure, but for NB-IoT test cases it should be “Ncell1”. The procedure numbering in the recently added 22.4.26 is incorrect. The number of entries for sib-TypeAndInfo-r13 under Table 22.4.26.3.3-1 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Testing of spare field handling under SIB1-NB has been replaced with a test for spare field handling under MIB-NB. References to “Cell1” have been changed to “Ncell1”. Numbering has been corrected in Tables 22.4.26.3.2-1 and 22.4.26.3.3-2. Changed the value of sib-TypeAndInfo-r13 under Table 22.4.26.3.3-1 from “1 entry” to “2 entries”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect numbering and references will remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

22.4.26.3.1	Pre-test conditions

System Simulator:

-	Ncell 1

<Start of first modified section>

<Start of second modified section>

Table 22.4.26.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH. | <-- | RRC: SYSTEM INFORMATION (BCCH) | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3 | Does the UE transmit an RRCConnectionRequest-NB message on Ncell 1? | --> | RRCConnectionRequest-NB | 1 | P |
| 4 | The SS transmits an RRCConnectionSetup-NB message on Ncell 1. | <-- | RRCConnectionSetup-NB | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete-NB message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message? The PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST. | --> | RRCConnectionSetupComplete-NB NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 1 | P |
| 6-15 | Steps 5 to 14b1 of the registration procedure described in TS 36.508 subclause 8.1.5.2 are performed on Ncell 1. | - | - | - | - |
| NOTE: sib-TypeAndInfo-r13 in SystemInformation-NB and si-WindowLength-r13 in SystemInformationBlockType1-NB are set to values in Rel-15 to check spare and extended fields in UE up to Rel-14. |  |  |  |  |  |

<End of second modified section>

<Start of third modified section>

22.4.26.3.3			Specific message contents

Table 22.4.26.3.3-1: Message SystemInformation-NB (step 1, Table 22.4.26.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformation-NB::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r13 SEQUENCE { |  |  |  |
| sib-TypeAndInfo-r13 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { | 2 entries |  |  |
| sib2-r13[1] | SystemInformationBlockType2-NB-r13 | SIB2-NB |  |
| sib23-v1530[1] SEQUENCE { |  | In the ASN.1, sib23-v1530 is considered an extended field from Rel-13 to Rel-14. | If Rel-13, Rel-14 |
| ul-ConfigList-v1530 | Not present |  |  |
| ul-ConfigListMixed-v1530 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.26.3.3-2: Void

Table 22.4.26.3.3-3: Message MasterInformationBlock-NB (step 1, Table 22.4.26.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |  |
| systemFrameNumber-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |  |
| hyperSFN-LSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |  |
| schedulingInfoSIB1-r13 | 2 |  |  |  |
| systemInfoValueTag-r13 | 0 |  |  |  |
| ab-Enabled-r13 | FALSE |  |  |
| spare | ‘1111 1111 111’B | From Rel-13 to Rel-14, the lowest 11 bits are considered spares according to TS 36.331 [17]. | If Rel-13, Rel-14 |
| } |  |  |  |

<End of third modified section>