3GPP TSG-RAN5 Meeting #89-e	R5-205262

# Online, , 9th Nov 2020 - 20th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 0909 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RRM TC 5.5.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some general test parameters in Table 5.5.5.4.4.1-3 don`t align with the ones in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | General test parameters were corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect paramenters will be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.5.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

5.5.5.4	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- Connection diagram is TBD.

- RAN4 dependency: Test parameters have brackets and TBDs.

5.5.5.4.1	Test purpose

The purpose of this test is:

To verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1.

To test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used.

To partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

5.5.5.4.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

5.5.5.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.5.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.5.5.5.4.

5.5.5.4.4	Test description

There is one E-UTRAN PCell and one NR PSCell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 5.5.5.4.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate CSI-RS based beam failure.

Figure 5.5.5.4.4-1: SNR variation CSI-RS for EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX

5.5.5.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.5.5.4.4.1-1.

Table 5.5.5.4.4.1-1: Supported test configurations for EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 5.5.5.4-1 | TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |

Configure the test equipment and the DUT according to the parameters in Table 5.5.5.4.4.1-2.

Table 5.5.5.4.4.1-2: Initial conditions for EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 5.5.5.4.4.1-3. The measurement gap configuration is according to Table 5.5.5.4.4.1-4. The NZP-CSI-RS configuration is according to Table 5.5.5.4.4.1-3. The DRX configuration for is according to Table 5.5.5.4.4.1-3. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

2. Message contents are defined in clause 5.5.5.4.4.3.

3. There is one E-UTRAN cell and one NR cell specified in the test. E-UTRAN Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 5.5.5.4.4.1-3: General test parameters for EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | A.3.1.2 |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | A.3.10 |
| SMTC Configuration | Config 1 |  | SMTC.3 | A.3.11 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  | 0 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |
| OCNG parameters |  |  | OP.1 | A.3.2.1 |
| CP length |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  | *[gp0] |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | dBm | -94.5 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration | Config 1 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 5.43 |  |
| T3 |  | s | 5.16 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Editor’s note: An additional RS for RLM, different from BFD-RS at constant high SNR shall be configured as part of the test configuration.

Table 5.5.5.4.4.1-4: Measurement gap configuration for EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |

5.5.5.4.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.4

2.	Set the parameters of NR Cell 1 according to T1 in Table 5.5.5.4.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 5.5.5.4.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 5.5.5.4.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 5.5.5.4.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 5.5.5.4.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-2 [18] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power on NR carrier higher than OFF power defined in TS 38.521-2 [18] clause 6.3.2.5 from time point C until T3 expires

and

c) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-2 [18] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point F (D1 after the start of T5) until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

1.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 5.5.5.4.5-1.

2.	Wait [1s] for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within [1s] continue to step 11. Otherwise continue to step 10.

3.	Switch the UE on and off. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

4.	Repeat steps 2-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.5.5.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 5.5.5.4.4.3-1: Common Exception messages for EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

5.5.5.4.5	Test requirement

Tables 5.5.5.4.4.1-3 and 5.5.5.4.5-1 define the primary level settings including test tolerances for EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX.

Table 5.5.5.4.5-1: NR Cell specific test parameters for EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Test 1 |  |  |  |  | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CSI-RS of set q0 |  |  |  |  | CSI-RS of set q1 |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  | dB |  |  |  |  |  |  |  |  |  |  |
| SNR_CSI-RS | Config 1 | dB | [5] | [-3] | [-12] | [-12] | [-12] | [-12] | [-12] | [-12] | [-3] | [10] |
|  | Config 2 |  | [5] | [-3] | [-12] | [-12] | [-12] | [-12] | [-12] | [-12] | [-3] | [10] |
|  | Config 3 |  | [5] | [-3] | [-12] | [-12] | [-12] | [-12] | [-12] | [-12] | [-3] | [10] |
|  | Config 1 | dBm/15KHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 2 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
|  | Config 3 |  | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  |  | [TDLA30-75] |  |  |  |  | [TDLA30-75] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<End of modified section>