3GPP TSG-RAN WG5 #89-e		R5-205622
Electronic Meeting, 9 - 20 Nov 2020

Agenda item:	5.3.2.17

Source:	Rohde & Schwarz

Title:	On the influence of the XPD for Enhanced-IFF

Document for:	Endorsement

# Introduction

The value for the uncertainty source Influence of the XPD for enhanced IFF is TBD. This paper presents measurement results and proposes a value for the Influence of the XPD.

# Discussion

Enhanced IFF is one of the permitted test setups for RRM OTA testing in TS 38.508-1 [1]. The Influence of the XPD definition and measurement procedure is captured in Annex B.2.1.10 of TR 38.903 [2].

As AUT, a standard gain horn antenna fitting the masks required in TS 38.521-2 [3] O.2.1 has been selected, with an approximate gain of 12dBi at 23.45 GHz, 12.5 dBi at 32.125 GHz and 14 dBi at 40.8 GHz.

Observation 1: The AUT has been selected to fit the directivity and HPBW masks in TS 38.521-2 [3] O.2.1.

The enhanced IFF setup has a QZ of 30 cm. Therefore, the reference AUT positions (see the QoQZ measurement procedure in TS 38.521-2 [3] Annex O.2) have been selected with R = 15 cm.

Observation 2: The Influence of the XPD has been measured at P1-P7 with R = 15 cm, i.e. for a 30 cm QZ size.

The measurement results are summarized in Table 1.

Table 1: Summary of the Influence of the XPD Measurement results

| Frequency | 23.45 GHz |  | 32.125 GHz | 40.8 GHz |
| --- | --- | --- | --- | --- |
| Influence of the XPD [dB] |  | 0.0066 | 0.0089 | 0.0095 |

Based on the results in Table 1, it is proposed to use a MU value of 0.01 dB for the Influence of the XPD for enhanced IFF.

Proposal 1: define an uncertainty value of 0.01 dB for the Influence of the XPD for enhanced IFF.

# Conclusion

Proposal 1: define an uncertainty value of 0.01 dB for the Influence of the XPD for enhanced IFF.

# References

- TS 38.508-1, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”

- TR 38.903, “NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”

- TS 38.521-2, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”