3GPP TSG-RAN WG5 #89-e		R5-205623
Electronic Meeting, 9 - 20 Nov 2020

Agenda item:	5.3.2.17

Source:	Rohde & Schwarz

Title:	On the DL AWGN absolute power or wanted DL signal absolute power

Document for:	Endorsement

# Introduction

The value for the DL AWGN absolute power (or wanted DL signal absolute power) for RRM is still TBD.

For Enhanced-IFF, there were a few TBD values that have been proposed in other contributions [1], [2].

This contribution calculates the DL AWGN absolute power MU value for enhanced IFF.

# Discussion

The uncertainty assessment for DL AWGN absolute power is done in Table E.3.1.4-2 of TR 38.903 [3].

The DL AWGN absolute power expanded uncertainty is calculated as the root sum square of the standard deviations of the Stage 1 and Stage 2 contributions (expanded by 1.96 to reach the confidence level of 95%) plus the systematic uncertainties. This gives the following equation (contributors with value of zero have not been taken into account for readability):

DL AWGN MU=1.96σQoQZ2+σMism.2+σgNB2+σAmp.2+σrand.2+σXPD2+σm.ant2+σrepos.2+σNet.Analiz.2+σcal.ant.2+σQoQZcal2+σfeedcable2+Syst. error

Taking the values from E.3.1.4-2, this gives:

DL AWGN MU=1.960.72+1.32+1.452+1.052+0.252+0.012+0.152+0.052+0.372+0.32+0.42+0.072+0.5=5.25 dB

Note: values in yellow depend on [1] and [2].

Proposal 1: Use 5.25 dB as DL AWGN absolute power MU for enhanced IFF.

# Conclusion

Proposal 1: Use 5.25 dB as DL AWGN absolute power MU for enhanced IFF.

# References

- R5-205622, “On the influence of the XPD for Enhanced-IFF”, R&S

- R5-205624, “On the QoQZ for enhanced IFF“, R&S

- TR 38.903, “NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”