3GPP TSG-RAN5 Meeting #89-e	R5-205239

# Online, , 9th Nov 2020 - 20th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 0902 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RRM TC 4.5.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Timer value of T310 doesn`t align with the one in TS 38.133. 2. In radio link monitor test case, test equipment may check the CSI report from UE to identify whether radio link is failure or not. However, in the test case 4.5.1.8, the measure gap pattern is fully overlapped with on duration period of the DRX cycle. Thus, it may cause UE cannot transmit the CSI report to test equipment during duration ON. As a result, we propose a new DRX configuration to guarantee the CSI report can be received by test equipment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. T310 value was corrected. 2. Propose new DRX configuration DRX.12 used for TC 4.5.1.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect paramenters will be used and a UE will fail this case unfairly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.1.8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.133 CR 1188 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 4.5.1.8	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode

4.5.1.8.1	Test purpose

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PSCell when DRX is used. This test will partly verify the FR1 PSCell CSI-RS in-sync radio link monitoring requirements in TS 38.133 clause 8.1.

4.5.1.8.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward supporting EN-DC, CSI-RS based RLM and long DRX cycle.

4.5.1.8.3	Minimum conformance requirements

The minimum requirements are specified in clause 4.5.1.0.4. DRX configuration is used for this test.

The normative reference for this requirement is TS 38.133 [6] clause A.4.5.1.8.

4.5.1.8.4	Test description

There are two cells configured in this test, the E-UTRA PCell and NR PSCell. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 4.5.1.8.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate in-sync states.

Figure 4.5.1.8.4-1: SNR variation for In-sync testing

4.5.1.8.4.1	Initial conditions

Test 4.5.1.8 can be run in one of the configurations defined in Table 4.5.1.8.4.1-1.

Table 4.5.1.8.4.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 4.5.1.8-1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.5.1.8-2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.5.1.8-3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4.5.1.8-4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.5.1.8-5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.5.1.8-6 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configue the test equipment and the DUT according to the parameters in Table 4.5.1.8.4.1-2.

Table 4.5.1.8.4.1-2: Initial conditions for CSI-RS In-sync radio link monitoring in DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.5.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The test parameters are given in Table 4.5.1.8.4.1-3 below.

2. Message contents are defined in clause 4.5.1.8.4.3.

3. There are two cells in the test, where Cell 1 is the E-UTRAN PCell on the E-UTRA carrier, and Cell 2 is the NR PSCell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table A.6.1.1-1 for this test. Cell 2 is configured according to Annex C.1.1 and C.1.2.

Table 4.5.1.8.4.1-3: General test parameters for FR1 PSCell for CSI-RS In-sync testing in DRX mode

| Parameter |  |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |
| Active E-UTRA PCell |  |  |  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  |  |  |  | 1 |
| Active PSCell |  |  |  |  |  | Cell 2 |
| RF Channel Number |  |  |  |  |  | 2 |
| Duplex mode |  |  | Config 1, 4 |  |  | FDD |
|  |  |  | Config 2, 3, 5, 6 |  |  | TDD |
| TDD Configuration |  |  | Config 1, 4 |  |  | Not Applicable |
|  |  |  | Config 2, 5 |  |  | TDDConf.1.1 |
|  |  |  | Config 3, 6 |  |  | TDDConf.2.1 |
| DL initial BWP configuration |  |  | Config 1, 2, 3, 4, 5, 6 |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3, 4, 5, 6 |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3, 4, 5, 6 |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3, 4, 5, 6 |  |  | ULBWP.1.1 |
| RMC CORESET Reference Channel |  |  | Config 1, 4 |  |  | CCR.1.1 FDD |
|  |  |  | Config 2, 5 |  |  | CCR.1.1 TDD |
|  |  |  | Config 3, 6 |  |  | CCR.2.1 TDD |
| SSB Configuration |  |  | Config 1, 4 |  |  | SSB.1 FR1 |
|  |  |  | Config 2, 5 |  |  | SSB.1 FR1 |
|  |  |  | Config 3, 6 |  |  | SSB.2 FR1 |
| SMTC Configuration |  |  | Config 1, 2, 4, 5 |  |  | SMTC.1 |
|  |  |  | Config 3, 6 |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2, 4, 5 |  |  | 15 KHz |
|  |  |  | Config 3, 6 |  |  | 30 KHz |
| TRS configuration |  |  | Config 1, 4 |  |  | TRS.1.1 FDD |
|  |  |  | Config 2, 5 |  |  | TRS.1.1 TDD |
|  |  |  | Config 3, 6 |  |  | TRS.1.2 TDD |
| CSI-RS for RLM |  |  |  | Config 1, 4 |  | Resource #4 in TRS.1.1 FDD |
|  |  |  |  | Config 2, 5 |  | Resource #4 in TRS.1.1 TDD |
|  |  |  |  | Config 3, 6 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  |  |  |  | TCI.State.0 |
| OCNG parameters |  |  |  |  |  | OP.1 |
| CP length |  |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low |
| Out of sync transmission parameters |  | DCI format |  |  |  | 1-0 |
|  |  | Number of Control OFDM symbols |  |  |  | 2 |
|  |  | Aggregation level |  |  | CCE | 8 |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 4 |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 4 |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size |
|  |  | REG bundle size |  |  |  | 6 |
| In sync transmission parameters |  | DCI format |  |  |  | 1-0 |
|  |  | Number of Control OFDM symbols |  |  |  | 2 |
|  |  | Aggregation level |  |  | CCE | 4 |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size |
|  |  | REG bundle size |  |  |  | 6 |
| DRX |  |  |  |  |  | DRX.12 |
| Gap pattern ID |  |  |  |  |  | gp0 |
| Layer 3 filtering |  |  |  |  |  | Enabled |
| T310 timer |  |  |  |  | ms | 2000 |
| T311 timer |  |  |  |  | ms | 1000 |
| N310 |  |  |  |  |  | 1 |
| N311 |  |  |  |  |  | 1 |
| CSI for reporting | Config 1, 4 |  |  |  |  | CSI-RS.1.1 FDD |
|  | Config 2, 5 |  |  |  |  | CSI-RS.1.1 TDD |
|  | Config 3, 6 |  |  |  |  | CSI-RS.2.1 TDD |
| T1 |  |  |  |  | s | 0.2 |
| T2 |  |  |  |  | s | 0.2 |
| T3 |  |  |  |  | s | 1.24 |
| T4 |  |  |  |  | s | 0.2 |
| T5 |  |  |  |  | s | 1.88 |
| T6 |  |  |  |  | s | 1.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table 4.5.1.8.4.1-4: Measurement gap configuration for FR1 CSI-RS In-sync radio link monitoring in DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. |  |

4.5.1.8.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity defined in CSI-RS configuration. In the test, DRX configuration is enabled in PSCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms). In the test, SSB0 is configured as the BFD-RS.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG_and_SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters of Cell 2 according to T1 in Table 4.5.1.8.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3. When T1 expires the SS shall change the SNR value to T2 as specified in Table 4.5.1.8.5-1. T2 starts.

4. When T2 expires the SS shall change the SNR value to T3 as specified in Table 4.5.1.8.5-1. T3 starts.

5. When T3 expires the SS shall change the SNR value to T4 as specified in Table 4.5.1.8.5-1. T4 starts.

6. When T4 expires the SS shall change the SNR value to T5 as specified in Table 4.5.1.8.5-1. T5 starts.

7. If the SS detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in the On-duration part of every DRX cycle in the configured slots for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point F (T6 after the start of time duration T5) the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	If the UE has not re-established the connection in at least 1s, the SS shall ensure PSCell is released.

9.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

10.	If the Reconfiguration fails, switch off and on the UE and ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG_and_SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat steps 2-7 for both subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

4.5.1.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 4.5.1.8.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED; Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1 and RLM for configuration 4.5.1.8-1, 4.5.1.8-2, 4.5.1.8-4, and 4.5.1.8-5 Table H.3.1-3 with Condition INTER-FREQ MO, SSB.2 FR1 and RLM for configuration 4.5.1.8-3 and 4.5.1.8-6 Table H.3.1-4 with a3-offset = -4.5dB; Table H.3.1-8 with Condition CSI-RS RLM Table H.3.1-9 Table H.3.7-1 with condition DRX.12 |

Table 4.5.1.8.4.3-2: MeasConfig for E-UTRAN PCell

| Derivation Path: TS 36.508, Table 4.6.6-1 with condition RF |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| measGapConfig | MeasGapConfig-GP1 | TS 36.508, table Table 4.6.6-1A |  |
| } |  |  |  |

4.5.1.8.5	Test requirement

Tables 4.5.1.8.4.1-2 and 4.5.1.8.5-1 define the primary level settings including test tolerances for Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode.

Table 4.5.1.8.5-1: Cell specific test parameters for FR1 for CSI-RS In-sync radio link monitoring in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta |  | dB | 4 |  |  |  |  |
| PDCCH_DMRS_beta |  | dB | 4 |  |  |  |  |
| PBCH_beta |  | dB | 0 |  |  |  |  |
| PSS_beta |  | dB |  |  |  |  |  |
| SSS_beta |  | dB |  |  |  |  |  |
| PDSCH_beta |  | dB |  |  |  |  |  |
| OCNG_beta |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1, 4 | dB | 1.9 | -6.1 | -15.9 | -5.4 | 1.9 |
|  | Config 2, 5 |  | 1.9 | -6.1 | -15.9 | -5.4 | 1.9 |
|  | Config 3, 6 |  | 1.9 | -6.1 | -15.9 | -5.4 | 1.9 |
| SNR on other channels and signals | Config 1, 4 | dB | 1 |  |  |  |  |
|  | Config 2, 5 |  | 1 |  |  |  |  |
|  | Config 3, 6 |  | 1 |  |  |  |  |
|  | Config 1, 4 | dBm/15KHz | -98 |  |  |  |  |
|  | Config 2, 5 |  | -98 |  |  |  |  |
|  | Config 3, 6 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 4.5.1.8.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |  |  |

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the ON-duration part of the cycle in the slots configured for CQI transmission according to the configured CQI reporting mode on PUCCH.

The uplink signal is verified on the basis of the UE output power:

-	UE output power equal to or higher than Transmit minimum power (as defined in TS 38.521-1 [17] clause 6.3.1.5) means uplink signal

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

<End of modified section>