3GPP TSG-RAN5 Meeting #89-e	R5-206172

# Online, , 9th Nov 2020 - 20th Nov 2020

Title:							On XPD MU for DFF

Source:			Anritsu

Agenda Item:			5.3.2.17

Document for:			Endorsement

# 1.	Introduction

During the discussion on FR2 RRM MU in RAN5#88e, it was requested to determine XPD MU based on the actual measurement of the system. This paper provides the measured result for the XPD MU for DFF.

# 2.	Discussion

# XPD MU is measured using the procedure outlined in TR 38.903[1\.

The XPD of the measurement system shall be determined from the quality of quiet zone measurements, see clause O.2 of [7], at the 7 reference points, P1 through P7, specifically with reference AUT orientations ==0o for distributed axes systems, Section O.2.6.1 [7], or reference AUT orientations ==0o for combined-axes systems, Section O.2.6.2 [7]. Alternatively, it can be determined using a reference antenna optimized for XPD measurements and with the corresponding alignment to achieve optimal polarization matching between the reference and the measurement antenna.

The XPD for each reference point shall be calculated as the ratio of cross-polarized to co-polarized measured powers and the largest XPD from the 7 different reference points shall be used to determine the XPD MU, i.e.,

(1-17)

where

Table 1 shows the result of measured MUXPD using DFF.

Table 1 Measured MUXPD using DFF(QZ=30cm)

| 23.45 GHz | 32.125 GHz | 40.80 GHz |
| --- | --- | --- |
| 0.043 dB | 0.033 dB | 0.054 dB |

# 4.	Proposals

RAN5 is asked to endorse following proposals.

Proposal 1 : Apply XPD MU with 0.06 dB for DFF

# 5.	References

[x]